|  | Tube Number Systems |  |
| --- | --- | --- |

 European system after 1934  Philips system before 1934  Mazda numbering system (EdiSwan)  Russian numbering system  Brimar type designation code  Tesla numbering system RETMA West European Mullard

3.1 North American systems

3.1.1 RMA system (1942)

3.1.2 RETMA receiving tubes system (1953)

3.1.3 EIA professional tubes system

3.1.4 Eimac high power RF tubes system

3.2 West European systems

3.2.1 Mullard–Philips system

3.2.1.1 Standard tubes

3.2.1.2 Special quality tubes

3.2.1.3 Professional tubes

3.2.1.4 Transmitting tubes

3.2.1.5 Phototubes and photomultipliers

3.2.1.6 Voltage stabilizers

3.2.2 Compagnie des Lampes (1888, "Métal") system

3.2.3 EdiSwan ("British Mazda") systems

3.2.3.1 Signal tubes

3.2.3.2 Power rectifiers

3.2.4 EEV system

3.2.5 ETL computing tubes system

3.2.6 Marconi-Osram system

3.2.7 Mullard designations before 1934

3.2.8 Philips system before 1934

3.2.9 STC/Brimar receiving tubes system

3.2.10 Valvo system before 1934

3.3 East European systems

3.3.1 Lamina transmitter tube system

3.3.2 RFT transmitter tube system

3.3.3 Tesla systems

3.3.3.1 Signal tubes

3.3.3.2 Power tubes

3.3.4 Tungsram receiving tubes system before 1934

3.4 Russian systems

3.4.1 GOST standard tubes system

3.4.2 Professional tubes system

3.5 Japanese systems

3.5.1 Older numbering system 1941–51

3.5.2 JIS C 7001 system

3.6 Military naming systems

3.6.1 British CV naming system

3.6.2 U.S. naming systems

3.7 Other numeral-only systems

3.8 Other letter followed by numerals

4 List of American RETMA tubes, with European equivalents

4.1 "0 volt" gas-filled cold cathode tubes

4.2 1 volt heater/filament tubes

4.3 1.25 volt filament subminiature tubes

4.4 1 prefix for home receivers

4.5 2 volt heater/filament tubes

4.6 3 volt heater/filament high voltage rectifier tubes

4.7 Nominally 5 volt heater/filament tubes

4.8 5 volt heater/filament full wave rectifier tubes

4.9 6 volt heater/filament tubes

4.10 "7" prefix loctal tubes

4.11 12 volt heater/filament tubes

4.12 "14" prefix loctal tubes

4.13 25 volt heater/filament tubes

4.14 35 volt heater/filament tubes

4.15 50 volt heater/filament tubes

4.16 117 volt heater tubes

4.17 Other tubes with nonstandard filament voltages

5 List of EIA professional tubes

List of vacuum tubes

From Wikipedia, the free encyclopedia

Jump to navigation

Jump to search

This is a list of vacuum tubes or thermionic valves, and low-pressure gas-filled tubes, or discharge tubes. Before the advent of semiconductor devices, thousands of tube types were used in consumer electronics. Many industrial, military or otherwise professional tubes were also produced. Only a few types are still used today, mainly in high-power, high-frequency applications.

Contents

1 Heater or filament ratings

2 Tube bases and envelopes

2.1 Abbreviations used in this list

3 Numbering systems

3.1 North American systems

3.1.1 RMA system (1942)

3.1.2 RETMA receiving tubes system (1953)

3.1.3 EIA professional tubes system

3.1.4 Eimac high power RF tubes system

3.2 West European systems

3.2.1 Mullard–Philips system

3.2.1.1 Standard tubes

3.2.1.2 Special quality tubes

3.2.1.3 Professional tubes

3.2.1.4 Transmitting tubes

3.2.1.5 Phototubes and photomultipliers

3.2.1.6 Voltage stabilizers

3.2.2 Compagnie des Lampes (1888, "Métal") system

3.2.3 EdiSwan ("British Mazda") systems

3.2.3.1 Signal tubes

3.2.3.2 Power rectifiers

3.2.4 EEV system

3.2.5 ETL computing tubes system

3.2.6 Marconi-Osram system

3.2.7 Mullard designations before 1934

3.2.8 Philips system before 1934

3.2.9 STC/Brimar receiving tubes system

3.2.10 Valvo system before 1934

3.3 East European systems

3.3.1 Lamina transmitter tube system

3.3.2 RFT transmitter tube system

3.3.3 Tesla systems

3.3.3.1 Signal tubes

3.3.3.2 Power tubes

3.3.4 Tungsram receiving tubes system before 1934

3.4 Russian systems

3.4.1 GOST standard tubes system

3.4.2 Professional tubes system

3.5 Japanese systems

3.5.1 Older numbering system 1941–51

3.5.2 JIS C 7001 system

3.6 Military naming systems

3.6.1 British CV naming system

3.6.2 U.S. naming systems

3.7 Other numeral-only systems

3.8 Other letter followed by numerals

4 List of American RETMA tubes, with European equivalents

4.1 "0 volt" gas-filled cold cathode tubes

4.2 1 volt heater/filament tubes

4.3 1.25 volt filament subminiature tubes

4.4 1 prefix for home receivers

4.5 2 volt heater/filament tubes

4.6 3 volt heater/filament high voltage rectifier tubes

4.7 Nominally 5 volt heater/filament tubes

4.8 5 volt heater/filament full wave rectifier tubes

4.9 6 volt heater/filament tubes

4.10 "7" prefix loctal tubes

4.11 12 volt heater/filament tubes

4.12 "14" prefix loctal tubes

4.13 25 volt heater/filament tubes

4.14 35 volt heater/filament tubes

4.15 50 volt heater/filament tubes

4.16 117 volt heater tubes

4.17 Other tubes with nonstandard filament voltages

5 List of EIA professional tubes

5.1 4000s

5.2 5000s

5.3 6000s

5.4 7000s

5.5 8000s

6 List of European Mullard–Philips tubes, with American equivalents

7 List of Pro Electron professional tubes

7.1 X - Electro-optical devices

7.1.1 XA

7.1.2 XG

7.1.3 XL

7.1.4 XM

7.1.5 XP

7.1.6 XQ

7.1.7 XR

7.1.8 XX

7.2 Y - Vacuum tubes

7.2.1 YA

7.2.2 YD

7.2.3 YG

7.2.4 YH

7.2.5 YJ

7.2.6 YK

7.2.7 YL

7.3 Z - Gas-filled tubes

7.3.1 ZA

7.3.2 ZC

7.3.3 ZM

7.3.4 ZP

7.3.5 ZT

7.3.6 ZX

7.3.7 ZY

7.3.8 ZZ

8 List of European transmitting tubes

8.1 B - Backward-wave amplifier

8.1.1 BA

8.2 D - Rectifier incl. grid-controlled

8.2.1 DA

8.2.2 DC

8.2.3 DCG

8.2.4 DCX

8.2.5 DE

8.3 J - Magnetron

8.3.1 JP

8.3.2 JPT

8.4 K - Klystron

8.4.1 KB

8.4.2 KS

8.5 L - Traveling-wave tube

8.5.1 LA

8.6 M - AF modulator Triode

8.6.1 MA

8.6.2 MB

8.6.3 MY

8.6.4 MZ

8.7 P - Pentode

8.7.1 PA

8.7.2 PAL

8.7.3 PAW

8.7.4 PB

8.7.5 PC

8.7.6 PE

8.8 Q - Tetrode

8.8.1 QB

8.8.2 QBL

8.8.3 QBW

8.8.4 QC

8.8.5 QE

8.8.6 QEL

8.8.7 QEP

8.8.8 QQC

8.8.9 QQE

8.8.10 QQV

8.8.11 QQZ

8.8.12 QV

8.8.13 QY

8.8.14 QYS

8.8.15 QZ

8.9 R - Rectifier

8.9.1 RG

8.9.2 RGQ

8.10 T - AF/RF/oscillator Triode

8.10.1 TA

8.10.2 TB

8.10.3 TBL

8.10.4 TBW

8.10.5 TC

8.10.6 TD

8.10.7 TE

8.10.8 TX

8.10.9 TY

8.10.10 TYS

8.11 X - Thyratron

8.11.1 XGQ

8.11.2 XR

9 List of other letter tubes

9.1 A

9.1.1 AC*/

9.1.2 ACT

9.1.3 AP

9.1.4 AT

9.2 B

9.2.1 BA

9.2.2 BG

9.2.3 BT

9.3 C

9.3.1 CH

9.3.2 CK

9.3.3 CL

9.4 D

9.4.1 DDR

9.4.2 DZ

9.5 E

9.5.1 EN

9.6 G

9.6.1 GC

9.6.2 GCA

9.6.3 GD

9.6.4 GDT

9.6.5 GE

9.6.6 GK

9.6.7 GN

9.6.8 GR

9.6.9 GRD

9.6.10 GS

9.6.11 GSA

9.6.12 GTE

9.6.13 GTR

9.7 K

9.7.1 KN

9.7.2 KT

9.8 M

9.8.1 M8000s

9.8.2 MC

9.8.3 ME

9.9 P

9.9.1 PBG

9.9.2 PD

9.9.3 PL

9.10 Q

9.10.1 QK

9.11 R

9.11.1 RK

9.12 S

9.12.1 SB

9.12.2 SU

9.13 T

9.13.1 TH

9.13.2 TM

9.13.3 TT

9.14 V

9.14.1 VHT

9.14.2 VS

10 List of other number tubes

10.1 1

10.1.1 1600s

10.2 2

10.2.1 200s

10.3 3

10.3.1 300s

10.4 4

10.4.1 400s

10.5 5

10.5.1 500s

10.6 6

10.7 7

10.7.1 700s

10.8 8

10.8.1 800s

10.9 9

10.9.1 900s

10.9.2 9000s

11 List of tubes used in 1920s and 1930s radio receivers

11.1 Tubes with directly heated cathodes

11.1.1 With 1.1 Volt DC filaments

11.1.2 With 2.0 Volts DC filaments

11.1.3 With 3.3 Volts DC filaments

11.1.4 With 5.0 Volts DC filaments

11.1.5 Other directly DC-heated tubes

11.1.6 Directly AC-heated power tubes

11.1.7 Directly AC-heated rectifier tubes

11.2 Tubes with indirectly heated cathodes

11.2.1 With DC heaters

11.2.2 For use with an AC heating transformer

11.2.3 For use with AC/DC or vehicle-based storage-battery power supplies

11.3 Shielded tubes for Majestic radios

11.4 Rarely used tubes

12 Lettered loctal tubes used in Philco radios

13 Russian tubes

13.1 List of standard tubes, with American equivalents

13.2 List of professional tubes

13.3 List of indicator tubes

13.4 List of other tubes

14 Compagnie des Lampes (1921, "French Mazda") and Mazda-Belvu

15 References and footnotes

15.1 Specific items

15.2 General literature and data sheets

16 See also

17 External links

Heater or filament ratings

Main article: Hot cathode

Receiving tubes have heaters or filaments intended for direct battery operation, parallel operation off a dedicated winding on a supply transformer, or series string operation on transformer-less sets. High-power RF power tubes are directly heated; the heater voltage must be much smaller than the signal voltage on the grid and is therefore in the 5...25 V range, drawing up to hundreds of amperes from a suitable heater transformer. In some valve part number series, the voltage class of the heater is given in the part number, and a similar valve might be available with several different heater voltage ratings.

Tube bases and envelopes

Main article: Tube socket

Abbreviations used in this list

ST – Shouldered tube

GT – Glass tube

MT – Miniature tube, such as Noval B9A or Miniature 7-pin B7G

FL – Subminiature all-glass elliptical body and flat bases with long, inline "flying leads" (pigtails) that are soldered into the circuit

SL – Subminiature all-glass elliptical body and flat bases with short inline leads that can be soldered or can be mated with a special socket. (Flying leads can be cut short to fit into inline sockets.)

R8 – Subminiature all-glass round body and base with 8 flying leads or stiff pins arranged in a circle

Numbering systems

North American systems

RMA system (1942)

Main article: RMA tube designation

The system assigned numbers with the base form "1A21", and is therefore also referred to as the "1A21 system".[1] The first numeric character indicated the filament/heater power rating, the second alphabetic character was a code for the function, and the last 2 digits were sequentially assigned, beginning with 21

RMA tubes

1B23 – 20 kW, 400 to 1500 MHz Gas-filled, cold-cathode Transmit/Receive Tube (TR cell)

1B41 – Gas-filled, cold-cathode 9.5 kV, 450 A spark gap

1B45 – Gas-filled, cold-cathode 14 kV, 450 A spark gap

1B49 – Gas-filled, cold-cathode 12 kV, 450 A spark gap

1C21 – Gas-filled, 25 mAavg, 100 mApeak, triode thyratron

1D21 – Strobotron Gas-filled, 50 mAavg, 5 Apeak, luminiscent tetrode thyratron for use as a stroboscope lamp

1P21 – 9-stage Photomultiplier, spectral S4 response, 11-pin base

1P25 – Infrared image converter used in World War II night vision "sniperscopes".

1P29 – Gas-filled phototube, spectral S3 response, 4-pin base

1P39 – Vacuum Phototube, spectral S4 response, 4-pin base

1S22 – 10 kV, 20 A Vacuum SPDT switch

2C21 – Dual transmitting triode, indirectly heated, 7-pin base plus a single top cap for one of the grids

2C22 – Transmitting triode, indirectly heated, 8-pin base plus dual top cap for grid and anode

2C36 – Rocket-type disk-seal UHF triode with an internal feedback circuit between cathode and anode, for use as UHF oscillator up to 1.75 GHz

2C37 – Rocket triode for use as SHF oscillator up to 3.3 GHz

2C39A – Oil can-type disk-seal UHF power triode with glass spacers up to 3 GHz, Panode = 100 W

2C39B – 2C39A with ceramic spacers

2C40 – Lighthouse-type disk-seal UHF power triode for continuous operation, Panode = 6.5 W at 3370 MHz

2C41 – Oil can UHF power triode for pulsed operation, 2200 Wpeak at 3 GHz

2C42 – Lighthouse UHF power triode for pulsed operation, 1750 Wpeak at 1050 MHz; improved 446

2C43 – Lighthouse UHF power triode, indirectly heated, up to 3.37 GHz, 6-pin base

2C46 – Lighthouse UHF power triode

2C51 – Dual shielded triode, indirectly heated, 9-pin base

2D21/EN91 (PL21, PL2D21, CV797) – 100 mAavg, 500 mApeak, 10 Asurge, Gas-filled, indirectly heated tetrode thyratron, negative starter voltage, miniature 7-pin base, for relay and grid-controlled rectifier service, used in jukeboxes and computer equipment.

2E22 – 53 W Power pentode, 5-pin base with anode on top cap

2E26 – Popular amateur 5.3 W VHF beam power tetrode up to 175 MHz, octal base

2E30 – 10 W Directly heated beam power tetrode with deflection screens available on separate pin, miniature 7-pin base

2E31 – Subminiature, directly heated, fully shielded sharp-cutoff RF/IF pentode, 5-pin all-glass pigtailed, FL

2E32 – Similar to 2E31, SL

2E35 – 6 mW Subminiature directly heated power pentode, 5-pin all-glass pigtailed, FL

2E36 – Similar to 2E35, SL

2E41 – Diode, pentode, FL

2E42 – Similar to 2E42, SL

2F21 – Indirectly heated hexode monoscope, Indian Head test pattern, 6-pin base with dual top caps for grid4 and anode

2G21 – Directly heated triode-heptode mixer, 7-pin all-glass pigtailed

2G41 – Triode-heptode converter, FL

2G42 – Similar to type 2G42, SL

2H21 – Phasitron, a magnetically controlled beam-deflection phase modulator tube[2] similar to the 5593, used in early FM broadcast transmitters[3][4][5]

2J30 to 2J34 – 300 kW S-band Magnetrons

2J55 and 2J56 – 40 kW X-band Magnetrons for use as pulsed oscillator

2K25 – 25 mW 8.5 to 9.66 GHz reflex Klystron

2K50 – 15 mW 23.5 to 24.5 GHz reflex Klystron

2P23 – Early image orthicon TV camera tube.

3B28 – Xenon half wave rectifier; ruggedized replacement for mercury vapor type 866.

3C22 – Disk-seal UHF power triode, Panode = 125 W with forced-air cooling, 1.4 GHz

3C23 – 1.5 Aavg, 6 Apeak, Mercury-vapor triode thyratron, 4-pin base with anode top cap

3C45 – 45 mAavg, 1.5 ARMS, 35 Apeak, Half-indirectly heated hydrogen triode thyratron, 4-pin base with anode top cap

3D21 – Indirectly heated beam power tetrode, 8-pin base with anode top cap

3D22 – Gas-filled, 800 mAavg, 8 Apeak, tetrode thyratron, 7-pin base

3E29 – Dual beam power tube used in radar equipment; a pulse rated variant of the earlier 829B, 7-pin base with dual anode top cap.

4B32 – 10 kV, 1.25 Aavg, 5 Apeak Xenon half wave rectifier

4D21 (6155, Eimac 4-125A) – 125 W Glass VHF beam power tetrode

4E27 – 125 W Glass radial-beam power pentode

4J31 to 4J35 – 1 MW S-band Magnetrons

5B24 – Tungar bulb, a low-voltage, mercury-vapor, full wave rectifier for charging 60-cell lead-acid batteries at 6 A; 2.5 V, 24 A heater[6]

5C22 – Half-indirectly heated, hydrogen triode thyratron for radar modulators.

5D22 (6156, Eimac 4-250A) – 250 W, 110 MHz Glass beam power tetrode

5J26 – 500 kW, 1.22 to 1.35 GHz S-band Magnetrons

5K70 – 30 kW S-band reflex Klystron

6C21 – Triode radar modulator for "hard tube" pulsers.

7C23 – 120 kW Power triode for high voltage pulse operation.

8D21 – Internally water cooled dual tetrode used in early VHF TV transmitters.

9C21 – 100 kW Water-cooled power triode, directly heated, 4-pin base with dual top caps for grid and anode

RETMA receiving tubes system (1953)

Main article: RETMA tube designation

RETMA is the acronym for the Radio Electronic Television Manufacturers Association formed in 1953.

The first character group is a number representing the heater voltage rounded to the nearest whole number; 0 indicates a cold-cathode tube.[1]

One or two letters assigned to the devices in order of development.

A single numeral that represents the number of active elements in the tube.

Suffix letters distinguish revisions or variants:

A, B, C – Improved backward compatible versions

E – Export version

G – Glass bulb, ST-12 to ST-16 size

GT – Glass bulb, T-9 size

GT/G – Glass bulb, T-9 size interchangeable with G and GT types

L – Loktal

LM – Loktal-metal

LT – Locking base

M – Metal envelope

MG – Metal-glass

ML – Metal-Loktal

S – Spray shielded

W – Ruggedised, or military grade

WA, WB – Improved, backward compatible military/industrial variants

X – Low loss ceramic base for RF use

Y – Low loss mica-filled phenolic resin base for RF use

Lastly, manufacturers may decide to combine two type numbers into a single name, which their one device can replace, such as: 6DX8/ECL84 (6DX8 and ECL84 being identical devices under different naming schemes) or 6BC5/6CE5 (sufficiently identical devices within the RETMA naming system) and even 3A3/3B2, or 6AC5-GT/6AC5-G (where the single type number, 6AC5-GT/6AC5-G, supersedes both the 6AC5-G and the 6AC5-GT).

Often designations that differed only in their initial numerals would be identical except for heater characteristics.

For examples see below

EIA professional tubes system

A four-digit system was maintained by the EIA for special industrial, military and professional vacuum and gas-filled tubes, and all sorts of other devices requiring to be sealed off against the external atmosphere.

Some manufacturers preceded the EIA number with a manufacturer's code:

CK, RK – Raytheon Company

ECG – Philips/Sylvania

F – Federal Telephone and Radio (ITT division)

GL – General Electric Corp. (not British General Electric Company)

ML – Machlett Laboratories, Inc.

NL – National Electronics, Inc. (Geneva, Illinois, USA)

NU – National Union Electric Corp. (Orange, New Jersey, USA)

PL – Philips N.V.

SV – Svetlana:

formerly only PJSC "Svetlana/ПАО Светлана", St. Petersburg, Russia

now also a brand of New Sensor Corp., Long Island City, New York, USA, manufacturing in Saratov, Russia

SY – Standard Telephones and Cables/Brimar (not Sylvania Inc.)

WL – Westinghouse Electric Corp.

For examples see below.

Eimac high power RF tubes system

Eitel/McCullough and other manufacturers of high power RF tubes use the following code:[7]

An initial digit denoting the number of electrodes:

2 – Diode

3 – Triode

4 – Tetrode

5 – Pentode

Up to 2 letters denoting the construction type and the cooling method:

R or a hyphen ("-") – Glass envelope, radiation cooling

C – Ceramic envelope

P – Primarily for pulse applications

L – External anode, liquid convection cooling

N – External anode, natural convection air cooling

S – External anode, conduction cooling

V – Vapor cooled (anode is immersed in boiling water, and the steam is collected, condensed and recycled)

W – Water cooled (water is pumped through an outer metal jacket thermically connected to the anode)

X – Forced-air cooled (air is blown through cooling fins thermally connected to the anode)

A number to indicate the maximum anode dissipation in watts. This can be exceeded for a short time, as long as the average is not exceeded over the anode's thermal time constant (typically 0.1 sec). In Class-C applications, the amplifier output power delivered to the load may be higher than the device dissipation

One or more manufacturer-proprietary letters denoting the construction variant

An optional digit denoting the gain group:

1 – ≤10

2 – 11...20

3 – 21...30

4 – 31...50

5 – 51...100

6 – 101...200

7 – 201...500

8 – 501...1000

Optionally a slash "/" followed by the EIA equivalent.

Examples:

3CW5000A3 – 5 kW Ceramic triode, water cooled, variant 'A', gain group 3

3CX100A5 – 100 W Ceramic UHF triode, forced-air cooled, variant 'A', gain group 5; often used by radio amateurs for 23cm-band microwave amplifiers.

3CX1500A7 (8877) – 1.5 kW Ceramic triode, forced air cooled, variant 'A', gain group 7

3CX2500A3 – 2.5 kW Ceramic triode, forced air cooled, variant 'A', gain group 3

4-65A (8165) – 65 W Glass beam tetrode

4-125A (4D21, 6155) – 125 W Glass beam tetrode

4-250A (5D22, 6156) – 110 MHz, 250 W Glass beam tetrode

4-400A – 400 W Glass beam tetrode

4-1000A (8166) – 1 kW Glass beam tetrode popular in broadcast and amateur transmitters.

4CX250B – 250 W Ceramic tetrode, forced-air cooled, version 'B', favored by radio amateurs as a final amplifier.

4CX250DC – 250 W Ceramic tetrode, forced-air cooled, version 'DC'

4CX35000 – Ceramic tetrode used in numerous 50-kW broadcast transmitters, forced-air cooled, often in a Doherty configuration as in the Continental Electronics 317C series.

5-125B/4E27A – 75 MHz, 125 W Glass power pentode

5-500A – 500 W Glass radial-beam pentode

5CX1500A – 110 MHz, 1.5 kW Ceramic radial-beam pentode, forced air cooled

5CX3000A – 150 MHz, 4.0 kW Ceramic radial-beam pentode, forced air cooled

Media related to Eimac vacuum tubes at Wikimedia Commons

West European systems

Mullard–Philips system

Main article: Mullard–Philips tube designation

This system is very descriptive of what type of device (triode, diode, pentode etc.) it is applied to, as well as the heater/filament type and the base type (octal, noval, etc.).[1][8] Adhering manufacturers include AEG (de), Amperex (us), CdL (1921, French Mazda brand), CIFTE (fr, Mazda-Belvu brand), EdiSwan (uk, British Mazda brand), Radiotechnique (fr, Coprim, Miniwatt-Dario and RTC brands), Lorenz (de), MBLE(fr, nl) (be, Adzam brand), Mullard (uk), Philips (nl, Miniwatt brand), RCA (us), RFT(de, sv) (de), Siemens (de), Telefunken (de), Tesla (cz), Toshiba (ja), Tungsram (hu), Unitra (pl, Dolam, Polam and Telam brands) and Valvo(de, it) (de).

Standard tubes

This part dates back to the joint valve code key (German: Röhren-Gemeinschaftsschlüssel) negotiated between Philips and Telefunken in 1933–34. Like the North American system the first symbol describes the heater voltage, in this case, a Roman letter rather than a number. Further Roman letters, up to three, describe the device followed by one to four numerals assigned in a semi-chronological order of type development within number ranges assigned to different base types.

If two devices share the same type designation other than the first letter (e.g. ECL82, PCL82, UCL82) they will usually be identical except for heater specifications; however there are exceptions, particularly with output types (for example, both the PL84 and UL84 differ significantly from the EL84 in certain major characteristics, although they have the same pinout and similar power rating). However, device numbers do not reveal any similarity between different type families; e.g. the triode section of an ECL82 is not related to either triode of an ECC82, whereas the triode section of an ECL86 does happen to be similar to those of an ECC83.

Pro Electron maintained a subset of the M-P system after their establishment in 1966, with only the first letters E, P for the heater, only the second letters A, B, C, D, E, F, H, K, L, M, Y, Z for the type, and issuing only three-digit numbers starting with 1, 2, 3, 5, 8, 9 for the base.[9]

Notes: Tungsram preceded the M-P designation with the letter T, as in TAD1 for AD1; VATEA Rádiótechnikai és Villamossági Rt.-t. (VATEA Radio Technology and Electric Co. Ltd., Budapest, Hungary) preceded the M-P designation with the letter V, as in VEL5 for EL5.

First letter: heater/filament type

Heater ratings for series-string, AC/DC tubes are given in milliamperes; heater ratings for parallel-string tubes are given in volts

A – 4 V heater for 2-cell lead-acid batteries and for AC mains transformers

B – 180 mA DC series heater

C – 200 mA AC/DC series heater

D – 1.4 V DC filament for Leclanché cells, later low-voltage/low power filament/heater:

0.625 V DC directly heated for NiCd battery, series-heated two-tube designs such as hearing aids. If either filament breaks, further draining of all batteries stops[10]

Wide range 0.9 V to 1.55 V DC directly heated for dry cells

1.25 V DC directly heated for NiCd batteries

1.25 V or 1.4 V AC from a separate heater winding on CRT horizontal-output transformers, in half-indirectly heated EHT rectifiers

E – 6.3 V parallel heater; for 3-cell lead-acid vehicle crank batteries (mobile equipment) and for AC mains or horizontal-output transformers

F – 12.6 V DC parallel heater for 6-cell lead-acid vehicle crank batteries

G – 5.0 V AC from a separate heater winding on a mains or horizontal-output transformer for the anode voltage rectifier; later misc.

H – 150 mA AC/DC series heater

In 1938, Philips tried to define this letter as "4 V battery", as opposed to A for "4 V AC"[11]:2

I – 20 V heater

K – 2.0 V filament for 1-cell lead-acid batteries, later for AC transformers

L – 450 mA AC/DC series heater; was shifted here from Y

M – 1.9 V, directly heated

N – 12.6 V, indirectly heated

O – Cold cathode

by 1955 this also included semiconductors as these had no heater

Philips sold a family of 150mA series heater tubes under this letter in South America

P – 300 mA AC/DC series heater

Q – 2.4 V, indirectly heated

R – Not assigned to avoid any confusion with the older Telefunken "R" system

S – 1.9 V, indirectly heated

T – Custom heater

U – 100 mA AC/DC series heater

V – 50 mA AC/DC series heater

X – 600 mA AC/DC series heater

Y – 450 mA AC/DC series heater, shifted to L to avoid conflicts with the professional tubes system

Z – Cold cathode tube; was shifted here from O after the advent of semiconductors

Second and subsequent letters: system type

A – Small signal diode

B – Dual small signal diode

C – Small signal triode

D – Power output triode

E – Small signal tetrode

F – Small signal pentode

H – Mixer hexode, special purpose heptode

K – Mixer heptode or octode

L – Power output, beam tetrode or pentode

M – Optical tuning/level indicator

N – Gas-filled thyratron

P – Secondary emission tube – mostly used as third letter

Q – Nonode

R – Resistive element (ballast tube, barretter, photoresistor)

S – Special tube (German: Sonderröhre)

T – Beam deflection tube, or misc.

W – Gas-filled half-wave rectifier

X – Gas-filled full-wave rectifier

Y – Vacuum half-wave rectifier (power diode)

Z – Vacuum full-wave rectifier (dual power diode with common cathode)

E.g. ECCnn is a 6.3 V dual triode; EABCnn has a single detector diode, a common-cathode pair of diodes, and a triode.

Following digits: model number and base type

For signal pentodes, an odd model number most often identified a variable-mu (remote-cutoff) tube, whereas an even number identified a 'high slope' (sharp-cutoff) tube

For power pentodes and triode-pentode combinations, even numbers usually indicate linear (audio power amplifier) devices while odd numbers were more suited to video signals or situations where more distortion could be tolerated.

1–9 – Pinch-type construction tubes, mostly P8 bases (P base, 8-pin side-contact) or European 5-pin (B base) and various other European pre-octal designs

10–19 – 8-pin German metal octal, G8A

20–29 – Loctal B8G; some octal; some 8-way side contact (exceptions are DAC21, DBC21, DCH21, DF21, DF22, DL21, DLL21, DM21 which have octal bases)

30–39 – International Octal (IEC 67-I-5a), also known as IO or K8A

40–49 – Rimlok (Rimlock) B8A All-glass miniature tubes

41w – Battery-heated bowl tube[12] (German: Pressnapfröhre)

50–59 – "Special construction types fitted with bases applicable to design features used";[13] mostly locking bases: "9-pin Loctal" (B9G) or 8-pin Loctal (B8G); but also used for Octal and others (3-pin glass; Disk-seal incl. Lighthouse tubes; German 10-pin with spigot; min. 4-pin; B26A; Magnoval B9D)

60–69 – Pencil tubes – sub-miniature all-glass tubes, pigtailed (inline fly-leads in place of pins)

—Before the 1950s:

60–64 – All-glass tubes fitted with 9-pin (B9G) bases

70–79 – Pencil tubes with circular pins or fly-leads

—Before the 1950s:

70–79 – Loctal Lorenz

80–89 – Noval B9A (9-pin; IEC 67-I-12a)

90–99 – "Button" B7G (miniature 7-pin; IEC 67-I-10a)

100–109 – B7G; Wehrmacht base; German PTT base

110–119 – 8-pin German octal; Rimlok B8A

130–139 – Octal

150–159 – German 10-pin with spigot; 10-pin glass with one big pin; Octal

160–169 – Inline wire-ended Pencil tubes; 8-pin German octal

170–179 – RFT 8-pin; RFT 11-pin all-glass gnome tube with one offset pin

180–189 – Noval B9A

190–199 – Miniature 7-pin B7G

200–209 – Decal B10B; Pro Electron-issued

230–239 – Octal

270–279 – RFT 11-pin all glass with one offset pin

280–289 – Noval B9A

300–399 – Octal; Pro Electron-issued

400–499 – Rimlok B8A

500–529 – Magnoval B9D, Novar; Pro Electron-issued

600–699 – Inline wire-ended Pencil tubes

700–799 – Circular wire-ended Pencil tubes

800–899 – Noval B9A; Pro Electron-issued

900–999 – Miniature 7-pin B7G; Pro Electron-issued

—Special quality:

1000– Round wire-ended; special Nuvistor base

2000– Decal B10B

3000– Octal

5000– Magnoval B9D

8000– Noval B9A

For examples see below

Special quality tubes

Vacuum tubes which had special qualities (French: Securité - Qualité) of some sort, very often long-life designs, particularly for computer and telecommunications use, had the numeric part of the designation placed immediately after the first letter. They were usually special-quality versions of standard types. Thus the E82CC was a long-life version of the ECC82 intended for computer and general signal use, and the E88CC a high quality version of the ECC88/6DJ8. While the E80F pentode was a high quality development of the EF80, they were not pin-compatible and could not be interchanged without rewiring the socket (the E80F is commonly sought after as a high quality replacement for the similar EF86 type in guitar amplifiers). The letters "CC" indicated the two triodes and the "F", the single pentode inside these types.

A few special-quality tubes did not have a standard equivalent, e.g. the E55L, a broadband power pentode used as the output stage of oscilloscope amplifiers and the E90CC, a double triode with a common cathode connection and seven pin base for use in cathode-coupled Flip-flops in early computers. The E91H is a special heptode with a passivated third grid designed to reduce secondary emission; this device was used as a "gate", allowing or blocking pulses applied to the first, (control) grid by changing the voltage on the third grid, in early computer circuits (similar in function to the U.S. 6AS6).

Many of these types had gold-plated base pins and special heater configurations inside the nickel cathode tube designed to reduce hum pickup from the A.C. heater supply, and also had improved oxide insulation between the heater and cathode so the cathode could be elevated to a greater voltage above the heater supply. (Note that elevating the cathode voltage above the average heater voltage, which in well-designed equipment was supplied from a transformer with an earthed center-tapped secondary, was less detrimental to the oxide insulation between heater and cathode than lowering the cathode voltage below the heater voltage, helping to prevent pyrometallurgical electrolytic chemical reactions where the oxide touched the nickel cathode that could form conductive aluminium tungstate and which could ultimately develop into a heater-cathode short-circuit.)

Better, often dual, getters were implemented to maintain a better vacuum, and more-rigid electrode supports introduced to reduce microphonics and improve vibration and shock resistance. The mica spacers used in "SQ" and "PQ" types did not possess sharp protrusions which could flake off and become loose inside the bulb, possibly lodging between the grids and thus changing the characteristics of the device. Some types, particularly the E80F, E88CC and E90CC, had a constricted section of bulb to firmly hold specially shaped flakeless mica spacers.[14]

For examples see below, starting at DC

Later special-quality tubes had not base and function swapped but were assigned a 4-digit number,[8] such as ECC2000 or ED8000, the first digit of which again denoting the base:

1 – Miscellaneous

2 – Miniature 10-pin base (JEDEC F10-61)

3 – Octal base (IEC 67-1-5a)

5 – Novar/magnoval base (JEDEC E9-75 and E9-23)

8 – Noval base (IEC 67-1-12a)

9 – Miniature 7-pin base (IEC 67-1-10a)

For examples see below, starting at EC

"Z" Cold-cathode SQ tubes had a different function letter scheme:[15]

A – Long-life amplifier tube

B – Binary counter or switching tube

C – Common-cathode Counter Dekatron that makes only carry/borrow cathodes separately available for cascading

E – Electrometer tube

G – Amplifier tube

M – Optical indicator

S – Separate-cathode Counter/Selector Dekatron that makes all cathodes available on individual pins for displaying, divide-by-n counter/timer/prescalers, etc.

T – Relay triode, a low-power triode thyratron, one starter electrode, may need illumination for proper operation if not radioactively primed

U – Low-power tetrode thyratron, may mean:

Trigger tetrode, one starter electrode and a primer (keep-alive) electrode for ion availability to keep the ignition voltage constant, for analog RC timers, voltage triggers, etc.

Relay tetrode, two starter electrodes to make counters bidirectional or resettable

W – Trigger pentode, two starter electrodes and a primer electrode

X – Shielded Trigger pentode, two starter electrodes, a primer electrode and a conductive coating of the glass envelope inside connected to a separate pin

For examples, see below under Z

Professional tubes

In use since at least 1961, this system was maintained by Pro Electron after their establishment in 1966.[9]

Both letters together indicate the type:

X – High vacuum electro-optical devices

XA – Phototube

XG – Miscellaneous

XM – Character generating cathode ray tube

XP – Photomultiplier

XQ – Camera tube

XR – Monoscope

XS – Cathode ray charge storage tube

XT – Memory display tube

XV – Infrared detector

XW – Infrared imaging device

XX – Image intensifier or image converter

Y – Vacuum tubes

YA – Diode

YD – Transmitting or industrial, single or dual triode

YG – Electrometer tube, vacuum gauge

YH – Traveling-wave tube

YJ – Magnetron

YK – Klystron

YL – Transmitting or industrial, single or dual tetrode or pentode

YN – Backward-wave oscillator

YP – Electron multiplier

YR – Crossed-field amplifier

YT – Pulse modulator tube

YY – High vacuum rectifier

Z – Gas-filled tubes not employing photosensitive materials

ZA – Cold cathode indicator tube

ZB – Microwave switching tube (TR/ATR cells, etc.)

ZC – Trigger tube

ZD – Surge arrester

ZE – Glow modulator tube, a linear light source for rotating-drum FAX receivers, film soundtrack recording, etc.

ZF – Flash tube

ZL – Gas laser

ZM – Cold cathode character display tube or counter display tube

ZP – Radiation counter tube (Geiger-Müller counter tube or proportional counter tube)

ZQ – Mixed analogue and digital display

ZR – Plasma display panel

ZS – Bar graph

ZT – Thyratron

ZX – Ignitron

ZY – Mercury-vapor rectifier

ZZ – Voltage stabilizer or corona discharge tube

Then follows a 4-digit sequentially assigned number.

Optional suffixes for camera tubes:

Version letter:

B – Blue

G – Green

L – Luminance

R – Red

T – Reticule

X – Medical X-ray

Letter for variants derived by selection:

D – High resolution

M – Blemish standard

For examples see below

Transmitting tubes

The first letter (or letter pair, in the case of a dual-system device) indicates the general type:

B – Backward-wave amplifier

D – Rectifier, including grid-controlled types

J – Magnetron

K – Klystron

L – Traveling-wave tube

M – Triode (AF amplifier or modulator)

P – Pentode

Q – Tetrode

R – Rectifier

T – Triode (AF, RF, oscillator)

X – Large thyratron (including all hydrogen thyratrons and high-current types)

The following letter indicates the filament or cathode type, or the fill gas or other construction detail. The coding for vacuum devices differs between Philips (and other Continental European manufacturers) on the one hand and its Mullard subsidiary on the other.

Philips vacuum devices:

A

Backward-wave amplifier or Traveling-wave tube: Output power <1W

Other tubes: Directly heated tungsten filament

B

Backward-wave amplifier or Traveling-wave tube: Output power ≥1W

Other tubes: Directly heated thoriated tungsten filament

C – Directly heated oxide-coated filament

D – Disk-seal construction

E – Indirectly heated oxide-coated cathode

Mullard vacuum devices:

G – Directly heated oxide-coated filament (only mercury-vapor rectifiers)

N – External magnet required (magnetrons)

P – Packaged construction (magnetrons)

S – Reflex klystron

T – Multiple resonator (klystrons)

V – Indirectly heated oxide-coated cathode

X – Directly heated tungsten filament

Y – Directly heated thoriated tungsten filament

Z – Directly heated oxide-coated filament (except mercury-vapor rectifiers)

Gas-filled devices:

G – Mercury-vapor filling

H – Hydrogen filling

R – Inert-gas filling

X – Xenon filling

The next letter indicates the cooling method or other significant characteristic:

H – Helix or other integral cooler

L – Forced-air cooling

Q – Shield-grid (tetrode) thyratron (thyratrons only)

S – Silica envelope, to allow for a glowing anode

T – Tunable microwave device

W – Water cooling

The following group of digits indicate:

Microwave tubes: Frequency in GHz

Rectifying tubes: DC output voltage in kV in a three-phase half-wave configuration

Thyratrons: Peak inverse voltage in kV

Transmitting tubes: Maximum anode voltage in kV

The following group of digits indicate the power:

Backward-wave amplifier or Traveling-wave tube: Output power

2nd letter: A – in mW

2nd letter: B – in W

Klystrons: Output power

Magnetrons: Pulse output power in kW

Continuously transmitting tubes: Maximum anode dissipation in W or kW in Class-C amplifier telegraphy

Pulsed transmitting tubes: Maximum peak anode current in A (number preceded by "P")

Rectifiers: Maximum average anode current in mA

Thyratrons: Maximum average anode current:

Less than 3 digits: in mA

3 or more digits:

1st digit: =0 – in mA

1st digit: >0 – in A

An optional following letter indicates the base or connection method:

B – Cables

E – Medium 7-pin base

ED – Edison screw lamp base

EG – Goliath base

F – Medium 8-pin base

G – Medium 4-pin base

GB – Jumbo 4-pin base

GS – Super jumbo 4-pin base

N – Medium 5-pin base

P – P-base

For examples see below

Phototubes and photomultipliers

The first digit indicates the tube base:

2 – Loctal 8-pin base

3 – Octal 8-pin base

5 – Special base

8 – Noval base

9 – Miniature 7-pin base

The second digit is a sequentially assigned number.

The following letter indicates the photocathode type:

A – Caesium-activated antimony cathode. Used for reflective-mode photocathodes. Response range from ultraviolet to visible. Widely used.

C – Caesium-on-oxidated-silver cathode, also called S1. Transmission-mode, sensitive from 300...1200 nm. High dark current; used mainly in near-infrared, with the photocathode cooled.

T – Multialkali sodium-potassium-antimony-caesium cathode, wide spectral response from ultraviolet to near-infrared; special cathode processing can extend range to 930 nm. Used in broadband spectrophotometers.

U – Caesium-antimony cathode with a quartz window

The following letter indicates the filling:

G – Gas-filled

V – High-vacuum

A following letter P indicates a photomultiplier.

Examples:

50AVP – 11-stage photomultiplier for scintillation counters, duodecal base

51UVP – 11-stage photomultiplier, duodecal base

52AVP/XP1180 – 10-stage photomultiplier, 13-pin base

53AVP, 153AVP – 10-stage photomultiplier, diheptal 14-pin base

53UVP – 11-stage photomultiplier, diheptal 14-pin base

54AVP – 11-stage photomultiplier, diheptal 14-pin base

55AVP – 15-stage photomultiplier, bidecal 20-pin base

56AVP – 14-stage photomultiplier, bidecal 20-pin base

56UVP – 14-stage photomultiplier, duodecal base

57AVP – 11-stage photomultiplier, bidecal 20-pin base

58AVP – 14-stage photomultiplier, bidecal 20-pin base

150AVP – 10-stage photomultiplier, bidecal 20-pin base

150CVP – 10-stage photomultiplier, bidecal 20-pin base

57CV – Photometric cell

58CG – Gas-filled phototube, Red/IR sensitive, all-glass pigtailed

58CV – Vacuum phototube, Red/IR sensitive, all-glass pigtailed

90AG – Gas-filled phototube, daylight/blue sensitive, miniature 7-pin base

90AV – Vacuum phototube, blue sensitive, miniature 7-pin base

90CG – Gas-filled phototube, Red/IR sensitive, miniature 7-pin base

90CV – Vacuum phototube, Red/IR sensitive, miniature 7-pin base

92AG – Gas-filled phototube, blue sensitive, miniature 7-pin base

92AV – Vacuum phototube, blue sensitive, miniature 7-pin base

61SV/7634 – PbS infrared (300...3500 nm) photoresistor, 2-pin all-glass pigtailed

Voltage stabilizers

The first number indicates the burning voltage

The following letter indicates the current range:

A – max. 10mA

B – max. 22mA

C – max. 40mA

D – max. 100mA

E – max. 200mA

The following digit is a sequentially assigned number.

An optional, following letter indicates the base:

E – Edison screw lamp base

K – Octal 8-pin base

P – P-base

Examples:

75B1 – Voltage reference tube, miniature 7-pin base

75C1 – Voltage reference tube, miniature 7-pin base

83A1 – Voltage reference tube, miniature 7-pin base

85A1/0E3 – Voltage reference tube, Loctal B8G base

85A2/0G3 – Voltage reference tube, miniature 7-pin base

90C1 – Voltage reference tube, miniature 7-pin base

95A1 – Voltage reference tube, miniature 7-pin base

100E1 – Voltage reference tube, "A" Base

108C1/0B2 – Voltage reference tube, miniature 7-pin base

150A1 – Voltage reference tube, "P" base

150B2 – Voltage reference tube, miniature 7-pin base

150B3 – Voltage reference tube, miniature 7-pin base

150C1 – Voltage reference tube, "P" base

150C2/0A2 – Voltage reference tube, miniature 7-pin base

150C4 – Voltage reference tube, miniature 7-pin base

Compagnie des Lampes (1888, "Métal") system

The first (1888) incarnation of La Compagnie des Lampes produced the TM tube since 1915 and defined one of the first French systems;[1][16] not to be confused with Compagnie des Lampes (1921, "French Mazda", see below).

First letter: Heater or filament voltage

A – 1 V

B – 2 V

D – 4 V

E – 5 V

F – 6 V

G – 7 V

Second letter: Heater or filament current

W – ≥200 mA

X – 150 mA

Y – 100...140 mA

Z – 50 mA

Next number: Gain

Next number: Internal resistance in kΩ

Examples:

BW604 – Métal secteur indirectly AC-heated AF power triode[17]

BW1010 – Métal secteur indirectly AC-heated AF triode[18]

EdiSwan ("British Mazda") systems

EdiSwan (British Mazda) is not to be confused with other licensees of General Electric's Mazda brand:

GE's own subsidiary British Thomson-Houston

Cie des Lampes (1921, French Mazda, see below)

Cie Industrielle Française des Tubes Electroniques – CIFTE (Mazda-Belvu – originating from Societé Radio Belvu; see below)[19]

Manufacture Belge des Lampes Électriques,(fr, nl) producing:

Light bulbs since 1911 under the Belgian Mazda brand

Electronic tubes since 1924 under the Adzam ("Mazda" spelled backwards) brand[20]

Note: EdiSwan also used the Mullard–Philips scheme.

Signal tubes

First number: Heater or filament rating[1]

0 – Misc. higher voltages

1 – 1.4 V

6 – 6.3 V

10 – 100 mA

20 – 200 mA

30 – 300 mA

Following letter or letter sequence: Type

C – Frequency changer with special oscillator section

D – Signal diode(s)

F – Tetrode or pentode

FD – Tetrode or pentode and diode(s)

FL – Tetrode or pentode, and triode

K – Small gas triode or tetrode thyratron

L – Single or dual triode, including oscillator triode

LD – Triode and diode(s)

M – Optical tuning/level indicator

P – Power tetrode or pentode

PL – Power tetrode or pentode, and signal triode

Final number: Sequentially assigned number

Power rectifiers

Letter(s): Type

U – High-vacuum half-wave rectifier

UU – High-vacuum full-wave rectifier

Number: Sequentially assigned number

Examples:

Note: "AC/"-series receiver tubes are listed under other letter tubes - AC/

6C10 (6CU7/ECH42) – Triode/hexode frequency converter, 8-pin Rimlock base

6F22 (6267/EF86) – Low-noise A.F. pentode, 9-pin noval base

6F33 – Shielded pentode, 7-pin base

6L12 (6AQ8/ECC85) – Dual triode, 9-pin noval base

6L19 – Dual triode, 8-pin base

6M2 (6CD7/EM34) – Dual-sensitivity tuning indicator, 8-pin octal base

6P9 (6BM5) – Power pentode, 7-pin base

6P15 (6BQ5/EL84) – Power pentode, 9-pin noval base

10PL12 (50BM8/UCL82) – Triode/power pentode, 9-pin noval base

U381 (38A3/UY85) – Half-wave rectifier, 9-pin noval base

UU9 (6BT4/EZ40) – Full-wave rectifier, 8-pin rimlock base

EEV system

This system consists of one or more letters followed by a sequentially assigned number[21]

A – High vacuum rectifier

AFX – Rare gas filled triode thyratron

AH – Mercury-vapor rectifier

AX – Xenon filled rectifier

B – Radiation-cooled triode

BD – Mercury vapor rectifier

BK – Ignitron

BM – Magnetron

BR – Forced air cooled triode

BS – TR (Transmit/receive) cell, TB cell, Solid-state microwave device

BT – Mercury vapor or xenon filled thyratron

BW – Water cooled triode

BY – Vapor cooled triode

C – Radiation-cooled tetrode

CR – Forced air cooled tetrode

CW – Water cooled tetrode

CX – Hydrogen tetrode thyratron

E – Storage tube

FX – Hydrogen triode thyratron

GX – Spark gap

K – Klystron

M – Magnetron

NFT – Nernst filament, a source of mid-infrared radiation

P – Video camera tube

QS – Voltage-regulator tube

QT – Cold-cathode trigger tube

T – CRT

U – Vacuum capacitor

XL – Glow modulator tube, flash tube, gas laser

Examples:

B142 – 400 W RF power triode up to 50 MHz similar to 833A

B1109 = 3C24 – 25 W VHF power triode up to 60 MHz

B1135 = 5867 = CV1350 – VHF power triode up to 100 MHz

B1152 – 500W RF power triode up to 50 MHz

QT1257 – Touch button tube, an illuminated capacitance touch switch; a cold-cathode DC relay tube, external (capacitive) starter activated by touching; then the cathode glow is visible. 6-pin octal base

XL601, XL602, XL603, XL627, XL628, XL631 and XL632 – Cold-cathode, linear light source (glow modulator tube), gas diode with a blue-violet glow, modulation up to 1 MHz, 8-pin base, for rotating-drum FAX receivers, etc.

ETL computing tubes system

The British Ericsson Telephones Limited (ETL), of Beeston, Nottingham (not to be confused with the Swedish TelefonAB Ericsson), original holder of the now-generic trademark Dekatron, used the following system:

An initial letter denoting the filling:

G – Gas-filled

V – Vacuum

One letter denoting the type:

C – Common-cathode Counter Dekatron that makes only carry/borrow cathodes separately available for cascading

D – Diode, voltage reference, etc.

R – Register (Readout) – Digital indicator

S – Separate-cathode Counter/Selector Dekatron that makes all cathodes available on individual pins for displaying, divide-by-n counter/timer/prescalers, etc.

TE – Trigger tetrode, one starter electrode and a keep-alive (primer) electrode for ion availability

TR – Trigger triode, one starter electrode only

A digit group:

Dekatrons: Stage count

Digital indicators: Display cathode count

Diodes, voltage references: Nominal voltage

Trigger tubes: Ignition voltage

An optional digit group after a slash: Pin count

One letter denoting the type:

A – Plastic base

B – Plastic base

C – Plastic base

D – Plastic base

E – Plastic base

G – 26-pin B26A base

H – 27-pin B27A base

M – B7G base

P – B7G base

Q – B7G base

W – Pigtails

X – Pigtails

Y – Pigtails

Marconi-Osram system

The British GEC–Marconi–Osram designation from the 1920s uses one or two letter(s) followed by two numerals and sometimes by a second letter identifying different versions of a particular type.[1]

The letter(s) generally denote the type or use:

Note: A preceding letter M indicates a 4-volts AC indirectly heated tube

A – General professional tube

B – Dual triode

D – Detector diode

GU – Gas-filled rectifier

GT – Gas-filled triode

H – High-impedance signal triode

L – Low-impedance signal triode

N – Power pentode

P – Power triode up to 3 W

PT – 3...6 W Power triode

PX – 9...25 W Power triode

QP – Dual pentode

S – Tetrode

U – Rectifier

VS – Remote-cutoff tetrode

W – Remote-cutoff pentode

X – Triode/hexode frequency-changer

Y – Optical tuning/level indicator

Z – Sharp-cutoff RF pentode

The following numbers are sequentially assigned for each new device.

Examples:

Note: Kinkless Tetrode beam power tubes are listed under other letter tubes - KT

A1834 = 6AS7G/ECC230 = CV2523 – Dual power triode (series regulator), octal base.

B309 = 12AT7/ECC81 – High-mu dual triode. Commonly used as R.F. amplifier/mixer in VHF circuits.

B719 = 6AQ8/ECC85 – Dual RF triode, RF Amp & Mixer in FM receivers, noval base.

D41 = V914 – Indirectly heated, Double Detector Diode, British 5-pin base.

D42 – Indirectly heated, Single Detector Diode, British 4-pin base.

GU21 = AH221 = RG4-1250 – Half-wave mercury-vapor rectifier, Edison screw lamp base.

H63 = 6F5 – High-mu triode, octal base.

H610 – Directly heated, high-mu AF triode, British 4-pin base.

L63 = 6J5 – Low-mu triode, octal base.

L610 – Directly heated, Low-mu RF triode, British 4-pin base.

MT7A, MT7B – Large radiation-cooled transmitting triodes used in the 1920s and 1930s.

MU14 = UU5 = IW4-500 – Indirectly heated full-wave rectifier, British 4-pin base.

N77 = 6AM5/EL91 – Power pentode, 7-pin miniature base.

P425 = PM254 – Power triode with a 4 V/200 mA battery heater and a British 4-pin base

P610 – Directly heated, AF power triode, British 4-pin base.

P625 – AF power triode.

PX4 – AF power triode designed in the 1930s. Capable of providing about 4.5 W of audio.

QP21 – Directly heated, dual AF (push-pull) power pentode, British 7-pin base.

QP240 – Directly heated, dual AF (push-pull) power pentode, British 9-pin base.

S610 – Directly heated, Sharp-cutoff RF tetrode, British 4-pin base.

U52 = 5U4G = 5AS4A/5U4GB – Full-wave rectifier, octal base.

VS24 – Directly heated, Remote-cutoff RF tetrode, British 4-pin base.

W727 = 6BA6/EF93 = 5749 – Remote-cutoff RF pentode, 7-pin miniature base.

X41 – Triode/hexode mixer designed to be a direct plug-in replacement for the MX40 pentagrid converter.

X61, X61M = 6J8G – British triode/heptode mixer, octal based.

X63 = 6A8 – Heptode pentagrid converter, octal based.

X727 = 6BE6/EK90 = 5750 – Pentagrid converter, 7-pin miniature base.

Y61, Y63 = 6U5G = VI103 – Optical tuning/level indicator, octal base, similar to 6G5.

Z77 = 6AM6/EF91 – Sharp-cutoff RF pentode, 7-pin miniature base.

Media related to Marconi vacuum tubes at Wikimedia Commons

Mullard designations before 1934

Older Mullard tubes were mostly designated PM, followed by a number containing the filament voltage.

Many later tubes were designated one to three semi-intuitive letters, followed by a number containing the heater voltage. This was phased out after 1934 when Mullard adopted the Mullard–Philips scheme.

Examples:[22]

2D4 – Dual Diode with a 4 V/650 mA heater and a British 5-pin base

FC4 = MX40 – Octode Frequency Converter with a 4 V/650 mA heater and a British 7-pin base

Pen20 – Power Pentode with a 20 V/180 mA heater and a British 5- or 7-pin base

PM254 = P425 – "Super Power" triode with a 4 V/200 mA battery heater and a British 4-pin base

TDD4 = MHD4 = AC/HLDD – Triode, dual Diode with a 4 V/550 mA heater and a British 7-pin base

TH21C – Triode/Hexode frequency converter with a 21 V/200 mA series heater and a British 7-pin base

TP4 = AC/TP – Triode, Pentode with a 4 V/1.25 A heater and a British 7-pin base

VP2 = VP21 = VP215 – Variable-mu Pentode with a 2 V/180 mA heater and a British 7-pin base

Media related to Mullard vacuum tubes at Wikimedia Commons

Philips system before 1934

The system consisted of one letter followed by 3 or 4 digits. It was phased out after 1934 when Philips adopted the Mullard–Philips scheme.

1st letter: Heater current[23][1]

A – 60...90 mA

B – 100...190 mA (This designation lived on as the "B" (180 mA) in the Mullard–Philips system)

C – 200...390 mA (This designation lived on as the "C" (200 mA) in the Mullard–Philips system)

D – 400...690 mA

E – 700...990 mA

F – 1...2 A

1 or 2 digit(s): Heater voltage

Last 2 digits: Type

00–40, 99: Triode amplification factor

41–98:

second-last digit: sequentially assigned, starting at 4

last digit:

1 – Tetrode with a space charge grid (the 2nd grid is the control grid)

2 – Tetrode with a screen grid (the 1st grid is the control grid)

3 – Power pentode

4 – Binode, a diode/triode or diode/tetrode

5 – Remote-cutoff RF tetrode

6 – Signal pentode

7 – Remote-cutoff RF pentode

8 – Sharp-cutoff hexode frequency changer

9 – Remote-cutoff hexode

Examples:[24]

A106 – Directly heated triode, 1 V, 60 mA filament, amplification factor = 6

A425 = RE034 = HR406 – RF triode, 4 V, 60 mA filament

A435 – Directly heated triode, 4 V, 60 mA filament, amplification factor = 35

A441 – Directly heated tetrode with a space charge grid, 4 V, 60 mA filament

A442 = RES094 = S406 – Directly heated tetrode with a screen grid, 4 V, 60 mA filament

B409 = RE134 = L414 – Triode, 4 Volt, 140 mA filament

B2038 = REN1821 = R2018 = A2118 – Triode, 180 mA heater

B2043 = RENS1823D = PP2018D = L2318D – Indirectly heated power pentode, 20 V, 180 mA DC series heater

B2044 = RENS1854 = DS2218 – Indirectly heated diode/tetrode, 20 V, 180 mA DC series heater

B2044S = REN1826 – Indirectly heated diode/triode, 20 V, 180 mA DC series heater

B2045 = RENS1819 – Indirectly heated remote-cutoff RF tetrode, 20 V, 180 mA DC series heater

B2048 = RENS1824 = MH2018 – Hexode mixer, 20 V, 180 mA heater

B2099 = REN1814 – Indirectly heated triode, 20 V, 180 mA DC series heater, amplification factor = 99

E443H = RES964 = PP4101 = L496D – Power pentode, 4 V heater

E446 = RENS1284 = HP4101 – Indirectly heated RF pentode, 4 V, 1.1 A heater

E447 = RENS1294 = HP4106 – Indirectly heated remote-cutoff RF pentode, 4 V, 1.1 A heater

E448 = RENS1224 = MH4100 – Indirectly heated sharp-cutoff hexode frequency changer, 4 V, 1.2 A heater

E449 = RENS1234 = FH4105 – Indirectly heated remote-cutoff hexode, 4 V, 1.2 A heater

F215 – Indirectly heated triode, 2.5 V, 1.5 A heater, amplification factor = 15

Media related to Philips vacuum tubes at Wikimedia Commons

Media related to Philips gas discharge tubes at Wikimedia Commons

STC/Brimar receiving tubes system

First number: Type[1]

1 – Half-wave rectifier

2 – Diode

3 – Power triode

4 – High-mu triode

5 – Sharp-cutoff tetrode

6 – Vari-mu tetrode

7 – Power or video pentode

8 – Sharp-cutoff RF pentode

9 – Vari-mu RF pentode

10 – Dual diode

11 – Triode and dual diode

12 – AF Pentode and dual diode

13 – Dual high-mu triode

14 – Dual Class-B power triode

15 – Heptode

16 – DC-coupled power triode

17 – RF pentode and dual diode

18 – Pentode and triode

20 – Hexode/heptode and triode

Next letter: Heater rating

A – 3.6 to 4.4 V Indirectly heated

B – 2 V Directly heated

C – Directly heated other than 2 or 4 V

D – All other heater ratings, indirectly heated other than 4 V

Number: Sequentially assigned number

Examples:

1D6 – Indirectly heated, half-wave rectifier, 5-pin base

4D1 – Indirectly heated triode, 7-pin base

7A3 – Indirectly heated power pentode, 7-pin base

8A1 – Indirectly heated RF sharp-cutoff pentode, 5-pin base with anode top cap

9A1 – Indirectly heated RF/IF remote-cutoff pentode, 5-pin base with anode top cap

10D1 – Indirectly heated, common-cathode dual diode, 5-pin base

11A2 – Indirectly heated, common-cathode dual diode and triode, 7-pin base

13D3 – Indirectly heated, common-cathode dual triode, 9-pin base

15A2 – Indirectly heated, heptode pentagrid converter, 7-pin base

20D4 – Indirectly heated, triode/heptode frequency mixer, 9-pin base

Valvo system before 1934

Valvo(de, it) was a major German electronic components manufacturer from 1924 to 1989; a Philips subsidiary since 1927, Valvo was one of the predecessors of NXP Semiconductors.

The system consisted of one or two letters followed by 3 or 4 digits. It was phased out after 1934 when Valvo adopted the Mullard–Philips scheme.

First letter(s): Type[25]

A – Triode

AN – Binode, a diode/triode or diode/tetrode

G – Rectifier

H – RF tube

L – Power tube

LK – Power amplifier

U – Triode with a space charge grid

W – Triode for resistor-coupled amplifiers

X – Hexode

Number:

If the first digit is 4, the tube has a 4 V heater

Otherwise, the last two digits give the heater current in tens of mA.

A following letter D indicates more than one grid, not counting a space charge grid

Examples:[24]

A2118 = B2038 = REN1821 = R2018 – Triode, 180 mA (=18⋅10 mA) heater

H2018D = B2042 = RENS1820 = S2018 – RF Tetrode, 180 mA heater

L496D = E443H = RES964 = PP4101 – Power pentode, 4 V heater

L2318D = B2043 = RENS1823D = PP2018D – Power pentode, 180 mA heater

Media related to Valvo vacuum tubes at Wikimedia Commons

Media related to Valvo gas discharge tubes at Wikimedia Commons

East European systems

Lamina transmitter tube system

Polish Lamina(pl) transmitter tube designations consist of one or two letters, a group of digits and an optional letter and/or two digits preceded by a "/" sign.

The first letter indicates the tube type, two equal letters denoting a dual tube:

P – Pentode

Q – Tetrode

T – Triode

A group of digits represents the maximum anode power dissipation in kW

An optional letter specifies the cooling method:

<none> – Radiation

P – Forced air

W – Water

The first of the two digits after the "/" sign means:

1 – Tube for radio broadcasting and radiocommunication equipment

2 – Tube for industrial equipment

3 – Tube used in TV broadcasting equipment

4 – Tube for radiocommunication equipment with unbalanced modulation

5 – Modulator or pulse tube

The second digit after the "/" is sequentially assigned.

Examples:

Q01 – Power tetrode up to 125 MHz, 0.1 kW (=100 W)

Q3.5 – Power tetrode up to 220 MHz, 3.5 kW

QQ-004/11 – Dual beam power tetrode up to 500 MHz, 0.04 kW (=40 W)

T01 – Power triode up to 200 MHz, 135 W

T015/21 – Power triode up to 150 MHz, 150 W

T02 – Power triode up to 60 MHz, 200 W

T05P/31 – Forced air cooled power triode up to 1 GHz, 1 kW

T2/22 – Power triode up to 60 MHz, 3 kW

T6 – Power triode up to 30 MHz, 6 kW

T8P/21 – Forced air cooled power triode up to 120 MHz, 8 kW

T10P/22 – Power triode up to 30 MHz, 10 kW

T-25P – Forced air cooled power triode up to 30 MHz, 25 kW

T60W/21 – Water cooled power triode up to 30 MHz, 6 kW

Media related to Lamina vacuum tubes at Wikimedia Commons

Media related to Lamina gas discharge tubes at Wikimedia Commons

RFT transmitter tube system

Rundfunk- und Fernmelde-Technik(de, sv) was the brand of a group of telecommunications manufacturers in the German Democratic Republic. The designation consists of a group of three letters and a group of three or four digits.

The first two letters determine the tube type:

SR – Transmitter tube

VR – Amplifier tube

The third letter specifies the cooling method:

L – Forced air

S – Radiation

V – Vapor (the anode is immersed in evaporating water, and the steam is collected, condensed and recycled)

W – Water

The first digit (or the first two digits in double tubes) indicates the number of electrodes:

2 – Diode

3 – Triode

4 – Tetrode

5 – Pentode

The last two digits are sequentially assigned.

Examples:

SRS301 – Radiation-cooled triode up to 40 MHz, 900 W

SRS464 – Radiation-cooled, vibration-resistant pulse tetrode up to 300 kW

SRS4451 – Radiation-cooled dual tetrode up to 500 MHz, 60 W

SRS4452 = QQE03/20 = 6252 – Radiation-cooled dual tetrode up to 600 MHz, 20 W

SRS4452 – Radiation-cooled dual tetrode up to 600 MHz, 20 W

SRS501 – Radiation-cooled pentode up to 50 MHz, 100 W

SRS552N = ГУ-50 – Radiation-cooled pentode up to 120 MHz, 50 W

VRS303 – Radiation-cooled AF triode, 1 kW

VRS328 – Radiation-cooled AF triode, 150 W

VRS331 – Radiation-cooled AF triode, 450 W

Note: RFT used the Mullard–Philips and RETMA schemes for their low-power tubes.

Media related to RFT vacuum tubes at Wikimedia Commons

Tesla systems

Signal tubes

Besides the genuine Mullard–Philips system, Tesla also used an M-P/RETMA hybrid scheme:[1]

First number: Heater voltage, as in the RETMA system

Next letter(s): Type, subset of the Mullard–Philips system

Next digit: Base

1 – Octal K8A, A08

2 – Loctal W8A

3 – Miniature 7-pin B7G

4 – Noval B9A

5 – Special, mostly 9 out of 10 1.25mm pins on a 25mm-diameter circle

6 – Submagnal B11A

7 – Duodecal B12A

8 – Diheptal B14A

9 – Pigtails

Last digit: Sequentially assigned number

Examples:

1M90 (DM70/1M3) – Subminiature indicator tube, 1.4V/25 mA filament, all-glass pigtailed

4L20 – Directly heated RF power pentode; filament 2x 2.4V/325mA; Soviet 4П1Л, German RL4,2P6 with Loctal base

6B31 – Dual diode up to 700 MHz; 6.3V/300mA heater, miniature 7-pin base

6BC32 (6AV6, EBC91) – Dual diode and triode; 6.3V/300mA heater, miniature 7-pin base

6CC31 (6J6, ECC91) – 600 MHz dual triode; 6.3V/450mA heater, miniature 7-pin base

6CC42 (2C51) – VHF dual triode; 6.3V/350mA heater, noval base

6F24 – Telecom pentode, 6.3V/450mA heater, Loctal base

6F36 (6AH6) – Sharp-cutoff IF/video pentode, 6.3V/450mA heater, miniature 7-pin base

6H31 (6BE6, EK90) – Heptode mixer; 6.3V/300mA heater, miniature 7-pin base

6L36 (6AQ5, EL90) – Power pentode, 6.3V/450mA heater, miniature 7-pin base

6L41 (5763) – Beam tetrode, 6.3V/750mA heater, noval base

35Y31 – Half-wave rectifier, miniature 7-pin base; 35V/150mA series heater; UY1N with 7-pin base

Power tubes

First letter:

R – Rectifier or RF tube

U – Gas-filled power rectifier

Z – Modulator tube

Next letter(s): Type, subset of the Mullard–Philips scheme

Next number: Anode dissipation in W (if radiation cooled) or kW (otherwise)

The next letter specifies the cooling method:

<none> – Radiation

V – Vapor

X – Forced air

Y – Water

Examples:

RA0007B – Directly heated saturated-emission ballast diode. Acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits; UAmax 600 V IAmax 700 µA, miniature 9-pin noval base

RA100A – 40 kV, 100 mA Half-wave rectifier with an E40 Goliath Edison lamp screw base and an anode top cap

RC5B – Bowl-type UHF power triode up to 5 W

RD27AS – Radiation-cooled power triode up to 25 MHz, 27 W

RD200B – Radiation-cooled power triode up to 60 MHz, 200 W

RD300S – Radiation-cooled power triode up to 200 MHz, 300 W

RD150YA – Water-cooled power triode up to 3 MHz, 150 kW

RE40AK = KT88

RE65A – Radiation-cooled beam tetrode up to 260 MHz, 65 W

RE125C – Radiation-cooled beam tetrode up to 235 MHz, 125 W

RE400C – Radiation-cooled beam tetrode up to 235 MHz, 400 W

RE20XL – Air-cooled beam tetrode up to 220 MHz, 20 kW

REE30A – Radiation-cooled dual beam tetrode up to 250 MHz, 20 W

RL15A – Radiation-cooled power pentode up to 60 MHz, 20 W

RL40A – Radiation-cooled power pentode up to 120 MHz, 40 W

RL65A – Radiation-cooled power pentode up to 15 MHz, 65 W

UA025A – 10 kV, 250 mA Argon-filled, half-wave rectifier with an E24 Edison lamp screw base and an anode screw top cap

UA5A – 11 kV, 5 A Half-wave mercury-vapor rectifier with a 2-pin base and an anode screw top cap

ZD1000F – Radiation-cooled power triode up to 60 MHz, 1 kW

ZD1XB – Air-cooled AF power triode up to 1.2 kW

ZD3XH – Air-cooled power triode up to 60 MHz, 3 kW

ZD8XA – Air-cooled power triode up to 20 MHz, 8 kW

ZD12YA – Air-cooled AF power triode up to 20 MHz, 12 kW

ZE025XS – Air-cooled beam tetrode up to 400 MHz, 250 W

Media related to Tesla vacuum tubes at Wikimedia Commons

Media related to Tesla gas discharge tubes at Wikimedia Commons

Tungsram receiving tubes system before 1934

The Tungsram system was composed of a maximum of three letters and three or four digits.[26][25] It was phased out after 1934 when Tungsram adopted the Mullard–Philips scheme, frequently preceding it with the letter T, as in TAD1 for AD1.

Letter: System type:

Note: A preceding letter A indicates an indirectly heated tube

D – Detector diode

DD – Dual diode

DG – Tetrode with a space charge grid (the 2nd grid is the control grid)

DS – Diode-tetrode

FH – Remote-cutoff hexode pentagrid converter

G – Preamplifier triode

H – Voltage amplifier triode or grid-leak detector

HP – RF pentode

HR – RF triode

L – AF power triode

MH – Hexode pentagrid converter

MO – Octode pentagrid converter

O – Transmitting tube

P – Power triode

PP – Power pentode

PV – Full-wave rectifier

R – High-Mu triode

S – Tetrode

V – Half-wave rectifier

X – US-licensed tube

Number:

First digit (or the first two digits): Heater voltage

Remaining digits: Heater current in tens of mA, but the last digit is sequentially assigned

Examples:[24]

AS4100 – Tetrode, 4 V, 1 A (=100⋅10 mA) indirect heater

FH4105 = E449 = RENS1234 – Indirectly heated remote-cutoff hexode, 4 V, 1.2 A heater

HP4101 = E446 = RENS1284 – RF pentode, 4 V, 1 A filament

HP4106 = E447 = RENS1294 – Indirectly heated remote-cutoff RF pentode, 4 V, 1.1 A heater

HR406 = A425 = RE034 – RF triode, 4 V, 60 mA (=6⋅10 mA) filament

L414 = B409 = RE134 – Triode, 4 Volt, 140 mA (=14⋅10 mA) filament

MH2018 = B2048 = RENS1824 – Hexode mixer, 20 V, 180 mA (=18⋅10 mA) heater

MH4100 = E448 = RENS1224 – Indirectly heated sharp-cutoff hexode frequency changer, 4 V, 1.2 A heater

PP4101 = E443H = RES964 = L496D – Power pentode, 4 V heater

PV4200 – Full-wave rectifier, 4 V, 2 A (=200⋅10 mA) filament

PP2018D = B2043 = RENS1823D = L2318D – Indirectly heated power pentode, 20 V, 180 mA DC series heater

R2018 = B2038 = REN1821 = A2118 – Triode, 180 mA heater

S406 = A442 = RES094 – Directly heated tetrode with a screen grid, 4 V, 60 mA filament

S2018 = B2042 = RENS1820 = H2018D – RF Tetrode, 180 mA heater

Media related to Tungsram vacuum tubes at Wikimedia Commons

Russian systems

Main article: Russian tube designations

Vacuum tubes produced in the former Soviet Union and in present-day Russia are designated in Cyrillic. Some confusion has been created in transliterating these designations to Latin.

The first system was introduced in 1929. It consisted of one or two letters (designating system type and, optionally, type of cathode), a hyphen, then a sequentially assigned number with up to 3 digits.[25]

In 1937, the Soviet Union purchased a tube assembly line from RCA (who at the time had difficulties raising funds for their basic operations), including production licenses and initial staff training, and installed it on the Svetlana/Светлана plant in St. Petersburg, Russia. US-licensed tubes were produced since then under an adapted RETMA scheme.

Examples:[27]

6Ф5 = 6F5 – High-mu triode

6Ф6 = 6F6 – Power pentode

6Х6 = 6H6 – Dual diode

6Ж7 = 6J7/EF37 – Sharp-cutoff pentode

6Л6 = 6L6 – Beam tetrode

6Л7 = 6L7 – Pentagrid converter

6Н7 = 6N7 – Dual power triode

GOST standard tubes system

In the 1950s a 5-element system (Russian: Государственный Стандарт "State standard" ГОСТ/GOST 5461-59, later 13393-76) was adopted in the (then) Soviet Union for designating receiver vacuum tubes.[28][29]

The first element is a number specifying filament voltage. The second element is a Cyrillic letter specifying the type of device. The third element is a sequentially assigned number that distinguishes between different devices of the same type. The fourth element denotes the type of envelope. An optional fifth element consists of a hyphen followed by one or more characters to designate special characteristics of the tube. This usually implies construction differences, not just selection from regular quality production.

Professional tubes system

There is another designation system for professional tubes such as transmitter ones.[30][25]

The first element designates function. The next elements varies in interpretation. For ignitrons, rectifiers, and thyratrons, there is a digit, then hyphen, then the anode current in amperes, a slash, anode voltage in kV. A letter may be attached to designate water cooling (no letter designates a radiation cooled device). For transmitting tubes in this system, the second element starts with a hyphen, a sequentially assigned number, then an optional letter specifying cooling method. For phototubes and photomultipliers, the second element is a sequential number and then a letter code identifying vacuum or gas fill and the type of cathode.

Japanese systems

Older numbering system 1941–51

A letter: Structure and usage[31]

E – Electron ray tube

K – Kenotron (rectifier)

U – General-purpose tube

Then a letter: Base and outline

N – Pigtailed (Acorn tubes, etc.)

S – Octal glass/metal

T – ST large 7-pin

t – ST small 7-pin

V – 4-pin

X – S/ST 4-pin

x – Peanut 4-pin

Y – S/ST 5-pin

y – Peanut 5-pin

Z – S/ST 6-pin U6A

Then a hyphen ("-"), followed by a sequentially assigned number or the designation of the American original

Then an optional hyphen ("-"), followed by a letter: Version

Examples:[32]

EZ-6G5 = 6G5 – Variable-mu "Magic Eye"-type tuning indicator

KX-80-B – Kenotron

UN-954 = 954 – Acorn sharp-cutoff pentode

UN-955 = 955 – Acorn triode

US-6A8 = 6A8 – Pentagrid converter

US-6L7G = 6L7G – Pentagrid converter

UX-26-B – Medium-mu RF triode

UX-167 – Sharp-cutoff RF pentode

UY-47B – Pentode

UZ-58-A – Remote-cutoff RF/IF pentode

JIS C 7001 system

JIS C 7001 was published in 1951 and modified in 1965 and 1970[31]

A number: Heater voltage range, as in the RETMA scheme

1 – 1 V ≤ Uf < 2 V

2 – 2 V ≤ Uf < 2.5 V

3 – 2.5 V ≤ Uf < 4 V

4 – 4 V ≤ Uf < 5 V

5 – 5 V ≤ Uf < 6 V

6 – 6 V ≤ Uf < 7 V

etc.

Then a letter: Base and Outline

A – Special base

B – Other

C – Compactron (Duodecal)

D – Subminiature button base

E – Subminiature flat base

F – European 4-pin ST

G – Octal base glass tube (GT)

H – Magnoval

K – Ceramic

L – Lock-In (Loktal)

M – Miniature (7-pin)

N – Nuvistor

Q – Acorn tube

R – Noval (9-pin miniature) or Neonoval (9T9)

S – Octal (US)

T – Large 7-pin ST

W – 7-pin ST

X – 4-pin ST

Y – 5-pin ST

Z – 6-pin ST

Then a hyphen ("-"), followed by a letter: Structure and usage

A – Power triode

B – Beam power tube

C – Pentagrid converter

D – Diode

E – Optical tuning/level indicator

G – Gas-filled rectifier

H – High-mu triode (μ>30)

K – Kenotron (rectifier)

Even number after K: Full-wave rectifier

Odd number after K: Half-wave rectifier

L – Low-mu triode (μ<30)

P – Power tetrode or pentode

R – Sharp-cutoff tetrode or pentode

S – Tetrode with a space charge grid (the 2nd grid is the control grid)

T – Gas-filled, grid-controlled

V – Variable-mu (remote-cutoff) tetrode and pentode

X – Other

Then a sequentially assigned number

Then an optional letter: Version

Examples:[32]

2N-H12 – Nuvistor

2X-L2A – Low-mu triode

6C-A10 – Power triode

6G-A4 – Power triode

6G-B8 – Beam power tube

6G-E12A – 2-channel "Magic Eye"-type tuning indicator, rectangular target

6H-B26 – Beam power tube

6M-DE1 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

6M-E4 – "Magic Finger"-type tuning indicator, miniature 7-pin base B7G

6M-E5 = 6ME5 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

6M-E10 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

6N-H10 – Nuvistor

6R-A8 – Power triode

6R-B10 – Beam power tube

6R-B11 – Beam power tube

Military naming systems

British CV naming system

This system prefixes a three- or four-digit number with the letters "CV", meaning "civilian valve" i.e. common to all three armed services. It was introduced during the Second World War to rationalise the previous nomenclatures maintained separately by the War Office/Ministry of Supply, Admiralty and Air Ministry/Ministry of Aircraft Production on behalf of the three armed services (e.g. "ACR~", "AR~", "AT~", etc. for CRTs, receiving and transmitting valves used in army equipments, "NC~", "NR~" and "NT~" similarly for navy equipments and "VCR~", "VR~" and "VT~" etc. for air force equipments), in which three separate designations could in principle apply to the same valve (which often had at least one prototype commercial designation as well). These numbers generally have identical equivalents in both the North American, RETMA, and West European, Mullard–Philips, systems but they bear no resemblance to the assigned "CV" number.

Examples:

CV1988 = 6SN7GT = ECC32 (not a direct equivalent as heater current is different and bulb is larger)

CV2729 = E80F – An SQ version of EF80 but with revised pin-out and a base screen substituted for the RF screen

CV4007 = E91AA – SQ version of 6AL5

CV4010 = E95F – SQ version of 6AK5 or EF95

CV4014 = M8083 – SQ version of EF91 or 6AM6 (The 'M' in the part number denotes that it was developed by the military)

Note: The 4000 numbers identify special-quality valves though SQ valves CV numbered before that rule came in retain their original CV number.

The principle behind the CV numbering scheme was also adopted by the US Joint Army-Navy JAN numbering scheme which was later considerably expanded into the US Federal and then NATO Stock Number system used by all NATO countries. This part-identification system ensures that every particular spare part (not merely thermionic valves) receives a unique stock number across the whole of NATO irrespective of the source, and hence is not held inefficiently as separate stores. In the case of CV valves, the stock number is always of the format 5960-99-000-XXXX where XXXX is the CV number (with a leading 0 if the CV number only has 3 digits).

U.S. naming systems

One system prefixes a three-digit number with the letters "VT", presumably meaning "Vacuum Tube". Other systems prefix the number with the letters "JHS" or "JAN". The numbers following these prefixes can be "special" four-digit numbers, or domestic two- or three-digit numbers or simply the domestic North American "RETMA" numbering system. Like the British military system, these have many direct equivalents in the civilian types. Confusingly, the British also had two entirely different "VT" nomenclatures, one used by the Royal Air Force (see the preceding section) and the other used by the General Post Office, responsible for post and telecommunications at the time, where it may have stood for "valve, telephone"; none of these schemes corresponded in any way with each other.

Examples:

"VT" numbering systems

North American VT90 = 6H6

British (RAF) VT90 – VHF Transmitting triode

British (GPO) VT90 = ML4 = CV1732 – Power triode

VT104 – RF pentode

VT105 – RF triode

Other numeral-only systems

Various numeral-only systems exist. These tend to be used for devices used in commercial or industrial equipment. The oldest numbering systems date back to the early 1920s, such as a two-digit numbering system, starting with the UV-201A, which was considered as "type 01", and extended almost continuously up into the 1980s. Three- and four-digit numeral-only systems were maintained by R.C.A., but also adopted by many other manufacturers, and typically encompassed rectifiers and radio transmitter output devices. Devices in the low 800s tend to be transmitter output types, those in the higher 800s are not vacuum tubes, but gas-filled rectifiers and thyratrons, and those in the 900s tend to be special-purpose and high-frequency devices. Use was not rigorously systematic: the 807 had variants 1624, 1625, and 807W.

Other letter followed by numerals

There are quite a number of these systems from different geographical realms, such as those used on devices from contemporary Russian and Chinese production. Other compound numbering systems were used to mark higher-reliability types used in industrial or commercial applications. Computers and telecommunication equipment also required tubes of greater quality and reliability than for domestic and consumer equipment.

Some letter prefixes are manufacturer's codes:

C – RCA/Cunningham

CK, QK, RK – Raytheon Company

ECG – Philips/Sylvania

EM – Eitel McCullough

F – Federal Telephone and Radio

GE, GL – General Electric Corp. (not British General Electric Company)

HK – Heintz & Kaufman, Ltd. (San Francisco, California, USA)

HY – CBS/Hytron

ML – Machlett Laboratories, Inc.

NL – National Electronics, Inc.

NU – National Union Electric Corp.

PL – Philips N.V.

RCA – RCA/Radiotron

SV – Svetlana/Светлана

SY – Standard Telephones and Cables Ltd./Brimar

TH – Compagnie Française Thomson-Houston

WE – Western Electric Company

WL – Westinghouse Electric Corp.

XD – Central Electronic Manufacturers (Denville, New Jersey, USA)

For examples, see below

Some designations are derived from the behavior of devices considered to be exceptional.

Mazda/EdiSwan sold their first tubes for 4-volts AC mains transformer (as opposed to home storage battery) heating with the prefix AC/ (for examples see below).

The first beam tetrodes manufactured in the UK in the late 1930s by M-OV, carried a "KT" prefix meaning Kinkless Tetrode (for examples see below).

List of American RETMA tubes, with European equivalents

Note: Typecode explained above. See also RETMA tube designation

"0 volt" gas-filled cold cathode tubes

First character is numeric zero, not letter O.

Voltage stabilisers and references. Function in a similar way to a Zener diode, at higher voltages. Letter order (A-B-C) indicates increasing voltage ratings on octal-based regulators and decreasing voltage ratings on miniature-based regulators.

0A2 – 150 volt regulator, 7-pin miniature base

0A3 – 75 volt regulator, octal base, aka VR75

0B2 – 105 volt regulator, 7-pin miniature base

0B3 – 90 volt regulator, octal base, aka VR90

0C2 – 75 volt regulator, 7-pin miniature base

0C3 – 105 volt regulator, octal base, aka VR105

0D3 – 150 volt regulator, octal base, aka VR150

Other cold-cathode tubes

0A4G – 25 mAavg, 100mApeak Gas triode designed for use as a ripple control receiver; with the cathode tied to the midpoint of a series-resonance LC circuit across live mains, it would activate a relay in its anode circuit while fres is present

0Y4 – 40 ≤ I ≤ 75 mA Half-wave gas rectifier with a starter anode, 5-pin octal base

0Z4 – 30 ≤ I ≤ 90 mA Argon-filled, full-wave gas rectifier, octal base. Widely used in vibrator power supplies in early automobile radio receivers.

1 volt heater/filament tubes

Tubes with up to 1.4 volt heaters

1B3GT – High-voltage rectifier diode with 1.25 V filament common in monochrome TV receivers of the 1950s and early 1960s. Peak inverse voltage of 30 kV. Anode current 2 mA average, 17 mA peak. Derived from the earlier industrial type 8016. (International Octal base.)

1V2 – High voltage rectifier with 0.625 V/300 mA filament (B7G base)

Tubes with 1.4 volt DC heaters

1A3 – High frequency diode with indirectly heated cathode. Used as a detector in some portable AM/FM receivers.

1A7GT/DK32 – Pentagrid converter

1G6-G – Dual power triode. "GT" version also available.

1L6 – Pentagrid frequency changer for battery radios with 50 mA filament

1LA6 (loctal) and later 1L6 (7-pin miniature) – Battery pentagrid converter for Zenith Trans-Oceanic shortwave radio, 50 mA filament

1LB6 – Superheterodyne mixer for battery-operated radios

1LC6 – Similar to type 1LA6, but with higher conversion transconductance

1R5/DK91 – Pentagrid converter, anode voltage in the 45...90 volt range.

1S4 – Power output pentode Class-A amplifier, anode voltage in the 45...90 volt range.

1S5 – Sharp-cutoff pentode Class-A amplifier, and diode, used as detector and first A.F. stage in battery radio receivers. Anode voltage in the 67...90 volt range.

1T4/DF91 – Remote-cutoff R.F. Pentode Class-A amplifier, used as R.F. and I.F. amplifier in battery radio receivers (B7G base).

1U4 – Sharp-cutoff R.F. Pentode Class-A amplifier, used as R.F. and I.F. amplifier in battery radio receivers, similar characteristics to 6BA6 (B7G base).

1U6 – Nearly identical to type 1L6, but with a 1.4 V/25 mA filament

1.25 volt filament subminiature tubes

The following tubes were used in post-World War II walkie-talkies and pocket-sized portable radios. All have 1.25 volt DC filaments and directly heated cathodes. Some specify which end of the filament is to be powered by the positive side of the filament power supply (usually a battery). All have glass bodies that measure from 0.285 to 0.400 inches (7.2 to 10.2 millimetres) wide, and from 1.25 to 2.00 inches (32 to 51 millimetres) in overall length.

1AC5 – Power pentode, FL

1AD4 – Sharp-cutoff pentode, FL

1AD5 – Sharp-cutoff pentode, R8

1AE5 – Heptode mixer, FL

1AG4 – Power pentode, FL

1AG5 – Diode, pentode, FL

1AH4 – RF pentode, FL

1AJ5 – Diode, sharp-cutoff pentode, FL

1AK4 – Sharp-cutoff pentode, FL

1AK5 – Diode, sharp-cutoff pentode, FL

1C8 – Pentagrid converter, R8

1D3 – Low-mu high-frequency triode, R8

1E8 – Pentagrid converter, R8

1Q6 – Diode, pentode, R8

1S6 – Diode, pentode, R8

1T6 – Diode, pentode, R8

1V5 – Power pentode, R8

1V6 – Triode-pentode converter, FL

1W5 – Sharp-cutoff pentode, R8

1 prefix for home receivers

These tubes were made for home storage battery receivers manufactured during the early to mid-1930s. The numbers of the following tubes all start with 1, but these tubes all have 2.0 volt DC filaments. This numbering scheme was intended to differentiate these tubes from the tubes with 2.5 volt AC heaters listed below.

1A4-p – Remote-cutoff pentode

1A4-t – Remote-cutoff tetrode

1A6 – Pentagrid converter up to only 10 MHz due to low heater power (2 V/60 mA) and consequent low emission in the oscillator section; also occasionally used as a grid-leak detector

1A7-GT – Re-engineered version of types 1A6 and 1D7-G, designed for use in portable AC/DC/Dry-cell battery radios introduced in 1938. Has 1.4 V/50 mA filament.

1B4-p – Sharp-cutoff pentode

1B4-t – Sharp-cutoff tetrode

1B5 – Dual detector diode, medium-mu triode. Usually numbered 1B5/25S

1B7-GT – Re-engineered version of types 1C6 and 1C7-G, designed for use in dry-cell battery radios with shortwave bands. Has 1.4 V/100 mA filament

1C5 – Power pentode (similar to 3Q5 except for filament)

1C6 – Pentagrid converter; 1A6, with double the heater power and double the frequency range

1C7-G – Octal version of type 1C6.

1D5-Gp – Octal version of type 1A4-p.

1D5-Gt – Octal version of type 1A4-t. (Note: This is a shouldered "G" octal, not a cylindrical "GT" octal.)

1D7-G – Octal version of type 1A6.

1E5-Gp – Octal version of type 1B4-p.

1E5-Gt – Octal version of type 1B4-t. (Note: This is a shouldered "G" octal, not a cylindrical "GT" octal.)

1E7-G – Dual power pentode for use as a driver when parallel-connected, or as a push-pull output. "GT" version also available

1F4 – Power pentode

1F5-G – Octal version of 1F4.

1F6 – Duplex diode, sharp-cutoff pentode

1F7-G – Octal version of type 1F6

1G5-G – Power pentode

1H4-G – Medium-mu triode, can be used as a power triode. Octal version of type 30, which is an upgraded version of type 01-A. "GT" version also available.

1H6-G – Octal version of type 1B5/25S. "GT" version also available.

1J5-G (950) – AF Power pentode

1J6-G – Dual power triode, octal version of type 19. "GT" version also available.

2 volt heater/filament tubes

Tubes used in AC-powered radio receivers of the early 1930s. All have 2.5 volt heaters.

2A3 – Directly heated power triode, used for AF output stages in 1930s–1940s audio amplifiers and radios.

2A5 – Power Pentode (Except for heater, electronically identical to types 42 and 6F6)

2A6 – Dual diode, high-mu triode (Except for heater, electronically identical to type 75)

2A7 – Dual-tetrode-style pentagrid converter (Except for heater, electronically identical to types 6A7, 6A8 and 12A8)

2B7 – Dual diode and remote-cutoff pentode (Except for heater, electronically identical to type 6B7)

2E5 and 2G5 – Electron-ray indicators ("Eye tube") with integrated control triode. (Except for heater, electronically identical to types 6E5 and 6G5)

Tubes used in television receivers

2AF4 – UHF triode oscillator

2BN4 – VHF/RF triode

2CW4 – Nuvistor high-mu VHF triode, 6CW4 with a 2.1 volt/450 mA heater; used in TV receivers with series heater strings

2CY5 – VHF sharp-cutoff RF tetrode

2EA5 – VHF sharp-cutoff RF tetrode

2EN5 – Dual-diode

2ER5 – VHF RF triode

2ES5 – VHF RF triode

2EV5 – VHF sharp-cutoff RF tetrode

2FH5 – VHF RF triode

2FQ5 – VHF RF triode

2FV6 – VHF sharp-cutoff RF tetrode

2FY5 – VHF RF triode

2X2 – High Vacuum High Peak inverse voltage diode, used as rectifier in CRT EHT supplies. Similar to 1B3 and 1S2 except for heater voltage.

3 volt heater/filament high voltage rectifier tubes

3A3/3B2/3AW3/ - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.22 amps.

3CA3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.6 volts and 0.225 amps.

3CN3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

3CU3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.28 amps.

3CZ3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

3AT2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. It comes in the variation as the 3AT2B, mainly for color television sets with a large picture tube. The 3AT2B comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.22 amps.

3AW2 - High Voltage rectifier. A compactron used for color and black and white television sets. It comes in the variation as the 3AW2A as a replacement for the 3AW2 after the 1967 General Electric X-radiation scandal. The 3AW2A comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.22 amps.

3BF2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. This tube is very rare, and very special, because it implements an indirectly heated cathode, not connected to the filament. No data is found on this tube, except for the filament power (which is 3.6 volts, 0.225 amps) and the base (which is the 12GQ type). The only reason why we know it is a high voltage rectifier is that the base tells us so.

3BL2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. It comes in the variation as the 3BL2A as a replacement for the 3BL2 after the 1967 General Electric X-radiation scandal. The 3BL2A comes with X-radiation shielding on the inside. The heater power is 3.3 volts and 0.285 amps.

3BM2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. It comes in the variation as the 3BM2A as a replacement for the 3BM2 after the 1967 General Electric X-radiation scandal. The 3BM2A comes with X-radiation shielding on the inside. The heater power is 3 volts and 0.3 amps.

3BN2 - High Voltage rectifier. A compactron used for color television sets. It comes in the variation as the 3BN2A as a replacement for the 3BN2 after the 1967 General Electric X-radiation scandal. The 3BN2A comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.22 amps.

3BS2 - High Voltage rectifier. A compactron used for color television sets. It comes in the variation as the 3BS2A and 3BS2B as a replacement for the 3BN2 after the 1967 General Electric X-radiation scandal. The 3BS2A and 3BS2B tubes are identical, maybe a small difference in ratings and characteristics. We do not know these differences as the 3BS2B tube data is not available. The 3BS2A and 3BS2B comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

3BT2 - High Voltage rectifier. A compactron used for color television sets. It comes in the variation as the 3BT2A as a replacement for the 3BT2 after the 1967 General Electric X-radiation scandal. The 3BT2A comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

3BW2 - High Voltage rectifier. A compactron used for color and black and white television sets. The 3BW2 comes with X-radiation shielding on the inside. It also comes with diffusion bonded cathode (a type of cathode that prevents the back-emission of the anode). This tube was designed in the December of 1970, after the 1967 General Electric X-radiation scandal. All high voltage rectifier tube types that were designed before 1967 had no X-radiation protection internally. That is why all these tubes made during and after 1967 have a suffix showing they had internal X-radiation protection. This is why there is no '3BW2A' type since it was made after 1967. The heater power is 3.15 volts and 0.22 amps.

Nominally 5 volt heater/filament tubes

5J6 – General purpose RF dual triode with common cathodes, a 6J6 with a 4.7 volt 600 mA controlled warm up heater[33]

5 volt heater/filament full wave rectifier tubes

5AR4, GZ34 – Full wave rectifier

5AS4 – Full wave rectifier

5R4 – Full wave rectifier

5U4 – Full wave rectifier

5V4, GZ32 – Full wave rectifier

5Y3 – Full-wave rectifier, octal base version of type 80

6 volt heater/filament tubes

6A6 – Dual Power Triode, used as a Class-A audio driver or a Class-B audio output. UX6 base. 6.3 volt heater version of type 53 which had a 2.5 volt heater. Octal version – 6N7.

6A7 and 6A8 (PH4, X63) – Superheterodyne Pentagrid converter – dual tetrode style. Based on type 2A7, which had a 2.5 volt heater. 6A7 has a UX7 base with top cap connection for control grid (grid 4). 6A8 is octal version with top cap connection for control grid. Loctal version: type 7B8.

6AB4/EC92 – High-mu triode (Pinout same as 6C4 except for pin 5 not having a connection)

6AB5/6N5 – "Magic Eye" cathode ray tuning indicator

6AC5-G – High-mu Power Triode

6AC7, 1852 – TV sharp-cutoff R.F. Pentode. (Often encountered in a black metal envelope, not to be confused with the 6CA7.)

6AC10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

6AD6-G and 6AF6-G – "Magic Eye" tuning indicators. Both have two "pie wedge" shadow indicators, one each on opposite sides of a single circular indicator target. Both shadows may be used in tandem or may be driven by two different signal sources. Type 6AE6-G is specifically made to drive each indicator with different signals. May also be driven by separate pentodes with different characteristics. E.g., a sharp-cutoff pentode like a 6J7 – which would be hyper-sensitive to any signal change – would drive one shadow, while a remote-cutoff pentode like a 6K7 – which would only react to stronger signals – would drive the other shadow. Both tubes have octal bases. Type 6AD6-G, with a target voltage rated from 100 to 150 volt, is designed for AC/DC radios. Type 6AF6-G, with a target voltage rated at 250 volt, is designed for larger AC radios.

6AE6-G – A driver triode specially designed for "Magic Eye" tuning indicator types 6AD6-G and 6AF6-G. Has a common heater and indirectly heated cathode, two internally connected triode grids – one with sharp-cutoff characteristics, one with remote-cutoff characteristics – and two anodes, one for each grid. The sharp-cutoff grid reacts to any signal change, while the remote-cutoff grid reacts only to stronger signal changes.

6AE7-GT – Dual Triode with a common, single anode, for use as a power triode driver

6AF4 – UHF Medium-mu Triode, commonly found in TV UHF tuners and converters.

6AF11 – Compactron High-mu dual triode and sharp-cutoff pentode

6AG11 – Compactron High-mu dual triode and dual diode

6AH5-G – Beam power tube for early TV use. Same as type 6L6-G, but with scrambled pinout. Used in some Philco receivers.

6AK5, EF95, 5654, CV4010, 6Ж1П – Miniature V.H.F. Sharp-cutoff pentode (Used in old Radiosonde weather balloon transmitters, receiver front ends and contemporary audio equipment) B7G, (Miniature 7-pin) base

6AK6 – Power pentode. 7-pin miniature version of type 6G6-G. Unusual low-power consumption output tube with 150 mA heater.

6AK8/EABC80 – Triple Diode, High-mu Triode. Diodes have identical characteristics – two have cathodes connected to the triode's cathode, one has a separate cathode. Used as a combination AM detector/AVC rectifier/FM ratio detector/A.F. amplifier in AM/FM radios manufactured outside of North America. Triode amplification factor: 70. North American type 6T8 is identical (but for a shorter glass envelope) and may be used as a substitute.

6AK9 – Compactron 1x high-mu + 1x medium-mu dual triode and beam power pentode, 12-pin base

6AK10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

6AL3, EY88 – TV "Damper/Efficiency" Diode

6AL5, EAA91, D77 – Dual Diode, Detector. Often used in vacuum tube volt meters (VTVMs). Miniature version of type 6H6.

6AL6-G – Beam power tube for early TV use. Same as type 6L6-G, but with scrambled pinout and anode connected to top cap.

6AL7-GT – Tuning indicator used in many early AM/FM Hi-Fi radios. Similar in function to "Magic Eye" tubes. Has two bar-shaped shadows; one grows to indicate signal strength, the other moves to indicate center tuning on FM.

6AM6, EF91, Z77 – Sharp-cutoff R.F. pentode used in receiver front ends and test gear such as VTVMs and TV broadcast modulation monitors.

6AN7, ECH80 – Triode-Hexode Local Oscillator/Mixer (radio)

6AN8 – Triode-Pentode used in frame timebase circuits for television. Electrically fairly similar to ECL80 but with a different pinout.

6AQ5 – Beam-power pentode, 7-pin miniature similar of type 6V6.

6AQ8/ECC85 – Dual triode with internal shield. Designed for use as oscillator and mixer in FM receivers. The heater to cathode insulation is inadequate for use in cascode operation

6AR8, 6JH8, 6ME8 – Beam deflection tubes for use as NTSC chroma signal demodulators in analog color TV receivers

6AS6 – Pentode with a fine-pitched suppressor grid which could serve as a second control grid. Used in radar phantastron circuits.

6AS7, 6080 – Dual low-mu Triode, low impedance, mostly used for voltage regulation circuits.

6AS11 – Compactron 1x high-mu + 1x medium-mu dual triode and sharp-cutoff pentode, 12-pin base

6AT6 – Dual Diode, High-mu Triode, miniature version of type 6Q7. Triode amplification factor: 70.

6AU4 – TV "Damper/Efficiency" Diode

6AU6, EF94, 6AU6A – Sharp-cutoff pentode

6AV6 – Dual Diode, High-mu Triode, miniature version of type 75. Triode amplification factor: 100. (Triode section similar in characteristics to one half of a 12AX7.)

6AV11 – Compactron Medium-mu triple triode, 12-pin base

6AX4 – TV "Damper/Efficiency" Diode

6AX5 – Full-wave rectifier. Octal base. Similar in structure to type 6X5, but with higher voltage and current ratings which are comparable to those of types 5Y3 and 80.

6B6-G – Double-Diode High-mu Triode. Octal version of type 75. Has top-cap connection for triode grid. Later octal version, type 6SQ7, has under-chassis connection for triode grid. Miniature version: 6AV6.

6B7 and 6B8 (EBF32) – Double-Diode, Semiremote-cutoff Pentodes. Based on type 2B7 which had a 2.5 volt heater. Type 6B7 has a UX7 base with a top-cap connection for the control grid (grid 1). Type 6B8 has an octal base with a top cap. The diode anodes are most commonly used as (second) detectors and AVC rectification in superheterodyne receivers. Because their control grids have both sharp-cutoff and remote-cutoff characteristics, these types were used as I.F. amplifiers with AVC bias to the control grid, and as A.F. amplifiers. These types were also used in reflex radios. In a typical 2B7/6B7/6B8 reflex circuit, the I.F. signal from the converter is injected into the pentode and is amplified. The diodes then act as detectors, separating the A.F. signal from the R.F. signal. The A.F. signal is then re-injected into the pentode, amplified, and sent to the audio output tube.[34]

6BA6, EF93, W727, 5790 – Semiremote-cutoff R.F. Pentode (Often encountered in car radios)

6BD11 – Compactron 1x high-mu + 1x medium-mu dual triode and sharp-cutoff pentode, 12-pin base

6BE6, EK90, 5750, X727 – Pentagrid Converter (Often encountered in car radios)

6BF6 – Dual Diode, Medium-mu triode. Miniature version of octal type 6R7.

6BF8 – Sextuple diode with a common cathode

6BG6 – Beam tetrode, anode cap. Used in early TV magnetic-deflection horizontal-output stage.

6BH11 – Compactron Medium-mu dual triode and sharp-cutoff pentode

6BK4 – High Voltage beam Triode (30 kV anode voltage). Used as shunt regulator in color TV receivers and measurement equipment such as high voltage meters

6BK7 – Dual Triode with Internal shield between each section, used in RF circuits (Similar to 6BQ7)

6BK8, EF86, Z729 – Audio Pentode used in microphone preamplifiers and audiophile equipment

6BK11 – Compactron 2x High-mu + 1x medium-mu triple triode preamplifier, 12-pin base; used in some guitar amps made by Ampeg.

6BL6 (5836) – Sutton tube, a reflex klystron used as a 250 mW CW microwave source, 1.6 to 6.5 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

6BL8, ECF80 – General-purpose Triode pentode used in TV, audio and test gear

6BM6 (5837) – Sutton tube used as a 150 mW CW microwave source, 550 MHz to 3.8 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

6BM8, ECL82 – Triode pentode used as the driver and output stages in audio amplifiers, audio output and vertical output stages in TV receivers and has even been seen in an electronic nerve stimulator.

6BN6 – Gated-beam discriminator pentode, used in radar, dual channel oscilloscopes and F.M. quadrature detectors (cf. 6DT6, nonode).

6BQ5, EL84,(N709) – 5.7 Watts AF Power pentode, noval base

6BQ6-GT – Beam Power Pentode, used as a Horizontal Deflection Output tube in monochrome TV receivers of the 1950s. Most commonly used in receivers with diagonal screen sizes less than 19 inches (48 cm). (However, may be found in some larger models.) Larger receivers often used similar type 6DQ6. Later versions of this tube branded as 6BQ6-GTB/6CU6.

6BQ7 – Dual RF/VHF triode with internal screen. The two sections can be used independently or in a cascode stage

6BQ7A – Improved 6BQ7 capable of operation at UHF frequencies

6BU8 – Split Anode TV Sync Separator

6BX6, EF80 – Sharp-cutoff RF/IF/Video pentode, noval base

6BY6 – Similar to type 6CS6, but with higher transconductance. 3BY6 with a different heater

6BY7, EF85, W719 – Remote-cutoff R.F. Pentode (TV IF)

6BZ6 – Sharp-cutoff R.F. pentode used in video I.F. circuits of the 1960s.

6BZ7 – Dual Triode. See 6BK7

6C4/EC90 – 3.6 W small-power V.H.F. triode up to 150 MHz; single 12AU7/ECC82 system

6C6 – Sharp-cutoff R.F. Pentode. Most common commercial uses were as a tuned R.F. amplifier, a detector, and an A.F. amplifier. Also used in test equipment. Has UX6 base with top cap. Based on type 57, which had a 2.5 volt heater. Similar to types 1603, 77 and octal types 6J7 and 6SJ7

6C10 – Compactron High-mu triple triode, 12-pin base – not related to the Mazda/EdiSwan 6C10 triode-hexode

6CA4, EZ81 – Full Wave Rectifier

6CA7, EL34 – Audio Power Output Pentode

6CA11 – Compactron High-mu dual triode and sharp-cutoff pentode

6CB6 – Remote-cutoff R.F. Pentode used in video I.F. circuits of the 1950s and early 1960s.

6CG7 – Dual Triode (used in TV and some audio amplifiers including modern solid-state designs often as a cathode follower, similar to 6SN7)

6CJ6 – Line Output Pentode

6CL6 – Power pentode

6CM5, EL36, EL360 – Audio and TV Line Output Beam Power Tetrode.

6CW4 – Nuvistor high-mu VHF triode, most common one in consumer electronics

6CZ5 - Beam pentode for use in vertical deflection or audio amplifier. In certain applications, it can be used in place of a 6973.

6D4 – 25 mAavg, 100 mApeak Indirectly heated, argon triode thyratron, negative starter voltage, miniature 7-pin base; found an additional use as a 0 to 10 MHz noise source, when operated as a diode (starter tied to cathode) in a transverse 375 G magnetic field. Sufficiently filtered for "flatness" ("white noise") in a band of interest, such noise was used for testing radio receivers, servo systems and occasionally in analog computing as a random value source.

6D6 – Remote-Cutoff R.F. Pentode. Most common commercial uses were as an I.F. amplifier or as a superheterodyne mixer, aka 1st detector. Also used in test equipment. Has UX6 base with top cap. Based on type 58, which had a 2.5 volt heater. Similar to type 78. Octal version: 6U7-G.

6D8-G – Superheterodyne Pentagrid converter, similar to type 6A8. Octal base with top cap. Has 150 mA heater. Used in pre-war 6-volt farm radios.

6D10 – High-mu triple triode for use as oscillator, mixer, amplifier or AGC tube, 12-pin base

6DA6, EF89 – R.F. Pentode used in AM/FM radios manufactured outside North America.

6DJ8, ECC88, E88CC, 6922, 6N23P, 6N11 – Dual Audio and R.F. Triode (often used in TV broadcast equipment, test gear, oscilloscopes and audiophile gear) similar to 6ES8

6DQ6 – Beam Power Pentode, used as a Horizontal Deflection Output tube in monochrome TV receivers of the 1950s. Most often found in receivers with diagonal screen measurements larger than 17 inches (43 cm). Smaller receivers often used similar type 6BQ6-GT. Also used as Audio Output tubes in Standel guitar amplifiers. Later versions branded as 6DQ6-B/6GW6.

6DR8, EBF83 – R.F. pentode which will operate with 12 V anode supply, used as I.F. amplifier in car radios which run directly off the 13.5 volt supply.

6DS4 – Nuvistor R.F. triode used in TV tuners immediately prior to the introduction of solid state tuning circuits. (RCA TVs equipped with a 6DS4 tuner bore the trademark "Nu-Vista Vision"); successor of the 6CW4.

6DS8, ECH83 – Triode-heptode Local oscillator-Mixer which will operate with 12 V anode supply, used in car radios which run directly off the 13.5 volt supply.

6DT6 – Quadrature detector used in TV audio circuits of the 1950s and early 1960s; cf. 6BN6, nonode.

6DV4 – Medium-mu Nuvistor triode for UHF oscillators; some versions had a gold-plated envelope

6DX8 – Triode pentode

6E5 – "Magic Eye" Tuning indicator. Has incorporated driver triode with sharp-cutoff grid which makes it extremely sensitive to any changes in signal strength. Has UX6 base. Based on type 2E5, which had a 2.5 volt heater.

6EM5 – TV Vertical Output Pentode

6ES6, EF98 – R.F. pentode which will operate with 12 V anode supply, used as tuned R.F. amplifier in car radios which run directly off the 13.5 volt supply.

6ES8, ECC89, E89CC – Dual Triode used as cascode R.F. amplifier in TV tuners and V.H.F. receiver front ends, also used as general-purpose dual triode in test gear, similar to 6DJ8

6EZ8 – High-mu triple triode, 9-pin base

6F4 – Acorn UHF triode up to 1.2 GHz, for use as an oscillator

6F5 – High-mu triode, equal to triode section of type 6Q7

6F6, KT63 – Power Pentode. Octal base version of type 42. Moderate power output rating – 9 watts max. (Single-ended Class-A circuit); 11 watts max. (Push-pull Class-A circuit); 19 watts max. (push-pull Class-AB2 circuit). Available in metal (numbered "6F6"), shouldered glass ("6F6-G"), and cylindrical glass ("6F6-GT"). Sometimes used as a transformer-coupled audio driver for types 6L6-GC and 807 when those tubes were used in Class-AB2 or Class-B amplifiers. Also used as a Class-C oscillator/amplifier in transmitters.

6F7 – Remote-cutoff Pentode, Medium-mu Triode. Has UX7 base with top-cap connection for the pentode's control grid (grid 1). Most common uses were as superheterodyne mixer ("first detector") and local oscillator, or as a combination I.F. amplifier (pentode) and (second) detector or A.F. amplifier (triode). Octal version: 6P7-G.

6FH8 – Medium-mu triode and three-anode sharp-cutoff tetrode for use in TV receivers and complex wave generators

6G5 – "Magic Eye" Tuning indicator. Has incorporated triode with remote-cutoff grid, which makes it less reactive to low-level changes in signal strength. Has UX6 base. Electronically identical to type 6U5 except for indicator. Both types had "pie wedge" shadow indicators. At first, the shadow indicator for type 6G5 was fully closed at zero signal and opened as signal strength increased. For type 6U5, the shadow indicator was fully open at zero signal and closed as signal strength increased. After World War II, type 6G5 was discontinued as a unique tube and all 6U5s were doubled branded either as 6G5/6U5 or 6U5/6G5.

6G6-G – Power pentode. Octal base. Low power output – 1.1 watt max. output. Has 150 mA heater. Used in pre-war 6-volt farm radios. Miniature version – 6AK6.

6G8-G – Double-Diode Sharp-cutoff Pentode (Used as Detector and first A.F. stage in Australian 1940s radios)

6GK5 – Miniature V.H.F. triode (Used as V.H.F. local oscillator in some T.V. Turret Tuners)

6GM5 – Beam power pentode, identical to 7591 and 7868 with a mini-noval pin base

6GV8, ECL85 – Triode Pentode (TV vertical output)

6GW8, ECL86 – Audio Triode Pentode (audio, TV vertical output)

6GY8 – High-mu triple triode for use as oscillator, mixer, RF amplifier or AGC tube, 9-pin base

6H6, D63, EB34, OSW3109 – Dual diode. Octal base. Most commonly found as a "stubby" metal envelope tube. Glass versions 6H6-G and 6H6-GT are also found.

6HS8 – Dual-anode pentode for TV receiver sync separation service or as a two-channel VCA

6J5, L63 – Medium-mu triode.

6J5WGT – Heater cathode type, medium-mu triode, identical to 12J5WGT except heater characteristics

6J6 – Dual general purpose RF triode with common cathodes, operates over much of the UHF band (up to 600 MHz), equivalent to ECC91

6J7, EF37 – Sharp-cutoff Pentode. Most common commercial uses were as a tuned R.F. amplifier, a (second) detector, or an A.F. amplifier. Octal version of type 77. This type included a top-cap connection for the control grid. Later version, type 6SJ7, had its control grid connection on pin 4.

6J8-G – Triode-Heptode (radio local oscillator/mixer)

6JU8A – 9 mA, Four-diode bridge rectifier

6K6-G – Power Pentode, octal version of type 41. Low-to-moderate power output rating – 0.35 to 4.5 watts (single-ended Class-A circuit); 10.5 watts max. (push-pull Class-A circuit).

6K7, EF39 – Remote-cutoff R.F. pentode. Most common commercial uses were as an I.F. amplifier or as a superheterodyne mixer, aka 1st detector. Also used in test equipment. Octal version of type 78. This type included a top-cap connection for the control grid. Later version, type 6SK7, had its control grid connection on pin 4.

6K8 and 12K8 – American Triode-Hexode mixer, 1938

6K11 – Compactron 2x High-mu + 1x medium-mu triple triode, 12-pin base

6KM8 – Diode and three-anode sharp-cutoff tetrode for use in musical instruments, frequency dividers and complex wave generators

6L4 – Acorn UHF triode for use as an oscillator

6L5-G – Medium-mu triode (Similar to type 6J5-G, available only in ST shape)

6L6/EL37 – High-powered beam tetrode.

There are several variations. Except for types 6L6-GC and 6L6-GX, all have the same maximum output ratings:

11.5 watts (single-ended Class-A circuit)

14.5 watts (push-pull Class-A circuit)

34 watts (push-pull Class-AB1 circuit)

60 watts (push-pull Class-AB2 circuit)

6L6 (metal envelope) and 6L6-G (shouldered glass envelope) were used in pre-World War II radios and Public Address amplifiers.

6L6 and 25L6 were introduced in 1935 as the first beam tetrodes. Both types were branded with the L6 ending to signify their (then) uniqueness among audio output tubes. However, this is the only similarity between the two tubes. (Type 6W6-GT is the 6.3 volt heater version of types 25L6-GT and 50L6-GT.)

6L6GA – Post-war version of type 6L6-G, in smaller ST-14 shape with Shouldered Tubular, (ST), shaped bulb, revision A.

6L6GB – Post-war improved version in a cylindrical glass envelope. Similar to type 5881.

6L6GTB – 6L6 with Tubular, (T), shaped bulb, revision B, (higher power rating, as it happens. The 6L6GTB can always replace the 6L6, 6L6G, and 6L6GT, but a 6L6GTB running at maximum rating should not be replaced with another subtype).

6L6-WGB – "Industrial" version of type 6L6GB.

6L6GC – Final and highest-powered audio version of the tube. Max. outputs:

17.5 watts (single-ended Class-A circuit)

32 watts (push-pull Class-A circuit)

55 watts (push-pull Class-AB1 circuit)

60 watts (push-pull Class-AB2 circuit)

6L6-GX – Class-C oscillator/amplifier used in transmitters. Max. output 30 watts. (All versions may be used as a Class-C oscillator/amplifier, but this version is specifically designed for this purpose, has a special ceramic base.)

6L7 – Pentagrid converter often used in console radios of the late 1930s. Similar in structure to pentode-triode pentagrid converters 6SA7 and 6BE6, except that a separate oscillator – usually type 6C5 – is required. Also, grid 1 is remote-cutoff control grid, grid 3 is oscillator input grid. (In types 6SA7 and 6BE6, grid 1 is the internal oscillator grid, grid 3 is the control grid.) Because of low conversion transconductance, radios using type 6L7 typically have either a tuned RF pre-amplifier stage, or at least two stages of I.F. amplification. (A few models have both.)

6LF6 – Beam power tetrode with a duodecar Compactron base and anode cap, for CRT horizontal-deflection amplifiers

6M5 – Audio Output Pentode (Used as Class-A or C output stages of 1950s Australian radiograms) similar to 6BQ5

6M11 – Compactron Dual triode and pentode

6MD8 – Medium-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 9-pin base

6ME5 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

6MK8 – Dual-anode pentode for TV receiver sync separation service or as a two-channel VCA

6MJ8 – Medium-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

6MN8 – High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

6N3, EY82 – Half-Wave Rectifier

6N5/6N5P – Tuning indicator

6N7 – Dual Power Triode, used as Class-A audio driver or as Class-B power output (also 6N7-G and 6N7-GT). Max. output (Class-B) – 10 watts. Octal version of type 6A6.

6N8, EBF80 – Remote-cutoff pentode, dual diode. (detector plus RF or AF amplifier in radios)

6P5-G/GT – Medium-mu triode, Octal version of type 76, often used as driver for type 6AC5-G.

6P7-G – Rarely seen octal version of type 6F7.

6Q5-G – Triode gas thyratron used in DuMont oscilloscopes as a sweep generator. Identical to type 884.

6Q11 – Medium-mu triple triode, 12-pin base, for use as a sync clipper and gated AGC amplifier in TV receivers

6R3, EY81 – TV "Damper/Efficiency" Diode

6R7 – Dual Diode, Medium-mu Triode (also 6R7-G and 6R7-GT). Octal base with top cap. Miniature version – 6BF6. Amplification factor: 16.

6S7-G – Remote-cutoff RF Pentode, similar to type 6K7. Octal base with top cap. Has 150 ms heater. Used in pre-war 6-volt farm radios.

6S8-GT – Triple Diode, High-mu Triode. Octal tube with top-cap connection to triode grid. Has three identical diodes – two diodes share a cathode with the triode, one has a separate cathode. Used as a combined AM detector/AVC rectifier/FM ratio detector/A.F. amplifier in AM/FM radios. Typically, all sections of this tube are arranged around a single heater.

6SA7 – First pentode-triode style pentagrid converter. Octal type. Miniature version: 6BE6.

6SB7Y (octal), 6BA7 and 12BA7 (9-pin miniatures) – VHF pentagrids, 1946

6SC7 – High-mu dual triode (Both sections share a single cathode)

6SK7 – Remote-cutoff pentode (Used in I.F. stages of North American radios) Miniature version: 6BD6

6SL7, ECC35 – Dual triode (Used in TV and general electronics)

6SN7, ECC32, B65, 13D2, CV1986, 6042? – Medium-mu dual triode (Used in Audio Amplifiers, Hammond Organs and Television; extensive use in World War II radar) Each section is equivalent to a 6J5. Miniature version: 12AU7

6SS7 – Remote-cutoff pentode (150 mA filament version of the 6SK7, found in some AA6 radios as both the RF amplifier and first IF). This is the only tube to have two of the same letters in its type.

6T5 – "Magic Eye" Tuning indicator. Has incorporated driver triode with remote-cutoff grid. Has UX6 base. Shadow indicator is fully closed at zero signal. As signal increases, shadow grows outward from the center, covering the entire circumference of the indicator. Electronically identical to types 6G5 and 6U5, which may be used as substitutes.

6T7-G – Dual diode, high-mu triode, similar to type 6Q7. Octal base with top cap. Has 150 mA heater. Used in pre-war farm radios.

6T8 – Triple Diode, High-mu Triode. Has three identical diodes – two have cathodes connected to the triode's cathode, one has a separate cathode. Triode amplification factor: 70. Used as an AM detector/AVC rectifier/FM ratio detector/A.F. amplifier in North American AM/FM radios. Identical to type 6AK8/EABC80, but with a shorter glass envelope.

6U5 – "Magic Eye" Tuning indicator. Has incorporated driver triode with remote-cutoff grid. Has UX6 base. Has "pie wedge" shadow indicator that is open at zero signal and closes as signal increases. Electronically identical to types 6G5 and 6T5 and may be used as a substitute for those types. After World War II, most new 6U5s were double-branded as either 6G5/6U5 or 6U5/6G5.

6U5G – "Magic Eye" Tuning indicator with triode, International Octal, (IO), base

6U7-G – Remote-cutoff R.F. Pentode. Most common commercial uses were as an I.F. amplifier or as a superheterodyne mixer, aka 1st detector. Also used in test equipment. Octal version of type 6D6. Most direct substitute: 6K7. Similar to types 58, 78 and 6SK7.

6U8A – Split triode-pentode, mini-noval pinbase. Audio preamplifier.

6U10 – 1x High-mu + 2x medium-mu triple triode, 12-pin base

6V4 (EZ80) - Noval-base, indirectly-heated, full-wave rectifier. EZ80 rated at 90mA, but 6V4 only rated for 70. Some brands were identical.

6V6 – Beam power tetrode, used in single-ended Class-A audio output stages of radios and sometimes seen in Class-B audio amplifiers (see also: 5V6 and 12V6). Electrically similar to 6AQ5/EL90.

6V6G – 6V6 with Shouldered Tubular, (ST), shaped bulb.

6V6GT – 6V6 with Tubular, (T), shaped bulb.

6V7-G – Dual diode, Medium-mu Triode. Octal version of type 85. Amplification factor: 8.3. Similar to type 6R7.

6W6-GT – Beam power pentode, used most often as a Vertical Deflection Output tube in monochrome TV receivers of the 1950s. Can also used as an Audio Output tube. This is the 6.3 volt heater version of types 25L6-GT and 50L6-GT.

6X4 (EZ90) and 6X5 (EZ35) – Full-wave rectifiers with indirectly heated common cathode. Type 6X4 has a 7-pin miniature base, the 6X5 has an octal base. Based on type 84/6Z4. No longer in production.

"7" prefix loctal tubes

These tubes all have 6.3 volt AC/DC heaters.

7A4 – Medium-mu triode, loctal version of type 6J5, often numbered 7A4/XXL

7A5 – Beam power pentode, loctal version of type 6U6GT

7A6 – Dual detector diode, similar to type 6H6

7A7 – Remote-cutoff pentode, loctal version of type 6SK7

7A8 – The only octode pentagrid converter produced in America by Sylvania, 1939. Used mostly in Philco radios.

7AB7 – Sharp-cutoff pentode

7AD7 – Power pentode

7AF7 – Dual medium-mu triode

7AG7 – Sharp-cutoff pentode

7AH7 – Remote-cutoff pentode

7AJ7 – Sharp-cutoff pentode

7AK7 – Sharp-cutoff, dual control pentode for computer service. Perhaps the first active device specifically designed for computer use.

7B4 – High-mu triode, loctal version of types 6F5 and 6SF5

7B5 – Power pentode, loctal version of types 6K6 and 41

7B6 – High-mu triode, dual detector diodes, loctal version of type 75, similar to types 6AV6 and 6SQ7

7B7 – Remote-cutoff pentode

7B8 – Pentagrid converter, loctal version of types 6A7 and 6A8

7C4 – High frequency diode

7C5 – Beam power pentode, loctal version of type 6V6

7C6 – High-mu triode, dual detector diode

7C7 – Sharp-cutoff pentode

7E5 – Medium-mu high-frequency triode

7E6 – Medium-mu triode, dual detector diode, loctal version of types 6R7 and 6SR7, electronically identical to miniature type 6BF6.

7E7 – Semiremote-cutoff pentode, dual detector diode, similar to types 6B7 and 6B8

7F7 – High-mu dual triode, loctal version of type 6SL7-GT

7F8 – Medium-mu high-frequency triode, used as FM RF amplifier and converter

7G7 – Sharp-cutoff pentode

7G8 – Sharp-cutoff dual tetrode

7H7 – Semiremote-cutoff pentode

7J7 – Triode-heptode converter, similar to type 6J8-G

7K7 – High-mu triode, dual detector diode, similar to types 6AT6 and 6Q7

7L7 – Sharp-cutoff pentode

7N7 – Dual medium-mu triode, loctal version of type 6SN7-GT

7Q7 – Pentagrid converter, similar to type 6SA7

7R7 – Remote-cutoff pentode, dual detector diode

7S7 – Triode-heptode converter

7T7 – Sharp-cutoff pentode

7V7 – Sharp-cutoff pentode

7W7 – Sharp-cutoff pentode

Note: Types 7V7 and 7W7 are electronically identical except for base connections of pins 4, 5 and 7. On type 7V7, the suppressor grid (grid 3) is connected to pin 4, an internal shield is connected to pin 5, and the cathode is connected to pin 7. On type 7W7, the suppressor grid and internal shield are connected to pin 5, and the cathode is connected to pins 4 and 7. All other pin connections are the same. If interchanging these tube types is necessary, confirm that pins 4 and 7 are connected at the socket. (Pin 5 is usually connected to the chassis.)

7X6 – Dual rectifier diode

7X7 – High-mu triode, dual detector diodes on separate cathodes, used as FM discriminator and AF amplifier, often numbered 7X7/XXFM

7Y4 – Dual rectifier diode

7Z4 – Dual rectifier diode

12 volt heater/filament tubes

12A5 – Power pentode. UX7 base. Single-section tube with two side-by-side 6.3 volt heater-cathode structures. Each is connected together to form (1) a 12.6 volt 300 mA heater for series heater strings, or (2) a 6.3 volt 600 mA heater for parallel heater circuits. Mostly used in pre-war car radios.

12A7 – Power pentode, rectifier diode. Pentode section is similar to type 38. Diode has a low power rating – 120 volt, 30 mA – that limits the number of tubes that can be tied to its B+ circuit. Used in one-tube portable phonographs and a few two- and three-tube radios. Forerunner of such types as 32L7-GT, 70L7-GT and 117L7-GT. UX7 base with top cap. Not related to types 2A7 and 6A7.

12AB5 – Beam Power Tube[35]

12AC10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

12AE10 – Compactron Beam power tube and sharp-cutoff pentode[35]

12AL5 – Dual diode (similar to 6AL5 except for heater)

12AT6 – Dual diode/triode (Commonly replaced by 12AV6 in consumer radios)

12AT7, ECC81, 6060, B309, M8162 – High-mu dual triode. Commonly used as R.F. amplifier/mixer in VHF circuits.[35]

12AU7, ECC82, 6067, B329, M8136 – Medium-mu dual triode.[35] Two 6C4/EC90s in one envelope;[36] however, it is only specified as an audio frequency device. Commonly used in audio applications and TV receivers.

12AV6 – Dual diode/High-mu triode (see also: 6AV6)

12AV7, 5965 – Medium-mu dual triode. Principally designed for VHF amplifier/mixer operation.[37]

12AX7, ECC83, 6057, B327, M8137 – High-mu dual triode. Very similar to triode section of 6AV6. Commonly used in high-gain audio stages.[35]

12AY7 – Dual Triode. Medium gain but low noise, intended for low-level/preamplifier use.[35]

12AZ7 – Double Triode. Medium-mu, AF Amplifier, or combined oscillator and mixer, 9-pin.[38][35]

12B4A – Low-mu triode, noval base.[35]

12BA6 – Remote-cutoff pentode, 6BA6/EF93 with a different heater

12BE6 – Pentagrid converter, 6BE6/EK90 with a different heater

12BH7 – Dual Triode, Medium-mu, designed for use in equipment having series heater-string arrangement.[39][35]

12BY7 – Video Amplifier Pentode

12DT6 – Sharp-cutoff pentode

12DW7 – Dissimilar triodes. One half 12AX7 value, other half 12AU7 value. (also known as 7247 or ECC832)

12EG6 – Pentagrid converter, both grids 1 and 3 are sharp-cutoff, has 12.6 volt anode and screen grid voltage, for use with audio equipment powered by a car battery

12FA6 – Low-anode voltage, car radio version of 12BE6 pentagrid converter

12FQ8 – Common-cathode, dual split-anode triode for use in musical instruments, frequency dividers and complex wave generators[40]

12FX8 – Low-anode voltage, triode-heptode converter for car radios

12GA6 – Similar to type 12FA6, but with lower conversion transconductance

12J5WGT – Heater cathode type, medium-mu triode, identical to 6J5WGT except heater characteristics

12K5 – Tetrode with a space charge grid, one of a few tubes that can function with low anode voltages

12MD8 – Medium-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 9-pin base

12MN8 – High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

12SA7 – Pentagrid converter (Octal version of 12BE6)

12SK7 – Remote-cutoff Pentode (Octal version of 12BA6)

12SQ7 – Dual diode, triode (Octal version of 12AV6)

12U5G – Tuning indicator identical to 6U5G except heater characteristics

12Z3 – Half-wave rectifier, UX4 base

"14" prefix loctal tubes

These tubes all have 12.6 volt AC/DC heaters

14A4 – Medium-mu triode, loctal version of type 12J5

14A5 – Beam power pentode

14A7 – Remote-cutoff pentode, often numbered 14A7/12B7

14AF7 – Dual medium-mu triodes, often numbered 14AF7/XXD

14B6 – High-mu triode, dual detector diode, similar to types 12AV6 and 12SQ7

14B8 – Pentagrid converter, loctal version of type 12A8

14C5 – Beam power pentode, loctal version of type 12V6-GT

14C7 – Sharp-cutoff pentode

14E6 – Medium-mu triode, dual detector diode, loctal version of 12SR7

14E7 – Semiremote-cutoff pentode, dual detector diode, similar to type 12C8

14F7 – High-mu dual triode, loctal version of type 12SL7-GT

14F8 – Medium-mu high frequency triode, used as FM RF amplifier and converter

14H7 – Semiremote-cutoff pentode

14J7 – Triode-heptode converter

14N7 – Dual medium-mu triode, loctal version of type 12SN7-GT

14Q7 – Pentagrid converter, similar to type 12SA7

14R7 – Remote-cutoff pentode, dual detector diode

14S7 – Triode-heptode converter

14W7 – Sharp-cutoff pentode

14X7 – High-mu triode, dual detector diodes on separate cathodes, used as FM discriminator and AF amplifier

14Y4 – Dual rectifier diode

25 volt heater/filament tubes

25A6 – Power pentode, octal version of type 43

25C5 – Beam Power Pentode (Identical to the 50C5 but with a 25 V 300 mA filament)

25F5 – Beam Power Pentode (Identical to the 50C5, but with a 25 V 150 mA filament, used in some AA5 type radios using push-pull output)

25L6 – Beam-power pentode (Except for heater, electrically identical to type 50L6)

25Z5 – Dual rectifier diode

25Z6 – Octal version of 25Z5

35 volt heater/filament tubes

35A5 – Beam Power Tube (Loctal, Similar to 35L6)

35B5 – Beam power tube

35C5 – Identical to 35B5 except for basing ("pin-out") arrangement (HL92)

35HB8 – Triode/Beam Power tube (Used primarily as both the audio amplifier and output)

35DZ8 – High-mu Triode/Beam Power tube (Like the 35HB8, used for audio)

35L6-GT – Beam power pentode similar to, but not electronically identical to, types 25L6-GT and 50L6-GT

35W4 – Rectifier diode

35Z4-GT – Rectifier diode

35Z5-GT – Similar to 35Z4-GT, but equipped with a heater tap used to power a pilot light

35Z3 – Rectifier Diode (Loctal, Similar to 35Z4)

35Y4 – Rectifier Diode (Loctal, similar to 35Z5)

50 volt heater/filament tubes

50B5 – Beam power tube, similar to 35B5 but with 50 volt filament

50C5 – Similar to 35C5 but with 50 volt filament, and 50B5 except for basing ("pin-out") arrangement (HL92)

50EH5 – Beam Power tube, (Similar to 50C5 but with higher gain, some radios that use this tube do not have an audio amplifier section.)

50L6 – Beam power tube (see also 25L6)

50HK6 – Power pentode (Filament is tapped for use with a dial lamp)

50A5 – Beam Power Tube (Loctal, similar to 50L6)

50X6 – Dual Diode (Loctal, commonly used as a rectifier-doubler)

50DC4 – Rectifier diode (Similar to 35W4 except for filament)

117 volt heater tubes

All of the following tubes are designed to operate with their heaters connected directly to the 117 volt (now 120 volt) electrical mains of North America. All of them use indirectly heated cathodes. All of them incorporate at least one rectifier diode.

Rectifier diode – Beam power pentode combinations

117L7GT

117M7GT

117N7GT

117P7GT

Rectifier tubes

117Z3 – Single diode, 7-pin miniature version of 117Z4GT

117Z4GT

117Z6GT – Dual diode, can be used as a voltage doubler

Other tubes with nonstandard filament voltages

The tubes in this list are most commonly used in series-wired circuits.

4CB6 – Sharp-cutoff pentode

7AU7 – Medium-mu Dual triode with a tapped filament like its more common brother, the 12AU7. 7.0/3.5 V

7KY6 – Sharp-cutoff frame-grid pentode with a 7.3 volt nominal heater voltage for use as video output tube in TV receivers, 9-pin miniature socket[41]

8AC10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

8B10 – Compactron Dual triode and dual diode

8FQ7/8CG7 – Dual triode (8 V version of the common 6CG7)

10DE7 – Dual triode (dissimilar triode sections)

11DS5 – Beam Power tube (11 V filament version of the 50B5/35B5)

13CW4 – Nuvistor used as a preamplifier in Neumann condenser microphones U-47 and U-48 after the production of the VF14 ceased

18FX6 – Pentagrid converter (18 V version of the 12BE6)

18FY6 – Dual diode/triode (18 V version of the 12AV6)

34GD5 – Beam power tube (34 V version of the 35C5/50C5)

36AM3 – Half-wave rectifier (36 V version of the 35W4)

38HE7 – Compactron Diode and beam power tube

38HK7 – Compactron Diode and beam power tube

List of EIA professional tubes

Note: Most of these are special quality versions of the equivalents given. Some manufacturers preceded the EIA number with a manufacturer's code, as explained above.

4000s

4042 – Ceramic/metal pencil-type disk-seal UHF power triode for pulsed operation up to 425 W

4062A – Ceramic/metal pencil-type disk-seal SHF power triode up to 4 GHz, mu = 100, Panode = 10 W

4065 – Directly heated electrometer triode, grid current ≤125 fA, 4-pin all-glass pigtail, for probe amplifiers

4205 – Directly heated power triode, 4-pin bayonet base with offset pin

4270A (3C/350E) – Directly heated power triode, 4-pin base

4275 – Directly heated power triode, 4-pin base

4300 – Directly heated power triode, 4-pin base

4307A – Power pentode similar to the output beam-tetrode type 807. It differs from an 807 by being a directly heated pentode rather than an indirectly heated beam-tetrode. Both types are contained in an ST-16 bulb with an anode cap and 5-pin "American" UY base

The SY4307A made by Standard Telephones and Cables/Brimar is historically notable because a pair of them in parallel Class-C was used as the output stage in a transmitter built in secret by Australian soldiers in Japanese-occupied Portuguese Timor during World War II in 1942. This transmitter, now reconstructed and on display at the Australian War Memorial in Canberra, was called "Winnie the War Winner".[42]

4307AF – 4307A qualified for use in standard aircraft radio

4560 – Character generator monoscope for text mode video rendering in early computer monitors, with a square target having letters, digits and symbols stenciled into it in a customer-supplied 8x8 array. An electron beam selects and scans a character, both by appropriate electrostatic deflection, and generates an analog video signal;[43] cf. CK1414, TH9503

4598, 7539, 7828, 8087, 8098 – Graphechon dual-electron gun scan conversion tubes, analog video transcoders with simultaneous R/W capability for realtime resolution and frame rate transcoding between different analog video standards. This was achieved by a CRT/camera tube combination; the CRT part writes onto a thin, dielectric target; the camera part reads the generated charge pattern at a different scan rate from the back side of this target.[44][45][46][47][48] The setup could also be used as a genlock

4613 – Directly heated power triode, 4-pin base

4614 – Indirectly heated power triode, 5-pin base

4641 – Directly heated power triode, 4-pin base

4671/E1C (955) – Indirectly heated Acorn triode

4672/E1F (954) – Indirectly heated Acorn pentode

4674 – Indirectly heated Acorn diode

4675 – 4671/E1C with a 4 Volts heater

4676 – 4672/E1F with a 4 Volts heater

4678 (EM1) – Indirectly heated tuning indicator

4683 – Directly heated power triode, 8-pin base

4695/E2F (956) – Indirectly heated Acorn pentode

5000s

5651

5331, 5332, 5514 – Directly heated power triodes, 4-pin base with anode top cap

5556 – Directly heated power triode, 4-pin base

5593 – Phasitron, a magnetically controlled beam-deflection phase modulator tube[49] similar to the 2H21, used in early FM broadcast transmitters[3][4][5]

5608 – Double power triode, designed for use with AC anode voltage and critical grid leak requirements

5651 – 86-volts, cold-cathode, glow-discharge voltage reference, 7-pin miniature base

5678 (CK5678 Raytheon) – 5 leads subminiature shielded pentode for RF applications

5691 – Special Red ruggedized long-life high-mu triode for industrial applications

5692 – Special Red ruggedized long-life medium-mu triode for industrial applications

5693 – Special Red ruggedized long-life sharp-cutoff pentode for industrial applications

5654, 6AK5, EF95, CV4010, 62H1P, 408A – VHF pentode; common in vintage radar IF amplifiers.

5704 – Subminiature diode, all-glass pigtailed

5727 – 650 V, 100 mAavg, 500 mApeak, 10 Asurge Indirectly heated tetrode thyratron, positive starter voltage, miniature 7-pin base

5729 – Beam-deflection, 30-channel analog multiplexer for telecomms transmitting channel banks, internal electrostatic focusing and deflection to determine through which one out of 30 grids the electron beam passes to the common anode.[50] Cf. 5738, 6090, 6091, 6170, 6324

5731 – Narrow-tolerance selected 955 Acorn triode for use in Radiosonde weather balloon transmitters

5734 – Mechano-electronic displacement sensor; a vacuum triode with its anode mounted on a shaft that extends through a thin, flexible metal diaphragm; shaft movement is reflected in anode current; Fres = 12 kHz[51][52]

5738 – Beam-deflection, secondary emission, 25-channel analog multiplexer, internal electrostatic focusing and deflection to determine which one out of 25 individually controllable dynodes receives the electron beam controlled by a common grid.[53] Cf. 5729, 6090, 6091, 6170, 6324

5749, 6BA6, EF93, W727 – RF pentode

5750, 6BE6, EK90, X727 – Heptode mixer

5751 – Low-voltage, low-noise avionics dual triode with separate cathodes

5814A – Industrial, computer-rated version of 12AU7/ECC82

5836, 6BL6

Raytheon RK5836

– Sutton tube, a reflex klystron used as a 250 mW CW microwave source, 1.6 to 6.5 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

5837, 6BM6 – Sutton tube used as a 150 mW CW microwave source, 550 MHz to 3.8 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

5845 – Dual directly heated saturated-emission diode. Acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits.

5876A – Glass pencil-type disk-seal UHF power triode up to 2 GHz

5930 – Ruggedized, directly heated power triode, 4-pin base

5962 – 700 V/2...55 µA Corona voltage reference, miniature 7-pin base with anode top cap

5963, 5964, 5965 – Dual triode, designed for high speed digital computers, has a high zero-bias anode current, identical to 12AV7

5998, 6336A, 6394, 6520, 6528, 7802 – Double power triodes, designed for series voltage regulator applications

6000s

6047 – Additron, a triple-control grid, split-anode tetrode for use as a single-bit digital full adder[54] (technically a hexode)

6057, 12AX7, ECC83, M8137, B339 – High-mu double triode

6059, 6BR7 – Low-microphonics pentode

6060, 12AT7, ECC81, M8162, B309 – High-mu double triode

6064, 6AM6, EF91, M8083, Z77 – R.F. pentode

6067, 12AU7, ECC82, M8136, B329 – Medium-mu double triode

6080, 6AS7 – Very-low impedance double power triode, designed for series voltage regulator applications, now popular for output transformerless audio amplifiers

6082 – Ruggedized, indirectly heated power triode, octal base

6090 – Beam-deflection, 18-channel analog demultiplexer for telecomms receiving channel banks, internal electrostatic focusing and deflection to determine which one out of 18 anodes receives the electron beam controlled by a common grid.[55] Cf. 5729, 5738, 6091, 6170, 6324

6091 – Beam-deflection, 25-channel analog multiplexer for telecomms transmitting channel banks, internal electrostatic focusing and deflection to determine through which one out of 25 grids the electron beam passes to the common anode.[56] Cf. 5729, 5738, 6090, 6170, 6324

6146 – 60 MHz, 120 W Power AF/RF/VHF beam pentode

6146B (8298A) – Improved version of 6146, 6146A and 8298.

6170 and 6324 – Beam-deflection, 25-channel analog multiplexer for telecomms transmitting channel banks, external focusing and deflection by a multiphase, rotating magnetic field to determine through which one out of 25 grids the electron beam passes to the common anode.[57] Cf. 5729, 5738, 6090, 6091

6173 – Pencil-type disk-seal UHF diode up to 3.3 GHz

6196 – Directly-heated dual, compensating electrometer tetrode with space charge grids for use in the 2 branches of a differential-in, differential-out bridge circuit[58]

6218/E80T (CV5724) – Modulated, single-anode beam deflection tube for pulse generation up to 375 MHz; shock resistant up to 500 g[59][60]

6351 – Secondary emission pentode for wide band RF amplifiers

6353 – 19.3 kV/25...1000 µA Corona voltage reference, miniature 7-pin base with anode top cap

6263 – Pencil-type disk-seal UHF power triode up to 500 MHz, Panode = 8 W

6361 – Convectron, an inclinometer tube that senses tilt from the vertical by means of different gas convections around a heating wire in a glass envelope, of two 6361s aligned in a 90° V-shaped position to each other and the heating wires connected in a bridge circuit

6391 – Subminiature low-microphonics pentode, 8-pin all-glass pigtailed

6441 – 650 V, 100 mAavg, 300 mAsurge Tacitron, a grid turn-off hydrogen thyratron with a grid that forms a shield around both the cathode and anode and separates the two by a wire mesh, so the arc discharge can be extinguished by a negative grid that surrounds the positive anode with a field of opposing polarity and inhibits conduction, taking over part of the anode current during deionisation – similar to today's GTOs; Octal base[61]

6462 – Magnetic pickup tube, a 1-axis beam-deflection magnetometer with approx. 1 G (100 μT) resolution; an electron beam is electrostatically centered between two anodes while no magnetic field is present; the magnetic field to be detected will then deflect the beam more towards one of the anodes, resulting in an imbalance between the two anode currents[62]

6550 – 20 W AF beam tetrode for high fidelity amplifiers

6571 – Williams-type computer memory tube[63]

6577 – Typotron, a charactron for text mode video rendering in early computer monitors[64]

6700 – 200 ns Decade counter Magnetron Beam Switching Tube, 6.3 V, 300 mA heater[65]

6701 – Low-voltage 500 ns decade counter Magnetron Beam Switching Tube, 6.3 V, 300 mA heater[66]

6703 – 500 ns Decade counter Magnetron Beam Switching Tube, 6.3 V, 300 mA heater

6704 – 100 ns Decade counter Magnetron Beam Switching Tube with internal spade load resistors, 6.3 V, 300 mA heater

6710, 6711, 6712 (High current), 6714 (Low voltage) – 2 MHz Decade counter Beam-X Switch, 6.3 V heater

6762 – Wamoscope, a TWT/CRT combination used to directly visualize an incoming microwave signal by electron velocity-sorting[67]

6835, 7570, 7571 – Single-electron gun recording storage tube, an analog video frame freezer tube. This was achieved by a CRT that writes the video image onto a thin, dielectric target and subsequently can read the generated charge pattern up to 30000 times from that target, producing a video signal containing a static shot that resembles a still photograph[68][69][70]

6846 – Gas-filled, three-cathode 1-bit binary counter or switching tube, 7-pin base[71]

6877, 7233 – Power triodes, designed for series voltage regulator applications

6900 – Dual power triode for pulse applications in missiles, avionics and industrial systems; noval base

6922 (E88CC, industrial version of 6DJ8/ECC88)

6973 – Power pentode similar in shape, size, and pinbase to the EL84/6BQ5, but with a high gain for more than double the output range. Popular in some makes of 1960s era guitar amplifiers, though rarely implemented in modern times.

7000s

7077 – Miniature all-ceramic disk-seal UHF triode

7105 – 12.6-volts version of 6080

7189/6BQ5/EL84 – AF Power pentode

7189A – Improved 7189

7199 – Split triode-pentode, noval pinbase. Similar to 6U8.

7229,[72] 7230,[73] 7231,[74] 7232,[75] 7439, 7440, 7441, 7595, 7596, 7597, 7598, 7599, 7600, 7602 – Krytrons, cold-cathode gas-filled trigger tubes with a primer electrode for use as a very high-speed, high-surge current switch[76] – second source to EG&G

7236 – Double power triode for use as long-life power amplifier in computer applications

7241, 7242 – Triple-grid/cathode power triodes, designed for hi-rel series voltage regulator applications

7266 – Miniature all-ceramic disk-seal UHF diode

7289 – 3 GHz, 40 W UHF planar power triode

7308/E188CC – Premium version of 6922

7360 – Beam deflection tube, used as balanced modulator/mixer up to 100 MHz[77][78][79]

7414 – Time Totalizer, a metal-vapor coulometer, a cold-cathode gas-discharge tube where metal is constantly sputtered off the cathode and deposited on a collector element whose resistance therefore decreases with elapsed time[80]

7430 – Flat-envelope version of the 6AK5/EF95 sharp-cutoff pentode for use on PCBs in Radiosonde weather balloon transmitters

7548 – Secondary emission hexode for pulse generator and pulse amplifier applications

7551 - Noval-base beam power pentode with 12-15 volt filament. 6.3 volt filament version was 7558. Used in telephony, RF amplification, and more rarely AF amplification.

7554 – Ceramic/metal pencil-type disk-seal SHF power triode up to 5 GHz

7572, 7575, 7702 – Dual-electron gun recording storage tube, a realtime analog video frame freezer tube with simultaneous R/W, and storing capability. This was achieved by a CRT/camera tube combination; the CRT part writes the video signal onto a thin, dielectric target, which can hold the generated charge pattern for many hours; the camera part reads the charge pattern from the back side of this target, producing a video signal containing a static shot that resembles a still photograph[81][82][83]

7586 – First Nuvistor available on the market, medium-mu triode

7587 – Nuvistor Sharp cutoff tetrode

7591 – Beam power pentode, octal pinbase. Found in many guitar amps made by Gibson and Ampeg.

7688, 7690 (Medium-mu), 7689 (high-mu) – triple triodes

7699 – Dual tetrode for wide band push-pull amplifiers

7762 – Shock-proof avionics AF beam power pentode

7763 – Beam deflection tube, used as IF amplifier/limiter where a constant phase shift over a wide range of input signal amplitudes is required[84]

7768 – Miniature all-ceramic disk-seal SHF triode up to 4 GHz

7868 – Beam power pentode, magnoval pinbase version of 7591. Found in many of the once popular Challenger series PA amps made by Bogen Communications, also found in some guitar amplifiers made by Ampeg.

7895 – Improved 7586 Nuvistor with higher mu

8000s

8011 – Micropup-type UHF power triode up to 600 MHz

8056 – Nuvistor triode for low supply voltage

8058 – Nuvistor triode with grid on envelope and an anode cap, for grounded-grid UHF circuits

8069 – 8 kV/23...1000 µA Corona voltage reference, cathode cylinder and anode top cap

8089 – 1.6 kV/20...800 µA Corona voltage reference, 2-pin all-glass pigtailed

8090 – 3.5 kV/50...1000 µA Corona voltage reference, 9-pin with anode top cap

8091 – 4 kV/50...1000 µA Corona voltage reference, 9-pin with anode top cap

8122 – Forced-air cooled, 300 W@470 MHz beam power tetrode

8256 – 3.5 kV/35...1900 µA Corona voltage reference, 2-pin all-glass pigtailed

8257 – 1.2 kV/15...750 µA Corona voltage reference, 2-pin all-glass pigtailed

8393 – Nuvistor Medium-mu triode, used in Tektronix oscilloscopes, 12.6 Volt heater

8469 – 400 V/5...400 µA Corona voltage reference, 2-pin all-glass pigtailed

8506 – Miniature all-ceramic disk-seal UHF triode

8514 – 1 kV/10...800 µA Corona voltage reference, 7-pin with anode top cap

8515 – 1.6 kV/20...950 µA Corona voltage reference, 7-pin with anode top cap

8526 – Nuvistor-type medium-mu dual triode

8873 – 500 MHz, 200 W anode dissipation power triode

8874 – 500 MHz, 400 W anode dissipation power triode

8875 – 500 MHz, 300 W anode dissipation power triode

8877 = 3CX1500A7 – Ceramic, forced air cooled, 1.5 kW power triode

8974 (X-2159) – Giant water-cooled megawatt-class tetrode used for very high-power broadcast and industrial service; possibly the most powerful tube ever commercially produced

List of European Mullard–Philips tubes, with American equivalents

Main article: List of Mullard–Philips vacuum tubes

List of Pro Electron professional tubes

Note: Typecode explained above.

X - Electro-optical devices

XA

XA1003 – Phototube, caesium-on-oxydated-silver cathode, 2-pin all-glass pigtailed

XG

XG2000 – Image converter for x-ray diagnostics

XL

XL7900 – Vibrating-capacitor chopper front end for dosimeters, electrometers, pH meters etc., Magnoval base with gold-plated pins[85]

XM

XM1000 – Nimo tube, directly heated cathode-ray 1-digit numeric display tube, decimal points on both sides, hence 12 stenciled electron guns, top-viewing, green, 15 mm high Futura Medium font, oval envelope for easy horizontal stacking, 14-pin base

XP

XP1000 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal (14-pin) base

XP1001 – 10-stage photomultiplier for gamma ray scintillation spectrometry, Sb-Cs cathode, Ag-Mg-O-Cs dynodes

XP1002 – 10-stage photomultiplier, blue/green/yellow/orange-sensitive Sb-Na-K-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

XP1003 – 10-stage photomultiplier with quartz window, UV/blue/green/yellow/orange-sensitive Sb-Na-K-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

XP1004 – 10-stage photomultiplier with quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

XP1005 – 10-stage Ag-O-Cs (800±100 nm) photomultiplier, IR/red-sensitive Ag-O-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

XP1010 – 10-stage photomultiplier for r-ray and gamma ray scintillation spectrometry, selected 150AVP for low noise and resolution, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, duodecal (12-pin) base

XP1011 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, shock and vibration-proof, duodecal base

XP1020 – 12-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, 100 O output, duodecal (20-pin) base

XP1021 – 12-stage photomultiplier, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, 50 O output, duodecal base

XP1023 – 12-stage photomultiplier with quartz window Sb-Cs cathode, Ag-Mg-O-Cs dynodes, UV/blue-sensitive, 50 O output, duodecal base

XP1030 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal (14-pin) base

XP1031 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, for gamma ray scintillation spectrometry

XP1032 – 10-stage photomultiplier with 3 mm quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

XP1033 – 10-stage photomultiplier with 10 mm quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

XP1040 – 14-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, concave window, duodecal base

XP1110 – Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

XP1111 – Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, pigtails

XP1113 – 6-stage Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

XP1114 – 4-stage Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

XP1115 – Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, pigtails, shock and vibration-proof

XP1116 – Photomultiplier, red-sensitive Ag-O-Cs cathode, Ag-Mg-O-Cs dynodes, shock and vibration-proof

XP1117 – 9-stage photomultiplier, blue/green/yellow/orange-sensitive Sb-Na-K-Cs cathode, Ag-Mg-O-Cs dynodes

XP1118 – Photomultiplier with quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

XP1120 – 17-stage photomultiplier for x-ray (λ > 200 pm) or UV (λ < 150 nm) photon counting in a high-vacuum environment, Nickel cathode, Cu-Be-O dynodes, coaxial outputs, built-in resistor ladder

XP1121 – 17-stage photomultiplier for ion (> 10 keV) or electron (0.1...10 keV) photon counting in a high-vacuum environment, Cu-Be-O cathode and dynodes, coaxial outputs, built-in resistor ladder

XP1122 – 17-stage photomultiplier for x-ray (λ > 200 pm) or UV (λ < 150 nm) photon counting in a high-vacuum environment, Nickel cathode, Cu-Be-O dynodes, coaxial outputs, built-in resistor ladder

XP1123 – 17-stage photomultiplier for ion (> 10 keV) or electron (0.1...10 keV) photon counting in a high-vacuum environment, Cu-Be-O cathode and dynodes, coaxial outputs, built-in resistor ladder

XP1130 – 17-stage photomultiplier for x-ray (λ > 200 pm) or UV (λ < 150 nm) photon counting in a high-vacuum environment, Nickel cathode, Cu-Be-O dynodes, coaxial outputs, built-in resistor ladder

XP1131 – 17-stage photomultiplier for ion (> 10 keV) or electron (0.1...10 keV) photon counting in a high-vacuum environment, Cu-Be-O cathode and dynodes, coaxial outputs, built-in resistor ladder

XP1140 – 6-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, fast, diheptal base

XP1141 – 7-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, fast, diheptal base

XP1180/52AVP – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, fast, diheptal base

XP1240 – Photomultiplier

XQ

XQ1023 – Camera tube

XQ1029R – Camera tube

XQ1032 – 1" Vidicon; magnetic focusing and deflection

XQ1053 – Camera tube

XQ1072 – 1" Plumbicon

XQ1073 – XQ1072 with higher resolution and improved low level contrast

XQ1200 – Vidicon, silicon target

XQ1270 – ⅔" Vidicon; Overall length 108mm (4¼")

XQ1272 – Camera tube

XQ1274 – ⅔" Newvicon, magnetic focussing and deflection, zinc telluride, zinc selenide target

XQ1275 – Vidicon, silicon target

XQ1276 – XQ1274 with high sensitivity into the near infrared

XQ1277 – XQ1275 with high sensitivity into the near infrared

XQ1278 – XQ1275 with better geometry and uniform signal

XQ1285 – 1" Vidicon; magnetic focusing and deflection, precision electron gun

XQ1290 – Camera tube

XQ1293 – Camera tube

XQ1300 – Saticon Camera Tube

XQ1340 – Low-light vidicon

XQ1371 – Resistron camera tube

XQ1380 – XQ1274 with radiation resistant (anti-browning) faceplate

XQ1381 – ⅔" Newvicon; electrostatioc focusing and magnetic deflection with radiation-resistant (anti-browning) faceplate

XQ1395 – High-resolution Resistron camera tube

XQ1410B/G/R – Plumbicon with color filter for color TV broadcast

XQ1412 – 6/5" Plumbicon; low lag, unity gamma matched to P20 phosphor

XQ1413B/G/R – Plumbicon with color filter for color TV broadcast

XQ1415B/G/R – Plumbicon with color filter for color TV broadcast

XQ1427 – ⅔" Plumbicon; low lag

XQ1427B/G/R – Plumbicon with color filter for color TV broadcast

XQ1430B/G/R – Plumbicon with color filter for color TV broadcast

XQ1435B/G/R – Plumbicon with color filter for color TV broadcast

XQ1440 – 1" Newvicon, separate mesh, zinc telluride, zinc selenide target

XQ1500B/G/R – Plumbicon with color filter for color TV broadcast

XQ1505B/G/R – Plumbicon with color filter for color TV broadcast

XQ1560 – 1" Saticon

XQ1565 – 1" Saticon

XQ1570 – 1" Saticon

XQ1575 – 1" Saticon

XQ1585 – 1" Saticon

XQ1600 – ½" Vidicon; separate mesh, electrostatic focusing and magnetic deflection

XQ1601 – ½" Newvicon; separate mesh, electrostatic focusing and magnetic deflection

XQ2070/02B/G/R – Plumbicon with color filter for color TV broadcast

XQ2070/05B/G/R – Plumbicon with color filter for color TV broadcast

XQ2075/02B/G/R – Plumbicon with color filter for color TV broadcast

XQ2075/05B/G/R – Plumbicon with color filter for color TV broadcast

XQ2172 – 1" Plumbicon; wide dynamic range matched to digital radiography applications

XQ2182 – 1" Plumbicon; wide dynamic range matched to digital radiography applications

XQ2427B/G/R – Plumbicon with color filter for color TV broadcast

XQ3070/02B/G/R – Plumbicon with color filter for color TV broadcast

XQ3070/05B/G/R – Plumbicon with color filter for color TV broadcast

XQ3075/02B/G/R – Plumbicon with color filter for color TV broadcast

XQ3075/05B/G/R – Plumbicon with color filter for color TV broadcast

XQ3427B/G/R – Plumbicon with color filter for color TV broadcast

XQ3430B/G/R – Plumbicon with color filter for color TV broadcast

XQ3435B/G/R – Plumbicon with color filter for color TV broadcast

XQ3440B/G/R – Plumbicon with color filter for color TV broadcast

XQ3445B/G/R – Plumbicon with color filter for color TV broadcast

XQ3457B/G/R – Plumbicon with color filter for color TV broadcast

XQ3467B/G/R – Plumbicon with color filter for color TV broadcast

XQ3477B/G/R – Plumbicon with color filter for color TV broadcast

XQ3487B/G/R – Plumbicon with color filter for color TV broadcast

XQ3550B/G/R – Plumbicon with color filter for color TV broadcast

XQ3555B/G/R – Plumbicon with color filter for color TV broadcast

XQ4187B/G/R – Plumbicon with color filter for color TV broadcast

XQ4502 – 2" Plumbicon; Highest resolution, low lag

XQ5002 – 2" Plumbicon; Electrostsatic deflection for improved corner resolution, low output capacitance

XQ7002 – 1" Plumbicon; Low output capacitance

XQ8002 – 1" Plumbicon

XQ9002 – 1" Plumbicon

XR

XR1000 – Monoscope, test pattern specified by suffix

XX

XX1000 – 2-stage image intensifier

XX1010 – Image intensifier

XX1020 – Image intensifier

XX1030 – Image intensifier

XX1050 – Image intensifier

XX1060 – Image intensifier

XX1066 – 1. Gen. 3-stage image intensifier

XX1140 – 1. Gen. 3-stage image intensifier

XX1190 – 1. Gen. inverter, 1-stage image intensifier

XX1192 – 1. Gen. inverter, 1-stage image intensifier

XX1200 – 1. Gen. inverter, 1-stage image intensifier

XX1211 – 1. Gen. inverter, 3-stage image intensifier

XX1270 – 1. Gen. inverter, 2-stage image intensifier

XX1400 – 2. Gen. inverter, 1-stage image intensifier

XX1430 – 1. Gen. inverter, 1-stage image intensifier

XX1510 – 1. Gen. 3-stage image intensifier

XX1610 – 2. Gen. image intensifier

XX1800 – 2. Gen. proximity focused, 1-stage image intensifier

Y - Vacuum tubes

YA

YA1000 – 5 kV, 5mA, Directly heated saturated-emission diode with pure-metal cathode for use in RMS converters of AC voltage/current stabilizer circuits, noval base[86]

YD

YD1000 – 120 kW, Water-cooled RF power triode

YD1001 – 120 kW, Air-cooled RF power triode

YD1012 – 360 kW, Vapor-cooled RF power triode

YD1130 – 580 W, Air-cooled, linear RF/AF power triode

YD1252 (RS 2051 V) – 420 kW, Water-cooled, modulator power triode

YD1300 – 35 W, Air-cooled, UHF power triode

YD1301 – 50 W, Air-cooled, UHF power triode

YD1302 – 55 W, Air-cooled, UHF power triode

YD1332 – 250 W, Air-cooled, UHF power triode

YD1333 – 100 W, Air-cooled, UHF power triode

YD1334 – 110 W, Air-cooled, UHF power triode

YD1335 – 550 W, Air-cooled, UHF power triode

YD1336 – 220 W, Air-cooled, UHF power triode

YD1342 – 30 MHz, 530 kW, Water-cooled RF power triode

YD1352S (8867, DX334) – Neotron, a field-effect tube, 5 MHz, 3 kW, water-cooled, magnetically beamed RF power pulse generator triode

YG

YG1000 – Directly heated electrometer tetrode with an oxide cathode and a space charge grid, grid current ≤600 fA, magnoval base with input grid on top cap

YH

YH1000 – Traveling-wave tube

YH1050 – Traveling-wave tube

YH1110 – Traveling-wave tube

YH1120 – Traveling-wave tube, >3 GHz

YH1131 – Traveling-wave tube, >3 GHz

YH1150 – Traveling-wave tube

YH1160 – Traveling-wave tube, >3 GHz

YH1181 – Traveling-wave tube, >3 GHz

YH1190 – Traveling-wave tube, >3 GHz

YH1200 – Traveling-wave tube, >3 GHz

YJ

YJ1000 – Indirectly heated, 2.5 kW magnetron for use as a pulsed X-band oscillator between 9.19 and 9.32 GHz

YJ1462 – Indirectly heated, 28 kW coaxial magnetron for use as a pulsed X-band oscillator at 9.375 GHz

YK

YK1000 – Water-cooled, permanent-magnet 11 kW UHF linear-beam Klystron for use in TV transmitters between 400 and 620 MHz

YK1004 – Water-cooled, permanent-magnet 11 kW UHF linear-beam Klystron for use in TV transmitters between 610 and 790 MHz

YK1005 – Water-cooled, permanent-magnet 11 kW UHF linear-beam Klystron for use in TV transmitters between 470 and 860 MHz

YK1046 – 35 mW X-band Klystron, 9.16 to 9.34 GHz

YK1151 – Forced-air cooled, permanent-magnet 25 kW UHF linear-beam Klystron for use in TV transmitters between 470 and 860 MHz

YK1190 – Water-cooled 40 kW UHF linear-beam Klystron for use in TV transmitters between 470 and 610 MHz

YK1191 – Water-cooled 40 kW UHF linear-beam Klystron for use in TV transmitters between 590 and 720 MHz

YK1192 – Water-cooled 40 kW UHF linear-beam Klystron for use in TV transmitters between 710 and 860 MHz

YL

YL1000/8463 – RF power pentode

YL1020/8118 – See QQZ03/20

YL1030 – See QQZ06/40

YL1050 – RF power tetrode

YL1060/7854 – See QQE06/40

YL1070/8117 – RF power pentode

YL1071 – YL1070 with a different heater

YL1080/8348 – RF power pentode

YL1120 – RF power tetrode

YL1130/8408 – RF power pentode

YL1150/8579 – RF power pentode

YL1190/8580 – RF power pentode

YL1200 – See PE1/100

YL1210 – QQE03/12 with a different heater

YL1220 – QQE02/5 with a different heater

YL1240/8458 – RF power pentode

YL1250/8505 – RF power pentode

YL1270/8581 – RF power pentode

YL1290 – QE08/200 with a different heater

YL1310/8603 – RF power pentode

YL1360 – QQE04/5 with a different heater

YL1570 (RS 1084 CJ) – RF power tetrode

Z - Gas-filled tubes

Note: See also standard M-P tubes under Z

ZA

ZA1000 – Neon-filled, coaxial, tritium-primed (half-life: 12.32 years), sputtered-molybdenum cold-cathode switching diode, meshed cylinder anode, all-glass pigtailed[87]

ZA1001 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode with traces of heavy gas (krypton/xenon) for slow de-ionization, e.g. for low-frequency relaxation oscillators; meshed cylinder anode, all-glass pigtailed

ZA1002 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode, large difference between burning and ignition voltage, meshed cylinder anode, 3-pin all-glass pigtailed

ZA1003 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode for use as indicator tube in transistorized circuits, meshed cylinder anode, 3-pin all-glass pigtailed

ZA1004 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode, small difference between burning and ignition voltage, for use as indicator tube in transistorized circuits or as 86.4 V Voltage reference, meshed cylinder anode, 3-pin all-glass pigtailed

ZA1005 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode for use like a DIAC in thyristor circuits, meshed cylinder anode, 2-pin all-glass pigtailed

ZC

ZC1010 (Z661W) – 8 mAavg, 50 mApeak, Gas-filled, cold-cathode AC trigger pentode, two starters and a primer electrode, positive starter voltage, 5-pin all-glass pigtailed, envelope inside radioactively coated for a constant ignition voltage, for use in bidirectional counters

ZC1040 – 25 mA, Gas-filled, cold-cathode AC trigger tetrode, one starter and a primer electrode, positive starter voltage, noval base

ZC1050 – 2 mA, Gas-filled, cold-cathode, luminiscent trigger tetrode, one starter and a primer, 300 mlm light output[88] for use as self-displaying shift register cells in large-format, crawling-text dot-matrix displays;[89] all-glass pigtailed

ZC1060 – 20 mAavg, 5 kApeak, Gas-filled, cold-cathode, high-current trigger triode for e.g. capacitor discharge circuits. One external (capacitive) starter electrode

ZM

ZM1000 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14 mm character height side-viewing, left decimal point

ZM1000R – ZM1000 with a red contrast filter coating

ZM1001 –  + - ~ X Y Z  Neon-filled digital indicator tube, 14mmCH side-viewing, for use with ZM1000

ZM1001R – ZM1001 with a red contrast filter coating, for use with ZM1000R

ZM1002 –  ns μs ms s Hz kHz MHz  Neon-filled digital indicator tube, 13mmCH side viewing, for use with ZM1000 in digital frequency counters

ZM1003 –  1 - +  Neon-filled digital indicator tube, 14mmCH side-viewing, for use with ZM1000

ZM1005 –  0 1 2 3 4 5 6 7 8 9  Long-life neon-filled digital indicator tube, 14mmCH side-viewing, left decimal point, multiplex-capable

ZM1005R – ZM1005 with a red contrast filter coating

ZM1006 –  1 2 3 4 5 6  Neon-filled digital indicator tube, side-viewing, left and right decimal point, for use in TV receivers

ZM1008 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14mmCH side-viewing

ZM1010 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14mmCH side-viewing, left decimal point

ZM1012 –  0 1 2 3 4 5 6 7 8  Neon-filled digital indicator tube, 14mmCH side-viewing

ZM1015 –  0 1 2 3 4 5 6 7 8  Neon-filled digital indicator tube, 14mmCH side-viewing

ZM1020 (Z520M) – ZM1022 with a red contrast filter coating

ZM1021 (Z521M) – ZM1023 with a red contrast filter coating, for use with ZM1020

ZM1022 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH top-viewing, no decimal point

ZM1023 –  A V Ω % + - ~  Neon-filled digital indicator tube, 15.5mmCH top-viewing, for use with ZM1022 in digital multimeters

ZM1024 – ZM1025 with a red contrast filter coating, for use with ZM1020

ZM1025 –  c/s Kc/s Mc/s μs ms ns s  Neon-filled digital indicator tube, 15.5mmCH top-viewing, for use with ZM1022 in digital frequency counters

ZM1030 – ZM1032 with a red contrast filter coating

ZM1031 – ZM1031/01 without the  ~

ZM1031/01 – ZM1033/01 with a red contrast filter coating, for use with ZM1030

ZM1032 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, no decimal point, 5 dual cathodes and separate odd/even anode compartments for biquinary multiplexing

ZM1033/01 –  + - ~  Neon-filled digital indicator tube, 15.5mmCH side-viewing, separate anode compartment for  + , for use with ZM1032

ZM1040 (Z522M) – ZM1042 with a red contrast filter coating

ZM1041 – ZM1043 with a red contrast filter coating, for use with ZM1040

ZM1041S – ZM1043S with a red contrast filter coating, for use with ZM1040

ZM1042 (Z5220M) –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 30mmCH side-viewing, no decimal point

ZM1043 –  + -  Neon-filled digital indicator tube, 30mmCH side-viewing, for use with ZM1042

ZM1043S –  Y X + W U Z -  Neon-filled digital indicator tube, 30mmCH side-viewing, for use with ZM1042

ZM1047 – ZM1049 with a red contrast filter coating, for use with ZM1040

ZM1049 –  T F S N Z Y G H M X  Neon-filled digital indicator tube, side-viewing, for use with ZM1042 in numerical control systems

ZM1050

ZM1070

(Z550M, 8453) – Neon-filled digital indicator tube, top-viewing, dekatron-type readout with common anode and common cathodes, pulsating anode voltage, controlled by 5-volts sensitive starter electrodes, for transistorized circuits

ZM1060 (Z505S) – Argon-filled, 50 kHz decade Counter/Selector Dekatron

ZM1070 (Z504S, 8433) – Neon-filled, 5 kHz decade Counter/Selector Dekatron

ZM1080 – ZM1082 with a red contrast filter coating

ZM1081 – ZM1083 with a red contrast filter coating, for use with ZM1080

ZM1082 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14mmCH side-viewing, no decimal point, probe electrode

ZM1083 –  + - ~  Neon-filled digital indicator tube, 14mmCH side-viewing, for use with ZM1082

ZM1100 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH top-viewing

ZM1120 – ZM1122 with a red contrast filter coating

ZM1122 –  0 1 2 3 4 5 6 7 8 9  Miniature neon-filled digital indicator tube, 7.8mmCH top-viewing

ZM1130 – ZM1132 with a red contrast filter coating

ZM1131 – ZM1133 with a red contrast filter coating, for use with ZM1080

ZM1132 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, side-viewing, left and right decimal point

ZM1133 –  + - ~  Neon-filled digital indicator tube, side-viewing, for use with ZM1132

ZM1136L/R – ZM1138L/R with a red contrast filter coating

ZM1137 – ZM1139 with a red contrast filter coating, for use with ZM1136L/R

ZM1138L/R –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 13mmCH side-viewing, left or right decimal points (specify)

ZM1139 –  + - ~ Ω  Neon-filled digital indicator tube, 13mmCH side-viewing, for use with ZM1138 in digital multimeters

ZM1162 –  0 1 2 3 4 5 6 7 8 9  Long-life neon-filled digital indicator tube, 15.5mmCH top-viewing, no decimal point, rectangular envelope for close stacking in both axes

ZM1170 – ZM1172 with a red contrast filter coating

ZM1172 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, no decimal point

ZM1174 – ZM1175 with a red contrast filter coating

ZM1175 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, left decimal point

ZM1176 – ZM1177 with a red contrast filter coating

ZM1177 – ZM1175, but right decimal point

ZM1180 – ZM1182 with a red contrast filter coating

ZM1181 – ZM1183 with a red contrast filter coating, for use with ZM1180

ZM1182 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16mmCH top-viewing, no decimal point, semi-rectangular envelope for close horizontal stacking

ZM1183 –  + - ~ Ω  Neon-filled digital indicator tube, top-viewing, 13mmCH for use with ZM1182 in digital multimeters

ZM1184D – ZM1185D with a red contrast filter coating

ZM1185A (GR1420) –  1 2 3 4 5 6 U K E R  Neon-filled digital indicator tube, 16mmCH top-viewing

ZM1185D (GR1430) –  ∇ Δ  Neon-filled digital indicator tube, 16mmCH top-viewing, for use in elevators

ZM1185E (GR1472) –  0 1 2 3 4 5 - t kg +  Neon-filled digital indicator tube, 16mmCH top-viewing

ZM1200 – Pandicon, multiplexed 14-digit display tube with decimal points and punctuation marks, pin connections on both ends

ZM1202 – 12-Digit Pandicon

ZM1204 – 10-Digit Pandicon

ZM1206 – 8-Digit Pandicon

ZM1210

ZM1212

– ZM1212 with a red contrast filter coating

ZM1212 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, left decimal point, all-glass pigtailed

ZM1220 – ZM1222 with a red contrast filter coating

ZM1222 –  0 1 2 3 4 5 6 7 8 9  Large neon-filled digital indicator tube, 40mmCH side-viewing

ZM1230 – ZM1232 with a red contrast filter coating

ZM1232 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH upside-down side-viewing, no decimal point

ZM1240 – ZM1242 with a red contrast filter coating

ZM1241 – ZM1243 with a red contrast filter coating, for use with ZM1240

ZM1242 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16mmCH side-viewing, right decimal point

ZM1243 –  + - ~ Ω  Neon-filled digital indicator tube, 16mmCH side-viewing, for use with ZM1242 in digital multimeters

ZM1263 –  ~ ⚫  Neon-filled digital indicator tube, 10mmCH side-viewing

ZM1290 – ZM1292 with a red contrast filter coating

ZM1292 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 10mmCH side-viewing

ZM1330 – ZM1332 with a red contrast filter coating

ZM1331 – ZM1333 with a red contrast filter coating, for use with ZM1330

ZM1332 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 13.1mmCH side-viewing, left and right decimal points, all-glass pigtailed

ZM1333 –  + - ~ Ω  Neon-filled digital indicator tube, 13.1mmCH side-viewing, all-glass pigtailed, for use with ZM1332 in digital multimeters

ZM1334 – ZM1336 with a red contrast filter coating

ZM1335 – ZM1337 with a red contrast filter coating, for use with ZM1334

ZM1336 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 13.1mmCH side-viewing, left and right decimal points, multiplex-capable

ZM1337 –  + - ~ Ω  Neon-filled digital indicator tube, 13.0mmCH side-viewing, right decimal point (!), all-glass pigtailed, red contrast filter coating, for use with ZM1336 in digital multimeters

ZM1350 – Varisymbol, planar neon-filled digital 40mm x 27mm fourteen-segment display tube, right decimal point, separate underscore text cursor, keep-alive cathode, multiplex-capable, viewing angle 160°

ZM1360 – ZM1350 with 60mm x 40mm characters

ZM1370 – ZM1350 with 20mm x 13mm characters

ZM1410 – ZM1412 with a red contrast filter coating

ZM1412 – Neon-filled digital seven-segment display tube, 8.6mmCH side-viewing, right decimal point and left punctuation mark, all-glass pigtailed

ZM1500 – Pandicon, multiplexed 12-digit, 7-segment display tube

ZM1550 – Planar neon-filled digital two-digit seven-segment display tube, right decimal points

ZM1551 – Planar neon-filled digital 1½-digit seven-segment display tube with  +  and  -  signs, right decimal points

Note: More Nixie tubes under standard - ZM and other letter - GR

ZP

ZP1000 – Borium tri-fluoride-filled Geiger-Müller tube, thermal neutrons

ZP1010 – Borium tri-fluoride-filled Geiger-Müller tube, thermal neutrons

ZP1020 – Borium tri-fluoride-filled Geiger-Müller tube, thermal neutrons

ZP1070 – Subminiature Geiger-Müller tube, all-glass pigtailed

ZP1080 – Halogen-quenched Geiger-Müller tube, β and γ

ZP1100 – Halogen-quenched Geiger-Müller tube, γ; pigtailed

ZP1200 – Halogen-quenched Geiger-Müller tube, γ

ZP1300 – Halogen-quenched Geiger-Müller tube, γ and high-energy β

ZP1330 – Halogen-quenched Geiger-Müller tube, for use in damp and/or saline atmosphere, β and γ

ZP1400 – Halogen-quenched Geiger-Müller tube, 9mm diameter mica window, β and γ

ZP1430 – Halogen-quenched Geiger-Müller tube, 27.8mm diameter mica window, α, β, γ

ZP1490 – Halogen-quenched Geiger-Müller tube, 28mm diameter mica window, low-level α, β and γ

ZP1600 – Halogen-quenched Geiger-Müller tube, 19.8 mm diameter mica window, X-rays, 6.0 to 20 keV energy, 60 to 200 pm wavelength range

ZP1610 – Side window, organically quenched Geiger-Müller tube. 7 x 18 mm mica window; X-rays, 2.5 to 40 keV energy, 30 to 500 pm wavelength range

ZP1700 – Halogen-quenched, cosmic-ray guard counter tube for low-background measurements; to be used with another radiation counter tube in an anticoincidence circuit

ZP1800 – Halogen-quenched Geiger-Müller tube for use at temperatures up to 200 °C, γ

ZP1810 – Halogen-quenched Geiger-Müller tube for use at temperatures up to 200 °C, γ, low sensitivity, up to 40 mGy/h

ZP1860 – Halogen-quenched Geiger-Müller tube, β and γ

ZT

ZT1000 – 21 kV, 10 A Mercury vapor triode thyratron

ZX

ZX1000 – 800 V, 1140 Apk, 13Aavg Ignitron

ZX1051 – Water-cooled, 56 Aavg Ignitron

ZX1052 – Water-cooled, 140 Aavg Ignitron

ZX1053 – Water-cooled, 355 Aavg Ignitron

ZX1060 – Water-cooled, 10 Aavg Ignitron

ZX1061 – Water-cooled, 10 Aavg Ignitron

ZX1062 – Water-cooled, 10 Aavg Ignitron

ZX1063 – Water-cooled, 10 Aavg Ignitron

ZY

ZY1000 (872B) – High voltage, half-wave mercury-vapor rectifier

ZY1001/8008A – High voltage, half-wave mercury-vapor rectifier, 4-pin base with anode top cap

ZY1002 – High voltage, half-wave mercury-vapor rectifier, E40 (Goliath) Edison screw lamp base with anode top cap

ZZ

ZZ1000 – 81 V Voltage reference, 2-pin all-glass pigtailed

ZZ1010 – 85 V Voltage reference

ZZ1020 (STV85-8) – 82 V Voltage reference with primer electrode, 3-pin all-glass pigtailed

ZZ1030 (STV500-0,1) – Quad 125 V Voltage references, noval base

ZZ1031 – Quad Voltage reference, noval base

ZZ1040 (STV100-60Z) – 100 V Voltage reference with primer electrode

ZZ1050 – 82 V Voltage reference, 2-pin all-glass pigtailed

List of European transmitting tubes

Note: Typecode explained above.

B - Backward-wave amplifier

BA

BA9/20 – X-band, 20 mW, Forced-air cooled backward-wave oscillator

D - Rectifier incl. grid-controlled

DA

DA1.5/75 – 1.5 kV, 75 W Half-wave power rectifier, triode TA1.5/75 without grid

DA12/24000 – 12 kV, 24 kW Water-cooled half-wave power rectifier

DC

DC1/50 – 1 kV, 50 mA Full-wave power rectifier, DC1/60 with dual anode top cap

DC1/60 – 1 kV, 60 mA Full-wave power rectifier

DC2/200 – 2 kV, 200 mA Full-wave power rectifier with dual anode top cap

DCG

DCG1/125 – 1 kV, 125 mA Half-wave mercury-vapor rectifier with Edison screw lamp base and anode top cap

DCG12/30 – 12 kV, 30 A Grid-controlled, half-wave mercury-vapor rectifier with anode top cap

DCX

DCX4/1000 – 4 kV, 1 kW Half-wave xenon rectifier with anode top cap

DCX4/5000 – 4 kV, 5 kW Half-wave xenon rectifier with anode top cap

DE

DE2/200 – 2 kV, 200 W Full-wave power rectifier with dual anode top cap

J - Magnetron

JP

JP8/02B – 8.8 GHz, 25 W Magnetron

JP9/15 – 9.345 to 9.405 GHz, 15 kW Forced-air cooled magnetron for pulsed service

JPT

JPT9/01 – 9.15 to 9.60 GHz, 5 W Magnetron

K - Klystron

KB

KB9/150W – X-band, 150 W Water-cooled, dual-resonator klystron

KS

KS7/85 – 6.5 to 7.5 GHz, 85 mW Reflex klystron

L - Traveling-wave tube

LA

LA9/3 – 7 to 11.5 GHz Forward-wave amplifier

LA16/2 – 11.5 to 18 GHz Forward-wave amplifier

M - AF modulator Triode

MA

MA4/600 – 4 kV, 600 W Radiation-cooled triode

MB

MB1/50 – 1 kV, 50 W Radiation-cooled triode

MB2/200 – 2 kV, 200 W Radiation-cooled triode

MY

MY3/275 – 3 kV, 275 W Radiation-cooled triode

MZ

MZ2/200 – 2 kV, 200 W Radiation-cooled triode

P - Pentode

PA

PA12/15 – 15 kW Water-cooled shortwave pentode

PA12/20 – 20 kW Water-cooled pentode made by Philips and used in the 1930s and 1940s

PAL

PAL12/15 – Air-cooled version of PAW12/15

PAW

PAW12/15 – 15 kW Water-cooled shortwave pentode

PB

PB2/200 – 200 W Shortwave pentode

PB3/1000 – 1 kW Shortwave pentode

PC

PC03/3 – 3 W Shortwave pentode

PC3/1000 – 1 kW Shortwave pentode

PE

PE04/10 – 10 W Shortwave pentode

PE1/100 (YL1200) – 100 W Shortwave pentode

Q - Tetrode

QB

QB2/75 – 75 W Beam-tetrode

QB5/2000 – 2 kW Beam-tetrode

QBL

QBL4/800 – Air-cooled 800 W beam-tetrode

QBL5/3500 – Air-cooled 3500 W beam-tetrode

QBW

QBW5/3500 – Water-cooled 3500 W beam-tetrode

QC

QC05/15 – 15 W Beam-tetrode

QC05/35 – 35 kW Beam-tetrode

QE

QE04/10 – 10 W Beam-tetrode

QE05/40 (6146) – 40 W Radiation-cooled output beam-tetrode, popular amongst radio amateurs as a final RF amplifier

QE08/200 – 200 W Beam-tetrode

QEL

QEL1/150 – Air-cooled 150 W beam-tetrode

QEL1/250 – Air-cooled 250 W beam-tetrode

QEP

QEP20/18 – 18 W Beam-tetrode for use as a pulse modulator

QQC

QQC03/14 – 14 W Dual beam-tetrode

QQE

QQE02/5 (6939) – 5 W Dual beam-tetrode

QQE03/12 (6360) – 12 W Dual beam-tetrode

QQE03/20 (6252) – 20 W Dual beam-tetrode

QQE04/5 (7377) – 5 W Dual beam-tetrode

QQE06/40 (5894, YL1060) – 40 W dual beam-tetrode, internally neutralized, Septar base with dual anode top cap

QQV

QQV02/6 – 6 W dual beam-tetrode

QQV03/20A – 20 W Radiation-cooled split-anode tetrode made by Mullard and used in the 1940s, 1950s and 1960s as a VHF frequency-doubling output stage with balanced output.

QQV07/50 – 50 W Dual beam-tetrode

QQZ

QQZ03/20 (8118, YL1020) – 20 W Dual beam-tetrode

QQZ06/40 (YL1030) – 40 W Dual beam-tetrode

QV

QV04/7 – 7 W Beam-tetrode

QV05/25 (807) – 25 W Radiation-cooled output beam-tetrode made by Mullard.

QV2/250C – 250 W Beam-tetrode

QY

QY3/65 – 65 W Beam-tetrode

QY5/3000A – 3 kW Beam-tetrode

QY5/3000W – Water-cooled version of QY5-3000A

QYS

QYS50/P40 – Pulsed power tetrode, Silica envelope, 50 kV anode voltage, considerable x-radiation, 810 °C anode temperature at 700 W anode dissipation, 40 A anode current at duty factor 0.0005, Vg1Cut-off (IA=1 mA@VA=55 kV): > -3.4 kV, gm: 38 mS

QZ

QZ06/20 – 25 W VHF Power tetrode up to 175 MHz

R - Rectifier

RG

RG1000/3000 – 1 kV, 3 A Half-wave mercury-vapor rectifier with anode top cap

RGQ

RGQ7.5/0.6 – 7.5 kV, 600 mA Half-wave mercury-vapor rectifier with anode top cap

RGQ20/5 – 20 kV, 5 A Half-wave mercury-vapor rectifier with anode top cap

T - AF/RF/oscillator Triode

TA

TA04/5 – 400 V, 50 W Radiation-cooled power triode

TA1.5/75 – 1.5 kV, 75 W Radiation-cooled power triode

TA4/2000K – 4 kV, 2 kW Air-cooled power triode made by Philips in the 1930s

TA18/100000 – 18 kV, 100 kW Water-cooled power triode

TB

TB04/8 – Directly heated Doorknob VHF power triode up to 600 MHz

TB2.5/400 – 2.5 kV, 300 W Radiation-cooled power triode

TB5/2500 – 5 kV, 2.5 kW Radiation-cooled power triode

TBL

TBL2/300 – 2 kV, 300 W Forced air-cooled power triode

TBL15/125 – 15 kV, 125 kW Forced air-cooled power triode

TBW

TBW6/14 – 6 kV, 14 kW Water-cooled power triode

TBW15/125 – 15 kV, 125 kW Water-cooled power triode

TC

TC03/5 – RF power triode up to 85 MHz, 5 W

TC2/250 – RF power triode up to 20 MHz, 250 W

TD

TD03/5 – Indirectly heated disk-seal UHF power triode up to 2 GHz

TD03/10 – Indirectly heated disk-seal UHF power triode up to 2.8 W, 3.75 GHz

TD03/10F – TD03/10 with internal feedback for use as an oscillator

TD04/20 – Indirectly heated disk-seal UHF power triode up to 13.5 W, 1 GHz

TD1/100C = 2C39BA – Indirectly heated, ceramic disk-seal UHF power triode up to 24 W, 3.5 GHz[90]

TD2/400 – Directly heated, ceramic disk-seal UHF power triode up to 600 W, 900 MHz

TD2/500 – Directly heated, ceramic disk-seal UHF power triode up to 500 W, 940 MHz

TE

TE05/10 – RF power triode up to 150 MHz

TX

TX12/12W – Water-cooled RF power triode

TX12/20W – Water-cooled RF power triode

TX10/4000 – Power triode, Silica envelope, 12 kV anode voltage, 4 kW anode dissipation, 1.6 A cathode current, gm: 4.5 mS, for use as self-excited high-power oscillator in induction heating equipment.

TY

TY2/125 – 135 W VHF power triode up to 200 MHz

TY12/50A – Forced-air cooled 45 kW RF power triode up to 30 MHz

TY12/50W – Water-cooled 50 kW RF power triode up to 30 MHz

TYS

TYS2/250 – Power triode, Silica envelope, 2.5 kV anode voltage, 250 W anode dissipation

TYS4/500 – Power triode, Silica envelope

TYS5/1000 – Power triode, Silica envelope

TYS5/2000 – Power triode, Silica envelope

TYS5/3000 – Power triode, Silica envelope, 6 kV anode voltage, 950 °C anode temperature at 3.5 kW anode dissipation, 2.8 A cathode current, gm: 15 mS. Used in RF generators for induction hardening.

X - Thyratron

XGQ

XGQ2/6400 – 2 kV, 6.4 kW Mercury-vapor tetrode thyratron with anode and grid1 top caps

XR

XR1/1600 (5545) – 1 kV, 1.6 kW Inert gas-filled triode thyratron with anode top cap

XR1/6400 – 1 kV, 6.4 kW Inert gas-filled triode thyratron with anode top cap

List of other letter tubes

A

Edison and Swan Electric Light Company (British Mazda/EdiSwan):

A40 – Acorn UHF triode up to 600 MHz, 4 Volts heater

A41 – Acorn UHF pentode up to 600 MHz, 4 Volts heater

AC*/

Mazda/EdiSwan 4-volts AC, indirectly heated receiver tubes:

AC/HL – AF triode, British 5-pin base

AC/HLDD = TDD4 = MHD4 – Dual diode and AF triode, British 7-pin base

AC/ME – Tuning indicator, British 7-pin base

AC/P, AC/P1 – AF triode, British 5-pin base

AC/P4 – CRT electrostatic-deflection output power triode, British 5-pin base

AC/PEN – AF power pentode, British 7-pin base

AC/S2PEN – RF pentode, British 7-pin base

AC/SP1 – RF pentode for use in squelch circuits or, as the reactance tube, in AFC circuits, British 7-pin base

AC/SP3 – RF pentode for shortwave and TV receivers, British 7-pin base

AC/SP3/RH – Low-noise, low-microphonics RF pentode for shortwave and TV receivers, British 7-pin base

AC/TH1 – Triode/hexode oscillator/mixer, British 9-pin base

AC/TP = TP4 – Triode/pentode oscillator/mixer, British 7-pin base

AC/VP1, AC/VP2 – RF pentode, British 7-pin base

AC2/HL – High-mu triode

AC2/PEN – AF Power pentode

AC2/PEN.DD – Dual diode and AF Power pentode

AC4/PEN – AF Beam power pentode

AC5/PEN – AF Beam power pentode

AC5/PEN.DD – Dual diode and AF Beam power pentode

AC6/PEN – Beam power pentode for use as a magnetic horizontal-deflection output amplifier

ACT

Marconi-Osram Valve Company:

ACT9 – 800 W Air cooled transmitting triode up to 15 MHz, with derating up to 80 MHz

AP

Mullard:

AP4 = 4676 – Acorn UHF pentode up to 430 MHz, 4 Volts heater

AT

Mullard:

AT4 = 4675 – Acorn UHF triode up to 430 MHz, 4 Volts heater

B

BA

Industrial Electronics Engineers:

BA-0000-P31 – Nimo tube, cathode-ray 1-digit numeric display tube, 10 stenciled electron guns aiming at a P31-phosphor (green, medium-persistence) fluorescent screen, top-viewing, Futura Medium font, 2.5 kV anode voltage, 12-pin base

BG

Burroughs:

BG08220-K – Neon-filled, planar, base-24 display dekatron with four sets of guiding cathodes and a carry/borrow cathode, for use e.g. in direction-finding equipment

BT

British Thomson-Houston (General Electric subsidiary):

BT1 – Thyratron used in Wynn-Williams' binary prescaler for the alpha particle counter that Rutherford, Chadwick et al. used for their nuclear research at the Cavendish Laboratory in the 1930s[91]

C

CH

Tung-Sol:

CH1027 – Curristor – Four types of nitrogen-filled, radioactive constant-current tubes with a current plateau from 25 to 500 V, all-glass pigtailed, active material is 226Ra with a half-life of 1601 years, for linear capacitor charging and draining in missile and ordnance mine timing circuits, instrumentation biasing, as current reference, etc.:

CH1027-9 – 10−9 A, 18.75 μCi (694 kBq)

CH1027-10 – 10−10 A, 1.875 μCi (69.4 kBq)

CH1027-11 – 10−11 A, 187.5 nCi (6.94 kBq)

CH1027-12 – 10−12 A, 18.75 nCi (694 Bq)

CK

Raytheon:

CK1414 EBCDIC target H 	I 	? 	. 	¤ 	[ 	< 	⊞

+ 	A 	B 	C 	D 	E 	F 	G

Q 	R 	! 	$ 	* 	] 	; 	△

- 	J 	K 	L 	M 	N 	O 	P

Y 	Z 	= 	, 	% 	≶ 	' 	⊠

/ 	S 	T 	U 	V 	W 	X

8 	9 	0 	# 	@ 	: 	> 	■

1 	2 	3 	4 	5 	6 	7

CK1022 – 1 kV/5...55 µA Corona voltage reference, miniature 7-pin base with anode top cap

CK1037 = 6437 – 700 V/5...125 µA Corona voltage reference, 3-pin all-glass pigtailed

CK1038 – 900 V/5...55 µA Corona voltage reference, 3-pin all-glass pigtailed

CK1039 = 6438 – 1.2 kV/5...125 µA Corona voltage reference, 3-pin all-glass pigtailed

CK1366, CK1367, CK1368, CK1369 – CRTs with an unphosphored front glass but with fine wires embedded in it for use as electrostatic print heads; the wires would pass the electron beam current through the glass onto a sheet of paper where the desired content was therefore deposited as an electrical charge pattern. The paper was then passed near a pool of liquid ink with the opposite charge. The charged areas of the paper attract the ink and thus form the image.[92][93]

CK1383 – Dual-electron gun recording storage tube, a realtime polar, radar PPI-to-rectangular, TV-type analog video transcoder similar to the 7702, with simultaneous R/W, and storing capability. This was achieved by a CRT/camera tube combination; the CRT part writes the PPI-format image onto a thin, dielectric target; the camera part reads the generated charge pattern in TV format from the back side of this target.[94]

CK1414 – Symbolray character generator monoscope for text mode video rendering in early computer monitors, with a square target having letters, digits and symbols patterned on it in a customer-supplied 8x8 or 8x12 array. An electron beam selects and scans a character, both by appropriate electrostatic deflection, and generates an analog video signal;[95][96] cf. 4560, TH9503

CL

Ferranti:

CL40 and CL41 – Indirectly heated, linear light source (glow modulator tube), mercury/argon-filled gas diode with primer electrode, 8-pin base, for rotating-drum FAX receivers, film soundtrack recording, etc.

CL42 and CL43 – Indirectly heated, low-noise linear light source, helium-filled gas diode with primer electrode, 8-pin base, for film soundtrack recording, interferometers, etc.

CL44 – Indirectly heated, low-noise linear light source, neon-filled gas diode with primer electrode, 8-pin base

CL50 and CL52 – Indirectly heated, linear light source, gas-filled diode with primer electrode, 7-pin base, for rotating-drum FAX receivers, film soundtrack recording, etc.

CL55 – Indirectly heated, spectrally pure light source, helium-filled gas diode with primer electrode, 7-pin base with anode top cap

CL56 – Indirectly heated, spectrally pure light source, krypton-filled gas diode with primer electrode, 7-pin base with anode top cap

CL57 – Indirectly heated, spectrally pure light source, neon-filled gas diode with primer electrode, 7-pin base with anode top cap

CL58 – Indirectly heated, spectrally pure light source, xenon-filled gas diode with primer electrode, 7-pin base with anode top cap

CL60 – Indirectly heated triode flood beam CRT-type stroboscope lamp with a green A-type phosphor with <1 μs decay time and 10 kCd light output, 20 kV anode voltage, 7-pin duodecal base

CL61 – CL60 with a blue P-type phosphor with 5 μs decay time and 16 kCd light output

CL62 – CL60 with an UV Q-type phosphor with 100 ns decay time and 240 Cd light output

CL63 – CL60 with a yellow-green C-type phosphor with 6 μs decay time and 24 kCd light output

CL64 – CL60 with a yellow V-type phosphor with 5 μs decay time and 12 kCd light output

CL65 – CL60 with a red R-type phosphor with 2 μs decay time and 14 kCd light output

CL66 – CL60 with a white T-type phosphor with 5 μs decay time and 12 kCd light output

D

Philips:

D1 – Early directly heated triode used in 1920s TRF and regenerative radios

DDR

Mullard:

DDR100 – 100 g max., 250 Hz max., 1-axis accelerometer double diode with elastically supported anodes, 6.3V/600mA indirect heater, fres = 1 kHz, B8G base[97]

DZ

Cerberus:

DZ10 – 3 kHz max. Decade Counter/Selector Dekatron, 14-pin diheptal base

E

EN

Ferranti:

EN10 – Neostron, 400 Apk Gas-filled, cold-cathode tetrode thyratron, differential trigger electrodes, 8-pin base, for use as a relay or as a reddish 700 Cd stroboscope lamp

EN15 – 80 Aavg Neon-filled, cold-cathode tetrode thyratron, differential trigger electrodes, miniature 9-pin base, for use as a stroboscope lamp

EN30 – 250 Apk Gas-filled, arc-discharge cold-cathode tetrode thyratron, differential trigger electrodes, miniature 7-pin base with anode cap, for use as a relay or as a stroboscope lamp

EN40 – 250 Apk Gas-filled, cold-cathode tetrode thyratron, differential trigger electrodes, 8-pin base, for use as a whitish stroboscope lamp with a high actinism for photographic film

EN55 Single, EDN10 dual xenon-filled, arc-discharge cold-cathode tetrode thyratron, external (capacitive) trigger, 12-pin base, for use as a white 140 kCd stroboscope lamp

EN60 – Gas-filled, arc-discharge cold-cathode tetrode thyratron, external (capacitive) trigger, Edison screw lamp base with anode cap, for use as a white 900 klm@10µF@800V stroboscope lamp

G

Standard Telephones and Cables/Brimar:

G10/241E – Nomotron, an unidirectional Dekatron with multi-alloy cathodes

Cerberus:

G11 – 5 mA Gas-filled, cold-cathode switching diode e.g. for relaxation oscillators, 2-pin all-glass pigtailed

G42 – 35 mApeak Gas-filled switching diode e.g. for relaxation oscillators, 2-pin all-glass pigtailed

GC

Ericsson Telephones Limited:

GC10/2P – Neon-filled, 1 kHz Miniature decade Counter Dekatron, a gas-filled, bidirecional decade counter tube

GC10A – Helium-filled, decade Counter Dekatron

GC10B – Neon-filled, 4 kHz Long life, decade Counter Dekatron

GC10/4B – 4 kHz Decade Computing Counter Dekatron with carry/borrow cathodes "0" and "9" and intermediate cathodes "3" and "5" wired to separate pins

GC10D – 20 kHz Decade Counter Dekatron, for single-pulse operation

GC12/4B – 4 kHz Duodecimal Counter Dekatron with carry/borrow cathodes 11 and 12 and intermediate cathodes 6 and 8 wired to separate pins

GCA

Ericsson Telephones Limited:

GCA10G – 10 kHz max. Decade Counter Dekatron with routing guides and aux anodes to directly drive Nixie tubes, B27A base without the inner pin ring

GD

Ericsson Telephones Limited:

GD2V – 2 kV, 16 J discharge tube, all-glass studded

GD75P – 75 V Voltage reference, miniature 7-pin base

GD90M – 90 V Voltage reference, miniature 7-pin base

GD340X – 345 V/3...200 µA Corona voltage reference, all-glass pigtailed

GD350X, GD350Y – 350 V/3...200 µA Corona voltage reference, all-glass pigtailed

GD550W – 550 V, 1.5 J Discharge tube, e.g. for power relaxation oscillators, all-glass pigtailed

GDT

Ericsson Telephones Limited:

GDT120M – 9 mA Gas-filled cold-cathode DC triode, one starter and a separate glow diode acting as an optical primer, miniature 7-pin base

GE

Ferranti:

GE10 – Directly heated saturated-emission diode. Acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits. It has two shorted pins that can be used to disable the circuit if the tube is removed from its socket

GK

Cerberus:

GK11 – Touch button tube, an illuminated capacitance touch switch; a cold-cathode DC relay tube, external (capacitive) starter activated by touching; then the cathode glow is visible as an orange ring. 2-pin all-glass pigtailed

GN

Ferranti:

GN10 – 250 Amps pulse-current, cold-cathode tetrode thyratron. Octal base

GR

Ericsson Telephones Limited:

GR2G –  + -  Neon-filled digital indicator tube, 18 x 18 mm characters, side-viewing

GR2H –  + -  Neon-filled digital indicator tube, 20 x 20 mm characters, top-viewing

GR4G –  ¼ ½ ¾ 1  Neon-filled digital indicator tube, 18 x 30 mm characters, side-viewing

GR7M –  + - V A Ω % ~  Neon-filled digital indicator tube, 15.5 mm character height, top-viewing

GR10A – Gas-filled digital indicator tube with a dekatron-type readout

GR10G –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16.86 x 30 mm characters, side-viewing

GR10H –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 12 x 19 mm characters, top-viewing

GR10J –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16.86 x 30 mm characters, side-viewing

GR10K –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 12 x 19 mm characters, top-viewing

GR10M –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 10 x 15.5 mm characters, top-viewing

GR10W –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 8.42 x 15 mm characters, side-viewing, all-glass pigtailed

GR12G –  A B C D E F G H I J K L  Neon-filled digital indicator tube, 16 x 30 mm characters, side-viewing

GR12H –  E L M N P R S T U V W X  Neon-filled digital indicator tube, 16 x 30 mm characters, side-viewing

Note: More Nixie tubes under standard - ZM and professional - ZM

Cerberus:

GR15 – 15 mA Gas-filled cold-cathode DC tetrode, one starter and one electrical primer and tritium-primed (half-life: 12.32 years), noval base, for voltage triggers, RC timers etc.

GR16 – 20 mA Gas-filled, cold-cathode, tritium-primed AC/DC triode, one starter and an EM shield, noval base, for voltage triggers, RC timers etc.

GR17 – 15 mA Gas-filled cold-cathode AC triode, one starter and an EM shield, noval base, for voltage triggers, RC timers etc.

GR31 – 15 mA Gas-filled cold-cathode DC tetrode, one starter and one electrical primer plus a tritium primer, noval base

GR44 – 12 mA Subminiature gas-filled cold-cathode DC pentode, two starters and one primer electrode plus a tritium primer, 5-pin all-glass pigtailed

GR46 – 12 mA Subminiature gas-filled cold-cathode DC tetrode, one starter and one primer electrode, 4-pin all-glass pigtailed

GRD

Ferranti:

GRD7 – Educational, directly heated saturated-emission guard ring diode

GS

Ericsson Telephones Limited:

GS10C – 4 kHz max. Decade Counter/Selector Dekatron, top-viewing, duodecal base

GS10D – Hydrogen-filled, 20 kHz max. Decade Counter/Selector Dekatron, duodecal base

GS10H – 4 kHz max. Decade Counter/Selector Dekatron with routing guides, B17A base

GS12C – 4 kHz max. Duodecimal Counter/Selector Dekatron, with solder lugs

GS12D – Neon-filled, 4 kHz max. duodecimal Counter/Selector Dekatron, duodecal base with two additional pigtails for the guide electrodes

GSA

Ericsson Telephones Limited:

GSA10G – 10 kHz max. Decade Counter/Selector Dekatron with routing guides and aux anodes to directly drive Nixie tubes, B27A base

GTE

Ericsson Telephones Limited:

GTE120Y – 5 mA Subminiature DC trigger tetrode, one starter and one primer, all-glass pigtailed

GTE130T – 8 mApeak DC trigger tetrode, one starter and one primer, close tolerance, low aging, quadrant I operation only, noval base

GTE175M – 3.5 mAavg, 50 mApeak DC Trigger tetrode, one starter and one primer, miniature 7-pin base, for Dekatron coupling circuits

GTR

Ericsson Telephones Limited:

GTR120W – 9 mA Subminiature DC trigger triode, 3-pin all-glass pigtailed, for computer applications

GTR75M – 75 V Voltage reference, Miniature 7-pin

GTR95M/S – 95 V Voltage reference, Miniature 7-pin

GTR150 – Subminiature, primed 150 V voltage reference, all-glass pigtailed

K

KN

KN2

Edgerton, Germeshausen, and Grier:

KN2 – 4 kV, 500 Asurge Krytron, a cold-cathode gas-filled tube with a primer electrode, for use as a very high-speed, high-surge current switch; similar to a thyratron, lifespan 107 shots, 4-pin all-glass pigtailed[98]

KN4 – 5 kV, 2.5 kAsurge Krytron with a primer electrode, lifespan 25000 shots, 4-pin all-glass pigtailed

KN6 – 5 kV, 3 kAsurge Krytron with a primer electrode, lifespan 35000 shots, 4-pin all-glass pigtailed

KN6B – 8 kV, 3 kAsurge Krytron with a primer electrode, lifespan 35000 shots, 4-pin all-glass pigtailed

KN9 – 4 kV, 500 Asurge Krytron with a primer electrode, lifespan 1.5⋅107 shots, 4-pin all-glass pigtailed

KN11B – 2.5 kV, 1.5 kAsurge Sprytron, lifespan 2000 shots, 3-pin all-glass pigtailed

KN12 – 5 kV, 3 kAsurge Sprytron, lifespan 500 shots, 3-pin all-glass pigtailed

KN22 – 5 kV, 100 Asurge Krytron with a primer electrode, lifespan 2⋅107 shots, 4-pin all-glass pigtailed, for laser pumping, to drive Pockels cells, also for educational purposes[99]

KN26 – 5 kV, 3 kAsurge Krytron with a primer electrode, lifespan 75000 shots, 4-pin all-glass pigtailed

KT

Marconi-Osram Kinkless Tetrode beam power tubes

KT32 (25L6, 25L6G, 25L6GT and 25W6GT)

KT33 (25A6GT)

KT41

KT61 (6M6G) in parallel filament circuits

KT63 (6F6, 6F6G, 6F6GT)

KT66 (6L6GC)

KT67 – Small transmitting valve

KT71 (50L6GT)

KT77 – Similar to EL34, 6CA7

KT81

KT88 = 6550A = CV5220 (12E13, 7D11) – AF beam power tube, two tubes are capable of providing 100W output, Class-AB1, octal base

KT90

"Tung-Sol":

KT120 – New production tube

KT150 – New production tube

M

M8000s

UK Military developed:

M8083 – Sharp-cutoff pentode, miniature 7-pin base (SQ version of EF91 = 6AM6 = Z77)

M8162 = 6060 – High-mu dual triode, for use as RF amplifier/mixer in VHF circuits, noval base (SQ version of ECC81 = 12AT7 = B309)

MC

Philips:

MC6-16, MC13-16 – CRT-type flying-spot scanners, P16-type phosphor (violet/near-ultraviolet, very short persistence), for use in a telecine

ME

Edison and Swan Electric Light Company (British Mazda/EdiSwan):

ME91 – AC/DC mains tuning indicator

P

PBG

Dale:

PBG12201 – Neon-filled, planar, dual 200-segment linear glow-transfer bar graph display tube with three cathode strings,[100] for use in VU meters etc.; cf. BG08220-K

PBG12203 – Neon-filled, planar, dual 201-segment linear glow-transfer bar graph display tube with three cathode strings, for use in VU meters etc.

PBG12205 – Neon-filled, planar, dual 200-segment linear glow-transfer bar graph display tube with five cathode strings, for use in VU meters etc.

PBG16101 – Neon-filled, planar, dual 101-segment linear glow-transfer bar graph display tube with three cathode strings, for use in VU meters etc.; similar to ИН-33

PD

Edison and Swan Electric Light Company (British Mazda/EdiSwan):

PD220 – Dual AF power triode for battery-supplied equipment (1939)

PL

Philips:

PL21 = 2D21 = EN91 – 100 mAavg, 500 mApeak, 10 Asurge, Gas-filled, indirectly heated tetrode thyratron, negative starter voltage, miniature 7-pin base, for relay and grid-controlled rectifier service

PL323 = 3C23 – 1.5 Aavg, 6 Apeak, Mercury-vapor triode thyratron, 4-pin base with anode top cap

PL5727 = 5727 – 100 mAavg, 500 mApeak, 10 Asurge, Tetrode thyratron, 7-pin miniature base

Q

Philips:

Q13-110GU – CRT-type flying-spot scanner, white phosphor, for use in a telecine

QK

Raytheon:

QK329 – Beam-deflection square-law tube for use as a function generator in analog computers. A flat sheet beam is electrostatically deflected across the anode which is partially covered by a parabolically stenciled screen "grid" that acts as the tube's output. Two tubes may be combined to form a 1-quadrant analog multiplier using the equation a⋅b = (a+b)2 − (a−b)2/4 where the deflection electrode signals a+b and a−b can be obtained directly from a fully balanced resistor bridge[101]

R

Marconi-Osram Valve Company:

R – Early directly heated triode derived from the French TM tube and used by many amateurs in the 1920s[102][103][104]

RK

Raytheon:

RK61 – Miniature, gas-filled, directly heated thyratron designed specifically to operate like a vacuum triode below its ignition voltage, allowing it to both amplify analog signals and work as a relaxation oscillator, for use as a self-quenching superregenreative detector up tp 100 MHz in radio control receivers, activating a relay in its anode circuit when a carrier wave is received; 4-pin all-glass pigtailed, 1.4 V, 45 mA filament, Ua=45 V, Ia=1.5 mA.[105]

RK62 – RK61's predecessor, marketed since 1938;[106] this was the major technical development which led to the wartime development of radio-controlled weapons and the parallel development of radio controlled modelling as a hobby.[107]

S

SB

Radio Corporation of America:

SB256 – 256-bit Selectron tube, an early form of digital computer memory[108][109]

SU

Cossor:

SU25 – EHT rectifier

SU2150 (CV1120) – High-voltage vacuum half-wave rectifier for use in CRT power supplies

T

British General Electric Company:

TuneOn – Early neon-filled bar graph tuning indicator, a glass tube with a short wire anode and a long wire cathode that glows partially; the glow length is proportional to the tube current[110]

TuneOn Button – Early glow modulator used as a budget-priced tuning indicator – a neon lamp whose brightness is proportional to the tube current[111]

Standard Telephones and Cables/Brimar:

Tunograph – Precursor of the "Magic Eye" tuning indicator first introduced in 1933; a tiny CRT with 1-axis electrostatic deflection and a phosphored target at 45° to the electron beam, so the projected green dot can be observed from the side[112]

TH

Compagnie Française Thomson-Houston:

TH9503 – Scripticon, a character generator monoscope for text mode video rendering in early computer monitors, with a square target having letters, digits and symbols patterned on it in an (optionally customer-supplied) 8x8 array. An electron beam selects and scans a character, both by appropriate magnetic deflection, and generates an analog video signal;[113] cf. 4560, CK1414

TM

"Loupiote" – a TM tube

E.C.&A. Grammont and Compagnie des Lampes (1888):

TM – Vacuum triode for amplification and detection of radio signals, developed in France and made since 1915.[114][115][116][117] It became the standard receiving and amplifying tube of the Entente countries during World War I, and the first mass-produced radio tube. TM's production volume in France alone is estimated at 1.1 million units; in addition, the production of TM and/or improved versions was started in the UK (Marconi–Osram R tube), the Netherlands (Philips E tube), the United States and the Soviet Union (R-5, Russian: Р-5).(ru)

The TM was developed in 1914–15 by the French military telecommunications service Télégraphie Militaire on the initiative of their technical director Gustave-Auguste Ferrié. He and his assistant, physicist Henri Abraham, visited the American laboratories on a number of occasions and were aware of the works of Lee de Forest, Reginald A. Fessenden and Irving Langmuir. They knew that de Forest's Audion and Henry Round's British tube were unreliable and imperfect, and Langmuir's Pliotron was too complex for mass production. They also knew about the latest German developments: Soon after the outbreak of the war, Ferrié received extensive information from a former Telefunken employee, the Frenchman Paul Pichon, who, upon return from a mission from his German employer to gather samples of the latest triodes from the USA, had to surrender himself and the samples to the French. The samples Pichon brought performed poorly due to insufficient vacuum. Following the ideas of Langmuir, Ferrié required the industry to guarantee a high vacuum in series production.

In October 1914, Ferrié, Abraham and François Péri from the radiotelegraph centre in Lyon/La-Doua(fr) went to the light bulb department of Société des Téléphones E.C.&Alexandre Grammont in Lyon to develop with them a triode suitable for mass production. The first prototypes, mere copies of de Forest's Audion, proved to be unreliable and unstable; the next ones were rejected for being too complex. Only the fourth prototype developed in December 1914, with a vertical coaxial system, an Edison screw lamp base for the filament and additional side terminals for anode and grid, was deemed suitable for series production, which started in February 1915 and stopped in October 1915 when it became clear that the vertical structure of "Abraham's Lamp" was too fragile and too many tubes were damaged during transport. Ferrié asked Péri to resolve the problem, and two days later Péri and Jacques Biguet came up with a horizontal coaxial system on the latest four-pin type "A" base. The series production of the Péri/Biguet tubes, named TM after Ferrié's service unit, began in November 1915 under Grammont's Radio Fotos brand; this variant became highly successful, and when demand started to exceed Grammont's production capacity, Compagnie des Lampes (1888) in Ivry-sur-Seine also started production under their Métal brand. Ferrié and Abraham were nominated for the 1916 Nobel Prize in Physics for their work in the field of radio communications.

The TM is a cylindrical/coaxial triode; the directly-heated cathode is a filament made of pure tungsten with a diameter of 60 μm, the anode is a nickel cylinder with a diameter of 10 mm and a length of 15 mm. The dimensions and material of the grid depend on the place of production – the Grammont plant in Lyon used molybdenum wire, the CdL plant in Ivry-sur-Seine used nickel. The diameter of the grid spiral is 4 resp. 4.5 mm. The filament required 4 V and 700 mA to bring it up to white heat; the bright glow prompted Grammont in 1923 to start producing TM tubes with dark blue glass envelopes to protect the eyes of radio operators from the blinding glare, and hide the harmless, but unsightly plaque of metal particles inevitably deposited on the inner wall of the bulb while evacuating during production – but also prevented the triodes' previous, secondary use as light sources, which had earned them their nickname Loupiote ("little lamp").

The TM could be used for their intended purpose, amplifying and detecting signals in radio receivers, or as power oscillators in low-power radio transmitters, and also, by paralleling of several tubes, as AF power amplifiers. The Soviet analogue of the TM, the triode R-5, could withstand anode voltages of up to 500...800 V, and was able to deliver a power of up to 1 W in Class-C mode, but only 40 mW in Class-A mode. A typical single-TM radio receiver of World War I ran at Ua=40 V, Ug=0 V, Ia≈2 mA, gm=400 μS, Ri=25 kΩ, μ=10. With an anode voltage of 160 V and a grid bias of -2 V, the anode current was 3...6 mA, while the reverse grid current reached 1 μA.[118]

The problem of TM tubes was their short service life of 100 hours maximum – if the tube was manufactured in strict accordance with the specifications. In wartime, this was not always possible; due to raw materials supply problems, plants sometimes had to use substandard materials. Such tubes were marked with a cross; they differed from the standard by a higher noise level and were prone to catastrophic failures due to cracks in the glass envelope.

Media related to French triode "Loupiote" at Wikimedia Commons

TT

Bendix:

TT8, TT9, TT13, TT15, TT17, TT18, TT20, TT21, TT22 – Chronotron, integrating, balanced-bridge hot-wire/PTC time delay devices

Marconi-Osram Valve Company:

TT11 – Low power VHF transmitting beam tetrode

TT21 – RF power beam-tetrode derived from KT88

TT100 – RF power beam-tetrode

V

VHT

Ferranti:

VHT1 – Pentagrid converter, 1933

VS

Ericsson Telephones Limited:

VS10G – Trochotron, an electron-beam decade counter tube

VS10G-M – VS10G with a magnetic shield

VS10H – High-current trochotron

VS10K – Low-voltage trochotron

List of other number tubes

1

175HQ – Ultra high reliability pentode for use in long-haul submarine communications cable repeaters[119]

1600s

1602 – Directly heated triode used for A.F. amplification with low microphonics. 7.5 volt heater/filament. 12 watts of A.F. operating in Class-A. 15 watts of low R.F. operating in Class-C. Similar to type 10.

1603 – Indirectly heated pentode used for A.F. amplification with low microphonics. Similar to types 6U7, 57, 6D6 and 6C6. UX6 Base.

1608 – Directly heated triode giving 20 watts at up to 45 MHz. 2.5 volt heater/filament. UX base.

1609 – Directly heated pentode used for A.F. amplification with low microphonics. American 5-Pin(UY)base.

1610 – Directly heated pentode specially designed for use as a crystal oscillator. 2.5 volt heater/filament, American 5-Pin base.

1612 – Pentagrid converter; low-microphonics version of type 6L7. Both control grids (1 and 3) are sharp-cutoff.

1619 – Beam Power Tetrode, similar to 6L6 with directly heated filament, common in World War II battle tank transmitters.

1624, 1625 – Very similar to the 807, but with different heater voltage

1626 – RF triode, very similar to 6J5 but with 12.6 volt filament

1629 – Tuning indicator tube with DC amplifier triode unit

1630 – Indirectly heated, orbital-beam, secondary-emission, 12-pin Jumbo Acorn-type UHF hexode

1633 – Dual triode, equivalent to 6SN7 with 26 volt filament (World War II aircraft use)

1635 – Indirectly heated, 10.4 W dual AF power triode, 8-pin base

1636 – Secondary emission UHF beam deflection tube, used as a balanced mixer up to 600 MHz[120]

1650 – High-altitude version of the 955 Acorn-type triode

1680 – Dual-control heptode for use as a NAND gate in a coincidence circuit in IBM computers, 6BE6/EK90 with a sharp-cutoff grid no.3

2

24B1 – Trigatron

24B9 – Trigatron

29C1 – Directly heated saturated-emission diode; acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits.

200s

203A – 40 W, Directly heated RF transmitter power triode, 4-pin base, anode on top cap

204A – 450 W, Directly heated RF transmitter power triode, 3-pin base, anode on top cap

205D – 1 W, Directly heated AF or modulator power triode, 4-pin base

207 – 22.5 kW, Water-cooled, directly heated RF transmitter power triode

210T – Directly heated RF transmitter power triode, 4-pin base, similar to type 10 triode with an isolantite base

210DET – Cossor directly heated, 2 volts, special detector

210HF – Cossor, directly heated, 2 volts, triode

210HL – Cossor, directly heated, 2 volts, triode

210LF – Cossor, directly heated, 2 volts, triode

210PG – Cossor, directly heated, 2 volts, variable-mu pentagrid

210RC – Cossor, directly heated, 2 volts, very high impedance triode

210SPT – Cossor, directly heated, 2 volts, HF pentode

210VPT – Cossor, directly heated, 2 volts, HF variable-mu shielded pentode

211 – 260 W, Directly heated AF or modulator power triode now favored by audiophiles; 4-pin base

212E – 250 W, Directly heated RF transmitter power triode, 4-pin base

215P – Directly heated AF power triode

220B – 5 kW, Water-cooled, directly heated AF/modulator power triode

228 – 2.5 kW, Directly heated RF/AF power triode

230XP – Directly heated power triode

232C – 8.5 kW, Water-cooled, directly heated RF transmitter power triode

236A – 12 kW, Water-cooled, directly heated RF transmitter power triode

240B – Directly heated dual AF power triode

241B – 300 W, Directly heated AF/modulator power triode, 3-pin base, anode on top cap

242A – Directly heated AF/modulator power triode, 4-pin base

250TH – 1.1 kW, Directly heated AF/modulator power triode, 4-pin base, anode on top cap

254A – 14 W, Directly heated RF transmitter power triode, 4-pin base, anode on top cap

261A – 170 W, Directly heated AF/modulator power triode, 4-pin base

268A – 20 W, Directly heated power triode, 4-pin base, anode on top cap

270A – 585 W, Directly heated AF/RF power triode, 4-pin base, anode on top cap

275A – 3 W, Directly heated AF power triode, 4-pin base

276A – 170 W, Directly heated AF/RF power triode, 4-pin base

279A – 1.75 kW, Directly heated AF/RF power triode

295A – 125 W, Directly heated AF/RF power triode, 4-pin base

298A – 5 kW, Water-cooled, directly heated power triode

3

300s

300B – 40 watt directly heated power triode, 4-pin base

316A = VT191 – Directly heated Doorknob-type UHF power triode up to 750 MHz

322 – Oil can-type disk-seal UHF clipper power diode, 800 VPIV, 15 W, 1500 MHz

328 – Tungar bulb, a low-voltage, gas-filled, full wave rectifier for charging 12V lead-acid batteries at 1.3 A

368A – Directly heated Doorknob UHF power triode, graphite anode, up to 1.7 GHz[121]

388A – Directly heated Doorknob UHF power triode, graphite anode, up to 1.7 GHz[121]

4

4XP – Directly heated power triode

41MP – Indirectly heated power triode

45 (CX345) – Directly heated power triode

400s

402P – Indirectly heated power triode, 7-pin base

416B – Planar SHF power triode, 500 mW output at 4 GHz

416D – Planar SHF power triode with BeO spacers, 5 W output at 4 GHz

446A – Early Lighthouse UHF triode, 10 dB noise figure at 1 GHz

450TH – Early Eimac high-mu power triode, 450 watt plate dissipation to 40 MHz[122]

455A – Ultra high reliability pentode for use in submarine communications cable repeaters[123]

5

5BP4 – Five-inch CRT used in pre-World War II television receivers, such as the RCA TRK-5 and in early radars such as the SCR-268 and SCR-270.[124]

5CEP11 (blue, short persistence); 10VP15, 5AKP15, 5DKP15, 5ZP15 (blue-green, extremely short); 5BNP16, 5CEP16, 5DKP16, 5ZP16 (violet/near-ultraviolet, very short); 5AKP24, 5AUP24, 5DKP24, 5ZP24 (green, short); 131QP55 (blue-green, very short); 131QP56 (blue-violet, very short) – CRT-type flying-spot scanners for use in a telecine

500s

527 – High-mu power triode up to 900 W

559 – Lighthouse-type disk-seal UHF diode

592 = 3-200A3 – Medium-mu power triode up to 200 W, 150 MHz

6

6P10 – Ultra high reliability pentode for use in short-haul submarine communications cable repeaters[119]

6P12 – Ultra high reliability pentode for use in long-haul submarine communications cable repeaters[119]

7

7JP1 – Monochrome cathode ray tube for use in early postwar oscilloscopes. Electrostatic deflection, P1 green, short-persistence phosphor, 7" screen.

7JP4 – Monochrome cathode ray tube common in early postwar TV receivers. Electrostatic deflection, P4 white, medium-persistence phosphor, 7" screen.

7JP7 – Monochrome cathode ray tube for use in early postwar radar displays. Electrostatic deflection, P7 blue-white, long-persistence phosphor, 5½" screen.

700s

703A – Directly heated Doorknob UHF power triode up to 1.5 GHz

713A – Indirectly heated Doorknob UHF power pentode

717A (CV3594, VT269) – Indirectly heated Doorknob UHF power pentode

8

800s

800 – Directly heated V.H.F. power triode, giving 35 watts up to 60 MHz and 18 watts at 180 MHz. American 4-Pin(UX)base with side locating pin.

801 – Directly heated power triode, used in pairs in Class-B in A.M. modulation sections of transmitters giving up to 45 watts of power at 60 MHz and 22 watts at 120 MHz.

802 – Indirectly heated H.F. power pentode, giving 8 watts up to 30 MHz and 4 watts at 110 MHz.

803 – Directly heated H.F. power pentode, giving 50 watts up to 20 MHz and 25 watts at 70 MHz.

804 – Directly heated H.F. power pentode, giving 20 watts up to 15 MHz and 10 watts at 10 MHz.

805 – Directly heated H.F. high-mu triode, giving 140 watts up to 30 MHz and 70 watts at 85 MHz.

806 – Directly heated H.F. high-mu triode, giving 390 watts up to 30 MHz 195 watts at 100 MHz.

807 – Indirectly heated H.F. beam power tetrode, giving 25 watts up to 30 MHz and 12 watts at 125 MHz. A variation of type 6L6 originally designed as a Class-C transmitter tube. Later used in pairs as push-pull outputs for high-wattage Class-AB2 audio amplifiers. Also used as a horizontal output tube in early TV receivers. One of the first commercial tubes that used the top cap to connect the anode (instead of the control grid) to the circuit.

808 – Directly heated H.F. high-mu triode, giving 140 watts up to 30 MHz and 70 watts at 130 MHz.

809 – Directly heated H.F. high-mu triode, giving 55 watts up to 27 MHz and 30 watts at 100 MHz.

810 – Directly heated H.F. triode, 10 volt filament and Zirconium Carbide anode. Base fits R.C.A. UT-541A Socket.

811A – Directly heated H.F. triode, 6.3 volt filament, 88 watts

813 – Beam Power Tetrode possessing about 5 times the Anode dissipation of an 807.

814 – A directly heated Beam Power Tetrode giving about 130 watts at 30 MHz and 65 watts at 100 MHz operating in Class-C.

815 – An indirectly heated dual Pentode. International Octal, (IO), base.

825 – First commercially available klystrode, a VHF/UHF linear-beam transmitting tube, similar to a klystron

829 – A dual indirectly heated beam power tetrode. Two 6.3 volt heaters sharing a common tap.

830 – A directly heated triode giving about 50 watts at 15 MHz and 7.5 watts at 60 MHz operating in Class-C.

831 – A directly heated triode giving about 400 watts at 20 MHz and 200 watts at 60 MHz operating in Class-C. 11 volt heater/filament.

833 – A larger directly heated high-mu triode giving about 1 kW at 30 MHz and 500 watts at 45 MHz operating in Class-C. Usable up to 100 MHz at reduced power, (400 W). 10 volt heater/filament drawing 10 A. The anode of this device is fabricated from tantalum. Anode current of 800 mA with an anode voltage of 3 kV and grid voltage of zero. Anode current of 4.3 A at a voltage of 750 with 350 volt on the grid. Uses two-part R.C.A socket assembly UT-103.[125]

833A – Improved 833.

834 – A directly heated triode giving 58 watts at 100 MHz and 25 watts at 350 MHz operating in Class-C. 7.5 volt heater/filament. Fitted with an American 4-Pin, (UX4), base with side locating pin.

836 – An indirectly heated high vacuum rectifier with a peak inverse voltage of 5 kV and peak anode current of 1 ampere. 2.5 volt heater.

837 – An indirectly heated pentode giving 11 watts at 20 MHz and 5 watts at 80 MHz. operating in Class-C. 12.6 volt heater.

838 – A directly heated triode giving about 100 watts at 30 MHz operating in Class-C. 10 volt heater/filament.

841 – A directly heated high-mu triode giving about 10 watts at 6 MHz and 5 watts at 170 MHz operating in Class-C. 7.5 volt heater/filament.

842 – A directly heated triode giving about 3 watts at 6 MHz operating in Class-C. 7.5 volt heater/filament.

843 – An indirectly heated tetrode giving gain at 6 MHz and usable up to 200 MHz operating in Class-C. 2.5 volt heater/filament.

844 – A directly heated triode giving gain at 6 MHz and usable up to 155 MHz operating in Class-C. 2.5 volt heater/filament.

845 – A directly heated triode giving up to 24 watts if undistorted power in Class-A at audio frequency with an anode voltage of 1250. 11 volt heater/filament.

849 – A directly heated triode giving gain at 3 MHz operating in Class-C. Two 849s, working in push-pull Class-B are capable of delivering 1.1 kW of audio output with an anode voltage of 3 kV. Usable up to 30 MHz. 11 volt filament/heater.

850 – A directly heated tetrode giving 120 watts of power gain up to 13 MHz and 50 watts at 100 MHz, operating in Class-C. 10 volt heater/filament.

851 – A directly heated triode giving 1.5 kW of power up to 3 MHz operating in Class-C. 11 volt heater/filament.

852 – A directly heated triode giving 75 W of power up to 30 MHz operating in Class-C. 10 volt heater/filament.

857B – Large mercury-vapor rectifier used in 50 kW class broadcast transmitters. 22 kV anode voltage, 10 A anode current. Filament 5 V @ 30 A

860 – A directly heated tetrode giving 105 W of power up to 30 MHz and 50 watts at 120 MHz operating in Class-C. 10 volt heater/filament.

861 – A directly heated triode giving 400 W of power up to 20 MHz and 200 watts at 60 MHz operating in Class-C. 11 volt heater/filament.

862 – Large water-cooled triode for broadcast/industrial applications. Used in experimental 500 kW transmitter at WLW.

864 – A directly heated general-purpose, low-microphonics triode with a maximum anode voltage of 135 volts and anode current of 3.5 mA. 1.1 volt heater/filament.

865 – A directly heated tetrode giving 30 W of power up to 15 MHz 15 watts at 70 MHz operating in Class-C. 11 volt heater/filament.

866

866A

– A mercury-vapor rectifier with a peak inverse voltage of 5 kV and peak anode current of 1 ampere. Average anode current, 250 mA, forward drop, 15 volt. Heater voltage and current, 2.5 at 5 A. American 4-Pin(UX) base.

866A – Improved 866 with a peak inverse voltage of 10 kV and a forward drop of 10 volt.

872 – A mercury-vapor rectifier with a peak inverse voltage of 5 kV and peak anode current of 5 amperes. Average anode current, 1250 mA, forward drop, 15 volt. Heater voltage, 5.0 at 10 A. Base fits R.C.A. UT-541A Socket.

872A – Improved 872 with a peak inverse voltage of 10 kV, a forward drop of 10 volt and a heater current of 6.25 A.

879 – A high vacuum rectifier with a peak inverse voltage of ca. 15 kV and peak anode current of ca. 5 mA. 2.5 volt heater and American 4-Pin, (UX) base. Used as half wave rectifier for high voltage cathode ray tube supplies. Similar to type 2X2.

884 – An indirectly heated triode thyratron. 6.3 volt heater/filament, International Octal, (IO), base. Electrically similar to type 885. Once commonly used as a sawtooth horizontal sweep waveform generator in recurrent-sweep oscilloscopes. Marketed by DuMont under the type number 6Q5.

885 – An indirectly heated triode thyratron. 2.5 volt heater/filament, American 5-Pin (UY) base. Otherwise similar to type 884.

898 – Large water-cooled triode for broadcast/industrial applications. Updated version of 862, with 3 phase filament structure.

9

900s

934 – Vacuum Phototube, spectral S4 response (maximum sensitivity at 400±50 nm), 3-pin Small-Shell Peewee base

935 – Vacuum Phototube, spectral S5 response (maximum sensitivity at 340±50 nm), 4-pin octal base

950 – Power pentode with directly heated cathode, used in storage battery home radios with 2.0 volt filament supply. Similar to types 1F4 and 1J5G

951 – Sharp-cutoff pentode with directly heated cathode, used in storage battery home radios with 2.0 volt filament supply. Similar to type 1B4P

953 – Acorn-type UHF diode; 6.3 V heater

954 (4672/E1F) – Indirectly heated Acorn-type sharp-cutoff pentode giving gains of 2...3 up to 300 MHz operating in Class-A and usable up to 600 MHz with careful stage design; 6.3 V heater

955 (4671/E1C) – Indirectly heated Acorn-type triode giving a power of 135 mW up to 600 MHz operating in Class-A and 500 mW in Class-C with careful stage design; 6.3 V heater

956 (4695/E2F) – Indirectly heated Acorn-type remote-cutoff pentode giving gains of 3...4 up to 600 MHz operating in Class-A with careful stage design; 6.3 V heater

957 (D1C) – Directly heated Acorn-type UHF receiving triode; 1.25 V filament for portable equipment

958 (D2C) – Directly heated Acorn-type UHF transmitting triode with dual, paralleled 1.25 V filaments for increased emission, for portable equipment

958A – 958 with tightened emission specs

959 (D3F) – Directly heated Acorn-type sharp-cutoff UHF pentode; 1.25 V filament for portable equipment

991 – 60-Volts Voltage reference, T4½ lightbulb with dual-contact, bayonet candelabra mount

9000s

9001 – 954 with a miniature 7-pin base

9002 – 955 with a miniature 7-pin base

9003 – 956 with a miniature 7-pin base

9004 – Acorn UHF diode

9005 – Acorn UHF diode with a 3.6 V heater

9006 – Detector diode with a miniature 7-pin base

List of tubes used in 1920s and 1930s radio receivers

[126]

Tubes with directly heated cathodes

Used with AC, DC or home-based storage battery power supplies (1927–31)

With 1.1 Volt DC filaments

Used in 1920s home radios. Filaments powered by 1.5 volt dry cells, anodes powered by storage batteries.

WD-11 – triode/detector

With 2.0 Volts DC filaments

Used in 1930s home radios powered by storage batteries.

19 – Dual power triode – also used in farm radios with 6-volt vibrator power supplies. Early version of octal type 1J6G.

20 – Power triode – Early versions numbered UX-120.

22 – Sharp-cutoff tetrode – Early versions numbered UX-222 or CX-322.

25S – Dual detector diode, medium-mu triode. Identical to type 1B5. Usually numbered 1B5/25S.

30 – Medium-mu triode, An upgraded version of type 01-A – Early versions numbered RCA-230 or CX-330. Can also be used as a power triode. The type 30 was popular amongst amateurs of the day. Early UX4 based version of octal type 1H4G.

31 – Power triode, UX4 based – Early versions numbered RCA-231 or CX-331.

32 – Sharp-cutoff tetrode – Early versions numbered RCA-232 or CX-332.

33 – Power pentode – Early versions numbered RCA-233 or C-333.

34 – Remote-cutoff tetrode – Early versions numbered RCA-234 or CX-334.

49 – Dual-grid power triode, similar to type 46

With 3.3 Volts DC filaments

Used in 1920s home radios powered by dry cells (filaments) and storage batteries (B-plus voltage).

V99 – Low-mu triode. Except for stub-pin bayonet base and pinout, electronically similar to X99

X99 – Similar to V99, but with standard pins and different basing arrangement (pinout).

With 5.0 Volts DC filaments

Used in 1920s home radios powered by storage batteries.

00-A – Detector triode with a trace of argon. "00-A" is the number used in most tube manuals. Numbers for earlier versions include UX-200-A and CX-300-A (long pins, push-in socket) and UV-200-A (stub pins, bayonet socket).

01-A – All-purpose low-mu triode, used as RF amplifier, detector, AF amplifier and power triode. The most popular tube of the 1920s. "01-A" is the number used for replacements manufactured after 1930 and in tube manuals. Numbers for early versions include UX-201-A and CX-301-A (long pins, push in socket) and UV-201-A (stub pins, bayonet socket).

Note: There were four tubes in the "01" series, each with different current ratings for their filaments. Type 01-A was the most commonly used.

Types UV 201 and UX 201 – 1.0 ampere

Type 01-A (UV 201-A, UX 201-A, etc.) – 250 milliampere

Type UX 201-B – 125 milliampere

Type UX 201-C – 60 milliampere

12-A – Medium-mu triode, often used as detector, audio driver or audio output, but not as an RF amplifier. This type is listed in tube manuals after 1930 for replacements purposes. Also referred to as type 112-A. Many early versions are marked UX-112-A or CX-112-A.

40 – Medium-mu triode – Early versions numbered UX-240. Introduced in 1927, this was an upgraded version of the "01" series. The "01" series had an amplification factor of 8, while type 40 had an amplification factor of 30. (By comparison, the two AC triodes introduced in the same time period – types 26 and 27 – had amplification factors of 8.3 and 9, respectively.) Because this was the highest-amplification triode available, advertising literature of the time lists it as a high-mu triode, although it is now classified as a medium-mu triode. Type 40 had the highest amplification factor of any triode until the introduction in 1932 of diode/triode complex type 2A6, which had an amplification factor of 100. It also had the highest amplification factor of any DC filament triode until the introduction in 1939 of complementary diode/triode complex types 1H5GT (octal) and 1LH4 (loctal), which both had amplification factors of 65.

Other directly DC-heated tubes

2HF – Tube-based "integrated circuit" with 2 tetrodes and passive components in the same envelope

3NF – Tube-based "integrated circuit" with 3 triodes and passive components in the same envelope. 4V heater

WG38 – Tube-based "integrated circuit" with 2 pentodes, a triode and passive components in the same envelope

Directly AC-heated power tubes

10 – Power triode – Early versions numbered UX-210 or CX-310.

26 – Medium-mu triode, used in early AC radio receivers manufactured in the late 1920s. Used as an RF or AF amplifier, but not as an dectetor or power output tube – Early versions numbered UX-226 or CX-326.

45 – Power Triode – Early versions numbered UX-245 or CX-345.

46 – Dual grid power triode – Grids 1 and 2 connected together for use as push-pull Class-B outputs, Grid 2 and anode connected together for use as single-tube audio driver.

47 – Power pentode – Early versions numbered RCA-247 or C-347.

50 – Power triode – Early versions numbered UX-250 or CX-350.

71-A – Power triode – This type listed in tube manuals after 1930 for replacements purposes. Also referred to as 171-A. Many early versions numbered as UX-171-A or CX-371-A.

Directly AC-heated rectifier tubes

80 – Full-wave rectifier used in early power supplies or battery eliminators (electronically similar to 5Y3G) – Early versions numbered UX-280 or CX-380

81 – Half-wave rectifier – Early versions numbered UX-281 or CX-381.

82 – Full-wave mercury-vapor rectifier

83 – Full-wave mercury-vapor rectifier

83-V – High-vacuum version of type 83, Early UX4 based version of octal type 5V4G.

Tubes with indirectly heated cathodes

With DC heaters

15 – Sharp-cutoff pentode, used in farm radios, in autodyne circuits requiring a separate cathode.

48 – Power tetrode, used in 32-volt farm radios. When two are parallel-connected, they can operate with anode and screen voltages as low as 28 volt.

For use with an AC heating transformer

Note: All have 2.5 volt heaters.

24 – Sharp-cutoff tetrode, UX5 based, Early versions numbered UY-224 and C-324

24-A – an upgraded version of type 24, see type 24 above. Early versions numbered UY-224A and C-324A

27 – Medium-mu triode, UX5 based, Early versions numbered UY-227 and C-327. The first North American tube with an indirectly heated cathode, which is necessary for detector circuits in AC powered tube radios.

35 – Remote-cutoff tetrode, UX5 based, (Commonly branded as 35/51). Early versions numbered UY-235 or C-335

51 – Similar to 35, see type 35 above. (Commonly branded as 35/51)

53 – Dual power triodes, Class-B, UX7 based, (Except for heater, electronically similar to 6A6 and octal based 6N7)

55 – Dual-diode, medium-mu triode, UX6 based, (Except for heater, electronically similar to type 85, and octal based 6V7G, but not to 75)

56 – Medium-mu triode, UX5 based, (Except for heater, electronically similar to 76, and octal based 6P5G)

57 – Sharp-cutoff pentode used in cabinet and mantel radio receivers, UX6 based, (Except for heater, electronically similar to 6C6 and octal based 6J7G, and somewhat similar to type 77)

58 – Remote-cutoff pentode, UX6 based, (Except for heater, electronically similar to 6D6 and octal based 6U7G, but not to 78)

59 – Power pentode, UX7 based.

For use with AC/DC or vehicle-based storage-battery power supplies

Note: All have 6.3 volt heaters except type 43

1-V – Half-wave rectifier with indirectly heated cathode, UX4 based, (often branded as type 1V/6Z3). Early version was KR-1.

36 – Sharp-cutoff tetrode, UX5 based. Early versions numbered RCA-236 or C-336

37 – Medium-mu triode, UX5 based. Early versions numbered RCA-237 or C-337

38 – Power pentode, UX5 based. Early versions numbered RCA-238

39 – Remote-cutoff pentode, UX5 based (Commonly branded as 39/44).

41 – Power pentode, Early UX6 based version of octal type 6K6G, and loctal type 7B5.

42 – Power pentode, Early UX6 based version of octal type 6F6G, Except for heater, similar to types 2A5 and 18.

43 – Power pentode, Early UX6 based version of octal type 25A6G, Commonly used in AC/DC radios.

44 – Similar to type 39, see type 39 above. (Commonly branded as 39/44).

75 – Dual-diode, high-mu triode. Early UX6 based version of octal types 6B6G & 6SQ7GT, and loctal type 7B6, and 7-pin miniature type 6AV6. Also except for heater, electronically similar to 2A6.

76 – Medium-mu triode, Early UX5 based version of octal type 6P5G.

77 – Sharp-cutoff pentode, Early UX6 based version of octal type 6J7G.

78 – Remote-cutoff pentode, Early UX6 based version of octal type 6K7G.

79 – Dual power triode, Early UX6 based version of octal type 6Y7G.

84 – Full-wave rectifier with indirectly heated cathode, (often branded as type 84/6Z4). Early UX5 based version of octal type 6X5GT, and loctal 7Y4, and 7-pin miniature 6X4.

85 – Dual-diode, medium-mu triode. Early UX6 based version of octal type 6V7G, except for heater voltage similar to type 55. Also somewhat similar to octal type 6SR7GT and 7-pin miniature types 6BF6.

89 – Power pentode, UX6 based.

Shielded tubes for Majestic radios

In the early 1930s, the Grigsby-Grunow Company – makers of Majestic brand radios – introduced the first American-made tubes to incorporate metal shields. These tubes had metal particles sprayed onto the glass envelope, copying a design common to European tubes of the time. Early types were shielded versions of tube types already in use. (The shield was connected to the cathode.) The Majestic numbers of these tube types, which are usually etched on the tube's base, have a "G" prefix (for Grigsby-Grunow) and an "S" suffix (for shielded). Later types incorporated an extra pin in the base so that the shield could be connected directly to the chassis.

Replacement versions from other manufacturers, such as Sylvania or General Electric, tend to incorporate the less expensive, form-fitting Goat brand shields that are cemented to the glass envelope.

Grigsby-Grunow did not shield rectifier tubes (except for type 6Y5 listed below) or power output tubes.

Early types based on existing tubes. (Non-shielded versions may be used, but add-on shielding is recommended.)

G-2A7-S – Pentagrid converter

G-2B7-S – Semiremote-cutoff pentode, dual detector diode

G-6A7-S – Pentagrid converter

G-6B7-S – Semiremote-cutoff pentode, dual detector diode

G-6F7-S – Remote-cutoff pentode, medium-mu triode

G-25-S – Medium-mu triode, dual detector diode for 2.0 volt storage battery radios. Glass type 1B5/25S used for replacement.

G-51-S – Remote-cutoff tetrode

G-55-S – Medium-mu triode, dual detector diode

G-56-S – Medium-mu triode

G-56A-S – Medium-mu triode, original version of type 76, but with 400 milliampere heater. (Not to be confused with types 56 or G-56-S, which has a 2.5 volt, 1.0 ampere heater.)

G-57-S – Sharp-cutoff pentode

G-57A-S – Sharp-cutoff pentode, original version of type 6C6, but with 400 milliampere heater. (Not to be confused with types 57 or G-57-S, which has a 2.5 volt, 1.0 ampere heater.)

G-58-S – Remote-cutoff pentode

G-58A-S – Remote-cutoff pentode, original version of type 6D6, but with 400 milliampere heater. (Not to be confused with types 58 or G-58-S, which has a 2.5 volt, 1.0 ampere heater.)

G-85-S – Similar to G-55-S, but with 6.3 volt heater.

Later types

6C7 – Medium-mu triode, dual detector diode, similar to later octal types 6R7 and 6SR7. Seven pin base. (Shield to pin 3.)

6D7 – Sharp-cutoff pentode, identical to type 6C6, but with 7-pin base. (Shield to pin 5.)

6E7 – Remote-cutoff pentode, identical to type 6D6, but with 7-pin base. (Shield to pin 5.)

6Y5 – Dual rectifier diode, similar to type 84/6Z4, but with 6-pin base. (Shield to pin 2.)

Other tubes unique to Majestic radios

G-2-S and G-4-S – Dual detector diodes with common cathodes. The first detector diodes packaged in a separate tube. Forerunners of octal type 6H6. Spray-shielded. Both tubes have 2.5 volt heaters. G-2-S is larger and has a 1.75 ampere heater. Type G-4-S has a 1.0 ampere heater. Later Sylvania replacement type 2S/4S has a 1.35 ampere heater.

2Z2/G-84 – Half-wave rectifier diode with 2.5 volt indirectly heated cathode. A lower-voltage version of type 81. Not interchangeable with type 6Z4/84.

6Z5 – Full-wave rectifier, similar to types 6Z4/84 and 6X5, but with 12.6 volt center-tapped heater.

Rarely used tubes

14 – Similar to 24-A but with a 14 volt, 300 milliampere heater. Used in Philco models 46 and 46E

17 – Similar to 27 but with a 14 volt, 300 milliampere heater. Used in Philco models 46 and 46E

18 – Similar to 2A5 and 42 but with a 14 volt, 300 milliampere heater. No known commercial use.

29 – Wunderlich detector. Known to have been manufactured by Sylvania.

52 – Dual grid power triode similar to types 46 and 49. Has 6.3 volt filament. Most commonly used in early car radios.

64 – Sharp-cutoff tetrode (Except for 400 milliampere heater, similar to 36)

65 – Remote-cutoff pentode (Except for 400 milliampere heater, similar to 39)

67 – Medium-mu triode (Except for 400 milliampere heater, similar 37)

68 – Power pentode (Except for 400 milliampere heater, similar to 38)

69 – Wunderlich detector

70 – Wunderlich detector used in Mission Bell model 19 car radio. Listed in early Philco tube lists.

90 – Wunderlich detector

92 – Wunderlich detector

95 – Original number of type 2A5

181 – Power triode

182-B – Similar to 482-B below.

183 – Similar to 483 below.

213 – Early version of type 80 – Often numbered UX-213

216 – Early version of type 81 – Often numbered as UX-216-B

482-B – Power triode with directly heated cathode. Used in Sparton AC radios, circa 1929. Replacements often numbered 182-B/482-B. Similar to type 71-A, but with higher anode voltage.

483 – Power triode with directly heated cathode. Used in Sparton AC radios, circa 1929. Replacements often numbered 183/483. Similar to type 45, but with a 5.0 volt, 1.25 ampere heater.

485 – Medium-mu triode with indirectly heated cathode. Used in Sparton AC radios, circa 1929. Similar to types 56 and 76, but with a 3.0 volt, 1.25 ampere heater, and lower anode voltage.

Lettered loctal tubes used in Philco radios

FM1000 – Unusual pentagrid for use as oscillator and coincidence-type phase detector in a PLL FM quadrature detector. The anode signal is loosely coupled into the oscillator tank and pulls it to stay quadrature-phase-locked with the IF; manufactured by Sylvania and used in Philco AM/FM radios of the late 1940s and early 1950s. Predecessor of the nonode approach

XXB – Medium-mu twin triode, also numbered 3C6/XXB

XXD – Medium-mu twin triode, also numbered 14AF7/XXD

XXFM – High-mu triode, twin diode (one shares its cathode with the triode, one with separate cathode), also numbered 7X7/XXFM

XXL – Medium-mu triode, also numbered 7A4/XXL

Russian tubes

Media related to Soviet vacuum tubes at Wikimedia Commons

Media related to Soviet gas discharge tubes at Wikimedia Commons

List of standard tubes, with American equivalents

Note: Typecode explained above.

6J1J 6Ж1Ж (954) – Indirectly heated Acorn-type sharp-cutoff pentode, 6.3 V heater

6K1J 6К1Ж (956) – Indirectly heated Acorn-type remote-cutoff pentode, 6.3 V heater

6L1P 6Л1П – Nonode for FM quadrature detection

6N1P 6Н1П – Dual triode, similar to 6DJ8/ECC88

6N2P 6Н2П – Dual triode, similar to 12AX7/ECC83

6N3P 6Н3П (2C51) – Dual triode

6N8S 6Н8С (6SN7/ECC33) – Dual triode

6N9S 6Н9С (6SL7) – Dual triode

6N13S 6Н13С (6AS7G) – Dual power triode

6N14P 6Н14П – Dual RF/VHF triode, similar to ECC84/6CW7

6N23P 6Н23П (6DJ8/ECC88) – Dual triode

6N24P 6Н24П (ECC89/6FC7/6ES8) – Dual RF/VHF triode for cascode amps

6P1P 6П1П – Power pentode, similar to 6AQ5/EL90

6P3S 6П3С – Beam-power tetrode, similar to 6L6GB

6P3S-E 6П3С-Е – Beam-power tetrode, similar to 5881/6L6WGB

6P6S 6П6С (6V6) – Beam-power tetrode

6P14P 6П14П (6BQ5/EL84) – Power pentode

6S1J 6С1Ж (955) – Indirectly heated Acorn-type triode, 6.3 V heater

6S19P 6С19П – Output triode

List of professional tubes

Note: Typecode explained above.

V1-0.15/55 В1-0.15/55 – 55 kV, 150 mA Half-wave rectifier

VI1-5/20 ВИ1-5/20 – 20 kV, 5 A Half-wave pulse rectifier

G-807 Г-807 – Shortwave transmitter tube (the Russian 807 analogue).

GI-7B ГИ-7Б – Impulse tube

GM-70 ГМ-70 – Modulator tube

GS-31B ГС-31Б – UHF transmitter tube

GU-29 ГУ-29 – VHF transmitter tube

GU-50 ГУ-50 – VHF transmitter pentode, similar to the German LS-50 (no direct U.S. equivalent)

GU-78B ГУ-78Б – VHF transmitter tetrode

I3-70-0.8A И3-70-0.8 – 800 V, 70 A Ignitron

I3-200-1.5A И3-200-1.5 – 1.5 kV, 200 A Ignitron

SG203K СГ203К – 82 V Voltage reference

SG204K СГ204К – 164 V Voltage reference

TGI1-270/12 ТГИ1-270/12 – 12 kV, 270 A Hydrogen thyratron

List of indicator tubes

IN-33 ИН-33 – Neon-filled, planar, dual 105-segment linear glow-transfer bar graph display tube with three cathode strings, for use in VU meters etc.; similar to PBG16101

⚫ 	⚫ 	⚫ 	⚫

⚫ 	⚫ 	⚫ 	⚫

⚫ 	⚫ 	⚫ 	⚫

⚫ 	⚫ 	⚫ 	⚫

ITM2-M ИТМ2-М – Four-color phosphored-thyratron latching pixel; 4x4 array of 4 subminiature dual-starter luminiscent thyratrons each for the colors red, yellow, green and blue (thus, 5 intensities per color yields 54 = 625 colors), 4x4 matrix of 10-volts sensitive starter electrodes, cubic envelope for easy stacking in both axes, 12-pin all-glass pigtaled,[127] similar to today's RGBA LEDs

ITS1A ИТС1А – Green phosphored-thyratron latching seven-segment display, no decimal point, 5-volts sensitive starter electrodes, all-glass pigtaled, rectangular envelope for easy stacking in both axes

MTX-90 МТХ-90 – Small neon-filled thyratron for use as a latching single-dot indicator, top-viewing, top of envelope acts as a magnifier, all-glass pigtaled, comes with a blob of solder on the end of each wire for rapid installing, like today's ball grid arrays

List of other tubes

LP-4 ЛП-4 – Linear trochotron, 26-pin Acorn-type all-glass pigtaled,[128]

Compagnie des Lampes (1921, "French Mazda") and Mazda-Belvu

Not to be confused with Compagnie des Lampes (1888, see above) nor with British Mazda (see above).

The 1921 incarnation of La Compagnie des Lampes (since 1953 as Lampe Mazda) made light bulbs and electronic tubes under the French Mazda brand. Many of their tubes were also available from Compagnie Industrielle Française des Tubes Electroniques (CIFTE)[129] under their Mazda-Belvu brand, which otherwise used mostly EIA, RETMA and Mullard–Philips tube designations.

Examples:

Before 1949:[130]

1883 – Indirectly-heated, 350 V/125 mA full-wave rectifier, 5 V/1.6 A heater

2XM600 – Directly-heated, 10 kV/250 mA half-wave mercury-vapor rectifier, 2.5 V/5 A heater

4Y25 = 807 – Indirectly-heated beam tetrode

RETMA tube 6H8G

RCA-800 tubes 879, 884

Since 1949 with a fire pot logo:[131]

RMA tube 2E30

3T20 – Directly-heated power triode, graphite anode

3T100 – Directly-heated power triode, graphite anode

4Y50 – Indirectly-heated beam tetrode

E1 – Electrometer tetrode

E2 – Dual electrometer tetrode

ST130 – 130 V Neon-filled voltage reference

Since 1953 as LAMPE MAZDA:[132]

RMA tube 2G21

4Y100 = 7745 – Dual beam tetrode

RCA-800 tubes 829, 832

927 – Gas-filled phototube

929 – Vacuum phototube

EIA tubes 6196, 6250

E5 – Subminiature electrometer tetrode, all-glass pigtailed

Since 1959 with a Faravahar logo related to Ahura Mazda:[133]

3T50 – Directly-heated power triode, thoriated-tungsten filament, graphite anode

4Y75 – Directly-heated power triode

RETMA tube 6K8

78A – Directly-heated, educational diode

EIA tubes 7233, 7242, 7377, 8418

E6 – Subminiature dual electrometer tetrode, all-glass pigtailed

E7, E9 – Subminiature electrometer pentodes, all-glass pigtailed

Mullard–Philips tubes ECF202, ECL802, ED501, EF816, EL503, EY81F, EY802, GY86, GY802, PY81F

F7024A (Diode), F7024C (Triode), F7024E (Tetrode), F7024L (Pentode) – Set of 4 educational tubes

F9116 – Electrometer tetrode

K25000A1 – Directly-heated, 25 kV/70 mA half-wave rectifier, 2.5 V/9 A heater

References and footnotes

Specific items

See "Decoding type numbers" ff.

"GL-2H21 Phasitron data sheet" (PDF). General Electric. September 1945. Retrieved 25 December 2016. (as JPGs) • RMA Release #486, 25 April 1946

Adler, Robert (January 1947). "A New System of Frequency Modulation" (PDF). Institute of Radio Engineers. Retrieved 25 December 2016.

Rider, John. F.; Seymour D. Uslan (1948). "FM Transmission and Reception" (PDF). John F. Rider Publisher, Inc. pp. 130–135. Retrieved 25 December 2016.

Dave Hershberger (W9GR): PHASITRON vacuum tube web page

"Tungar bulb data manual" (PDF). General Electric. Retrieved 12 February 2016.

"Eimac power grid tubes - Quick Reference Catalog 175" (PDF). Eitel McCullough. 1975. Retrieved 1 May 2017.

"Preferred Types of Electron Tubes 1967" (PDF). Retrieved 17 May 2013.

"European Type Designation Code System for Electronic Components" (PDF) (15 ed.). Pro-Electron, Brussels, Belgium. June 2008. Archived from the original (PDF) on 29 December 2013. Retrieved 25 December 2013.

"Akumed Berlin" hearing-aid eyeglasses schematic

"Philips Miniwatt 1938" (PDF). Retrieved 31 January 2016.

"Tabelle der Heeres-Batterie-Spezialröhren" (PDF) (in German). Lorenz. Retrieved 21 December 2015.

Miniwatt Technical Data, 6th Edition; 1958; Published by the "Miniwatt" Electronics Division of Philips Electrical Industries Pty. Limited, 20 Herbert Street, Artarmon, N,S,W., Australia

"Miniwatt" Premium Quality and Special Purpose Tubes, Philips Electrical Industries Pty. Ltd., Australia, November 1957.

"Cold cathode tubes ZnnnA". Retrieved 17 May 2013.

"Les lampes" (PDF) (in French). Retrieved 1 May 2017.

BW604 data sheet

BW1010 data sheet

"Belvu tubes electroniques, Licence R.C.A" (PDF) (in French). Retrieved 25 April 2017.

"Vade-mecum ADZAM" (PDF) (in French). 1957. Retrieved 1 May 2017.

"EEV Valve data book" (PDF). March 1966. Retrieved 1 May 2017.

"Master Valve Guide" (PDF). Mullard. 1935. Retrieved 12 February 2016.

"Philips radio-artikelen 1927" (PDF) (in Dutch). Retrieved 1 May 2017.p. 15

Дроздов, К. И. (1948). "Справочник по западно-европейским приёмным лампам (West-European receiving tubes)" (PDF) (in Russian). Retrieved 1 May 2017.

Georgescu, Aurel; Golea, Ion (1956). "Catalog De Tuburi Electronice" (PDF) (in Romanian). Editura Tehnikă Bukurești. Retrieved 10 September 2017.

"TUNGSRAM ELECTRON TUBE NUMBERING SYSTEM" (PDF). 2004. Retrieved 1 May 2017.

МЕТАЛЛИЧЕСКИЕ ЛАМПЫ (METAL TUBES)

ЭНЦИКЛОПЕДИЯ ЛАМПОВОЙ РАДИОАППАРАТУРЫ (Encyclopedia of tubes for radio equipment)

А.Л. Булыев; В.И. Галкин; В.А. Прохоренко (1982). "СПРАВОЧНИК ПО ЗЛЕКТРОВАКЧЧМНЫМ ПРИБОРАМ (HANDBOOK ON ELECTRONIC DEVICES)" (PDF) (in Russian). БЕЛАРУСЬ. pp. 10ff. Retrieved 1 May 2017.

Ewert, Jürgen. "Vacuum Tube Numbering Schemes, Bases & Bulbs".

HAYASHI, Koji; JAPAN, Ibaraki. "日本の真空管名称制度 (Tube Naming System Japan)" (in Japanese).

HAYASHI, Koji; JAPAN, Ibaraki. "Gallery on Tubes/真空管展示室" (in Japanese).

5J6 data sheet - this particular Tung-Sol datasheet contains a copy/paste error in the description where it cites 6J6's 450 mA heater current when it should read 5J6's 600 mA.

Schematic for General Electric model F-40, a 1938 reflex radio using a 6B7.

http://www.mif.pg.gda.pl/homepages/frank/sheets/191/1/12AB5.pdf

"12AU7 data sheet" (PDF). STC. August 1950. Retrieved 1 May 2017.

http://www.mif.pg.gda.pl/homepages/frank/sheets/127/1/12AV7.pdf

http://www.mif.pg.gda.pl/homepages/frank/sheets/127/1/12AZ7.pdf

http://www.mif.pg.gda.pl/homepages/frank/sheets/049/1/12BH7A.pdf

RCA: Receiving Tube Manual RC21, p.360

RCA: Receiving Tube Manual RC30, p.397

http://www.ww2australia.gov.au/japadvance/boots.html

"4560 Custom-built, 2" diameter, Electrostatic-Focus, Electrostatic-Deflection Monoscope Tubes For Use As Alphanumeric Character Generators data sheet" (PDF). RCA Electronic Components. May 1969. Retrieved 27 September 2017.

"4598 Graphechon Tube data sheet" (PDF). RCA Electronic Components. February 1971. Retrieved 1 May 2017.

"7539 Graphechon Tube data sheet" (PDF). RCA Electronic Components. March 1960. Retrieved 1 May 2017.

"GEC 7828 Scan conversion tube data sheet" (PDF). General Electric Corporation. 10 April 1961. Retrieved 1 May 2017.

"8087 Scan-Conversion Storage Tube data sheet" (PDF). Machlett Laboratories, Inc. 16 September 1963. Retrieved 1 May 2017.

"Rauland 8098 Signal Storage Tube data sheet" (PDF). Rauland Corporation. 8 January 1962. Retrieved 1 May 2017.

RMA Release #600, 2 September 1947

"5729 30 channel radial beam tube - collector type data sheet" (PDF). National Union Electric Corporation. 9 April 1951. Retrieved 1 May 2017.

"5734 Mechano-electronic transducer, triode type data sheet" (PDF). R.C.A. Manufacturing Company. November 15, 1948. Retrieved 1 May 2017.

"5734A Mechano-electronic transducer data sheet" (PDF). Toshiba Corp. March 14, 1964. Retrieved 1 May 2017.

"5738 Commutator tube data sheet" (PDF). Federal communication laboratories, Inc., Nutley, New Jersey, USA. 6 October 1948. Retrieved 1 May 2017.

"6047 Additron data sheet, RTMA Engineering Dept. Release #954" (PDF). Rogers Majestic Corp. March 20, 1951. Retrieved 14 August 2016.

"6090 18 channel radial beam tube - multiple anode type data sheet" (PDF). National Union Electric Corporation. January 1956. Retrieved 15 June 2013.

"6091 25 channel radial beam tube - multiple grid type data sheet" (PDF). National Union Electric Corporation. January 1956. Retrieved 1 May 2017.

"6170 & 6324 25 channel radial beam tube - multiple grid type data sheet" (PDF). National Union Electric Corporation. December 1955. Retrieved 15 June 2013.

"Tube Electrometre Double Tétrode à 2 Grilles de Charge d'Espace data sheet" (PDF) (in French). Compagnie Industrielle Française des Tubes Electroniques (CIFTE). January 1968. Retrieved 1 May 2017.

"6218 data sheet, RTMA Engineering Dept. Release #1115" (PDF). Rogers Majestic Corp. 25 August 1952. Retrieved 1 May 2017.

"E80T data sheet" (PDF). Philips. 4 April 1956. Retrieved 1 May 2017.

Richard G. Cumings (8 June 1956). "NRL Memorandum Report 606: Application of Tacitron Type RCA 6441 to Pulse Circuitry" (PDF). United States Naval Research Laboratory. Retrieved 19 November 2017.

"6462 Magnetic pick-up tube data sheet" (PDF). National Union Electric Corporation. 9 May 1956. Retrieved 1 May 2017.

"6571 Computer storage tube data sheet" (PDF). RCA Electron Tube Division. 21 March 1955. Retrieved 1 May 2017.

"6577 Typotron, 5" character-writing CRT-type storage tube data sheet" (PDF). Hughes Aircraft Corporation. 24 November 1954. Retrieved 29 August 2017.

"6700 Magnetron Beam Switching Tube data sheet" (PDF). Burroughs Corporation. August 1956. Archived from the original (PDF) on 4 March 2014. Retrieved 4 March 2014.

"6701 Magnetron Beam Switching Tube data sheet" (PDF). Burroughs Corporation. August 1956. Archived from the original (PDF) on 4 March 2014. Retrieved 4 March 2014.

"6762 Wamoscope data sheet" (PDF). Sylvania Electric Products. 17 January 1957. Retrieved 1 May 2017.

"CK6835 Recording storage tube data sheet" (PDF). Raytheon Company. 1 November 1959. Retrieved 1 May 2017.

"CK7570 Recording storage tube data sheet" (PDF). Raytheon Company. 1 November 1959. Retrieved 1 May 2017.

"CK7571 Recording storage tube data sheet" (PDF). Raytheon Company. 1 November 1959. Retrieved 1 May 2017.

"6846 Binary tube data sheet" (PDF). Sylvania Electric Products. July 1956. Retrieved 1 May 2017.

"7229 Cold-Cathode Trigger Tube data sheet E287B" (PDF). CBS/Hytron. 22 June 1958. Retrieved 11 September 2017.

"7230 Reliable Cold-Cathode Trigger Tube data sheet E287C" (PDF). CBS/Hytron. 25 August 1958. Retrieved 11 September 2017.

"7231 Subminiature Cold-Cathode Trigger Tube data sheet E287D" (PDF). CBS/Hytron. 22 June 1958. Retrieved 11 September 2017.

"7232 Reliable Subminiature Cold-Cathode Trigger Tube data sheet E287E" (PDF). CBS/Hytron. 22 June 1958. Retrieved 11 September 2017.

CBS/Hytron "Krytron Trigger Tubes" spec sheets E-337 (30 March 1959), E-337A-1 (20 June 1960), E-337A-2 (20 June 1960)

"7360 Beam Deflection Tube data sheet" (PDF). R.C.A. Manufacturing Company. March 1961. Retrieved 1 May 2017.

M. B. Knight (1960). "A new miniature beam deflection tube" (PDF). RCA Electron Tube Division. Retrieved January 22, 2017.

H. C. Vance K2FF (1960). "SSB Exciter Circuits Using a New Beam-Deflection Tube" (PDF). QST. Retrieved May 30, 2013.

"7414 Subminiature Time Totalizer data sheet" (PDF). Bendix Corporation. 14 March 1959. Retrieved 1 May 2017.

"CK7572 Recording storage tube data sheet" (PDF). Raytheon Company. 15 December 1959. Retrieved 1 May 2017.

"CK7575 Recording storage tube data sheet" (PDF). Raytheon Company. 15 December 1959. Retrieved 1 May 2017.

"CK7702 Recording storage tube data sheet" (PDF). Raytheon Company. 15 March 1960. Retrieved 1 May 2017.

"7763 Sheet Beam Tube data sheet" (PDF). General Electric. 5 March 1962. Retrieved 1 May 2017.

This tube's designation is inconsistent with the scheme

Wechselspannungs- und Wechselstrom-Stabilisierungsschaltungen mit der Diode YA1000. Telefunken Laborbuch (in German). IV. Ulm: AEG-Telefunken. 1967. pp. 189–195.

"The ZA100x series switching tubes from Philips". Retrieved 19 August 2013.

"ZC1050 data sheet" (PDF). Philips. February 1968. Retrieved 21 December 2013.

Thaens, J. G. M.; van Vlodrop, P. H. G. "Electronic Applications Vol. 27 No. 3: Running Text Display with Cold-Cathode Trigger Tubes" (PDF). Philips Elcoma Division, Central Application Laboratory, Eindhoven, The Nederlands. Retrieved 21 December 2013.

"Disc Seal Triodes" (PDF). Mullard. 1965. Retrieved 12 February 2016.

C. E. Wynn-Williams (2 May 1932). "Proceedings of the Royal Society of London: A Thyratron Scale of Two Automatic counter" (PDF). A, Containing Papers of a Mathematical and Physical Character. Royal Society. pp. 312–324. Retrieved 1 May 2017.

"CK1366 CK1367 Printer-type cathode ray tube data sheet" (PDF). Raytheon Company. 1 November 1960. Retrieved 1 May 2017.

"CK1368 CK1369 Printer-type cathode ray tube data sheet" (PDF). Raytheon Company. 1 November 1960. Retrieved 1 May 2017.

"CK1383 Recording storage tube data sheet" (PDF). Raytheon Company. 15 February 1963. Retrieved 1 May 2017.

"CK1414 Symbolray character generating cathode ray tube data sheet" (PDF). Raytheon Company components division, industrial components operation. 15 April 1966. Retrieved 1 May 2017.

"Symbolray™ application note" (PDF). Raytheon Company components division, industrial components operation. Retrieved 1 May 2017.

"DDR100 Accelerometer double diode data sheet" (PDF). Mullard. Retrieved 1 May 2017.

"Krytrons - Cold Cathode Switch Tubes data sheet K5500B-1" (PDF). EG&G Electro-Optics Division, Salem, Massachusetts, USA. September 1973. Retrieved 11 September 2016.

Wahl, Günter. "Hightech-Elektronik-Experimente" (PDF) (in German). Franzis Verlag. Retrieved 26 Dec 2014.

"Plasma Panel Displays - Dual Linear Bar Graph" (PDF). Vishay Dale, Columbus, Nebraska, USA. November 2000. Retrieved 8 March 2014.

Miller, Joseph A.; Soltes, Aaron S.; Scott, Ronald E. (February 1955). "Wide-band Analog Function Multiplier" (PDF). Electronics. Retrieved 15 June 2013.

Wyse, Barry (2000). "Extracts from "The Saga of Marconi Osram Valves", part 1" (PDF). The British Vintage Wireless Society. p. 12ff. Retrieved 1 May 2017.

R-type tube on The National Valve Museum

Lankshear, Peter (July 1996). "Valve filament/heater voltages" (PDF). Electronics Australia. Retrieved 1 May 2017.

"Subminiature gas triode type RK61 data sheet" (PDF). Raytheon Company. Retrieved 1 May 2017.

"Ed Lorenz Mystery Tube". Retrieved 1 May 2017.

George Honnest-Redlich Radio Control for Models (1950) p. 7

"SB256 Selective Electrostatic Storage Tube data sheet" (PDF). RCA Electron Tube Division. November 1951. Retrieved 4 November 2017.

Charles S. Osborne • lampes-et-tubes.info

TuneOn data sheet

TuneOn Button data sheet

BRIMAR (STC) Tunograph, Visual Tuning Indicator on lampes-et-tubes • Tunograph data sheet

"TH9503 Scripticon character generating cathode ray tube data sheet" (PDF). Compagnie Française Thomson-Houston, division tubes electroniques, Paris (France). January 1968. Retrieved 27 September 2017.

Van Bergen, Fons (2000). "About the French TM valve" (PDF). The British Vintage Wireless Society. p. 20ff. Retrieved 1 May 2017.

Champeix, Robert. "Grande et Petite Histoire de la Lampe TM" (in French). Les Anciens de la Radio et de l'Électronique. Retrieved 1 May 2017.

TM tube; Horned tube on The National Valve Museum

Gerald Garratt G5CS. "Why the French R valve?". Retrieved 1 May 2017.

Grid-anode curves for the Soviet R-5 triode, a licensed clone of the French TM triode made by La Compagnie des Lampes (1888)

McNally, J.O.; Metson, G.H.; Veazie, E.A.; Holmes, M.F. (January 1957). "Electron tubes for the transatlantic cable system" (PDF). The Bell system technical journal. pp. 163ff. Retrieved 9 February 2016.

"1636 U-H-F Beam deflection mixer" (PDF). RCA Electron Tube Division. 3 November 1944. Retrieved 1 May 2017.

"368A, 368AS and 388A data sheet" (PDF). Western Electric. Retrieved 19 January 2016.

http://www.tubecollectors.org/eimac/archives/450th(50).pdf

Holdaway, V.L.; Van Haste, W.; Walsh, E.J. (July 1964). "Electron tubes for the SD submarine cable system" (PDF). The Bell system technical journal. pp. 1311ff. Retrieved 9 Feb 2016.

https://www.crtsite.com/tv-crt.html

"RCA Air-Cooled Transmitting Tube Manual TT3" (PDF). R.C.A. Manufacturing Company, Harrison, New Jersey, USA. 1938. Retrieved 17 October 2013.

Lankshear, Peter (August 1988). "The Methuselah of valves" (PDF). Electronics Australia. Retrieved 1 May 2017.

"ИНДИКАТОР ИТМ2-М data sheet" (PDF) (in Russian). Moscow Electric Lamp Plant (МЭЛЗ/MELZ). 1944. Archived from the original (PDF) on 12 October 2013. Retrieved 9 May 2013.

"Линейный Трохотрон Типа ЛП-4 data sheet" (PDF) (in Russian). Moscow Electric Lamp Plant (МЭЛЗ/MELZ). Archived from the original (PDF) on 9 March 2014. Retrieved 8 March 2014.

La Compagnie des Lampes on radiomuseum.org

French Mazda datasheets before 1949: 18MA4 by CdL • 1883 (July 1948) by CdL, BELVU • 2XM400 (September 1947) by CdL • 2XM600 (September 1947) by CdL • 4Y25 (February 1949) by CdL, BELVU • 5Y35 (July 1948) by CdL • 6H8G (September 1947) by CdL • 879 (September 1947) by CdL • 884 (January 1949) by CdL • 8SAx by CdL • C75S (June 1947) by CdL • C95S (June 1947) by CdL

French Mazda datasheets 1949–53: 2E30 (November 1949) by CdL • 31MA4 (February 1950) by CdL • 3T20 (July 1949) by CdL • 3T100 (July 1949) by CdL, BELVU • 4Y50 (November 1950) by CdL, BELVU • C30S (January 1950) by CdL • C127S (January 1950) by CdL • C220MW1 (January 1950) by CdL • E1 (April 1950) by CdL • E2 (April 1950) by CdL • ST130 (September 1949) by CdL

French Mazda datasheets 1953–59: 2G21 (October 1953) by CdL, BELVU • 4Y100 (September 1960) by CdL, BELVU • 43MG4 (December 1954) by CdL • 43MH4 (March 1954) by CdL • 43MR4 (December 1954) by CdL • 54MS4 (June 1955) by CdL • 829 (June 1955) by CdL • 832 (June 1955) by CdL • 927 (July 1954) by CdL • 929 (June 1957) by CdL • 6196 (November 1959) by CdL • 6250 (November 1959) by CdL, BELVU • E5 (September 1960) by CdL • JA10 (September 1960) by CdL, BELVU

French Mazda datasheets since 1959: 3T50 (February 1966) by CdL, BELVU • 4Y75 (February 1964) by CdL, BELVU • 6K8 (June 1964) by CdL • 78A (September 1966) by CdL, BELVU • 7233 (April 1962) by CdL • 7242 (April 1965) by CdL • 7377 (April 1962) by CdL • 8418 (February 1963) by CdL, BELVU • E6 (February 1964) by CdL • E7 (June 1965) by CdL • E9 (September 1965) by CdL • ECF202 (April 1967) by CdL, BELVU • ECL802 (December 1966) by CdL, BELVU • ED501 (February 1966) by CdL • EF816 (April 1967) by CdL • EL183 (June 1959) by CdL, BELVU • EL503 (June 1966) by CdL, BELVU • EY81F (April 1967) by CdL, BELVU • EY802 (April 1967) by CdL, BELVU • F7024x (April 1967) by CdL, BELVU • F9102 (April 1965) by CdL, BELVU • F9116 (December 1965) by CdL • GY86 (June 1966) by CdL • GY802 (April 1967) by CdL • K25000A1 (June 1961) by CdL • PY81F (April 1967) by CdL, BELVU

General literature and data sheets

Frank Philipse's Tube Datasheet Archive

Mirrors in Brazil • Brazil searchable • Germany • Germany • Hong Kong • Hong Kong • Hungary • Poland[fast] • Poland searchable • Romania • Romania searchable • Sweden • USA • USA

Tubebooks.org datasheet collection

Roy J. Tellason's tube datasheet collection

Klausmobile Russian tube directory

General Electric Essential Characteristics, 1970

RCA Receiving Tube Manuals R10 (1932) • RC11 (1933) • RC12 (1934) • RC13 (1937) • RC14 (1942) • RC15 (1948) • RC16 (1951) • RC17 (1954) • RC18 (1956) • RC19 (1959) • RC20 (1960) • RC21 (1961) • RC22 (1963) • RC23 (1964) • RC24 (1965) • RC25 (1966) • RC26 (1968) • RC27 (1970) • RC28 (1971) • RC29 (1973) • RC30 (1975)

Scanned tube documentation (PDFs): Tubebooks • Frank Philipse • 4tubes

Sylvania Technical Manual, 1958

J. P. Hawker (ed), Radio and television servicing, Newnes, London, 1964

Camera tube datasheets

[1] • [2] Decoding type numbers

Decoding Valve, Transistor and CRT Numbers

Vacuum Tube Numbering Schemes, Bases & Bulbs

European tube designation systems: [3] • [4] • [5]

See also

iconElectronics portal

External links

Wikimedia Commons has media related to Vacuum tubes and Gas discharge tube.

Vacuum Tube Data Sheet Locator

Tube Substitution and Characteristics Guide

British virtual thermionic valve museum with good quality pictures and data

Belgian virtual thermionic valve museum with good quality pictures and data

Radio museum

Virtual Valve Museum

vte

Thermionic valves

Theoretical principles

Thermionic emission Work function Hot cathode Space charge Control grid Suppressor grid Anode Glowing anode Getter

Types

Diode Audion Triode Acorn tube Nuvistor Tetrode Beam tetrode Pentode Pentagrid (Hexode, Heptode, Octode) Nonode Cathode ray tube Additron Backward-wave oscillator Beam deflection tube Charactron Compactron Eidophor Iconoscope Inductive output tube Kinescope Klystron Magic eye Magnetron Monoscope Phototube Photomultiplier Selectron tube Storage tube Sutton tube Talaria projector Traveling-wave tube Trochotron Video camera tube Williams tube Fleming valve

CV Numbering System

Format is CV followed by up to 5 digits, e.g. CV4004.  Used by the British Military to codify vacuum tubes, gas tubes, and latterly, some semiconductor devices.

CV numbers can be directly mapped onto NATO numbers, as 5960-XX-000-YYYY where XX is the country code, and YYYY is the CV number.  For example, 5960-99-000-4004 = CV4004 (12AX7WA), with 99 indicating country of origin as UK.

Mullard–Philips tube designation

From Wikipedia, the free encyclopedia

Jump to navigation

Jump to search

This article needs additional citations for verification. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed.

Find sources: "Mullard–Philips tube designation" – news · newspapers · books · scholar · JSTOR (February 2012) (Learn how and when to remove this template message)

In Europe, the principal method of numbering vacuum tubes ("thermionic valves") was the nomenclature used by the Philips company and its subsidiaries Mullard in the UK, Valvo(de, it) in Germany, Radiotechnique (Miniwatt-Dario brand) in France, and Amperex in the United States, from 1934 on. Adhering manufacturers include AEG (de), CdL (1921, French Mazda brand), CIFTE (fr, Mazda-Belvu brand), EdiSwan (British Mazda brand), Lorenz (de), MBLE(fr, nl) (be, Adzam brand), RCA (us), RFT(de, sv) (de), Siemens (de), Telefunken (de), Tesla (cz), Toshiba (ja), Tungsram (hu), and Unitra (pl; Dolam, Polam, Telam brands). This system allocated meaningful codes to tubes based on their function and became the starting point for the Pro Electron naming scheme for active devices (including tubes and transistors).

Contents

1 Nomenclature systems

2 Symbol definitions

2.1 Single-digit numbers

2.2 Historical progression

2.2.1 The older Philips system

2.2.2 Single-digit numerical sequences

2.2.3 Double-digit numerical sequences

2.2.4 Triple-digit numerical sequences

2.2.5 Four-digit numerical sequences

3 Semiconductors

4 See also

5 References

6 External links

Nomenclature systems

The system allowed for cross-referencing with the American RETMA tube designation, the Marconi-Osram tube designation, and with military numbering systems such as civilian valve (CV) numbering in the United Kingdom and the Joint Army–Navy (JAN) tube designation in the US.

European tube manufacturers agreed on the system, but in the UK, MOV (Marconi-Osram Valve), STC/Brimar and Mazda/Ediswan maintained their own systems. Most MOV tubes were cross-licensed copies of RCA types, with a British designation. For example, an MOV X63 valve was the same as an RCA 6A8 tube. Brimar, which stood for "British Manufactured American Radio" (valves), used all American designations. STC/Brimar was a UK subsidiary of the American giant ITT (International Telephone and Telegraph).

Special quality tubes were sometimes identified by placing the numerical part of the designation between the heater rating and the tube type. The special quality could be anything, from rugged designs for military and industrial use, through devices with exceptionally low noise and microphony, to designs primarily optimised for long life without cathode poisoning when used for switching in a digital computer (but not necessarily with exceptional qualities as an amplifier). For example, an ECC81 manufactured as a special quality (SQ) tube would usually be designated 'E81CC'. The system was not universal as other special quality designation systems existed. An EF91 in its special quality version was designated 'M8083' (the 'M' meant Military) as in this case the standard EF91 was derived from the M8083 military design. Also, the SQ tube was not always designed for the same tasks or given the same Maximum Ratings (for example the E80F was more suitable for audio and electrometer applications, lacking the RF screening of the EF80, with anode and screen grid power ratings roughly half the EF80.[1][2]

Two or more elements in a single envelope were handled by adding letters after the heater identifier, in alphabetical order, so an ECH35 is a 6.3 volt heated triode plus a hexode with an octal base; a PABC80 is a Noval 300mA series-heater combination of a single low-power diode A, a pair of diodes with common cathode B, and a triode C. There were many ECCnn(n) 6.3V dual triodes.

It was the usual practice for power transformers to have a 5 volt insulated winding for rectifier filaments, and a 6.3 volt winding for all the other heaters; virtually all valves with 5V filament are rectifiers with cathode connected to heater, in practice full-wave (usable as half-wave by strapping both anodes together), e.g. GZ34. For lower-voltage lower-power requirements, rectifiers with 6.3V heaters and insulated cathodes such as the EZ80 were used, connected to the common filament supply. There is no special nomenclature for EHT rectifiers for cathode-ray tubes; the EY51 and EY86 were rated at 17kV with an average current of 350 microamps.[3] The GY501 is another example (31kV at 1.7mA and used the rare B9D base).[4] In practice most "xY" half-wave rectifiers are EHT types; but there are plenty of exceptions.[5]

Symbol definitions

1st letter: Heater rating

A	4V AC (See Note 1)

B	180mA DC (See Note 1, 2)

C	200mA AC/DC (See Note 1)

D	1.4V or less (normally 1.4V) or 2.8V/1.4V series/parallel filament

E	6.3V, or 12.6V/6.3V series/parallel heater; usually AC

F	12.6V (See Note 2)

G	Formerly 5V (often used for rectifiers) or later miscellaneous

H	150mA AC/DC

I	20V (See Note 2)

K	2V DC

L	450mA AC/DC

M	1.9V directly heated (See Note 2)

N	12.6V, indirectly heated (See Note 2)

O	Cold cathode (by 1955 this also included semiconductors as these had no heater)

P	300mA AC/DC

Q	2.4V indirectly heated (See Note 2)

S	1.9V indirectly heated (See Note 2)

T	7.4V (See Note 2, 3)

U	100mA AC/DC

V	50mA AC/DC

X	600mA AC/DC

Y	450mA AC/DC (replaced by L, to allow Y to be used for professional transmitting, microwave or industrial tubes)

Z	Cold cathode and/or gas-filled tubes

Notes:

Heater ratings for series-string, AC/DC tubes are given in milliamperes; heater ratings for parallel-string tubes are given in volts

(1) Letters A,B,and C were rarely used after the 1930's and discontinued long before the 1960's and so allocated to semiconductors in the Pro Electron naming scheme

(2) Letters B, F, I, M, N, Q, S and T were so very rarely used that most tubes beginning with these letters (such as the FC13 Octode, the function letter C of which doesn't match anyway) should not be assumed to be part of this naming scheme.

(3) Letter T was introduced solely for use with the TY86F EHT rectifier. This was an EY86 with a higher voltage heater specifically manufactured as a replacement for Ferguson television sets 306T and 308T manufactured in 1956. These produced excessive voltage from their line output circuits which rapidly destroyed the originally fitted EY86.[6]

The main letters used in the second half of the twentieth century for receiving tubes were: D, E, G, L, P and U [7] although X was also frequently found when 600mA heater chain versions were produced for the North American market.

Remaining letters: Type(s) of device(s)

All except special-quality "Z" cold cathode tubes:

A	Low-current diode

AA	Low-current double diode with separate cathodes

B	Low-current double diode with common cathode

C	Small-signal vacuum triode

D	Power triode

E	Small-signal tetrode

F	Small-signal pentode

H	Hexode or Heptode (of the Hexode type)

K	Octode or Heptode (of the Octode type)

L	Power tetrode, beam tetrode or power pentode

M	Tuning indicator

N	Gas-filled triode or thyratron

P	Tube designed for secondary emission

Q	Nonode (also called Enneode[8])

S	TV sync oscillator

T	(Deflection-controlled) beam tube, or miscellaneous

W	Gas-filled halfwave rectifier

X	Gas-filled fullwave rectifier

Y	Halfwave rectifier (power diode)

Z	Fullwave rectifier (double power diode)

Special quality "Z" cold cathode tubes:[9]

A	Long-life amplifier tube

B	Binary counter or switching tube

C	Common-cathode Counter Dekatron that makes only carry/borrow cathodes separately available for cascading

E	Electrometer tube

G	Amplifier tube

M	Optical indicator

S	Separate-cathode Counter/Selector Dekatron that makes all cathodes available on individual pins for displaying, divide-by-n counter/timer/prescalers, etc.

T	Relay triode, a low-power triode thyratron, one starter electrode, may need illumination for proper operation if not radioactively primed

U	Low-power tetrode thyratron, may mean:

One starter electrode and a primer (keep-alive) electrode for ion availability to keep the ignition voltage constant, for analog RC timers, voltage triggers, etc., or

Two starter electrodes to make counters bidirectional or resettable

W	Trigger pentode, two starter electrodes and a primer electrode

Numbers: Base type and serial number

1-9	Pinch-type construction valves, mostly P8 bases (P base, 8-pin side-contact) or European 5-pin (B base) and various other European pre-octal designs.

10–19	8-pin German metal octal, G8A

20–29	Loctal B8G; some octal; some 8-way side contact (exceptions are DAC21, DBC21, DCH21, DF21, DF22, DL21, DL21, DLL21, DM21 which have octal bases)

30–39	International Octal (IEC 67-I-5a), also known as IO or K8A

40–49	Rimlok (Rimlock) B8A All-glass miniature valves

50–59	"Special construction types fitted with bases applicable to design features used";[10] mostly locking bases: "9-pin Loctal" (B9G) or 8-pin Loctal (B8G); but also used for Octal and others (3-pin glass; Disk-seal; German 10-pin with spigot; min. 4-pin; B26A; Magnoval B9D)

60–64	All-glass valves fitted with 9-pin (B9G) bases

65-69	Sub-miniature all-glass valves with or without bases

70–79	Loctal Lorenz an all-glass wire (fly-leads in place of pins) subminiatures

80–89	Noval B9A (9-pin; IEC 67-I-12a)

90–99	"button" B7G (miniature 7-pin; IEC 67-I-10a)

100–109	B7G; Wehrmacht base; German PTT base

110–119	8-pin German octal; Rimlok B8A

130–139	Octal

150–159	German 10-pin with spigot; 10-pin glass with one big pin; Octal

160–169	Flat wire submins; 8-pin German octal

170–179	RFT 8-pin; RFT 11-pin all glass with one offset pin

180–189	Noval B9A

190–199	B7G

200–209	Decal B10B

230–239	Octal

270–279	RFT 11-pin all glass with one offset pin

280–289	Noval B9A

300–399	Octal

400–499	Rimlok B8A

500–529	Magnoval B9D; Novar

600–699	Flat wire-ended

700–799	Round wire-ended

800–899	Noval B9A

900–999	B7G

1000-	Round wire-ended; special nuvistor

2000–	Decal B10B

3000-	Octal

5000-	Magnoval B9D

8000-	Noval B9A

Notes:

For signal pentodes, an odd numerical identifier most often identified a variable transconductance (remote-cutoff) valve whereas an even number identified a 'high slope' (sharp-cutoff) valve.

For power pentodes and triode-pentode combinations, even numbers usually indicate linear (audio power amplifier) devices while odd numbers were more suited to video signals or situations where more distortion could be tolerated.

Single-digit numbers

The first types assigned using this sequence (in the mid to late 1930s) were less systematic and sometimes would append the US "G" and/or "GT" suffixes for octal base versions, although the base type was not always knowable from just the type number:

KK2 (Cap E) was a pinch-type valve fitted with an American 7-pin base.

Sometimes special versions were made with US (Ux-4 to 7) bases with no change in the type number (e.g. AF2, AK1, KK2), but

in the case of Octal (IO) often a "G" would be appended to the type number; examples are ECH3G, ECH4G, EK2G, EK2G/GT, EL3G, EL3NG, KF3G, KK2G and KL4G.

EBF2Gm EBF2GT/G and EBF35 had International Octal bases but European base connection sequences.

Versions without the pinch at the top and/or with a metal screen might have "N" appended, and letters "A", "B" or "X" would sometimes be used for variants (e.g. AL2X, ECH33B, ECH35A, EL3N and EL3NG). The AL2X differs from the AL2 in connecting the control grid to pin 6 instead of the top cap. EL33, EL33A and EL33B are octal power pentodes differing only in whether metallization shielding is connected to pin 1 or 8.

The AL3, AL4, EL3N and EL3NG have identical characteristics to the EL33, EL33A and EL33B but with different heater voltages and/or bases; the CL4 and CL33 are lower voltage and lower power devices that are only somewhat similar to the EL33 and PL33.

Historical progression

This section needs additional citations for verification. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed.

Find sources: "Mullard–Philips tube designation" – news · newspapers · books · scholar · JSTOR (January 2013) (Learn how and when to remove this template message)

The older Philips system

Prior to 1934, Phillips numbers were based on a sequence of one letter to indicate filament current range, followed by one or two digits for the filament voltage, then two digits that gave either the amplification factor (for triodes) or a code beginning with 41 to indicate tetrodes, pentodes and so on.[10] Examples are:

A409 (a 4Volt/65mA filament triode with a mu of 9)

B2043 (a 20V/180mA indirectly-heated power output pentode from 1931)

C243N (a 2V/200mA filament power output pentode from 1931 with the option of B5 and Ux5 bases).

Single-digit numerical sequences

An example of this format is "CL4". This format was used from 1934, when many European-specific bases existed. These bases included 5- and 8-pin side-contact, and 4- to 7-pin alternatives to incompatible US base types.[10] At this time there was pressure to produce devices compatible with wider markets, and so several versions of the same device might be produced with different bases, yet sometimes no change in type number.

Double-digit numerical sequences

An example of this designation format is "EL33A". After about 1938 the digits gave a more consistent definition of the base type. During the 1950s, most often two devices that shared the same number and all but the first letter of the name would be very similar except for heater voltage/current. During this time older filament voltage and current "families" were abandoned, so a device name beginning with "A", "B", "C" or "K" and ending in two or more digits is very unlikely to be part of this naming scheme. For example, the "KT61" is not a 2 Volt-filament beam tube within the Philips naming system, but a "kinkless tetrode" within the Marconi/EMI naming scheme.

Triple-digit numerical sequences

Examples of this format are "PL302" and "EF183". From about the start of the 1960s an extra digit was needed for new devices. Either a digit 1 was inserted before the 8 or other base-defining digit (e.g. an EF184 is a Noval pentode), or a three-digit sequence was used. For example, a PL500 is a power pentode in a Magnoval base.

Four-digit numerical sequences

Main article: Pro Electron

Numbering using four digits were never issued under the Mullard-Philips scheme. They were obtained from the successor scheme, Pro Electron.

European system after 1934 (pro-electron)

1st letter; heater indication. 0; tubes without filament. A; 4 V AC parallel connection. B; 180 mA DC. C; 200 mA AC/DC series or parallel connection. D; <= 1.4 V DC dry-battery, parallel connection. E; 6.3 V AC or carbattery, parallel connection. F; 13 V carbattery. G; 5V AC parallel connection. H; early: 4V DC battery. later: 150mA AC/DC series connection. I; 20V AC/DC parallel connection. K; 2 V battery. O; 150 mA AC/DC series connection. P; 300 mA AC/DC series connection. U; 100 mA AC/DC series connection. V; 50mA AC/DC series connection. X; 600 mA AC/DC series connection. Y; 450 mA AC/DC series connection. 2nd+next letters; tube systems. A; single detection diode. B; double detection diode. C; small-signal triode. D; power triode. E; small-signal tetrode (or 2nd emmission tube EE1). F; small-signal pentode. H; hexode or heptode. K; octode. L; power pentode or power tetrode. M; indicator tube. N; thyratron. Q; enneode. W; single gasfilled rectifier diode. X; double gasfilled rectifier diode. Y; single vacuum rectifier diode. Z; double vacuum rectifier diode. digits; socket & order. x; P (some are V) (except U-series (e.g.UBL1), those are octal). 1x; Y8A. 2x; W8A, Loctal (except D-series (e.g. DL21), those are octal). 3x; K8A, A08, octal. 4x; A8A, B8A, Rimlock. 5x; T9A, B9G, Enne-al and C2R and some special bases. 6x; Subminiature. 7x; Subminor8p. 8x, 18x, 8xx; B9A, noval. 9x, 19x, 9xx; B7G, miniature-7p (Exception: The ECC99 is a poorly chosen tube type number, as it comes with a noval base.). 2xx, 2xxx; decal. 5xx; B9D, magnoval. Examples; Description. CBL1; output pentode with dual detection diode, heater 200mA, P base. EABC80; tripple signal diode with triode, heater 6.3V, noval base. ECC83; dual triode, heater 6.3V, noval base. GZ34; dual vacuum rectifier, heater 5V, octal base. PFL200; pentode and power pentode, heater 300mA, decal base. UCH21; triode and heptode, heater 100mA, base loctal.

Philips system before 1934

1st letter; heater current. A; 0.06 to 0.10 A. B; 0.10 to 0.20 A. C; 0.20 to 0.40 A. D; 0.40 to 0.70 A. E; 0.70 to 1.25 A. F; > 1.25 A. 2nd digit or 2nd+3rd digits; heater voltage. x; heater voltage < 10 V. xx; heater voltage >= 10V. last 2 digits; description. xx; amplification factor for triodes. 41, 51 etc; tetrode with spacecharge grid (2nd grid is control grid). 42, 52 etc; tetrode with screen grid (1st grid is control grid). 43, 53 etc; power pentode. 44, 54 etc; triode with diode or tetrode with diode. 45, 55 etc; hf tetrode with variable gain. 46, 56 etc; hf pentode. 47, 57 etc; hf pentode with variable gain. 48, 58 etc; hexode frequency changer. 49, 59 etc; hexode with variable gain. Suffix letter; Description. H; ?. N; ? New, later version. S; ? Series connection allowed. T; ?. Examples; Bases usually: A or O. E499; triode, If=1A , Vf=4V, gain=99. F443N; power pentode, If=2A, Vf=4V. B2046; hf pentode, If=0.18A, Vf=20V.

MAZDA NUMBERING SYSTEM Signal valves

First number indicates heater or filament rating. 1; 1.4V (parallel or series). 6; 6.3V (parallel or series). 10; 0.1A (series). 20; 0.2A (series). 30; 0.3A (series). Following letter or letter sequence indicates class of valve. C; Frequency changer with special oscillator section. D; Signal diode(s). F; Voltage amplifier tetrode or pentode. FD; Voltage amplifier tetrode or pentode with diode(s). FL; Voltage amplifier tetrode or pentode with voltage amplifier triode. K; Small gas triode or tetrode. L; Voltage amplifier triode or double triode including oscillator triode. LD; Voltage amplifier triode with diode(s). M; Tuning indicator. P; Power amplifier valve, tetrode or pentode. PL; Power amplifier valve, tetrode or pentode with voltage amplifier triode. Final number distinguishes between different valves in the same class. MAZDA NUMBERING SYSTEM Power rectifier valves Letters indicates heater or filament rating. U; High vacuum half-wave. UU; High vacuum full-wave. Final number distinguishes between different valves in the same class.

RUSSIAN NUMBERING SYSTEM

The regulation GOST 5461-59 applies to tubes manufactured from 1959 and semiconductors manufactured from 1959 to 1963. Every symbol consists of four elements. If an element is absent, a dash should be written at its place.. Note: There are exceptions. Not all Russian tubes are numbered according this system.. First element. Generator (transmitting) tubes up to 25 MHz. UKF generator (transmitting) tubes 25-600 MHz. Centimeter range generation (transmitting) tubes. Modulator tubes. High power rectifiers. Voltage stabilizers. Current stabilizers. Thyratrons. Gas-discharge rectifiers. Mercury gas-discharge rectifiers. ,; Photocells and photomultipliers. Approximate (usually full volts) heater voltage; Receiving tubes. Screen diameter in cm; Picture tubes. Semiconductor diodes. Transistors. Second element. Diodes. Double diodes. Triodes. Tetrodes. Power pentodes and tetrodes. Remote cut-off pentodes. Sharp cut-off pentodes. Secondary emission pentodes. Mixing tubes with two signal grids. Pentodes with one or two diodes. Triodes with one or more diodes. Tuning indicators. Triodes with hexode, heptode or octode. Double triodes. Double tetrodes and pentodes. Triode - pentodes. Rectifiers. Type sequence number; Gas-discharge tubes, power rectifiers. Gas thyratrons. Mercury thyratrons. Cold cathode thyratrons. Oscillograph tubes with electrostatic deflection. Oscillograph tubes with electromagnetic deflection. TV picture tubes. Type sequence number; Semiconductors. Third element. Receiving, picture, generator and modulator tubes voltage and current stabilizers photocells and photomultipliers cold cathode thyratrons; Type sequence number. Semiconductors; Subtype, group (letter). Fourth element. none; Metal; Receiving tubes; voltage and current stabilizers. Glass. HF glass, side leads. Ceramic. Miniature glass 19 and 22.5 mm. Subminiature > 10 mm. Subminiature 10mm. Subminiature 4 mm. Subminiature 6 mm. lock-in-key. Disc leads. Water cooled; Generator (transmitting) and modulator tubes. Air cooled. letter; Luminophor color code; Picture tubes. X/Y; X = average current in A Y = reverse voltage amplitude in kV; Thyratrons (non cold cathode), Gas-discharge, power rectifiers. After symbol dash and postfix of one or more letters may be added:. high reliability tube (military). long life tube (military). designed for impulse operation. vibration resistant. Current Regulators. [A][V1-V2]; [A] is the current (in amperes) and [V1-V2] is voltage range.

Brimar type designation code for Receiving Valves

First Number; Indicates the Construction. 1; Half Wave Rectifiers. 2; Diodes. Single. 3; Triodes, Output. 4; Triodes, High-mu. 5; Tetrodes, Straight. 6; Tetrodes, Vari-mu. 7; Pentodes, Power and Video. 8; Pentodes, R.F. Straight. 9; Pentodes, R.F. Vari-mu. 10; Diodes, Double. 11; Triodes with Double Diode. 12; Pentodes, A.F. with Double Diode. 13; Triodes Double, High-mu. 14; Triodes Double, Class B Output. 15; Heptodes. 16; Triodes Output, D.C. Coupled. 17; Pentodes R.F. with Double Diode. 18; Pentodes with Triode. 19. 20; Hexode/Heptode with Triode. Letter; Indicates the heater rating. A; 3.6 to 4.4V Indirectly Heated. B; 2V Directly Heated. C; Directly Heated other then 2 or 4V. D; All other heater ratings Indirectly Heated other then 4V. Number; Serial number. Serial Numbers are allocated in chronological order as new valve types are introduced.

TESLA NUMBERING SYSTEM

leading digits; heater indication. Approximate (usually full volts) heater voltage. middle letters; tube systems (a subset of European system). A; single detection diode. B; double detection diode. C; small-signal triode. F; small-signal pentode. H; hexode or heptode. L; power pentode or power tetrode. M; indicator tube. Y; single vacuum rectifier diode. Z; double vacuum rectifier diode. trailing digits; socket and order. 1x; octal K8A, A08. 2x; loctal W8A. 3x; heptal B7G. 4x; noval B9A. 5x; special; mostly, 9 out of 10 pins 1.25mm on a circle diameter 25mm. 6x; submagnal B11A (no actual tube in this system known). 7x; duodecal B12A (no actual tube in this system known). 8x; diheptal B14A (no actual tube in this system known). 9x; free wires. Examples; Description. 35Y31; Single rectifier, heptal (7pin miniature) socket; 35V/150mA series heater; otherwise, European UY1N in heptal format. 4L20; HF power pentode; filament 2x2.4V / 325mA, Soviet 4П1Л; German RL4,2P6 in loctal format. 6CC42; VHF double triode; 6.3V/350mA heater, noval; equivalent to 2C51. 6F24; Telecom pentode, 6.3V/450mA heater, loctal; not similar to later Mazda's 6F24. 1M90; Subminiature indicator tube, 1.4V/25 mA filament, European DM70. leading 2 digits; screen specification. Approximate screen diameter or diagonal, in centimeters. 3rd digit; type order. middle letters; focusing and deflection. QP; magnetic focusing, magnetic deflection (early TV tubes). QQ; electrostatic focusig, magnetic deflection (later TV tubes). QR; electrostatic focusing and deflection (oscilloscope tubes). trailing digits; screen color and persistence. Examples; Description. 7QR20; Oscilloscope tube, 7cm diameter, green fluorescence, middle persistence. 430QP44; TV tube, 43cm diagonal, magnetic focusing and deflection, white fluorescence, pentode system, 90 degrees deflection angle. 430QQ44; TV tube, 43cm diagonal, electrostatic focusing, magnetic deflection, white fluorescence, tetrode system, 110 degrees deflection angle. 431QQ44; Identical to 430QQ44, except with metal-backed screen. 1st letter; tube category. R; HF tube. Z; modulator tube. U; gas-filled power rectifier. 2nd letter; tube systems. see receiving tubes. digits; power level. Anode dissipation in watts or kilowatts. trailing letters; cooling system (optional 1st letter), tube order. X; air cooling. Y; water cooling. Examples; Description. RA0007B; Ballast diode controlled by filament current; max anode current 0.7mA. RD1,5XA; Air-cooled power triode up to 30MHz, 1.5kW anode dissipation. RE40AK; KT88 by Tesla Vršovive (in production).

# List of vacuum tubes

From Wikipedia, the free encyclopedia

Jump to navigation Jump to search

This is a list of vacuum tubes or thermionic valves, and low-pressure gas-filled tubes, or discharge tubes. Before the advent of semiconductor devices, thousands of tube types were used in consumer electronics. Many industrial, military or otherwise professional tubes were also produced. Only a few types are still used today, mainly in high-power, high-frequency applications.

- Contents

- 1 Heater or filament ratings

- 2 Tube bases and envelopes

  - 2.1 Abbreviations used in this list

- 3 Numbering systems

  - 3.1 North American systems

    - 3.1.1 RMA system (1942)

    - 3.1.2 RETMA receiving tubes system (1953)

    - 3.1.3 EIA professional tubes system

    - 3.1.4 Eimac high power RF tubes system

  - 3.2 West European systems

    - 3.2.1 Mullard–Philips system

      - 3.2.1.1 Standard tubes

      - 3.2.1.2 Special quality tubes

      - 3.2.1.3 Professional tubes

      - 3.2.1.4 Transmitting tubes

      - 3.2.1.5 Phototubes and photomultipliers

      - 3.2.1.6 Voltage stabilizers

    - 3.2.2 Compagnie des Lampes (1888, "Métal") system

    - 3.2.3 EdiSwan ("British Mazda") systems

      - 3.2.3.1 Signal tubes

      - 3.2.3.2 Power rectifiers

    - 3.2.4 EEV system

    - 3.2.5 ETL computing tubes system

    - 3.2.6 Marconi-Osram system

    - 3.2.7 Mullard designations before 1934

    - 3.2.8 Philips system before 1934

    - 3.2.9 STC/Brimar receiving tubes system

    - 3.2.10 Valvo system before 1934

  - 3.3 East European systems

    - 3.3.1 Lamina transmitter tube system

    - 3.3.2 RFT transmitter tube system

    - 3.3.3 Tesla systems

      - 3.3.3.1 Signal tubes

      - 3.3.3.2 Power tubes

    - 3.3.4 Tungsram receiving tubes system before 1934

  - 3.4 Russian systems

    - 3.4.1 GOST standard tubes system

    - 3.4.2 Professional tubes system

  - 3.5 Japanese systems

    - 3.5.1 Older numbering system 1941–51

    - 3.5.2 JIS C 7001 system

  - 3.6 Military naming systems

    - 3.6.1 British CV naming system

    - 3.6.2 U.S. naming systems

  - 3.7 Other numeral-only systems

  - 3.8 Other letter followed by numerals

- 4 List of American RETMA tubes, with European equivalents

  - 4.1 "0 volt" gas-filled cold cathode tubes

  - 4.2 1 volt heater/filament tubes

  - 4.3 1.25 volt filament subminiature tubes

  - 4.4 1 prefix for home receivers

  - 4.5 2 volt heater/filament tubes

  - 4.6 3 volt heater/filament high voltage rectifier tubes

  - 4.7 Nominally 5 volt heater/filament tubes

  - 4.8 5 volt heater/filament full wave rectifier tubes

  - 4.9 6 volt heater/filament tubes

  - 4.10 "7" prefix loctal tubes

  - 4.11 12 volt heater/filament tubes

  - 4.12 "14" prefix loctal tubes

  - 4.13 25 volt heater/filament tubes

  - 4.14 35 volt heater/filament tubes

  - 4.15 50 volt heater/filament tubes

  - 4.16 117 volt heater tubes

  - 4.17 Other tubes with nonstandard filament voltages

- 5 List of EIA professional tubes

  - 5.1 4000s

  - 5.2 5000s

  - 5.3 6000s

  - 5.4 7000s

  - 5.5 8000s

- 6 List of European Mullard–Philips tubes, with American equivalents

- 7 List of Pro Electron professional tubes

  - 7.1 X - Electro-optical devices

    - 7.1.1 XA

    - 7.1.2 XG

    - 7.1.3 XL

    - 7.1.4 XM

    - 7.1.5 XP

    - 7.1.6 XQ

    - 7.1.7 XR

    - 7.1.8 XX

  - 7.2 Y - Vacuum tubes

    - 7.2.1 YA

    - 7.2.2 YD

    - 7.2.3 YG

    - 7.2.4 YH

    - 7.2.5 YJ

    - 7.2.6 YK

    - 7.2.7 YL

  - 7.3 Z - Gas-filled tubes

    - 7.3.1 ZA

    - 7.3.2 ZC

    - 7.3.3 ZM

    - 7.3.4 ZP

    - 7.3.5 ZT

    - 7.3.6 ZX

    - 7.3.7 ZY

    - 7.3.8 ZZ

- 8 List of European transmitting tubes

  - 8.1 B - Backward-wave amplifier

    - 8.1.1 BA

  - 8.2 D - Rectifier incl. grid-controlled

    - 8.2.1 DA

    - 8.2.2 DC

    - 8.2.3 DCG

    - 8.2.4 DCX

    - 8.2.5 DE

  - 8.3 J - Magnetron

    - 8.3.1 JP

    - 8.3.2 JPT

  - 8.4 K - Klystron

    - 8.4.1 KB

    - 8.4.2 KS

  - 8.5 L - Traveling-wave tube

    - 8.5.1 LA

  - 8.6 M - AF modulator Triode

    - 8.6.1 MA

    - 8.6.2 MB

    - 8.6.3 MY

    - 8.6.4 MZ

  - 8.7 P - Pentode

    - 8.7.1 PA

    - 8.7.2 PAL

    - 8.7.3 PAW

    - 8.7.4 PB

    - 8.7.5 PC

    - 8.7.6 PE

  - 8.8 Q - Tetrode

    - 8.8.1 QB

    - 8.8.2 QBL

    - 8.8.3 QBW

    - 8.8.4 QC

    - 8.8.5 QE

    - 8.8.6 QEL

    - 8.8.7 QEP

    - 8.8.8 QQC

    - 8.8.9 QQE

    - 8.8.10 QQV

    - 8.8.11 QQZ

    - 8.8.12 QV

    - 8.8.13 QY

    - 8.8.14 QYS

    - 8.8.15 QZ

  - 8.9 R - Rectifier

    - 8.9.1 RG

    - 8.9.2 RGQ

  - 8.10 T - AF/RF/oscillator Triode

    - 8.10.1 TA

    - 8.10.2 TB

    - 8.10.3 TBL

    - 8.10.4 TBW

    - 8.10.5 TC

    - 8.10.6 TD

    - 8.10.7 TE

    - 8.10.8 TX

    - 8.10.9 TY

    - 8.10.10 TYS

  - 8.11 X - Thyratron

    - 8.11.1 XGQ

    - 8.11.2 XR

- 9 List of other letter tubes

  - 9.1 A

    - 9.1.1 AC*/

    - 9.1.2 ACT

    - 9.1.3 AP

    - 9.1.4 AT

  - 9.2 B

    - 9.2.1 BA

    - 9.2.2 BG

    - 9.2.3 BT

  - 9.3 C

    - 9.3.1 CH

    - 9.3.2 CK

    - 9.3.3 CL

  - 9.4 D

    - 9.4.1 DDR

    - 9.4.2 DZ

  - 9.5 E

    - 9.5.1 EN

  - 9.6 G

    - 9.6.1 GC

    - 9.6.2 GCA

    - 9.6.3 GD

    - 9.6.4 GDT

    - 9.6.5 GE

    - 9.6.6 GK

    - 9.6.7 GN

    - 9.6.8 GR

    - 9.6.9 GRD

    - 9.6.10 GS

    - 9.6.11 GSA

    - 9.6.12 GTE

    - 9.6.13 GTR

  - 9.7 K

    - 9.7.1 KN

    - 9.7.2 KT

  - 9.8 M

    - 9.8.1 M8000s

    - 9.8.2 MC

    - 9.8.3 ME

  - 9.9 P

    - 9.9.1 PBG

    - 9.9.2 PD

    - 9.9.3 PL

  - 9.10 Q

    - 9.10.1 QK

  - 9.11 R

    - 9.11.1 RK

  - 9.12 S

    - 9.12.1 SB

    - 9.12.2 SU

  - 9.13 T

    - 9.13.1 TH

    - 9.13.2 TM

    - 9.13.3 TT

  - 9.14 V

    - 9.14.1 VHT

    - 9.14.2 VS

- 10 List of other number tubes

  - 10.1 1

    - 10.1.1 1600s

  - 10.2 2

    - 10.2.1 200s

  - 10.3 3

    - 10.3.1 300s

  - 10.4 4

    - 10.4.1 400s

  - 10.5 5

    - 10.5.1 500s

  - 10.6 6

  - 10.7 7

    - 10.7.1 700s

  - 10.8 8

    - 10.8.1 800s

  - 10.9 9

    - 10.9.1 900s

    - 10.9.2 9000s

- 11 List of tubes used in 1920s and 1930s radio receivers

  - 11.1 Tubes with directly heated cathodes

    - 11.1.1 With 1.1 Volt DC filaments

    - 11.1.2 With 2.0 Volts DC filaments

    - 11.1.3 With 3.3 Volts DC filaments

    - 11.1.4 With 5.0 Volts DC filaments

    - 11.1.5 Other directly DC-heated tubes

    - 11.1.6 Directly AC-heated power tubes

    - 11.1.7 Directly AC-heated rectifier tubes

  - 11.2 Tubes with indirectly heated cathodes

    - 11.2.1 With DC heaters

    - 11.2.2 For use with an AC heating transformer

    - 11.2.3 For use with AC/DC or vehicle-based storage-battery power supplies

  - 11.3 Shielded tubes for Majestic radios

  - 11.4 Rarely used tubes

- 12 Lettered loctal tubes used in Philco radios

- 13 Russian tubes

  - 13.1 List of standard tubes, with American equivalents

  - 13.2 List of professional tubes

  - 13.3 List of indicator tubes

  - 13.4 List of other tubes

- 14 Compagnie des Lampes (1921, "French Mazda") and Mazda-Belvu

- 15 References and footnotes

  - 15.1 Specific items

  - 15.2 General literature and data sheets

- 16 See also

- 17 External links

- Heater or filament ratings

Main article: Hot cathode

Receiving tubes have heaters or filaments intended for direct battery operation, parallel operation off a dedicated winding on a supply transformer, or series string operation on transformer-less sets. High-power RF power tubes are directly heated; the heater voltage must be much smaller than the signal voltage on the grid and is therefore in the 5...25 V range, drawing up to hundreds of amperes from a suitable heater transformer. In some valve part number series, the voltage class of the heater is given in the part number, and a similar valve might be available with several different heater voltage ratings.

- Tube bases and envelopes

Main article: Tube socket

- Abbreviations used in this list

- ST – Shouldered tube

- GT – Glass tube

- MT – Miniature tube, such as Noval B9A or Miniature 7-pin B7G

- FL – Subminiature all-glass elliptical body and flat bases with long, inline "flying leads" (pigtails) that are soldered into the circuit

- SL – Subminiature all-glass elliptical body and flat bases with short inline leads that can be soldered or can be mated with a special socket. (Flying leads can be cut short to fit into inline sockets.)

- R8 – Subminiature all-glass round body and base with 8 flying leads or stiff pins arranged in a circle

- Numbering systems

- North American systems

- RMA system (1942)

Main article: RMA tube designation

The system assigned numbers with the base form "1A21", and is therefore also referred to as the "1A21 system".[1] The first numeric character indicated the filament/heater power rating, the second alphabetic character was a code for the function, and the last 2 digits were sequentially assigned, beginning with 21

RMA tubes

- 1B23 – 20 kW, 400 to 1500 MHz Gas-filled, cold-cathode Transmit/Receive Tube (TR cell)

- 1B41 – Gas-filled, cold-cathode 9.5 kV, 450 A spark gap

- 1B45 – Gas-filled, cold-cathode 14 kV, 450 A spark gap

- 1B49 – Gas-filled, cold-cathode 12 kV, 450 A spark gap

- 1C21 – Gas-filled, 25 mAavg, 100 mApeak, triode thyratron

- 1D21 – Strobotron Gas-filled, 50 mAavg, 5 Apeak, luminiscent tetrode thyratron for use as a stroboscope lamp

- 1P21 – 9-stage Photomultiplier, spectral S4 response, 11-pin base

- 1P25 – Infrared image converter used in World War II night vision "sniperscopes".

- 1P29 – Gas-filled phototube, spectral S3 response, 4-pin base

- 1P39 – Vacuum Phototube, spectral S4 response, 4-pin base

- 1S22 – 10 kV, 20 A Vacuum SPDT switch

- 2C21 – Dual transmitting triode, indirectly heated, 7-pin base plus a single top cap for one of the grids

- 2C22 – Transmitting triode, indirectly heated, 8-pin base plus dual top cap for grid and anode

- 2C36 – Rocket-type disk-seal UHF triode with an internal feedback circuit between cathode and anode, for use as UHF oscillator up to 1.75 GHz

- 2C37 – Rocket triode for use as SHF oscillator up to 3.3 GHz

- 2C39A – Oil can-type disk-seal UHF power triode with glass spacers up to 3 GHz, Panode = 100 W

- 2C39B – 2C39A with ceramic spacers

- 2C40 – Lighthouse-type disk-seal UHF power triode for continuous operation, Panode = 6.5 W at 3370 MHz

- 2C41 – Oil can UHF power triode for pulsed operation, 2200 Wpeak at 3 GHz

- 2C42 – Lighthouse UHF power triode for pulsed operation, 1750 Wpeak at 1050 MHz; improved 446

- 2C43 – Lighthouse UHF power triode, indirectly heated, up to 3.37 GHz, 6-pin base

- 2C46 – Lighthouse UHF power triode

- 2C51 – Dual shielded triode, indirectly heated, 9-pin base

- 2D21/EN91 (PL21, PL2D21, CV797) – 100 mAavg, 500 mApeak, 10 Asurge, Gas-filled, indirectly heated tetrode thyratron, negative starter voltage, miniature 7-pin base, for relay and grid-controlled rectifier service, used in jukeboxes and computer equipment.

- 2E22 – 53 W Power pentode, 5-pin base with anode on top cap

- 2E26 – Popular amateur 5.3 W VHF beam power tetrode up to 175 MHz, octal base

- 2E30 – 10 W Directly heated beam power tetrode with deflection screens available on separate pin, miniature 7-pin base

- 2E31 – Subminiature, directly heated, fully shielded sharp-cutoff RF/IF pentode, 5-pin all-glass pigtailed, FL

- 2E32 – Similar to 2E31, SL

- 2E35 – 6 mW Subminiature directly heated power pentode, 5-pin all-glass pigtailed, FL

- 2E36 – Similar to 2E35, SL

- 2E41 – Diode, pentode, FL

- 2E42 – Similar to 2E42, SL

- 2F21 – Indirectly heated hexode monoscope, Indian Head test pattern, 6-pin base with dual top caps for grid4 and anode

- 2G21 – Directly heated triode-heptode mixer, 7-pin all-glass pigtailed

- 2G41 – Triode-heptode converter, FL

- 2G42 – Similar to type 2G42, SL

- 2H21 – Phasitron, a magnetically controlled beam-deflection phase modulator tube[2] similar to the 5593, used in early FM broadcast transmitters[3][4][5]

- 2J30 to 2J34 – 300 kW S-band Magnetrons

- 2J55 and 2J56 – 40 kW X-band Magnetrons for use as pulsed oscillator

- 2K25 – 25 mW 8.5 to 9.66 GHz reflex Klystron

- 2K50 – 15 mW 23.5 to 24.5 GHz reflex Klystron

- 2P23 – Early image orthicon TV camera tube.

- 3B28 – Xenon half wave rectifier; ruggedized replacement for mercury vapor type 866.

- 3C22 – Disk-seal UHF power triode, Panode = 125 W with forced-air cooling, 1.4 GHz

- 3C23 – 1.5 Aavg, 6 Apeak, Mercury-vapor triode thyratron, 4-pin base with anode top cap

- 3C45 – 45 mAavg, 1.5 ARMS, 35 Apeak, Half-indirectly heated hydrogen triode thyratron, 4-pin base with anode top cap

- 3D21 – Indirectly heated beam power tetrode, 8-pin base with anode top cap

- 3D22 – Gas-filled, 800 mAavg, 8 Apeak, tetrode thyratron, 7-pin base

- 3E29 – Dual beam power tube used in radar equipment; a pulse rated variant of the earlier 829B, 7-pin base with dual anode top cap.

- 4B32 – 10 kV, 1.25 Aavg, 5 Apeak Xenon half wave rectifier

- 4D21 (6155, Eimac 4-125A) – 125 W Glass VHF beam power tetrode

- 4E27 – 125 W Glass radial-beam power pentode

- 4J31 to 4J35 – 1 MW S-band Magnetrons

- 5B24 – Tungar bulb, a low-voltage, mercury-vapor, full wave rectifier for charging 60-cell lead-acid batteries at 6 A; 2.5 V, 24 A heater[6]

- 5C22 – Half-indirectly heated, hydrogen triode thyratron for radar modulators.

- 5D22 (6156, Eimac 4-250A) – 250 W, 110 MHz Glass beam power tetrode

- 5J26 – 500 kW, 1.22 to 1.35 GHz S-band Magnetrons

- 5K70 – 30 kW S-band reflex Klystron

- 6C21 – Triode radar modulator for "hard tube" pulsers.

- 7C23 – 120 kW Power triode for high voltage pulse operation.

- 8D21 – Internally water cooled dual tetrode used in early VHF TV transmitters.

- 9C21 – 100 kW Water-cooled power triode, directly heated, 4-pin base with dual top caps for grid and anode

- RETMA receiving tubes system (1953)

Main article: RETMA tube designation

RETMA is the acronym for the Radio Electronic Television Manufacturers Association formed in 1953.

- The first character group is a number representing the heater voltage rounded to the nearest whole number; 0 indicates a cold-cathode tube.[1]

- One or two letters assigned to the devices in order of development.

- A single numeral that represents the number of active elements in the tube.

- Suffix letters distinguish revisions or variants:

- A, B, C – Improved backward compatible versions

- E – Export version

- G – Glass bulb, ST-12 to ST-16 size

- GT – Glass bulb, T-9 size

- GT/G – Glass bulb, T-9 size interchangeable with G and GT types

- L – Loktal

- LM – Loktal-metal

- LT – Locking base

- M – Metal envelope

- MG – Metal-glass

- ML – Metal-Loktal

- S – Spray shielded

- W – Ruggedised, or military grade

- WA, WB – Improved, backward compatible military/industrial variants

- X – Low loss ceramic base for RF use

- Y – Low loss mica-filled phenolic resin base for RF use

- Lastly, manufacturers may decide to combine two type numbers into a single name, which their one device can replace, such as: 6DX8/ECL84 (6DX8 and ECL84 being identical devices under different naming schemes) or 6BC5/6CE5 (sufficiently identical devices within the RETMA naming system) and even 3A3/3B2, or 6AC5-GT/6AC5-G (where the single type number, 6AC5-GT/6AC5-G, supersedes both the 6AC5-G and the 6AC5-GT).

Often designations that differed only in their initial numerals would be identical except for heater characteristics.

For examples see below

- EIA professional tubes system

A four-digit system was maintained by the EIA for special industrial, military and professional vacuum and gas-filled tubes, and all sorts of other devices requiring to be sealed off against the external atmosphere.

Some manufacturers preceded the EIA number with a manufacturer's code:

- CK, RK – Raytheon Company

- ECG – Philips/Sylvania

- F – Federal Telephone and Radio (ITT division)

- GL – General Electric Corp. (not British General Electric Company)

- ML – Machlett Laboratories, Inc.

- NL – National Electronics, Inc. (Geneva, Illinois, USA)

- NU – National Union Electric Corp. (Orange, New Jersey, USA)

- PL – Philips N.V.

- SV – Svetlana:

- formerly only PJSC "Svetlana/ПАО Светлана", St. Petersburg, Russia

- now also a brand of New Sensor Corp., Long Island City, New York, USA, manufacturing in Saratov, Russia

- SY – Standard Telephones and Cables/Brimar (not Sylvania Inc.)

- WL – Westinghouse Electric Corp.

For examples see below.

- Eimac high power RF tubes system

Eitel/McCullough and other manufacturers of high power RF tubes use the following code:[7]

- An initial digit denoting the number of electrodes:

- 2 – Diode

- 3 – Triode

- 4 – Tetrode

- 5 – Pentode

- Up to 2 letters denoting the construction type and the cooling method:

- R or a hyphen ("-") – Glass envelope, radiation cooling

- C – Ceramic envelope

- P – Primarily for pulse applications

- L – External anode, liquid convection cooling

- N – External anode, natural convection air cooling

- S – External anode, conduction cooling

- V – Vapor cooled (anode is immersed in boiling water, and the steam is collected, condensed and recycled)

- W – Water cooled (water is pumped through an outer metal jacket thermically connected to the anode)

- X – Forced-air cooled (air is blown through cooling fins thermally connected to the anode)

- A number to indicate the maximum anode dissipation in watts. This can be exceeded for a short time, as long as the average is not exceeded over the anode's thermal time constant (typically 0.1 sec). In Class-C applications, the amplifier output power delivered to the load may be higher than the device dissipation

- One or more manufacturer-proprietary letters denoting the construction variant

- An optional digit denoting the gain group:

- 1 – ≤10

- 2 – 11...20

- 3 – 21...30

- 4 – 31...50

- 5 – 51...100

- 6 – 101...200

- 7 – 201...500

- 8 – 501...1000

- Optionally a slash "/" followed by the EIA equivalent.

Examples:

- 3CW5000A3 – 5 kW Ceramic triode, water cooled, variant 'A', gain group 3

- 3CX100A5 – 100 W Ceramic UHF triode, forced-air cooled, variant 'A', gain group 5; often used by radio amateurs for 23cm-band microwave amplifiers.

- 3CX1500A7 (8877) – 1.5 kW Ceramic triode, forced air cooled, variant 'A', gain group 7

- 3CX2500A3 – 2.5 kW Ceramic triode, forced air cooled, variant 'A', gain group 3

- 4-65A (8165) – 65 W Glass beam tetrode

- 4-125A (4D21, 6155) – 125 W Glass beam tetrode

- 4-250A (5D22, 6156) – 110 MHz, 250 W Glass beam tetrode

- 4-400A – 400 W Glass beam tetrode

- 4-1000A (8166) – 1 kW Glass beam tetrode popular in broadcast and amateur transmitters.

- 4CX250B – 250 W Ceramic tetrode, forced-air cooled, version 'B', favored by radio amateurs as a final amplifier.

- 4CX250DC – 250 W Ceramic tetrode, forced-air cooled, version 'DC'

- 4CX35000 – Ceramic tetrode used in numerous 50-kW broadcast transmitters, forced-air cooled, often in a Doherty configuration as in the Continental Electronics 317C series.

- 5-125B/4E27A – 75 MHz, 125 W Glass power pentode

- 5-500A – 500 W Glass radial-beam pentode

- 5CX1500A – 110 MHz, 1.5 kW Ceramic radial-beam pentode, forced air cooled

- 5CX3000A – 150 MHz, 4.0 kW Ceramic radial-beam pentode, forced air cooled

Media related to Eimac vacuum tubes at Wikimedia Commons

- West European systems

- Mullard–Philips system

Main article: Mullard–Philips tube designation

This system is very descriptive of what type of device (triode, diode, pentode etc.) it is applied to, as well as the heater/filament type and the base type (octal, noval, etc.).[1][8] Adhering manufacturers include AEG (de), Amperex (us), CdL (1921, French Mazda brand), CIFTE (fr, Mazda-Belvu brand), EdiSwan (uk, British Mazda brand), Radiotechnique (fr, Coprim, Miniwatt-Dario and RTC brands), Lorenz (de), MBLE(fr, nl) (be, Adzam brand), Mullard (uk), Philips (nl, Miniwatt brand), RCA (us), RFT(de, sv) (de), Siemens (de), Telefunken (de), Tesla (cz), Toshiba (ja), Tungsram (hu), Unitra (pl, Dolam, Polam and Telam brands) and Valvo(de, it) (de).

- Standard tubes

This part dates back to the joint valve code key (German: Röhren-Gemeinschaftsschlüssel) negotiated between Philips and Telefunken in 1933–34. Like the North American system the first symbol describes the heater voltage, in this case, a Roman letter rather than a number. Further Roman letters, up to three, describe the device followed by one to four numerals assigned in a semi-chronological order of type development within number ranges assigned to different base types.

If two devices share the same type designation other than the first letter (e.g. ECL82, PCL82, UCL82) they will usually be identical except for heater specifications; however there are exceptions, particularly with output types (for example, both the PL84 and UL84 differ significantly from the EL84 in certain major characteristics, although they have the same pinout and similar power rating). However, device numbers do not reveal any similarity between different type families; e.g. the triode section of an ECL82 is not related to either triode of an ECC82, whereas the triode section of an ECL86 does happen to be similar to those of an ECC83.

Pro Electron maintained a subset of the M-P system after their establishment in 1966, with only the first letters E, P for the heater, only the second letters A, B, C, D, E, F, H, K, L, M, Y, Z for the type, and issuing only three-digit numbers starting with 1, 2, 3, 5, 8, 9 for the base.[9]

Notes: Tungsram preceded the M-P designation with the letter T, as in TAD1 for AD1; VATEA Rádiótechnikai és Villamossági Rt.-t. (VATEA Radio Technology and Electric Co. Ltd., Budapest, Hungary) preceded the M-P designation with the letter V, as in VEL5 for EL5.

- First letter: heater/filament type

Heater ratings for series-string, AC/DC tubes are given in milliamperes; heater ratings for parallel-string tubes are given in volts

- A – 4 V heater for 2-cell lead-acid batteries and for AC mains transformers

- B – 180 mA DC series heater

- C – 200 mA AC/DC series heater

- D – 1.4 V DC filament for Leclanché cells, later low-voltage/low power filament/heater:

- 0.625 V DC directly heated for NiCd battery, series-heated two-tube designs such as hearing aids. If either filament breaks, further draining of all batteries stops[10]

- Wide range 0.9 V to 1.55 V DC directly heated for dry cells

- 1.25 V DC directly heated for NiCd batteries

- 1.25 V or 1.4 V AC from a separate heater winding on CRT horizontal-output transformers, in half-indirectly heated EHT rectifiers

- E – 6.3 V parallel heater; for 3-cell lead-acid vehicle crank batteries (mobile equipment) and for AC mains or horizontal-output transformers

- F – 12.6 V DC parallel heater for 6-cell lead-acid vehicle crank batteries

- G – 5.0 V AC from a separate heater winding on a mains or horizontal-output transformer for the anode voltage rectifier; later misc.

- H – 150 mA AC/DC series heater

- In 1938, Philips tried to define this letter as "4 V battery", as opposed to A for "4 V AC"[11]:2

- I – 20 V heater

- K – 2.0 V filament for 1-cell lead-acid batteries, later for AC transformers

- L – 450 mA AC/DC series heater; was shifted here from Y

- M – 1.9 V, directly heated

- N – 12.6 V, indirectly heated

- O – Cold cathode

- by 1955 this also included semiconductors as these had no heater

- Philips sold a family of 150mA series heater tubes under this letter in South America

- P – 300 mA AC/DC series heater

- Q – 2.4 V, indirectly heated

- R – Not assigned to avoid any confusion with the older Telefunken "R" system

- S – 1.9 V, indirectly heated

- T – Custom heater

- U – 100 mA AC/DC series heater

- V – 50 mA AC/DC series heater

- X – 600 mA AC/DC series heater

- Y – 450 mA AC/DC series heater, shifted to L to avoid conflicts with the professional tubes system

- Z – Cold cathode tube; was shifted here from O after the advent of semiconductors

- Second and subsequent letters: system type

- A – Small signal diode

- B – Dual small signal diode

- C – Small signal triode

- D – Power output triode

- E – Small signal tetrode

- F – Small signal pentode

- H – Mixer hexode, special purpose heptode

- K – Mixer heptode or octode

- L – Power output, beam tetrode or pentode

- M – Optical tuning/level indicator

- N – Gas-filled thyratron

- P – Secondary emission tube – mostly used as third letter

- Q – Nonode

- R – Resistive element (ballast tube, barretter, photoresistor)

- S – Special tube (German: Sonderröhre)

- T – Beam deflection tube, or misc.

- W – Gas-filled half-wave rectifier

- X – Gas-filled full-wave rectifier

- Y – Vacuum half-wave rectifier (power diode)

- Z – Vacuum full-wave rectifier (dual power diode with common cathode)

E.g. ECCnn is a 6.3 V dual triode; EABCnn has a single detector diode, a common-cathode pair of diodes, and a triode.

- Following digits: model number and base type

For signal pentodes, an odd model number most often identified a variable-mu (remote-cutoff) tube, whereas an even number identified a 'high slope' (sharp-cutoff) tube

For power pentodes and triode-pentode combinations, even numbers usually indicate linear (audio power amplifier) devices while odd numbers were more suited to video signals or situations where more distortion could be tolerated.

- 1–9 – Pinch-type construction tubes, mostly P8 bases (P base, 8-pin side-contact) or European 5-pin (B base) and various other European pre-octal designs

- 10–19 – 8-pin German metal octal, G8A

- 20–29 – Loctal B8G; some octal; some 8-way side contact (exceptions are DAC21, DBC21, DCH21, DF21, DF22, DL21, DLL21, DM21 which have octal bases)

- 30–39 – International Octal (IEC 67-I-5a), also known as IO or K8A

- 40–49 – Rimlok (Rimlock) B8A All-glass miniature tubes

- 41w – Battery-heated bowl tube[12] (German: Pressnapfröhre)

- 50–59 – "Special construction types fitted with bases applicable to design features used";[13] mostly locking bases: "9-pin Loctal" (B9G) or 8-pin Loctal (B8G); but also used for Octal and others (3-pin glass; Disk-seal incl. Lighthouse tubes; German 10-pin with spigot; min. 4-pin; B26A; Magnoval B9D)

- 60–69 – Pencil tubes – sub-miniature all-glass tubes, pigtailed (inline fly-leads in place of pins)

—Before the 1950s:

- 60–64 – All-glass tubes fitted with 9-pin (B9G) bases

- 70–79 – Pencil tubes with circular pins or fly-leads

—Before the 1950s:

- 70–79 – Loctal Lorenz

- 80–89 – Noval B9A (9-pin; IEC 67-I-12a)

- 90–99 – "Button" B7G (miniature 7-pin; IEC 67-I-10a)

- 100–109 – B7G; Wehrmacht base; German PTT base

- 110–119 – 8-pin German octal; Rimlok B8A

- 130–139 – Octal

- 150–159 – German 10-pin with spigot; 10-pin glass with one big pin; Octal

- 160–169 – Inline wire-ended Pencil tubes; 8-pin German octal

- 170–179 – RFT 8-pin; RFT 11-pin all-glass gnome tube with one offset pin

- 180–189 – Noval B9A

- 190–199 – Miniature 7-pin B7G

- 200–209 – Decal B10B; Pro Electron-issued

- 230–239 – Octal

- 270–279 – RFT 11-pin all glass with one offset pin

- 280–289 – Noval B9A

- 300–399 – Octal; Pro Electron-issued

- 400–499 – Rimlok B8A

- 500–529 – Magnoval B9D, Novar; Pro Electron-issued

- 600–699 – Inline wire-ended Pencil tubes

- 700–799 – Circular wire-ended Pencil tubes

- 800–899 – Noval B9A; Pro Electron-issued

- 900–999 – Miniature 7-pin B7G; Pro Electron-issued

—Special quality:

- 1000– Round wire-ended; special Nuvistor base

- 2000– Decal B10B

- 3000– Octal

- 5000– Magnoval B9D

- 8000– Noval B9A

For examples see below

- Special quality tubes

Vacuum tubes which had special qualities (French: Securité - Qualité) of some sort, very often long-life designs, particularly for computer and telecommunications use, had the numeric part of the designation placed immediately after the first letter. They were usually special-quality versions of standard types. Thus the E82CC was a long-life version of the ECC82 intended for computer and general signal use, and the E88CC a high quality version of the ECC88/6DJ8. While the E80F pentode was a high quality development of the EF80, they were not pin-compatible and could not be interchanged without rewiring the socket (the E80F is commonly sought after as a high quality replacement for the similar EF86 type in guitar amplifiers). The letters "CC" indicated the two triodes and the "F", the single pentode inside these types.

A few special-quality tubes did not have a standard equivalent, e.g. the E55L, a broadband power pentode used as the output stage of oscilloscope amplifiers and the E90CC, a double triode with a common cathode connection and seven pin base for use in cathode-coupled Flip-flops in early computers. The E91H is a special heptode with a passivated third grid designed to reduce secondary emission; this device was used as a "gate", allowing or blocking pulses applied to the first, (control) grid by changing the voltage on the third grid, in early computer circuits (similar in function to the U.S. 6AS6).

Many of these types had gold-plated base pins and special heater configurations inside the nickel cathode tube designed to reduce hum pickup from the A.C. heater supply, and also had improved oxide insulation between the heater and cathode so the cathode could be elevated to a greater voltage above the heater supply. (Note that elevating the cathode voltage above the average heater voltage, which in well-designed equipment was supplied from a transformer with an earthed center-tapped secondary, was less detrimental to the oxide insulation between heater and cathode than lowering the cathode voltage below the heater voltage, helping to prevent pyrometallurgical electrolytic chemical reactions where the oxide touched the nickel cathode that could form conductive aluminium tungstate and which could ultimately develop into a heater-cathode short-circuit.)

Better, often dual, getters were implemented to maintain a better vacuum, and more-rigid electrode supports introduced to reduce microphonics and improve vibration and shock resistance. The mica spacers used in "SQ" and "PQ" types did not possess sharp protrusions which could flake off and become loose inside the bulb, possibly lodging between the grids and thus changing the characteristics of the device. Some types, particularly the E80F, E88CC and E90CC, had a constricted section of bulb to firmly hold specially shaped flakeless mica spacers.[14]

For examples see below, starting at DC

Later special-quality tubes had not base and function swapped but were assigned a 4-digit number,[8] such as ECC2000 or ED8000, the first digit of which again denoting the base:

- 1 – Miscellaneous

- 2 – Miniature 10-pin base (JEDEC F10-61)

- 3 – Octal base (IEC 67-1-5a)

- 5 – Novar/magnoval base (JEDEC E9-75 and E9-23)

- 8 – Noval base (IEC 67-1-12a)

- 9 – Miniature 7-pin base (IEC 67-1-10a)

For examples see below, starting at EC

"Z" Cold-cathode SQ tubes had a different function letter scheme:[15]

- A – Long-life amplifier tube

- B – Binary counter or switching tube

- C – Common-cathode Counter Dekatron that makes only carry/borrow cathodes separately available for cascading

- E – Electrometer tube

- G – Amplifier tube

- M – Optical indicator

- S – Separate-cathode Counter/Selector Dekatron that makes all cathodes available on individual pins for displaying, divide-by-n counter/timer/prescalers, etc.

- T – Relay triode, a low-power triode thyratron, one starter electrode, may need illumination for proper operation if not radioactively primed

- U – Low-power tetrode thyratron, may mean:

- Trigger tetrode, one starter electrode and a primer (keep-alive) electrode for ion availability to keep the ignition voltage constant, for analog RC timers, voltage triggers, etc.

- Relay tetrode, two starter electrodes to make counters bidirectional or resettable

- W – Trigger pentode, two starter electrodes and a primer electrode

- X – Shielded Trigger pentode, two starter electrodes, a primer electrode and a conductive coating of the glass envelope inside connected to a separate pin

For examples, see below under Z

- Professional tubes

In use since at least 1961, this system was maintained by Pro Electron after their establishment in 1966.[9]

Both letters together indicate the type:

- X – High vacuum electro-optical devices

- XA – Phototube

- XG – Miscellaneous

- XM – Character generating cathode ray tube

- XP – Photomultiplier

- XQ – Camera tube

- XR – Monoscope

- XS – Cathode ray charge storage tube

- XT – Memory display tube

- XV – Infrared detector

- XW – Infrared imaging device

- XX – Image intensifier or image converter

- Y – Vacuum tubes

- YA – Diode

- YD – Transmitting or industrial, single or dual triode

- YG – Electrometer tube, vacuum gauge

- YH – Traveling-wave tube

- YJ – Magnetron

- YK – Klystron

- YL – Transmitting or industrial, single or dual tetrode or pentode

- YN – Backward-wave oscillator

- YP – Electron multiplier

- YR – Crossed-field amplifier

- YT – Pulse modulator tube

- YY – High vacuum rectifier

- Z – Gas-filled tubes not employing photosensitive materials

- ZA – Cold cathode indicator tube

- ZB – Microwave switching tube (TR/ATR cells, etc.)

- ZC – Trigger tube

- ZD – Surge arrester

- ZE – Glow modulator tube, a linear light source for rotating-drum FAX receivers, film soundtrack recording, etc.

- ZF – Flash tube

- ZL – Gas laser

- ZM – Cold cathode character display tube or counter display tube

- ZP – Radiation counter tube (Geiger-Müller counter tube or proportional counter tube)

- ZQ – Mixed analogue and digital display

- ZR – Plasma display panel

- ZS – Bar graph

- ZT – Thyratron

- ZX – Ignitron

- ZY – Mercury-vapor rectifier

- ZZ – Voltage stabilizer or corona discharge tube

Then follows a 4-digit sequentially assigned number.

Optional suffixes for camera tubes:

Version letter:

- B – Blue

- G – Green

- L – Luminance

- R – Red

- T – Reticule

- X – Medical X-ray

Letter for variants derived by selection:

- D – High resolution

- M – Blemish standard

For examples see below

- Transmitting tubes

The first letter (or letter pair, in the case of a dual-system device) indicates the general type:

- B – Backward-wave amplifier

- D – Rectifier, including grid-controlled types

- J – Magnetron

- K – Klystron

- L – Traveling-wave tube

- M – Triode (AF amplifier or modulator)

- P – Pentode

- Q – Tetrode

- R – Rectifier

- T – Triode (AF, RF, oscillator)

- X – Large thyratron (including all hydrogen thyratrons and high-current types)

The following letter indicates the filament or cathode type, or the fill gas or other construction detail. The coding for vacuum devices differs between Philips (and other Continental European manufacturers) on the one hand and its Mullard subsidiary on the other.

Philips vacuum devices:

- A

- Backward-wave amplifier or Traveling-wave tube: Output power <1W

- Other tubes: Directly heated tungsten filament

- B

- Backward-wave amplifier or Traveling-wave tube: Output power ≥1W

- Other tubes: Directly heated thoriated tungsten filament

- C – Directly heated oxide-coated filament

- D – Disk-seal construction

- E – Indirectly heated oxide-coated cathode

Mullard vacuum devices:

- G – Directly heated oxide-coated filament (only mercury-vapor rectifiers)

- N – External magnet required (magnetrons)

- P – Packaged construction (magnetrons)

- S – Reflex klystron

- T – Multiple resonator (klystrons)

- V – Indirectly heated oxide-coated cathode

- X – Directly heated tungsten filament

- Y – Directly heated thoriated tungsten filament

- Z – Directly heated oxide-coated filament (except mercury-vapor rectifiers)

Gas-filled devices:

- G – Mercury-vapor filling

- H – Hydrogen filling

- R – Inert-gas filling

- X – Xenon filling

The next letter indicates the cooling method or other significant characteristic:

- H – Helix or other integral cooler

- L – Forced-air cooling

- Q – Shield-grid (tetrode) thyratron (thyratrons only)

- S – Silica envelope, to allow for a glowing anode

- T – Tunable microwave device

- W – Water cooling

The following group of digits indicate:

- Microwave tubes: Frequency in GHz

- Rectifying tubes: DC output voltage in kV in a three-phase half-wave configuration

- Thyratrons: Peak inverse voltage in kV

- Transmitting tubes: Maximum anode voltage in kV

The following group of digits indicate the power:

- Backward-wave amplifier or Traveling-wave tube: Output power

- 2nd letter: A – in mW

- 2nd letter: B – in W

- Klystrons: Output power

- Magnetrons: Pulse output power in kW

- Continuously transmitting tubes: Maximum anode dissipation in W or kW in Class-C amplifier telegraphy

- Pulsed transmitting tubes: Maximum peak anode current in A (number preceded by "P")

- Rectifiers: Maximum average anode current in mA

- Thyratrons: Maximum average anode current:

- Less than 3 digits: in mA

- 3 or more digits:

- 1st digit: =0 – in mA

- 1st digit: >0 – in A

An optional following letter indicates the base or connection method:

- B – Cables

- E – Medium 7-pin base

- ED – Edison screw lamp base

- EG – Goliath base

- F – Medium 8-pin base

- G – Medium 4-pin base

- GB – Jumbo 4-pin base

- GS – Super jumbo 4-pin base

- N – Medium 5-pin base

- P – P-base

For examples see below

- Phototubes and photomultipliers

The first digit indicates the tube base:

- 2 – Loctal 8-pin base

- 3 – Octal 8-pin base

- 5 – Special base

- 8 – Noval base

- 9 – Miniature 7-pin base

The second digit is a sequentially assigned number.

The following letter indicates the photocathode type:

- A – Caesium-activated antimony cathode. Used for reflective-mode photocathodes. Response range from ultraviolet to visible. Widely used.

- C – Caesium-on-oxidated-silver cathode, also called S1. Transmission-mode, sensitive from 300...1200 nm. High dark current; used mainly in near-infrared, with the photocathode cooled.

- T – Multialkali sodium-potassium-antimony-caesium cathode, wide spectral response from ultraviolet to near-infrared; special cathode processing can extend range to 930 nm. Used in broadband spectrophotometers.

- U – Caesium-antimony cathode with a quartz window

The following letter indicates the filling:

- G – Gas-filled

- V – High-vacuum

A following letter P indicates a photomultiplier.

Examples:

- 50AVP – 11-stage photomultiplier for scintillation counters, duodecal base

- 51UVP – 11-stage photomultiplier, duodecal base

- 52AVP/XP1180 – 10-stage photomultiplier, 13-pin base

- 53AVP, 153AVP – 10-stage photomultiplier, diheptal 14-pin base

- 53UVP – 11-stage photomultiplier, diheptal 14-pin base

- 54AVP – 11-stage photomultiplier, diheptal 14-pin base

- 55AVP – 15-stage photomultiplier, bidecal 20-pin base

- 56AVP – 14-stage photomultiplier, bidecal 20-pin base

- 56UVP – 14-stage photomultiplier, duodecal base

- 57AVP – 11-stage photomultiplier, bidecal 20-pin base

- 58AVP – 14-stage photomultiplier, bidecal 20-pin base

- 150AVP – 10-stage photomultiplier, bidecal 20-pin base

- 150CVP – 10-stage photomultiplier, bidecal 20-pin base

- 57CV – Photometric cell

- 58CG – Gas-filled phototube, Red/IR sensitive, all-glass pigtailed

- 58CV – Vacuum phototube, Red/IR sensitive, all-glass pigtailed

- 90AG – Gas-filled phototube, daylight/blue sensitive, miniature 7-pin base

- 90AV – Vacuum phototube, blue sensitive, miniature 7-pin base

- 90CG – Gas-filled phototube, Red/IR sensitive, miniature 7-pin base

- 90CV – Vacuum phototube, Red/IR sensitive, miniature 7-pin base

- 92AG – Gas-filled phototube, blue sensitive, miniature 7-pin base

- 92AV – Vacuum phototube, blue sensitive, miniature 7-pin base

- 61SV/7634 – PbS infrared (300...3500 nm) photoresistor, 2-pin all-glass pigtailed

- Voltage stabilizers

The first number indicates the burning voltage

The following letter indicates the current range:

- A – max. 10mA

- B – max. 22mA

- C – max. 40mA

- D – max. 100mA

- E – max. 200mA

The following digit is a sequentially assigned number.

An optional, following letter indicates the base:

- E – Edison screw lamp base

- K – Octal 8-pin base

- P – P-base

Examples:

- 75B1 – Voltage reference tube, miniature 7-pin base

- 75C1 – Voltage reference tube, miniature 7-pin base

- 83A1 – Voltage reference tube, miniature 7-pin base

- 85A1/0E3 – Voltage reference tube, Loctal B8G base

- 85A2/0G3 – Voltage reference tube, miniature 7-pin base

- 90C1 – Voltage reference tube, miniature 7-pin base

- 95A1 – Voltage reference tube, miniature 7-pin base

- 100E1 – Voltage reference tube, "A" Base

- 108C1/0B2 – Voltage reference tube, miniature 7-pin base

- 150A1 – Voltage reference tube, "P" base

- 150B2 – Voltage reference tube, miniature 7-pin base

- 150B3 – Voltage reference tube, miniature 7-pin base

- 150C1 – Voltage reference tube, "P" base

- 150C2/0A2 – Voltage reference tube, miniature 7-pin base

- 150C4 – Voltage reference tube, miniature 7-pin base

- Compagnie des Lampes (1888, "Métal") system

The first (1888) incarnation of La Compagnie des Lampes produced the TM tube since 1915 and defined one of the first French systems;[1][16] not to be confused with Compagnie des Lampes (1921, "French Mazda", see below).

First letter: Heater or filament voltage

- A – 1 V

- B – 2 V

- D – 4 V

- E – 5 V

- F – 6 V

- G – 7 V

Second letter: Heater or filament current

- W – ≥200 mA

- X – 150 mA

- Y – 100...140 mA

- Z – 50 mA

Next number: Gain

Next number: Internal resistance in kΩ

Examples:

- BW604 – Métal secteur indirectly AC-heated AF power triode[17]

- BW1010 – Métal secteur indirectly AC-heated AF triode[18]

- EdiSwan ("British Mazda") systems

EdiSwan (British Mazda) is not to be confused with other licensees of General Electric's Mazda brand: - GE's own subsidiary British Thomson-Houston - Cie des Lampes (1921, French Mazda, see below) - Cie Industrielle Française des Tubes Electroniques – CIFTE (Mazda-Belvu – originating from Societé Radio Belvu; see below)[19] - Manufacture Belge des Lampes Électriques,(fr, nl) producing: - Light bulbs since 1911 under the Belgian Mazda brand - Electronic tubes since 1924 under the Adzam ("Mazda" spelled backwards) brand[20]

Note: EdiSwan also used the Mullard–Philips scheme.

- Signal tubes

First number: Heater or filament rating[1]

- 0 – Misc. higher voltages

- 1 – 1.4 V

- 6 – 6.3 V

- 10 – 100 mA

- 20 – 200 mA

- 30 – 300 mA

Following letter or letter sequence: Type

- C – Frequency changer with special oscillator section

- D – Signal diode(s)

- F – Tetrode or pentode

- FD – Tetrode or pentode and diode(s)

- FL – Tetrode or pentode, and triode

- K – Small gas triode or tetrode thyratron

- L – Single or dual triode, including oscillator triode

- LD – Triode and diode(s)

- M – Optical tuning/level indicator

- P – Power tetrode or pentode

- PL – Power tetrode or pentode, and signal triode

Final number: Sequentially assigned number

- Power rectifiers

Letter(s): Type

- U – High-vacuum half-wave rectifier

- UU – High-vacuum full-wave rectifier

Number: Sequentially assigned number

Examples:

Note: "AC/"-series receiver tubes are listed under other letter tubes - AC/

- 6C10 (6CU7/ECH42) – Triode/hexode frequency converter, 8-pin Rimlock base

- 6F22 (6267/EF86) – Low-noise A.F. pentode, 9-pin noval base

- 6F33 – Shielded pentode, 7-pin base

- 6L12 (6AQ8/ECC85) – Dual triode, 9-pin noval base

- 6L19 – Dual triode, 8-pin base

- 6M2 (6CD7/EM34) – Dual-sensitivity tuning indicator, 8-pin octal base

- 6P9 (6BM5) – Power pentode, 7-pin base

- 6P15 (6BQ5/EL84) – Power pentode, 9-pin noval base

- 10PL12 (50BM8/UCL82) – Triode/power pentode, 9-pin noval base

- U381 (38A3/UY85) – Half-wave rectifier, 9-pin noval base

- UU9 (6BT4/EZ40) – Full-wave rectifier, 8-pin rimlock base

- EEV system

This system consists of one or more letters followed by a sequentially assigned number[21]

- A – High vacuum rectifier

- AFX – Rare gas filled triode thyratron

- AH – Mercury-vapor rectifier

- AX – Xenon filled rectifier

- B – Radiation-cooled triode

- BD – Mercury vapor rectifier

- BK – Ignitron

- BM – Magnetron

- BR – Forced air cooled triode

- BS – TR (Transmit/receive) cell, TB cell, Solid-state microwave device

- BT – Mercury vapor or xenon filled thyratron

- BW – Water cooled triode

- BY – Vapor cooled triode

- C – Radiation-cooled tetrode

- CR – Forced air cooled tetrode

- CW – Water cooled tetrode

- CX – Hydrogen tetrode thyratron

- E – Storage tube

- FX – Hydrogen triode thyratron

- GX – Spark gap

- K – Klystron

- M – Magnetron

- NFT – Nernst filament, a source of mid-infrared radiation

- P – Video camera tube

- QS – Voltage-regulator tube

- QT – Cold-cathode trigger tube

- T – CRT

- U – Vacuum capacitor

- XL – Glow modulator tube, flash tube, gas laser

Examples:

- B142 – 400 W RF power triode up to 50 MHz similar to 833A

- B1109 = 3C24 – 25 W VHF power triode up to 60 MHz

- B1135 = 5867 = CV1350 – VHF power triode up to 100 MHz

- B1152 – 500W RF power triode up to 50 MHz

- QT1257 – Touch button tube, an illuminated capacitance touch switch; a cold-cathode DC relay tube, external (capacitive) starter activated by touching; then the cathode glow is visible. 6-pin octal base

- XL601, XL602, XL603, XL627, XL628, XL631 and XL632 – Cold-cathode, linear light source (glow modulator tube), gas diode with a blue-violet glow, modulation up to 1 MHz, 8-pin base, for rotating-drum FAX receivers, etc.

- ETL computing tubes system

The British Ericsson Telephones Limited (ETL), of Beeston, Nottingham (not to be confused with the Swedish TelefonAB Ericsson), original holder of the now-generic trademark Dekatron, used the following system:

- An initial letter denoting the filling:

- G – Gas-filled

- V – Vacuum

- One letter denoting the type:

- C – Common-cathode Counter Dekatron that makes only carry/borrow cathodes separately available for cascading

- D – Diode, voltage reference, etc.

- R – Register (Readout) – Digital indicator

- S – Separate-cathode Counter/Selector Dekatron that makes all cathodes available on individual pins for displaying, divide-by-n counter/timer/prescalers, etc.

- TE – Trigger tetrode, one starter electrode and a keep-alive (primer) electrode for ion availability

- TR – Trigger triode, one starter electrode only

- A digit group:

- Dekatrons: Stage count

- Digital indicators: Display cathode count

- Diodes, voltage references: Nominal voltage

- Trigger tubes: Ignition voltage

- An optional digit group after a slash: Pin count

- One letter denoting the type:

- A – Plastic base

- B – Plastic base

- C – Plastic base

- D – Plastic base

- E – Plastic base

- G – 26-pin B26A base

- H – 27-pin B27A base

- M – B7G base

- P – B7G base

- Q – B7G base

- W – Pigtails

- X – Pigtails

- Y – Pigtails

- Marconi-Osram system

The British GEC–Marconi–Osram designation from the 1920s uses one or two letter(s) followed by two numerals and sometimes by a second letter identifying different versions of a particular type.[1]

The letter(s) generally denote the type or use:

Note: A preceding letter M indicates a 4-volts AC indirectly heated tube

- A – General professional tube

- B – Dual triode

- D – Detector diode

- GU – Gas-filled rectifier

- GT – Gas-filled triode

- H – High-impedance signal triode

- L – Low-impedance signal triode

- N – Power pentode

- P – Power triode up to 3 W

- PT – 3...6 W Power triode

- PX – 9...25 W Power triode

- QP – Dual pentode

- S – Tetrode

- U – Rectifier

- VS – Remote-cutoff tetrode

- W – Remote-cutoff pentode

- X – Triode/hexode frequency-changer

- Y – Optical tuning/level indicator

- Z – Sharp-cutoff RF pentode

The following numbers are sequentially assigned for each new device.

Examples:

Note: Kinkless Tetrode beam power tubes are listed under other letter tubes - KT

- A1834 = 6AS7G/ECC230 = CV2523 – Dual power triode (series regulator), octal base.

- B309 = 12AT7/ECC81 – High-mu dual triode. Commonly used as R.F. amplifier/mixer in VHF circuits.

- B719 = 6AQ8/ECC85 – Dual RF triode, RF Amp & Mixer in FM receivers, noval base.

- D41 = V914 – Indirectly heated, Double Detector Diode, British 5-pin base.

- D42 – Indirectly heated, Single Detector Diode, British 4-pin base.

- GU21 = AH221 = RG4-1250 – Half-wave mercury-vapor rectifier, Edison screw lamp base.

- H63 = 6F5 – High-mu triode, octal base.

- H610 – Directly heated, high-mu AF triode, British 4-pin base.

- L63 = 6J5 – Low-mu triode, octal base.

- L610 – Directly heated, Low-mu RF triode, British 4-pin base.

- MT7A, MT7B – Large radiation-cooled transmitting triodes used in the 1920s and 1930s.

- MU14 = UU5 = IW4-500 – Indirectly heated full-wave rectifier, British 4-pin base.

- N77 = 6AM5/EL91 – Power pentode, 7-pin miniature base.

- P425 = PM254 – Power triode with a 4 V/200 mA battery heater and a British 4-pin base

- P610 – Directly heated, AF power triode, British 4-pin base.

- P625 – AF power triode.

- PX4 – AF power triode designed in the 1930s. Capable of providing about 4.5 W of audio.

- QP21 – Directly heated, dual AF (push-pull) power pentode, British 7-pin base.

- QP240 – Directly heated, dual AF (push-pull) power pentode, British 9-pin base.

- S610 – Directly heated, Sharp-cutoff RF tetrode, British 4-pin base.

- U52 = 5U4G = 5AS4A/5U4GB – Full-wave rectifier, octal base.

- VS24 – Directly heated, Remote-cutoff RF tetrode, British 4-pin base.

- W727 = 6BA6/EF93 = 5749 – Remote-cutoff RF pentode, 7-pin miniature base.

- X41 – Triode/hexode mixer designed to be a direct plug-in replacement for the MX40 pentagrid converter.

- X61, X61M = 6J8G – British triode/heptode mixer, octal based.

- X63 = 6A8 – Heptode pentagrid converter, octal based.

- X727 = 6BE6/EK90 = 5750 – Pentagrid converter, 7-pin miniature base.

- Y61, Y63 = 6U5G = VI103 – Optical tuning/level indicator, octal base, similar to 6G5.

- Z77 = 6AM6/EF91 – Sharp-cutoff RF pentode, 7-pin miniature base.

Media related to Marconi vacuum tubes at Wikimedia Commons

- Mullard designations before 1934

Older Mullard tubes were mostly designated PM, followed by a number containing the filament voltage.

Many later tubes were designated one to three semi-intuitive letters, followed by a number containing the heater voltage. This was phased out after 1934 when Mullard adopted the Mullard–Philips scheme.

Examples:[22]

- 2D4 – Dual Diode with a 4 V/650 mA heater and a British 5-pin base

- FC4 = MX40 – Octode Frequency Converter with a 4 V/650 mA heater and a British 7-pin base

- Pen20 – Power Pentode with a 20 V/180 mA heater and a British 5- or 7-pin base

- PM254 = P425 – "Super Power" triode with a 4 V/200 mA battery heater and a British 4-pin base

- TDD4 = MHD4 = AC/HLDD – Triode, dual Diode with a 4 V/550 mA heater and a British 7-pin base

- TH21C – Triode/Hexode frequency converter with a 21 V/200 mA series heater and a British 7-pin base

- TP4 = AC/TP – Triode, Pentode with a 4 V/1.25 A heater and a British 7-pin base

- VP2 = VP21 = VP215 – Variable-mu Pentode with a 2 V/180 mA heater and a British 7-pin base

Media related to Mullard vacuum tubes at Wikimedia Commons

- Philips system before 1934

The system consisted of one letter followed by 3 or 4 digits. It was phased out after 1934 when Philips adopted the Mullard–Philips scheme.

1st letter: Heater current[23][1]

- A – 60...90 mA

- B – 100...190 mA (This designation lived on as the "B" (180 mA) in the Mullard–Philips system)

- C – 200...390 mA (This designation lived on as the "C" (200 mA) in the Mullard–Philips system)

- D – 400...690 mA

- E – 700...990 mA

- F – 1...2 A

1 or 2 digit(s): Heater voltage

Last 2 digits: Type

- 00–40, 99: Triode amplification factor

- 41–98:

- second-last digit: sequentially assigned, starting at 4

- last digit:

- 1 – Tetrode with a space charge grid (the 2nd grid is the control grid)

- 2 – Tetrode with a screen grid (the 1st grid is the control grid)

- 3 – Power pentode

- 4 – Binode, a diode/triode or diode/tetrode

- 5 – Remote-cutoff RF tetrode

- 6 – Signal pentode

- 7 – Remote-cutoff RF pentode

- 8 – Sharp-cutoff hexode frequency changer

- 9 – Remote-cutoff hexode

Examples:[24]

- A106 – Directly heated triode, 1 V, 60 mA filament, amplification factor = 6

- A425 = RE034 = HR406 – RF triode, 4 V, 60 mA filament

- A435 – Directly heated triode, 4 V, 60 mA filament, amplification factor = 35

- A441 – Directly heated tetrode with a space charge grid, 4 V, 60 mA filament

- A442 = RES094 = S406 – Directly heated tetrode with a screen grid, 4 V, 60 mA filament

- B409 = RE134 = L414 – Triode, 4 Volt, 140 mA filament

- B2038 = REN1821 = R2018 = A2118 – Triode, 180 mA heater

- B2043 = RENS1823D = PP2018D = L2318D – Indirectly heated power pentode, 20 V, 180 mA DC series heater

- B2044 = RENS1854 = DS2218 – Indirectly heated diode/tetrode, 20 V, 180 mA DC series heater

- B2044S = REN1826 – Indirectly heated diode/triode, 20 V, 180 mA DC series heater

- B2045 = RENS1819 – Indirectly heated remote-cutoff RF tetrode, 20 V, 180 mA DC series heater

- B2048 = RENS1824 = MH2018 – Hexode mixer, 20 V, 180 mA heater

- B2099 = REN1814 – Indirectly heated triode, 20 V, 180 mA DC series heater, amplification factor = 99

- E443H = RES964 = PP4101 = L496D – Power pentode, 4 V heater

- E446 = RENS1284 = HP4101 – Indirectly heated RF pentode, 4 V, 1.1 A heater

- E447 = RENS1294 = HP4106 – Indirectly heated remote-cutoff RF pentode, 4 V, 1.1 A heater

- E448 = RENS1224 = MH4100 – Indirectly heated sharp-cutoff hexode frequency changer, 4 V, 1.2 A heater

- E449 = RENS1234 = FH4105 – Indirectly heated remote-cutoff hexode, 4 V, 1.2 A heater

- F215 – Indirectly heated triode, 2.5 V, 1.5 A heater, amplification factor = 15

Media related to Philips vacuum tubes at Wikimedia Commons

Media related to Philips gas discharge tubes at Wikimedia Commons

- STC/Brimar receiving tubes system

First number: Type[1]

- 1 – Half-wave rectifier

- 2 – Diode

- 3 – Power triode

- 4 – High-mu triode

- 5 – Sharp-cutoff tetrode

- 6 – Vari-mu tetrode

- 7 – Power or video pentode

- 8 – Sharp-cutoff RF pentode

- 9 – Vari-mu RF pentode

- 10 – Dual diode

- 11 – Triode and dual diode

- 12 – AF Pentode and dual diode

- 13 – Dual high-mu triode

- 14 – Dual Class-B power triode

- 15 – Heptode

- 16 – DC-coupled power triode

- 17 – RF pentode and dual diode

- 18 – Pentode and triode

- 20 – Hexode/heptode and triode

Next letter: Heater rating

- A – 3.6 to 4.4 V Indirectly heated

- B – 2 V Directly heated

- C – Directly heated other than 2 or 4 V

- D – All other heater ratings, indirectly heated other than 4 V

Number: Sequentially assigned number

Examples:

- 1D6 – Indirectly heated, half-wave rectifier, 5-pin base

- 4D1 – Indirectly heated triode, 7-pin base

- 7A3 – Indirectly heated power pentode, 7-pin base

- 8A1 – Indirectly heated RF sharp-cutoff pentode, 5-pin base with anode top cap

- 9A1 – Indirectly heated RF/IF remote-cutoff pentode, 5-pin base with anode top cap

- 10D1 – Indirectly heated, common-cathode dual diode, 5-pin base

- 11A2 – Indirectly heated, common-cathode dual diode and triode, 7-pin base

- 13D3 – Indirectly heated, common-cathode dual triode, 9-pin base

- 15A2 – Indirectly heated, heptode pentagrid converter, 7-pin base

- 20D4 – Indirectly heated, triode/heptode frequency mixer, 9-pin base

- Valvo system before 1934

Valvo(de, it) was a major German electronic components manufacturer from 1924 to 1989; a Philips subsidiary since 1927, Valvo was one of the predecessors of NXP Semiconductors.

The system consisted of one or two letters followed by 3 or 4 digits. It was phased out after 1934 when Valvo adopted the Mullard–Philips scheme.

First letter(s): Type[25]

- A – Triode

- AN – Binode, a diode/triode or diode/tetrode

- G – Rectifier

- H – RF tube

- L – Power tube

- LK – Power amplifier

- U – Triode with a space charge grid

- W – Triode for resistor-coupled amplifiers

- X – Hexode

Number:

- If the first digit is 4, the tube has a 4 V heater

- Otherwise, the last two digits give the heater current in tens of mA.

A following letter D indicates more than one grid, not counting a space charge grid

Examples:[24]

- A2118 = B2038 = REN1821 = R2018 – Triode, 180 mA (=18⋅10 mA) heater

- H2018D = B2042 = RENS1820 = S2018 – RF Tetrode, 180 mA heater

- L496D = E443H = RES964 = PP4101 – Power pentode, 4 V heater

- L2318D = B2043 = RENS1823D = PP2018D – Power pentode, 180 mA heater

Media related to Valvo vacuum tubes at Wikimedia Commons

Media related to Valvo gas discharge tubes at Wikimedia Commons

- East European systems

- Lamina transmitter tube system

Polish Lamina(pl) transmitter tube designations consist of one or two letters, a group of digits and an optional letter and/or two digits preceded by a "/" sign.

The first letter indicates the tube type, two equal letters denoting a dual tube:

- P – Pentode

- Q – Tetrode

- T – Triode

A group of digits represents the maximum anode power dissipation in kW

An optional letter specifies the cooling method:

- <none> – Radiation

- P – Forced air

- W – Water

The first of the two digits after the "/" sign means:

- 1 – Tube for radio broadcasting and radiocommunication equipment

- 2 – Tube for industrial equipment

- 3 – Tube used in TV broadcasting equipment

- 4 – Tube for radiocommunication equipment with unbalanced modulation

- 5 – Modulator or pulse tube

The second digit after the "/" is sequentially assigned.

Examples:

- Q01 – Power tetrode up to 125 MHz, 0.1 kW (=100 W)

- Q3.5 – Power tetrode up to 220 MHz, 3.5 kW

- QQ-004/11 – Dual beam power tetrode up to 500 MHz, 0.04 kW (=40 W)

- T01 – Power triode up to 200 MHz, 135 W

- T015/21 – Power triode up to 150 MHz, 150 W

- T02 – Power triode up to 60 MHz, 200 W

- T05P/31 – Forced air cooled power triode up to 1 GHz, 1 kW

- T2/22 – Power triode up to 60 MHz, 3 kW

- T6 – Power triode up to 30 MHz, 6 kW

- T8P/21 – Forced air cooled power triode up to 120 MHz, 8 kW

- T10P/22 – Power triode up to 30 MHz, 10 kW

- T-25P – Forced air cooled power triode up to 30 MHz, 25 kW

- T60W/21 – Water cooled power triode up to 30 MHz, 6 kW

Media related to Lamina vacuum tubes at Wikimedia Commons

Media related to Lamina gas discharge tubes at Wikimedia Commons

- RFT transmitter tube system

Rundfunk- und Fernmelde-Technik(de, sv) was the brand of a group of telecommunications manufacturers in the German Democratic Republic. The designation consists of a group of three letters and a group of three or four digits.

The first two letters determine the tube type:

- SR – Transmitter tube

- VR – Amplifier tube

The third letter specifies the cooling method:

- L – Forced air

- S – Radiation

- V – Vapor (the anode is immersed in evaporating water, and the steam is collected, condensed and recycled)

- W – Water

The first digit (or the first two digits in double tubes) indicates the number of electrodes:

- 2 – Diode

- 3 – Triode

- 4 – Tetrode

- 5 – Pentode

The last two digits are sequentially assigned.

Examples:

- SRS301 – Radiation-cooled triode up to 40 MHz, 900 W

- SRS464 – Radiation-cooled, vibration-resistant pulse tetrode up to 300 kW

- SRS4451 – Radiation-cooled dual tetrode up to 500 MHz, 60 W

- SRS4452 = QQE03/20 = 6252 – Radiation-cooled dual tetrode up to 600 MHz, 20 W

- SRS4452 – Radiation-cooled dual tetrode up to 600 MHz, 20 W

- SRS501 – Radiation-cooled pentode up to 50 MHz, 100 W

- SRS552N = ГУ-50 – Radiation-cooled pentode up to 120 MHz, 50 W

- VRS303 – Radiation-cooled AF triode, 1 kW

- VRS328 – Radiation-cooled AF triode, 150 W

- VRS331 – Radiation-cooled AF triode, 450 W

Note: RFT used the Mullard–Philips and RETMA schemes for their low-power tubes.

Media related to RFT vacuum tubes at Wikimedia Commons

- Tesla systems

- Signal tubes

Besides the genuine Mullard–Philips system, Tesla also used an M-P/RETMA hybrid scheme:[1]

First number: Heater voltage, as in the RETMA system

Next letter(s): Type, subset of the Mullard–Philips system

Next digit: Base

- 1 – Octal K8A, A08

- 2 – Loctal W8A

- 3 – Miniature 7-pin B7G

- 4 – Noval B9A

- 5 – Special, mostly 9 out of 10 1.25mm pins on a 25mm-diameter circle

- 6 – Submagnal B11A

- 7 – Duodecal B12A

- 8 – Diheptal B14A

- 9 – Pigtails

Last digit: Sequentially assigned number

Examples:

- 1M90 (DM70/1M3) – Subminiature indicator tube, 1.4V/25 mA filament, all-glass pigtailed

- 4L20 – Directly heated RF power pentode; filament 2x 2.4V/325mA; Soviet 4П1Л, German RL4,2P6 with Loctal base

- 6B31 – Dual diode up to 700 MHz; 6.3V/300mA heater, miniature 7-pin base

- 6BC32 (6AV6, EBC91) – Dual diode and triode; 6.3V/300mA heater, miniature 7-pin base

- 6CC31 (6J6, ECC91) – 600 MHz dual triode; 6.3V/450mA heater, miniature 7-pin base

- 6CC42 (2C51) – VHF dual triode; 6.3V/350mA heater, noval base

- 6F24 – Telecom pentode, 6.3V/450mA heater, Loctal base

- 6F36 (6AH6) – Sharp-cutoff IF/video pentode, 6.3V/450mA heater, miniature 7-pin base

- 6H31 (6BE6, EK90) – Heptode mixer; 6.3V/300mA heater, miniature 7-pin base

- 6L36 (6AQ5, EL90) – Power pentode, 6.3V/450mA heater, miniature 7-pin base

- 6L41 (5763) – Beam tetrode, 6.3V/750mA heater, noval base

- 35Y31 – Half-wave rectifier, miniature 7-pin base; 35V/150mA series heater; UY1N with 7-pin base

- Power tubes

First letter:

- R – Rectifier or RF tube

- U – Gas-filled power rectifier

- Z – Modulator tube

Next letter(s): Type, subset of the Mullard–Philips scheme

Next number: Anode dissipation in W (if radiation cooled) or kW (otherwise)

The next letter specifies the cooling method:

- <none> – Radiation

- V – Vapor

- X – Forced air

- Y – Water

Examples:

- RA0007B – Directly heated saturated-emission ballast diode. Acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits; UAmax 600 V IAmax 700 µA, miniature 9-pin noval base

- RA100A – 40 kV, 100 mA Half-wave rectifier with an E40 Goliath Edison lamp screw base and an anode top cap

- RC5B – Bowl-type UHF power triode up to 5 W

- RD27AS – Radiation-cooled power triode up to 25 MHz, 27 W

- RD200B – Radiation-cooled power triode up to 60 MHz, 200 W

- RD300S – Radiation-cooled power triode up to 200 MHz, 300 W

- RD150YA – Water-cooled power triode up to 3 MHz, 150 kW

- RE40AK = KT88

- RE65A – Radiation-cooled beam tetrode up to 260 MHz, 65 W

- RE125C – Radiation-cooled beam tetrode up to 235 MHz, 125 W

- RE400C – Radiation-cooled beam tetrode up to 235 MHz, 400 W

- RE20XL – Air-cooled beam tetrode up to 220 MHz, 20 kW

- REE30A – Radiation-cooled dual beam tetrode up to 250 MHz, 20 W

- RL15A – Radiation-cooled power pentode up to 60 MHz, 20 W

- RL40A – Radiation-cooled power pentode up to 120 MHz, 40 W

- RL65A – Radiation-cooled power pentode up to 15 MHz, 65 W

- UA025A – 10 kV, 250 mA Argon-filled, half-wave rectifier with an E24 Edison lamp screw base and an anode screw top cap

- UA5A – 11 kV, 5 A Half-wave mercury-vapor rectifier with a 2-pin base and an anode screw top cap

- ZD1000F – Radiation-cooled power triode up to 60 MHz, 1 kW

- ZD1XB – Air-cooled AF power triode up to 1.2 kW

- ZD3XH – Air-cooled power triode up to 60 MHz, 3 kW

- ZD8XA – Air-cooled power triode up to 20 MHz, 8 kW

- ZD12YA – Air-cooled AF power triode up to 20 MHz, 12 kW

- ZE025XS – Air-cooled beam tetrode up to 400 MHz, 250 W

Media related to Tesla vacuum tubes at Wikimedia Commons

Media related to Tesla gas discharge tubes at Wikimedia Commons

- Tungsram receiving tubes system before 1934

The Tungsram system was composed of a maximum of three letters and three or four digits.[26][25] It was phased out after 1934 when Tungsram adopted the Mullard–Philips scheme, frequently preceding it with the letter T, as in TAD1 for AD1.

Letter: System type:

Note: A preceding letter A indicates an indirectly heated tube

- D – Detector diode

- DD – Dual diode

- DG – Tetrode with a space charge grid (the 2nd grid is the control grid)

- DS – Diode-tetrode

- FH – Remote-cutoff hexode pentagrid converter

- G – Preamplifier triode

- H – Voltage amplifier triode or grid-leak detector

- HP – RF pentode

- HR – RF triode

- L – AF power triode

- MH – Hexode pentagrid converter

- MO – Octode pentagrid converter

- O – Transmitting tube

- P – Power triode

- PP – Power pentode

- PV – Full-wave rectifier

- R – High-Mu triode

- S – Tetrode

- V – Half-wave rectifier

- X – US-licensed tube

Number:

- First digit (or the first two digits): Heater voltage

- Remaining digits: Heater current in tens of mA, but the last digit is sequentially assigned

Examples:[24]

- AS4100 – Tetrode, 4 V, 1 A (=100⋅10 mA) indirect heater

- FH4105 = E449 = RENS1234 – Indirectly heated remote-cutoff hexode, 4 V, 1.2 A heater

- HP4101 = E446 = RENS1284 – RF pentode, 4 V, 1 A filament

- HP4106 = E447 = RENS1294 – Indirectly heated remote-cutoff RF pentode, 4 V, 1.1 A heater

- HR406 = A425 = RE034 – RF triode, 4 V, 60 mA (=6⋅10 mA) filament

- L414 = B409 = RE134 – Triode, 4 Volt, 140 mA (=14⋅10 mA) filament

- MH2018 = B2048 = RENS1824 – Hexode mixer, 20 V, 180 mA (=18⋅10 mA) heater

- MH4100 = E448 = RENS1224 – Indirectly heated sharp-cutoff hexode frequency changer, 4 V, 1.2 A heater

- PP4101 = E443H = RES964 = L496D – Power pentode, 4 V heater

- PV4200 – Full-wave rectifier, 4 V, 2 A (=200⋅10 mA) filament

- PP2018D = B2043 = RENS1823D = L2318D – Indirectly heated power pentode, 20 V, 180 mA DC series heater

- R2018 = B2038 = REN1821 = A2118 – Triode, 180 mA heater

- S406 = A442 = RES094 – Directly heated tetrode with a screen grid, 4 V, 60 mA filament

- S2018 = B2042 = RENS1820 = H2018D – RF Tetrode, 180 mA heater

Media related to Tungsram vacuum tubes at Wikimedia Commons

- Russian systems

Main article: Russian tube designations

Vacuum tubes produced in the former Soviet Union and in present-day Russia are designated in Cyrillic. Some confusion has been created in transliterating these designations to Latin.

The first system was introduced in 1929. It consisted of one or two letters (designating system type and, optionally, type of cathode), a hyphen, then a sequentially assigned number with up to 3 digits.[25]

In 1937, the Soviet Union purchased a tube assembly line from RCA (who at the time had difficulties raising funds for their basic operations), including production licenses and initial staff training, and installed it on the Svetlana/Светлана plant in St. Petersburg, Russia. US-licensed tubes were produced since then under an adapted RETMA scheme.

Examples:[27]

- 6Ф5 = 6F5 – High-mu triode

- 6Ф6 = 6F6 – Power pentode

- 6Х6 = 6H6 – Dual diode

- 6Ж7 = 6J7/EF37 – Sharp-cutoff pentode

- 6Л6 = 6L6 – Beam tetrode

- 6Л7 = 6L7 – Pentagrid converter

- 6Н7 = 6N7 – Dual power triode

- GOST standard tubes system

In the 1950s a 5-element system (Russian: Государственный Стандарт "State standard" ГОСТ/GOST 5461-59, later 13393-76) was adopted in the (then) Soviet Union for designating receiver vacuum tubes.[28][29]

The first element is a number specifying filament voltage. The second element is a Cyrillic letter specifying the type of device. The third element is a sequentially assigned number that distinguishes between different devices of the same type. The fourth element denotes the type of envelope. An optional fifth element consists of a hyphen followed by one or more characters to designate special characteristics of the tube. This usually implies construction differences, not just selection from regular quality production.

- Professional tubes system

There is another designation system for professional tubes such as transmitter ones.[30][25]

The first element designates function. The next elements varies in interpretation. For ignitrons, rectifiers, and thyratrons, there is a digit, then hyphen, then the anode current in amperes, a slash, anode voltage in kV. A letter may be attached to designate water cooling (no letter designates a radiation cooled device). For transmitting tubes in this system, the second element starts with a hyphen, a sequentially assigned number, then an optional letter specifying cooling method. For phototubes and photomultipliers, the second element is a sequential number and then a letter code identifying vacuum or gas fill and the type of cathode.

- Japanese systems

- Older numbering system 1941–51

A letter: Structure and usage[31]

- E – Electron ray tube

- K – Kenotron (rectifier)

- U – General-purpose tube

Then a letter: Base and outline

- N – Pigtailed (Acorn tubes, etc.)

- S – Octal glass/metal

- T – ST large 7-pin

- t – ST small 7-pin

- V – 4-pin

- X – S/ST 4-pin

- x – Peanut 4-pin

- Y – S/ST 5-pin

- y – Peanut 5-pin

- Z – S/ST 6-pin U6A

Then a hyphen ("-"), followed by a sequentially assigned number or the designation of the American original

Then an optional hyphen ("-"), followed by a letter: Version

Examples:[32]

- EZ-6G5 = 6G5 – Variable-mu "Magic Eye"-type tuning indicator

- KX-80-B – Kenotron

- UN-954 = 954 – Acorn sharp-cutoff pentode

- UN-955 = 955 – Acorn triode

- US-6A8 = 6A8 – Pentagrid converter

- US-6L7G = 6L7G – Pentagrid converter

- UX-26-B – Medium-mu RF triode

- UX-167 – Sharp-cutoff RF pentode

- UY-47B – Pentode

- UZ-58-A – Remote-cutoff RF/IF pentode

- JIS C 7001 system

JIS C 7001 was published in 1951 and modified in 1965 and 1970[31]

A number: Heater voltage range, as in the RETMA scheme

- 1 – 1 V ≤ Uf < 2 V

- 2 – 2 V ≤ Uf < 2.5 V

- 3 – 2.5 V ≤ Uf < 4 V

- 4 – 4 V ≤ Uf < 5 V

- 5 – 5 V ≤ Uf < 6 V

- 6 – 6 V ≤ Uf < 7 V

etc.

Then a letter: Base and Outline

- A – Special base

- B – Other

- C – Compactron (Duodecal)

- D – Subminiature button base

- E – Subminiature flat base

- F – European 4-pin ST

- G – Octal base glass tube (GT)

- H – Magnoval

- K – Ceramic

- L – Lock-In (Loktal)

- M – Miniature (7-pin)

- N – Nuvistor

- Q – Acorn tube

- R – Noval (9-pin miniature) or Neonoval (9T9)

- S – Octal (US)

- T – Large 7-pin ST

- W – 7-pin ST

- X – 4-pin ST

- Y – 5-pin ST

- Z – 6-pin ST

Then a hyphen ("-"), followed by a letter: Structure and usage

- A – Power triode

- B – Beam power tube

- C – Pentagrid converter

- D – Diode

- E – Optical tuning/level indicator

- G – Gas-filled rectifier

- H – High-mu triode (μ>30)

- K – Kenotron (rectifier)

- Even number after K: Full-wave rectifier

- Odd number after K: Half-wave rectifier

- L – Low-mu triode (μ<30)

- P – Power tetrode or pentode

- R – Sharp-cutoff tetrode or pentode

- S – Tetrode with a space charge grid (the 2nd grid is the control grid)

- T – Gas-filled, grid-controlled

- V – Variable-mu (remote-cutoff) tetrode and pentode

- X – Other

Then a sequentially assigned number

Then an optional letter: Version

Examples:[32]

- 2N-H12 – Nuvistor

- 2X-L2A – Low-mu triode

- 6C-A10 – Power triode

- 6G-A4 – Power triode

- 6G-B8 – Beam power tube

- 6G-E12A – 2-channel "Magic Eye"-type tuning indicator, rectangular target

- 6H-B26 – Beam power tube

- 6M-DE1 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

- 6M-E4 – "Magic Finger"-type tuning indicator, miniature 7-pin base B7G

- 6M-E5 = 6ME5 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

- 6M-E10 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

- 6N-H10 – Nuvistor

- 6R-A8 – Power triode

- 6R-B10 – Beam power tube

- 6R-B11 – Beam power tube

- Military naming systems

- British CV naming system

This system prefixes a three- or four-digit number with the letters "CV", meaning "civilian valve" i.e. common to all three armed services. It was introduced during the Second World War to rationalise the previous nomenclatures maintained separately by the War Office/Ministry of Supply, Admiralty and Air Ministry/Ministry of Aircraft Production on behalf of the three armed services (e.g. "ACR~", "AR~", "AT~", etc. for CRTs, receiving and transmitting valves used in army equipments, "NC~", "NR~" and "NT~" similarly for navy equipments and "VCR~", "VR~" and "VT~" etc. for air force equipments), in which three separate designations could in principle apply to the same valve (which often had at least one prototype commercial designation as well). These numbers generally have identical equivalents in both the North American, RETMA, and West European, Mullard–Philips, systems but they bear no resemblance to the assigned "CV" number.

Examples:

- CV1988 = 6SN7GT = ECC32 (not a direct equivalent as heater current is different and bulb is larger)

- CV2729 = E80F – An SQ version of EF80 but with revised pin-out and a base screen substituted for the RF screen

- CV4007 = E91AA – SQ version of 6AL5

- CV4010 = E95F – SQ version of 6AK5 or EF95

- CV4014 = M8083 – SQ version of EF91 or 6AM6 (The 'M' in the part number denotes that it was developed by the military)

Note: The 4000 numbers identify special-quality valves though SQ valves CV numbered before that rule came in retain their original CV number.

The principle behind the CV numbering scheme was also adopted by the US Joint Army-Navy JAN numbering scheme which was later considerably expanded into the US Federal and then NATO Stock Number system used by all NATO countries. This part-identification system ensures that every particular spare part (not merely thermionic valves) receives a unique stock number across the whole of NATO irrespective of the source, and hence is not held inefficiently as separate stores. In the case of CV valves, the stock number is always of the format 5960-99-000-XXXX where XXXX is the CV number (with a leading 0 if the CV number only has 3 digits).

- U.S. naming systems

One system prefixes a three-digit number with the letters "VT", presumably meaning "Vacuum Tube". Other systems prefix the number with the letters "JHS" or "JAN". The numbers following these prefixes can be "special" four-digit numbers, or domestic two- or three-digit numbers or simply the domestic North American "RETMA" numbering system. Like the British military system, these have many direct equivalents in the civilian types. Confusingly, the British also had two entirely different "VT" nomenclatures, one used by the Royal Air Force (see the preceding section) and the other used by the General Post Office, responsible for post and telecommunications at the time, where it may have stood for "valve, telephone"; none of these schemes corresponded in any way with each other.

Examples:

- "VT" numbering systems

- North American VT90 = 6H6

- British (RAF) VT90 – VHF Transmitting triode

- British (GPO) VT90 = ML4 = CV1732 – Power triode

- VT104 – RF pentode

- VT105 – RF triode

- Other numeral-only systems

Various numeral-only systems exist. These tend to be used for devices used in commercial or industrial equipment. The oldest numbering systems date back to the early 1920s, such as a two-digit numbering system, starting with the UV-201A, which was considered as "type 01", and extended almost continuously up into the 1980s. Three- and four-digit numeral-only systems were maintained by R.C.A., but also adopted by many other manufacturers, and typically encompassed rectifiers and radio transmitter output devices. Devices in the low 800s tend to be transmitter output types, those in the higher 800s are not vacuum tubes, but gas-filled rectifiers and thyratrons, and those in the 900s tend to be special-purpose and high-frequency devices. Use was not rigorously systematic: the 807 had variants 1624, 1625, and 807W.

- Other letter followed by numerals

There are quite a number of these systems from different geographical realms, such as those used on devices from contemporary Russian and Chinese production. Other compound numbering systems were used to mark higher-reliability types used in industrial or commercial applications. Computers and telecommunication equipment also required tubes of greater quality and reliability than for domestic and consumer equipment.

Some letter prefixes are manufacturer's codes:

- C – RCA/Cunningham

- CK, QK, RK – Raytheon Company

- ECG – Philips/Sylvania

- EM – Eitel McCullough

- F – Federal Telephone and Radio

- GE, GL – General Electric Corp. (not British General Electric Company)

- HK – Heintz & Kaufman, Ltd. (San Francisco, California, USA)

- HY – CBS/Hytron

- ML – Machlett Laboratories, Inc.

- NL – National Electronics, Inc.

- NU – National Union Electric Corp.

- PL – Philips N.V.

- RCA – RCA/Radiotron

- SV – Svetlana/Светлана

- SY – Standard Telephones and Cables Ltd./Brimar

- TH – Compagnie Française Thomson-Houston

- WE – Western Electric Company

- WL – Westinghouse Electric Corp.

- XD – Central Electronic Manufacturers (Denville, New Jersey, USA)

For examples, see below

Some designations are derived from the behavior of devices considered to be exceptional.

- Mazda/EdiSwan sold their first tubes for 4-volts AC mains transformer (as opposed to home storage battery) heating with the prefix AC/ (for examples see below).

- The first beam tetrodes manufactured in the UK in the late 1930s by M-OV, carried a "KT" prefix meaning Kinkless Tetrode (for examples see below).

- List of American RETMA tubes, with European equivalents

Note: Typecode explained above. See also RETMA tube designation

- "0 volt" gas-filled cold cathode tubes

First character is numeric zero, not letter O.

- Voltage stabilisers and references. Function in a similar way to a Zener diode, at higher voltages. Letter order (A-B-C) indicates increasing voltage ratings on octal-based regulators and decreasing voltage ratings on miniature-based regulators.

- 0A2 – 150 volt regulator, 7-pin miniature base

- 0A3 – 75 volt regulator, octal base, aka VR75

- 0B2 – 105 volt regulator, 7-pin miniature base

- 0B3 – 90 volt regulator, octal base, aka VR90

- 0C2 – 75 volt regulator, 7-pin miniature base

- 0C3 – 105 volt regulator, octal base, aka VR105

- 0D3 – 150 volt regulator, octal base, aka VR150

- Other cold-cathode tubes

- 0A4G – 25 mAavg, 100mApeak Gas triode designed for use as a ripple control receiver; with the cathode tied to the midpoint of a series-resonance LC circuit across live mains, it would activate a relay in its anode circuit while fres is present

- 0Y4 – 40 ≤ I ≤ 75 mA Half-wave gas rectifier with a starter anode, 5-pin octal base

- 0Z4 – 30 ≤ I ≤ 90 mA Argon-filled, full-wave gas rectifier, octal base. Widely used in vibrator power supplies in early automobile radio receivers.

- 1 volt heater/filament tubes

- Tubes with up to 1.4 volt heaters

- 1B3GT – High-voltage rectifier diode with 1.25 V filament common in monochrome TV receivers of the 1950s and early 1960s. Peak inverse voltage of 30 kV. Anode current 2 mA average, 17 mA peak. Derived from the earlier industrial type 8016. (International Octal base.)

- 1V2 – High voltage rectifier with 0.625 V/300 mA filament (B7G base)

- Tubes with 1.4 volt DC heaters

- 1A3 – High frequency diode with indirectly heated cathode. Used as a detector in some portable AM/FM receivers.

- 1A7GT/DK32 – Pentagrid converter

- 1G6-G – Dual power triode. "GT" version also available.

- 1L6 – Pentagrid frequency changer for battery radios with 50 mA filament

- 1LA6 (loctal) and later 1L6 (7-pin miniature) – Battery pentagrid converter for Zenith Trans-Oceanic shortwave radio, 50 mA filament

- 1LB6 – Superheterodyne mixer for battery-operated radios

- 1LC6 – Similar to type 1LA6, but with higher conversion transconductance

- 1R5/DK91 – Pentagrid converter, anode voltage in the 45...90 volt range.

- 1S4 – Power output pentode Class-A amplifier, anode voltage in the 45...90 volt range.

- 1S5 – Sharp-cutoff pentode Class-A amplifier, and diode, used as detector and first A.F. stage in battery radio receivers. Anode voltage in the 67...90 volt range.

- 1T4/DF91 – Remote-cutoff R.F. Pentode Class-A amplifier, used as R.F. and I.F. amplifier in battery radio receivers (B7G base).

- 1U4 – Sharp-cutoff R.F. Pentode Class-A amplifier, used as R.F. and I.F. amplifier in battery radio receivers, similar characteristics to 6BA6 (B7G base).

- 1U6 – Nearly identical to type 1L6, but with a 1.4 V/25 mA filament

- 1.25 volt filament subminiature tubes

The following tubes were used in post-World War II walkie-talkies and pocket-sized portable radios. All have 1.25 volt DC filaments and directly heated cathodes. Some specify which end of the filament is to be powered by the positive side of the filament power supply (usually a battery). All have glass bodies that measure from 0.285 to 0.400 inches (7.2 to 10.2 millimetres) wide, and from 1.25 to 2.00 inches (32 to 51 millimetres) in overall length.

- 1AC5 – Power pentode, FL

- 1AD4 – Sharp-cutoff pentode, FL

- 1AD5 – Sharp-cutoff pentode, R8

- 1AE5 – Heptode mixer, FL

- 1AG4 – Power pentode, FL

- 1AG5 – Diode, pentode, FL

- 1AH4 – RF pentode, FL

- 1AJ5 – Diode, sharp-cutoff pentode, FL

- 1AK4 – Sharp-cutoff pentode, FL

- 1AK5 – Diode, sharp-cutoff pentode, FL

- 1C8 – Pentagrid converter, R8

- 1D3 – Low-mu high-frequency triode, R8

- 1E8 – Pentagrid converter, R8

- 1Q6 – Diode, pentode, R8

- 1S6 – Diode, pentode, R8

- 1T6 – Diode, pentode, R8

- 1V5 – Power pentode, R8

- 1V6 – Triode-pentode converter, FL

- 1W5 – Sharp-cutoff pentode, R8

- 1 prefix for home receivers

These tubes were made for home storage battery receivers manufactured during the early to mid-1930s. The numbers of the following tubes all start with 1, but these tubes all have 2.0 volt DC filaments. This numbering scheme was intended to differentiate these tubes from the tubes with 2.5 volt AC heaters listed below.

- 1A4-p – Remote-cutoff pentode

- 1A4-t – Remote-cutoff tetrode

- 1A6 – Pentagrid converter up to only 10 MHz due to low heater power (2 V/60 mA) and consequent low emission in the oscillator section; also occasionally used as a grid-leak detector

- 1A7-GT – Re-engineered version of types 1A6 and 1D7-G, designed for use in portable AC/DC/Dry-cell battery radios introduced in 1938. Has 1.4 V/50 mA filament.

- 1B4-p – Sharp-cutoff pentode

- 1B4-t – Sharp-cutoff tetrode

- 1B5 – Dual detector diode, medium-mu triode. Usually numbered 1B5/25S

- 1B7-GT – Re-engineered version of types 1C6 and 1C7-G, designed for use in dry-cell battery radios with shortwave bands. Has 1.4 V/100 mA filament

- 1C5 – Power pentode (similar to 3Q5 except for filament)

- 1C6 – Pentagrid converter; 1A6, with double the heater power and double the frequency range

- 1C7-G – Octal version of type 1C6.

- 1D5-Gp – Octal version of type 1A4-p.

- 1D5-Gt – Octal version of type 1A4-t. (Note: This is a shouldered "G" octal, not a cylindrical "GT" octal.)

- 1D7-G – Octal version of type 1A6.

- 1E5-Gp – Octal version of type 1B4-p.

- 1E5-Gt – Octal version of type 1B4-t. (Note: This is a shouldered "G" octal, not a cylindrical "GT" octal.)

- 1E7-G – Dual power pentode for use as a driver when parallel-connected, or as a push-pull output. "GT" version also available

- 1F4 – Power pentode

- 1F5-G – Octal version of 1F4.

- 1F6 – Duplex diode, sharp-cutoff pentode

- 1F7-G – Octal version of type 1F6

- 1G5-G – Power pentode

- 1H4-G – Medium-mu triode, can be used as a power triode. Octal version of type 30, which is an upgraded version of type 01-A. "GT" version also available.

- 1H6-G – Octal version of type 1B5/25S. "GT" version also available.

- 1J5-G (950) – AF Power pentode

- 1J6-G – Dual power triode, octal version of type 19. "GT" version also available.

- 2 volt heater/filament tubes

- Tubes used in AC-powered radio receivers of the early 1930s. All have 2.5 volt heaters.

- 2A3 – Directly heated power triode, used for AF output stages in 1930s–1940s audio amplifiers and radios.

- 2A5 – Power Pentode (Except for heater, electronically identical to types 42 and 6F6)

- 2A6 – Dual diode, high-mu triode (Except for heater, electronically identical to type 75)

- 2A7 – Dual-tetrode-style pentagrid converter (Except for heater, electronically identical to types 6A7, 6A8 and 12A8)

- 2B7 – Dual diode and remote-cutoff pentode (Except for heater, electronically identical to type 6B7)

- 2E5 and 2G5 – Electron-ray indicators ("Eye tube") with integrated control triode. (Except for heater, electronically identical to types 6E5 and 6G5)

- Tubes used in television receivers

- 2AF4 – UHF triode oscillator

- 2BN4 – VHF/RF triode

- 2CW4 – Nuvistor high-mu VHF triode, 6CW4 with a 2.1 volt/450 mA heater; used in TV receivers with series heater strings

- 2CY5 – VHF sharp-cutoff RF tetrode

- 2EA5 – VHF sharp-cutoff RF tetrode

- 2EN5 – Dual-diode

- 2ER5 – VHF RF triode

- 2ES5 – VHF RF triode

- 2EV5 – VHF sharp-cutoff RF tetrode

- 2FH5 – VHF RF triode

- 2FQ5 – VHF RF triode

- 2FV6 – VHF sharp-cutoff RF tetrode

- 2FY5 – VHF RF triode

- 2X2 – High Vacuum High Peak inverse voltage diode, used as rectifier in CRT EHT supplies. Similar to 1B3 and 1S2 except for heater voltage.

- 3 volt heater/filament high voltage rectifier tubes

- 3A3/3B2/3AW3/ - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.22 amps.

- 3CA3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.6 volts and 0.225 amps.

- 3CN3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

- 3CU3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.28 amps.

- 3CZ3 - High Voltage rectifier. An octal type used in color television sets. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

- 3AT2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. It comes in the variation as the 3AT2B, mainly for color television sets with a large picture tube. The 3AT2B comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.22 amps.

- 3AW2 - High Voltage rectifier. A compactron used for color and black and white television sets. It comes in the variation as the 3AW2A as a replacement for the 3AW2 after the 1967 General Electric X-radiation scandal. The 3AW2A comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.22 amps.

- 3BF2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. This tube is very rare, and very special, because it implements an indirectly heated cathode, not connected to the filament. No data is found on this tube, except for the filament power (which is 3.6 volts, 0.225 amps) and the base (which is the 12GQ type). The only reason why we know it is a high voltage rectifier is that the base tells us so.

- 3BL2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. It comes in the variation as the 3BL2A as a replacement for the 3BL2 after the 1967 General Electric X-radiation scandal. The 3BL2A comes with X-radiation shielding on the inside. The heater power is 3.3 volts and 0.285 amps.

- 3BM2 - High Voltage rectifier. A compactron used in television sets to supply power to the anode of the picture tube. It comes in the variation as the 3BM2A as a replacement for the 3BM2 after the 1967 General Electric X-radiation scandal. The 3BM2A comes with X-radiation shielding on the inside. The heater power is 3 volts and 0.3 amps.

- 3BN2 - High Voltage rectifier. A compactron used for color television sets. It comes in the variation as the 3BN2A as a replacement for the 3BN2 after the 1967 General Electric X-radiation scandal. The 3BN2A comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.22 amps.

- 3BS2 - High Voltage rectifier. A compactron used for color television sets. It comes in the variation as the 3BS2A and 3BS2B as a replacement for the 3BN2 after the 1967 General Electric X-radiation scandal. The 3BS2A and 3BS2B tubes are identical, maybe a small difference in ratings and characteristics. We do not know these differences as the 3BS2B tube data is not available. The 3BS2A and 3BS2B comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

- 3BT2 - High Voltage rectifier. A compactron used for color television sets. It comes in the variation as the 3BT2A as a replacement for the 3BT2 after the 1967 General Electric X-radiation scandal. The 3BT2A comes with X-radiation shielding on the inside. The heater power is 3.15 volts and 0.48 amps. The large current is for the advantage of fast warm-up.

- 3BW2 - High Voltage rectifier. A compactron used for color and black and white television sets. The 3BW2 comes with X-radiation shielding on the inside. It also comes with diffusion bonded cathode (a type of cathode that prevents the back-emission of the anode). This tube was designed in the December of 1970, after the 1967 General Electric X-radiation scandal. All high voltage rectifier tube types that were designed before 1967 had no X-radiation protection internally. That is why all these tubes made during and after 1967 have a suffix showing they had internal X-radiation protection. This is why there is no '3BW2A' type since it was made after 1967. The heater power is 3.15 volts and 0.22 amps.

- Nominally 5 volt heater/filament tubes

- 5J6 – General purpose RF dual triode with common cathodes, a 6J6 with a 4.7 volt 600 mA controlled warm up heater[33]

- 5 volt heater/filament full wave rectifier tubes

- 5AR4, GZ34 – Full wave rectifier

- 5AS4 – Full wave rectifier

- 5R4 – Full wave rectifier

- 5U4 – Full wave rectifier

- 5V4, GZ32 – Full wave rectifier

- 5Y3 – Full-wave rectifier, octal base version of type 80

- 6 volt heater/filament tubes

- 6A6 – Dual Power Triode, used as a Class-A audio driver or a Class-B audio output. UX6 base. 6.3 volt heater version of type 53 which had a 2.5 volt heater. Octal version – 6N7.

- 6A7 and 6A8 (PH4, X63) – Superheterodyne Pentagrid converter – dual tetrode style. Based on type 2A7, which had a 2.5 volt heater. 6A7 has a UX7 base with top cap connection for control grid (grid 4). 6A8 is octal version with top cap connection for control grid. Loctal version: type 7B8.

- 6AB4/EC92 – High-mu triode (Pinout same as 6C4 except for pin 5 not having a connection)

- 6AB5/6N5 – "Magic Eye" cathode ray tuning indicator

- 6AC5-G – High-mu Power Triode

- 6AC7, 1852 – TV sharp-cutoff R.F. Pentode. (Often encountered in a black metal envelope, not to be confused with the 6CA7.)

- 6AC10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 6AD6-G and 6AF6-G – "Magic Eye" tuning indicators. Both have two "pie wedge" shadow indicators, one each on opposite sides of a single circular indicator target. Both shadows may be used in tandem or may be driven by two different signal sources. Type 6AE6-G is specifically made to drive each indicator with different signals. May also be driven by separate pentodes with different characteristics. E.g., a sharp-cutoff pentode like a 6J7 – which would be hyper-sensitive to any signal change – would drive one shadow, while a remote-cutoff pentode like a 6K7 – which would only react to stronger signals – would drive the other shadow. Both tubes have octal bases. Type 6AD6-G, with a target voltage rated from 100 to 150 volt, is designed for AC/DC radios. Type 6AF6-G, with a target voltage rated at 250 volt, is designed for larger AC radios.

- 6AE6-G – A driver triode specially designed for "Magic Eye" tuning indicator types 6AD6-G and 6AF6-G. Has a common heater and indirectly heated cathode, two internally connected triode grids – one with sharp-cutoff characteristics, one with remote-cutoff characteristics – and two anodes, one for each grid. The sharp-cutoff grid reacts to any signal change, while the remote-cutoff grid reacts only to stronger signal changes.

- 6AE7-GT – Dual Triode with a common, single anode, for use as a power triode driver

- 6AF4 – UHF Medium-mu Triode, commonly found in TV UHF tuners and converters.

- 6AF11 – Compactron High-mu dual triode and sharp-cutoff pentode

- 6AG11 – Compactron High-mu dual triode and dual diode

- 6AH5-G – Beam power tube for early TV use. Same as type 6L6-G, but with scrambled pinout. Used in some Philco receivers.

- 6AK5, EF95, 5654, CV4010, 6Ж1П – Miniature V.H.F. Sharp-cutoff pentode (Used in old Radiosonde weather balloon transmitters, receiver front ends and contemporary audio equipment) B7G, (Miniature 7-pin) base

- 6AK6 – Power pentode. 7-pin miniature version of type 6G6-G. Unusual low-power consumption output tube with 150 mA heater.

- 6AK8/EABC80 – Triple Diode, High-mu Triode. Diodes have identical characteristics – two have cathodes connected to the triode's cathode, one has a separate cathode. Used as a combination AM detector/AVC rectifier/FM ratio detector/A.F. amplifier in AM/FM radios manufactured outside of North America. Triode amplification factor: 70. North American type 6T8 is identical (but for a shorter glass envelope) and may be used as a substitute.

- 6AK9 – Compactron 1x high-mu + 1x medium-mu dual triode and beam power pentode, 12-pin base

- 6AK10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 6AL3, EY88 – TV "Damper/Efficiency" Diode

- 6AL5, EAA91, D77 – Dual Diode, Detector. Often used in vacuum tube volt meters (VTVMs). Miniature version of type 6H6.

- 6AL6-G – Beam power tube for early TV use. Same as type 6L6-G, but with scrambled pinout and anode connected to top cap.

- 6AL7-GT – Tuning indicator used in many early AM/FM Hi-Fi radios. Similar in function to "Magic Eye" tubes. Has two bar-shaped shadows; one grows to indicate signal strength, the other moves to indicate center tuning on FM.

- 6AM6, EF91, Z77 – Sharp-cutoff R.F. pentode used in receiver front ends and test gear such as VTVMs and TV broadcast modulation monitors.

- 6AN7, ECH80 – Triode-Hexode Local Oscillator/Mixer (radio)

- 6AN8 – Triode-Pentode used in frame timebase circuits for television. Electrically fairly similar to ECL80 but with a different pinout.

- 6AQ5 – Beam-power pentode, 7-pin miniature similar of type 6V6.

- 6AQ8/ECC85 – Dual triode with internal shield. Designed for use as oscillator and mixer in FM receivers. The heater to cathode insulation is inadequate for use in cascode operation

- 6AR8, 6JH8, 6ME8 – Beam deflection tubes for use as NTSC chroma signal demodulators in analog color TV receivers

- 6AS6 – Pentode with a fine-pitched suppressor grid which could serve as a second control grid. Used in radar phantastron circuits.

- 6AS7, 6080 – Dual low-mu Triode, low impedance, mostly used for voltage regulation circuits.

- 6AS11 – Compactron 1x high-mu + 1x medium-mu dual triode and sharp-cutoff pentode, 12-pin base

- 6AT6 – Dual Diode, High-mu Triode, miniature version of type 6Q7. Triode amplification factor: 70.

- 6AU4 – TV "Damper/Efficiency" Diode

- 6AU6, EF94, 6AU6A – Sharp-cutoff pentode

- 6AV6 – Dual Diode, High-mu Triode, miniature version of type 75. Triode amplification factor: 100. (Triode section similar in characteristics to one half of a 12AX7.)

- 6AV11 – Compactron Medium-mu triple triode, 12-pin base

- 6AX4 – TV "Damper/Efficiency" Diode

- 6AX5 – Full-wave rectifier. Octal base. Similar in structure to type 6X5, but with higher voltage and current ratings which are comparable to those of types 5Y3 and 80.

- 6B6-G – Double-Diode High-mu Triode. Octal version of type 75. Has top-cap connection for triode grid. Later octal version, type 6SQ7, has under-chassis connection for triode grid. Miniature version: 6AV6.

- 6B7 and 6B8 (EBF32) – Double-Diode, Semiremote-cutoff Pentodes. Based on type 2B7 which had a 2.5 volt heater. Type 6B7 has a UX7 base with a top-cap connection for the control grid (grid 1). Type 6B8 has an octal base with a top cap. The diode anodes are most commonly used as (second) detectors and AVC rectification in superheterodyne receivers. Because their control grids have both sharp-cutoff and remote-cutoff characteristics, these types were used as I.F. amplifiers with AVC bias to the control grid, and as A.F. amplifiers. These types were also used in reflex radios. In a typical 2B7/6B7/6B8 reflex circuit, the I.F. signal from the converter is injected into the pentode and is amplified. The diodes then act as detectors, separating the A.F. signal from the R.F. signal. The A.F. signal is then re-injected into the pentode, amplified, and sent to the audio output tube.[34]

- 6BA6, EF93, W727, 5790 – Semiremote-cutoff R.F. Pentode (Often encountered in car radios)

- 6BD11 – Compactron 1x high-mu + 1x medium-mu dual triode and sharp-cutoff pentode, 12-pin base

- 6BE6, EK90, 5750, X727 – Pentagrid Converter (Often encountered in car radios)

- 6BF6 – Dual Diode, Medium-mu triode. Miniature version of octal type 6R7.

- 6BF8 – Sextuple diode with a common cathode

- 6BG6 – Beam tetrode, anode cap. Used in early TV magnetic-deflection horizontal-output stage.

- 6BH11 – Compactron Medium-mu dual triode and sharp-cutoff pentode

- 6BK4 – High Voltage beam Triode (30 kV anode voltage). Used as shunt regulator in color TV receivers and measurement equipment such as high voltage meters

- 6BK7 – Dual Triode with Internal shield between each section, used in RF circuits (Similar to 6BQ7)

- 6BK8, EF86, Z729 – Audio Pentode used in microphone preamplifiers and audiophile equipment

- 6BK11 – Compactron 2x High-mu + 1x medium-mu triple triode preamplifier, 12-pin base; used in some guitar amps made by Ampeg.

- 6BL6 (5836) – Sutton tube, a reflex klystron used as a 250 mW CW microwave source, 1.6 to 6.5 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

- 6BL8, ECF80 – General-purpose Triode pentode used in TV, audio and test gear

- 6BM6 (5837) – Sutton tube used as a 150 mW CW microwave source, 550 MHz to 3.8 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

- 6BM8, ECL82 – Triode pentode used as the driver and output stages in audio amplifiers, audio output and vertical output stages in TV receivers and has even been seen in an electronic nerve stimulator.

- 6BN6 – Gated-beam discriminator pentode, used in radar, dual channel oscilloscopes and F.M. quadrature detectors (cf. 6DT6, nonode).

- 6BQ5, EL84,(N709) – 5.7 Watts AF Power pentode, noval base

- 6BQ6-GT – Beam Power Pentode, used as a Horizontal Deflection Output tube in monochrome TV receivers of the 1950s. Most commonly used in receivers with diagonal screen sizes less than 19 inches (48 cm). (However, may be found in some larger models.) Larger receivers often used similar type 6DQ6. Later versions of this tube branded as 6BQ6-GTB/6CU6.

- 6BQ7 – Dual RF/VHF triode with internal screen. The two sections can be used independently or in a cascode stage

- 6BQ7A – Improved 6BQ7 capable of operation at UHF frequencies

- 6BU8 – Split Anode TV Sync Separator

- 6BX6, EF80 – Sharp-cutoff RF/IF/Video pentode, noval base

- 6BY6 – Similar to type 6CS6, but with higher transconductance. 3BY6 with a different heater

- 6BY7, EF85, W719 – Remote-cutoff R.F. Pentode (TV IF)

- 6BZ6 – Sharp-cutoff R.F. pentode used in video I.F. circuits of the 1960s.

- 6BZ7 – Dual Triode. See 6BK7

- 6C4/EC90 – 3.6 W small-power V.H.F. triode up to 150 MHz; single 12AU7/ECC82 system

- 6C6 – Sharp-cutoff R.F. Pentode. Most common commercial uses were as a tuned R.F. amplifier, a detector, and an A.F. amplifier. Also used in test equipment. Has UX6 base with top cap. Based on type 57, which had a 2.5 volt heater. Similar to types 1603, 77 and octal types 6J7 and 6SJ7

- 6C10 – Compactron High-mu triple triode, 12-pin base – not related to the Mazda/EdiSwan 6C10 triode-hexode

- 6CA4, EZ81 – Full Wave Rectifier

- 6CA7, EL34 – Audio Power Output Pentode

- 6CA11 – Compactron High-mu dual triode and sharp-cutoff pentode

- 6CB6 – Remote-cutoff R.F. Pentode used in video I.F. circuits of the 1950s and early 1960s.

- 6CG7 – Dual Triode (used in TV and some audio amplifiers including modern solid-state designs often as a cathode follower, similar to 6SN7)

- 6CJ6 – Line Output Pentode

- 6CL6 – Power pentode

- 6CM5, EL36, EL360 – Audio and TV Line Output Beam Power Tetrode.

- 6CW4 – Nuvistor high-mu VHF triode, most common one in consumer electronics

- 6CZ5 - Beam pentode for use in vertical deflection or audio amplifier. In certain applications, it can be used in place of a 6973.

- 6D4 – 25 mAavg, 100 mApeak Indirectly heated, argon triode thyratron, negative starter voltage, miniature 7-pin base; found an additional use as a 0 to 10 MHz noise source, when operated as a diode (starter tied to cathode) in a transverse 375 G magnetic field. Sufficiently filtered for "flatness" ("white noise") in a band of interest, such noise was used for testing radio receivers, servo systems and occasionally in analog computing as a random value source.

- 6D6 – Remote-Cutoff R.F. Pentode. Most common commercial uses were as an I.F. amplifier or as a superheterodyne mixer, aka 1st detector. Also used in test equipment. Has UX6 base with top cap. Based on type 58, which had a 2.5 volt heater. Similar to type 78. Octal version: 6U7-G.

- 6D8-G – Superheterodyne Pentagrid converter, similar to type 6A8. Octal base with top cap. Has 150 mA heater. Used in pre-war 6-volt farm radios.

- 6D10 – High-mu triple triode for use as oscillator, mixer, amplifier or AGC tube, 12-pin base

- 6DA6, EF89 – R.F. Pentode used in AM/FM radios manufactured outside North America.

- 6DJ8, ECC88, E88CC, 6922, 6N23P, 6N11 – Dual Audio and R.F. Triode (often used in TV broadcast equipment, test gear, oscilloscopes and audiophile gear) similar to 6ES8

- 6DQ6 – Beam Power Pentode, used as a Horizontal Deflection Output tube in monochrome TV receivers of the 1950s. Most often found in receivers with diagonal screen measurements larger than 17 inches (43 cm). Smaller receivers often used similar type 6BQ6-GT. Also used as Audio Output tubes in Standel guitar amplifiers. Later versions branded as 6DQ6-B/6GW6.

- 6DR8, EBF83 – R.F. pentode which will operate with 12 V anode supply, used as I.F. amplifier in car radios which run directly off the 13.5 volt supply.

- 6DS4 – Nuvistor R.F. triode used in TV tuners immediately prior to the introduction of solid state tuning circuits. (RCA TVs equipped with a 6DS4 tuner bore the trademark "Nu-Vista Vision"); successor of the 6CW4.

- 6DS8, ECH83 – Triode-heptode Local oscillator-Mixer which will operate with 12 V anode supply, used in car radios which run directly off the 13.5 volt supply.

- 6DT6 – Quadrature detector used in TV audio circuits of the 1950s and early 1960s; cf. 6BN6, nonode.

- 6DV4 – Medium-mu Nuvistor triode for UHF oscillators; some versions had a gold-plated envelope

- 6DX8 – Triode pentode

- 6E5 – "Magic Eye" Tuning indicator. Has incorporated driver triode with sharp-cutoff grid which makes it extremely sensitive to any changes in signal strength. Has UX6 base. Based on type 2E5, which had a 2.5 volt heater.

- 6EM5 – TV Vertical Output Pentode

- 6ES6, EF98 – R.F. pentode which will operate with 12 V anode supply, used as tuned R.F. amplifier in car radios which run directly off the 13.5 volt supply.

- 6ES8, ECC89, E89CC – Dual Triode used as cascode R.F. amplifier in TV tuners and V.H.F. receiver front ends, also used as general-purpose dual triode in test gear, similar to 6DJ8

- 6EZ8 – High-mu triple triode, 9-pin base

- 6F4 – Acorn UHF triode up to 1.2 GHz, for use as an oscillator

- 6F5 – High-mu triode, equal to triode section of type 6Q7

- 6F6, KT63 – Power Pentode. Octal base version of type 42. Moderate power output rating – 9 watts max. (Single-ended Class-A circuit); 11 watts max. (Push-pull Class-A circuit); 19 watts max. (push-pull Class-AB2 circuit). Available in metal (numbered "6F6"), shouldered glass ("6F6-G"), and cylindrical glass ("6F6-GT"). Sometimes used as a transformer-coupled audio driver for types 6L6-GC and 807 when those tubes were used in Class-AB2 or Class-B amplifiers. Also used as a Class-C oscillator/amplifier in transmitters.

- 6F7 – Remote-cutoff Pentode, Medium-mu Triode. Has UX7 base with top-cap connection for the pentode's control grid (grid 1). Most common uses were as superheterodyne mixer ("first detector") and local oscillator, or as a combination I.F. amplifier (pentode) and (second) detector or A.F. amplifier (triode). Octal version: 6P7-G.

- 6FH8 – Medium-mu triode and three-anode sharp-cutoff tetrode for use in TV receivers and complex wave generators

- 6G5 – "Magic Eye" Tuning indicator. Has incorporated triode with remote-cutoff grid, which makes it less reactive to low-level changes in signal strength. Has UX6 base. Electronically identical to type 6U5 except for indicator. Both types had "pie wedge" shadow indicators. At first, the shadow indicator for type 6G5 was fully closed at zero signal and opened as signal strength increased. For type 6U5, the shadow indicator was fully open at zero signal and closed as signal strength increased. After World War II, type 6G5 was discontinued as a unique tube and all 6U5s were doubled branded either as 6G5/6U5 or 6U5/6G5.

- 6G6-G – Power pentode. Octal base. Low power output – 1.1 watt max. output. Has 150 mA heater. Used in pre-war 6-volt farm radios. Miniature version – 6AK6.

- 6G8-G – Double-Diode Sharp-cutoff Pentode (Used as Detector and first A.F. stage in Australian 1940s radios)

- 6GK5 – Miniature V.H.F. triode (Used as V.H.F. local oscillator in some T.V. Turret Tuners)

- 6GM5 – Beam power pentode, identical to 7591 and 7868 with a mini-noval pin base

- 6GV8, ECL85 – Triode Pentode (TV vertical output)

- 6GW8, ECL86 – Audio Triode Pentode (audio, TV vertical output)

- 6GY8 – High-mu triple triode for use as oscillator, mixer, RF amplifier or AGC tube, 9-pin base

- 6H6, D63, EB34, OSW3109 – Dual diode. Octal base. Most commonly found as a "stubby" metal envelope tube. Glass versions 6H6-G and 6H6-GT are also found.

- 6HS8 – Dual-anode pentode for TV receiver sync separation service or as a two-channel VCA

- 6J5, L63 – Medium-mu triode.

- 6J5WGT – Heater cathode type, medium-mu triode, identical to 12J5WGT except heater characteristics

- 6J6 – Dual general purpose RF triode with common cathodes, operates over much of the UHF band (up to 600 MHz), equivalent to ECC91

- 6J7, EF37 – Sharp-cutoff Pentode. Most common commercial uses were as a tuned R.F. amplifier, a (second) detector, or an A.F. amplifier. Octal version of type 77. This type included a top-cap connection for the control grid. Later version, type 6SJ7, had its control grid connection on pin 4.

- 6J8-G – Triode-Heptode (radio local oscillator/mixer)

- 6JU8A – 9 mA, Four-diode bridge rectifier

- 6K6-G – Power Pentode, octal version of type 41. Low-to-moderate power output rating – 0.35 to 4.5 watts (single-ended Class-A circuit); 10.5 watts max. (push-pull Class-A circuit).

- 6K7, EF39 – Remote-cutoff R.F. pentode. Most common commercial uses were as an I.F. amplifier or as a superheterodyne mixer, aka 1st detector. Also used in test equipment. Octal version of type 78. This type included a top-cap connection for the control grid. Later version, type 6SK7, had its control grid connection on pin 4.

- 6K8 and 12K8 – American Triode-Hexode mixer, 1938

- 6K11 – Compactron 2x High-mu + 1x medium-mu triple triode, 12-pin base

- 6KM8 – Diode and three-anode sharp-cutoff tetrode for use in musical instruments, frequency dividers and complex wave generators

- 6L4 – Acorn UHF triode for use as an oscillator

- 6L5-G – Medium-mu triode (Similar to type 6J5-G, available only in ST shape)

- 6L6/EL37 – High-powered beam tetrode.

There are several variations. Except for types 6L6-GC and 6L6-GX, all have the same maximum output ratings:

- 11.5 watts (single-ended Class-A circuit)

- 14.5 watts (push-pull Class-A circuit)

- 34 watts (push-pull Class-AB1 circuit)

- 60 watts (push-pull Class-AB2 circuit)

6L6 (metal envelope) and 6L6-G (shouldered glass envelope) were used in pre-World War II radios and Public Address amplifiers.

6L6 and 25L6 were introduced in 1935 as the first beam tetrodes. Both types were branded with the L6 ending to signify their (then) uniqueness among audio output tubes. However, this is the only similarity between the two tubes. (Type 6W6-GT is the 6.3 volt heater version of types 25L6-GT and 50L6-GT.)

- 6L6GA – Post-war version of type 6L6-G, in smaller ST-14 shape with Shouldered Tubular, (ST), shaped bulb, revision A.

- 6L6GB – Post-war improved version in a cylindrical glass envelope. Similar to type 5881.

- 6L6GTB – 6L6 with Tubular, (T), shaped bulb, revision B, (higher power rating, as it happens. The 6L6GTB can always replace the 6L6, 6L6G, and 6L6GT, but a 6L6GTB running at maximum rating should not be replaced with another subtype).

- 6L6-WGB – "Industrial" version of type 6L6GB.

- 6L6GC – Final and highest-powered audio version of the tube. Max. outputs:

- 17.5 watts (single-ended Class-A circuit)

- 32 watts (push-pull Class-A circuit)

- 55 watts (push-pull Class-AB1 circuit)

- 60 watts (push-pull Class-AB2 circuit)

- 6L6-GX – Class-C oscillator/amplifier used in transmitters. Max. output 30 watts. (All versions may be used as a Class-C oscillator/amplifier, but this version is specifically designed for this purpose, has a special ceramic base.)

- 6L7 – Pentagrid converter often used in console radios of the late 1930s. Similar in structure to pentode-triode pentagrid converters 6SA7 and 6BE6, except that a separate oscillator – usually type 6C5 – is required. Also, grid 1 is remote-cutoff control grid, grid 3 is oscillator input grid. (In types 6SA7 and 6BE6, grid 1 is the internal oscillator grid, grid 3 is the control grid.) Because of low conversion transconductance, radios using type 6L7 typically have either a tuned RF pre-amplifier stage, or at least two stages of I.F. amplification. (A few models have both.)

- 6LF6 – Beam power tetrode with a duodecar Compactron base and anode cap, for CRT horizontal-deflection amplifiers

- 6M5 – Audio Output Pentode (Used as Class-A or C output stages of 1950s Australian radiograms) similar to 6BQ5

- 6M11 – Compactron Dual triode and pentode

- 6MD8 – Medium-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 9-pin base

- 6ME5 – "Magic Eye"-type tuning indicator, miniature 7-pin base B7G

- 6MK8 – Dual-anode pentode for TV receiver sync separation service or as a two-channel VCA

- 6MJ8 – Medium-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 6MN8 – High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 6N3, EY82 – Half-Wave Rectifier

- 6N5/6N5P – Tuning indicator

- 6N7 – Dual Power Triode, used as Class-A audio driver or as Class-B power output (also 6N7-G and 6N7-GT). Max. output (Class-B) – 10 watts. Octal version of type 6A6.

- 6N8, EBF80 – Remote-cutoff pentode, dual diode. (detector plus RF or AF amplifier in radios)

- 6P5-G/GT – Medium-mu triode, Octal version of type 76, often used as driver for type 6AC5-G.

- 6P7-G – Rarely seen octal version of type 6F7.

- 6Q5-G – Triode gas thyratron used in DuMont oscilloscopes as a sweep generator. Identical to type 884.

- 6Q11 – Medium-mu triple triode, 12-pin base, for use as a sync clipper and gated AGC amplifier in TV receivers

- 6R3, EY81 – TV "Damper/Efficiency" Diode

- 6R7 – Dual Diode, Medium-mu Triode (also 6R7-G and 6R7-GT). Octal base with top cap. Miniature version – 6BF6. Amplification factor: 16.

- 6S7-G – Remote-cutoff RF Pentode, similar to type 6K7. Octal base with top cap. Has 150 ms heater. Used in pre-war 6-volt farm radios.

- 6S8-GT – Triple Diode, High-mu Triode. Octal tube with top-cap connection to triode grid. Has three identical diodes – two diodes share a cathode with the triode, one has a separate cathode. Used as a combined AM detector/AVC rectifier/FM ratio detector/A.F. amplifier in AM/FM radios. Typically, all sections of this tube are arranged around a single heater.

- 6SA7 – First pentode-triode style pentagrid converter. Octal type. Miniature version: 6BE6.

- 6SB7Y (octal), 6BA7 and 12BA7 (9-pin miniatures) – VHF pentagrids, 1946

- 6SC7 – High-mu dual triode (Both sections share a single cathode)

- 6SK7 – Remote-cutoff pentode (Used in I.F. stages of North American radios) Miniature version: 6BD6

- 6SL7, ECC35 – Dual triode (Used in TV and general electronics)

- 6SN7, ECC32, B65, 13D2, CV1986, 6042? – Medium-mu dual triode (Used in Audio Amplifiers, Hammond Organs and Television; extensive use in World War II radar) Each section is equivalent to a 6J5. Miniature version: 12AU7

- 6SS7 – Remote-cutoff pentode (150 mA filament version of the 6SK7, found in some AA6 radios as both the RF amplifier and first IF). This is the only tube to have two of the same letters in its type.

- 6T5 – "Magic Eye" Tuning indicator. Has incorporated driver triode with remote-cutoff grid. Has UX6 base. Shadow indicator is fully closed at zero signal. As signal increases, shadow grows outward from the center, covering the entire circumference of the indicator. Electronically identical to types 6G5 and 6U5, which may be used as substitutes.

- 6T7-G – Dual diode, high-mu triode, similar to type 6Q7. Octal base with top cap. Has 150 mA heater. Used in pre-war farm radios.

- 6T8 – Triple Diode, High-mu Triode. Has three identical diodes – two have cathodes connected to the triode's cathode, one has a separate cathode. Triode amplification factor: 70. Used as an AM detector/AVC rectifier/FM ratio detector/A.F. amplifier in North American AM/FM radios. Identical to type 6AK8/EABC80, but with a shorter glass envelope.

- 6U5 – "Magic Eye" Tuning indicator. Has incorporated driver triode with remote-cutoff grid. Has UX6 base. Has "pie wedge" shadow indicator that is open at zero signal and closes as signal increases. Electronically identical to types 6G5 and 6T5 and may be used as a substitute for those types. After World War II, most new 6U5s were double-branded as either 6G5/6U5 or 6U5/6G5.

- 6U5G – "Magic Eye" Tuning indicator with triode, International Octal, (IO), base

- 6U7-G – Remote-cutoff R.F. Pentode. Most common commercial uses were as an I.F. amplifier or as a superheterodyne mixer, aka 1st detector. Also used in test equipment. Octal version of type 6D6. Most direct substitute: 6K7. Similar to types 58, 78 and 6SK7.

- 6U8A – Split triode-pentode, mini-noval pinbase. Audio preamplifier.

- 6U10 – 1x High-mu + 2x medium-mu triple triode, 12-pin base

- 6V4 (EZ80) - Noval-base, indirectly-heated, full-wave rectifier. EZ80 rated at 90mA, but 6V4 only rated for 70. Some brands were identical.

- 6V6 – Beam power tetrode, used in single-ended Class-A audio output stages of radios and sometimes seen in Class-B audio amplifiers (see also: 5V6 and 12V6). Electrically similar to 6AQ5/EL90.

- 6V6G – 6V6 with Shouldered Tubular, (ST), shaped bulb.

- 6V6GT – 6V6 with Tubular, (T), shaped bulb.

- 6V7-G – Dual diode, Medium-mu Triode. Octal version of type 85. Amplification factor: 8.3. Similar to type 6R7.

- 6W6-GT – Beam power pentode, used most often as a Vertical Deflection Output tube in monochrome TV receivers of the 1950s. Can also used as an Audio Output tube. This is the 6.3 volt heater version of types 25L6-GT and 50L6-GT.

- 6X4 (EZ90) and 6X5 (EZ35) – Full-wave rectifiers with indirectly heated common cathode. Type 6X4 has a 7-pin miniature base, the 6X5 has an octal base. Based on type 84/6Z4. No longer in production.

- "7" prefix loctal tubes

These tubes all have 6.3 volt AC/DC heaters.

- 7A4 – Medium-mu triode, loctal version of type 6J5, often numbered 7A4/XXL

- 7A5 – Beam power pentode, loctal version of type 6U6GT

- 7A6 – Dual detector diode, similar to type 6H6

- 7A7 – Remote-cutoff pentode, loctal version of type 6SK7

- 7A8 – The only octode pentagrid converter produced in America by Sylvania, 1939. Used mostly in Philco radios.

- 7AB7 – Sharp-cutoff pentode

- 7AD7 – Power pentode

- 7AF7 – Dual medium-mu triode

- 7AG7 – Sharp-cutoff pentode

- 7AH7 – Remote-cutoff pentode

- 7AJ7 – Sharp-cutoff pentode

- 7AK7 – Sharp-cutoff, dual control pentode for computer service. Perhaps the first active device specifically designed for computer use.

- 7B4 – High-mu triode, loctal version of types 6F5 and 6SF5

- 7B5 – Power pentode, loctal version of types 6K6 and 41

- 7B6 – High-mu triode, dual detector diodes, loctal version of type 75, similar to types 6AV6 and 6SQ7

- 7B7 – Remote-cutoff pentode

- 7B8 – Pentagrid converter, loctal version of types 6A7 and 6A8

- 7C4 – High frequency diode

- 7C5 – Beam power pentode, loctal version of type 6V6

- 7C6 – High-mu triode, dual detector diode

- 7C7 – Sharp-cutoff pentode

- 7E5 – Medium-mu high-frequency triode

- 7E6 – Medium-mu triode, dual detector diode, loctal version of types 6R7 and 6SR7, electronically identical to miniature type 6BF6.

- 7E7 – Semiremote-cutoff pentode, dual detector diode, similar to types 6B7 and 6B8

- 7F7 – High-mu dual triode, loctal version of type 6SL7-GT

- 7F8 – Medium-mu high-frequency triode, used as FM RF amplifier and converter

- 7G7 – Sharp-cutoff pentode

- 7G8 – Sharp-cutoff dual tetrode

- 7H7 – Semiremote-cutoff pentode

- 7J7 – Triode-heptode converter, similar to type 6J8-G

- 7K7 – High-mu triode, dual detector diode, similar to types 6AT6 and 6Q7

- 7L7 – Sharp-cutoff pentode

- 7N7 – Dual medium-mu triode, loctal version of type 6SN7-GT

- 7Q7 – Pentagrid converter, similar to type 6SA7

- 7R7 – Remote-cutoff pentode, dual detector diode

- 7S7 – Triode-heptode converter

- 7T7 – Sharp-cutoff pentode

- 7V7 – Sharp-cutoff pentode

- 7W7 – Sharp-cutoff pentode

- Note: Types 7V7 and 7W7 are electronically identical except for base connections of pins 4, 5 and 7. On type 7V7, the suppressor grid (grid 3) is connected to pin 4, an internal shield is connected to pin 5, and the cathode is connected to pin 7. On type 7W7, the suppressor grid and internal shield are connected to pin 5, and the cathode is connected to pins 4 and 7. All other pin connections are the same. If interchanging these tube types is necessary, confirm that pins 4 and 7 are connected at the socket. (Pin 5 is usually connected to the chassis.)

- 7X6 – Dual rectifier diode

- 7X7 – High-mu triode, dual detector diodes on separate cathodes, used as FM discriminator and AF amplifier, often numbered 7X7/XXFM

- 7Y4 – Dual rectifier diode

- 7Z4 – Dual rectifier diode

- 12 volt heater/filament tubes

- 12A5 – Power pentode. UX7 base. Single-section tube with two side-by-side 6.3 volt heater-cathode structures. Each is connected together to form (1) a 12.6 volt 300 mA heater for series heater strings, or (2) a 6.3 volt 600 mA heater for parallel heater circuits. Mostly used in pre-war car radios.

- 12A7 – Power pentode, rectifier diode. Pentode section is similar to type 38. Diode has a low power rating – 120 volt, 30 mA – that limits the number of tubes that can be tied to its B+ circuit. Used in one-tube portable phonographs and a few two- and three-tube radios. Forerunner of such types as 32L7-GT, 70L7-GT and 117L7-GT. UX7 base with top cap. Not related to types 2A7 and 6A7.

- 12AB5 – Beam Power Tube[35]

- 12AC10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 12AE10 – Compactron Beam power tube and sharp-cutoff pentode[35]

- 12AL5 – Dual diode (similar to 6AL5 except for heater)

- 12AT6 – Dual diode/triode (Commonly replaced by 12AV6 in consumer radios)

- 12AT7, ECC81, 6060, B309, M8162 – High-mu dual triode. Commonly used as R.F. amplifier/mixer in VHF circuits.[35]

- 12AU7, ECC82, 6067, B329, M8136 – Medium-mu dual triode.[35] Two 6C4/EC90s in one envelope;[36] however, it is only specified as an audio frequency device. Commonly used in audio applications and TV receivers.

- 12AV6 – Dual diode/High-mu triode (see also: 6AV6)

- 12AV7, 5965 – Medium-mu dual triode. Principally designed for VHF amplifier/mixer operation.[37]

- 12AX7, ECC83, 6057, B327, M8137 – High-mu dual triode. Very similar to triode section of 6AV6. Commonly used in high-gain audio stages.[35]

- 12AY7 – Dual Triode. Medium gain but low noise, intended for low-level/preamplifier use.[35]

- 12AZ7 – Double Triode. Medium-mu, AF Amplifier, or combined oscillator and mixer, 9-pin.[38][35]

- 12B4A – Low-mu triode, noval base.[35]

- 12BA6 – Remote-cutoff pentode, 6BA6/EF93 with a different heater

- 12BE6 – Pentagrid converter, 6BE6/EK90 with a different heater

- 12BH7 – Dual Triode, Medium-mu, designed for use in equipment having series heater-string arrangement.[39][35]

- 12BY7 – Video Amplifier Pentode

- 12DT6 – Sharp-cutoff pentode

- 12DW7 – Dissimilar triodes. One half 12AX7 value, other half 12AU7 value. (also known as 7247 or ECC832)

- 12EG6 – Pentagrid converter, both grids 1 and 3 are sharp-cutoff, has 12.6 volt anode and screen grid voltage, for use with audio equipment powered by a car battery

- 12FA6 – Low-anode voltage, car radio version of 12BE6 pentagrid converter

- 12FQ8 – Common-cathode, dual split-anode triode for use in musical instruments, frequency dividers and complex wave generators[40]

- 12FX8 – Low-anode voltage, triode-heptode converter for car radios

- 12GA6 – Similar to type 12FA6, but with lower conversion transconductance

- 12J5WGT – Heater cathode type, medium-mu triode, identical to 6J5WGT except heater characteristics

- 12K5 – Tetrode with a space charge grid, one of a few tubes that can function with low anode voltages

- 12MD8 – Medium-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 9-pin base

- 12MN8 – High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 12SA7 – Pentagrid converter (Octal version of 12BE6)

- 12SK7 – Remote-cutoff Pentode (Octal version of 12BA6)

- 12SQ7 – Dual diode, triode (Octal version of 12AV6)

- 12U5G – Tuning indicator identical to 6U5G except heater characteristics

- 12Z3 – Half-wave rectifier, UX4 base

- "14" prefix loctal tubes

These tubes all have 12.6 volt AC/DC heaters

- 14A4 – Medium-mu triode, loctal version of type 12J5

- 14A5 – Beam power pentode

- 14A7 – Remote-cutoff pentode, often numbered 14A7/12B7

- 14AF7 – Dual medium-mu triodes, often numbered 14AF7/XXD

- 14B6 – High-mu triode, dual detector diode, similar to types 12AV6 and 12SQ7

- 14B8 – Pentagrid converter, loctal version of type 12A8

- 14C5 – Beam power pentode, loctal version of type 12V6-GT

- 14C7 – Sharp-cutoff pentode

- 14E6 – Medium-mu triode, dual detector diode, loctal version of 12SR7

- 14E7 – Semiremote-cutoff pentode, dual detector diode, similar to type 12C8

- 14F7 – High-mu dual triode, loctal version of type 12SL7-GT

- 14F8 – Medium-mu high frequency triode, used as FM RF amplifier and converter

- 14H7 – Semiremote-cutoff pentode

- 14J7 – Triode-heptode converter

- 14N7 – Dual medium-mu triode, loctal version of type 12SN7-GT

- 14Q7 – Pentagrid converter, similar to type 12SA7

- 14R7 – Remote-cutoff pentode, dual detector diode

- 14S7 – Triode-heptode converter

- 14W7 – Sharp-cutoff pentode

- 14X7 – High-mu triode, dual detector diodes on separate cathodes, used as FM discriminator and AF amplifier

- 14Y4 – Dual rectifier diode

- 25 volt heater/filament tubes

- 25A6 – Power pentode, octal version of type 43

- 25C5 – Beam Power Pentode (Identical to the 50C5 but with a 25 V 300 mA filament)

- 25F5 – Beam Power Pentode (Identical to the 50C5, but with a 25 V 150 mA filament, used in some AA5 type radios using push-pull output)

- 25L6 – Beam-power pentode (Except for heater, electrically identical to type 50L6)

- 25Z5 – Dual rectifier diode

- 25Z6 – Octal version of 25Z5

- 35 volt heater/filament tubes

- 35A5 – Beam Power Tube (Loctal, Similar to 35L6)

- 35B5 – Beam power tube

- 35C5 – Identical to 35B5 except for basing ("pin-out") arrangement (HL92)

- 35HB8 – Triode/Beam Power tube (Used primarily as both the audio amplifier and output)

- 35DZ8 – High-mu Triode/Beam Power tube (Like the 35HB8, used for audio)

- 35L6-GT – Beam power pentode similar to, but not electronically identical to, types 25L6-GT and 50L6-GT

- 35W4 – Rectifier diode

- 35Z4-GT – Rectifier diode

- 35Z5-GT – Similar to 35Z4-GT, but equipped with a heater tap used to power a pilot light

- 35Z3 – Rectifier Diode (Loctal, Similar to 35Z4)

- 35Y4 – Rectifier Diode (Loctal, similar to 35Z5)

- 50 volt heater/filament tubes

- 50B5 – Beam power tube, similar to 35B5 but with 50 volt filament

- 50C5 – Similar to 35C5 but with 50 volt filament, and 50B5 except for basing ("pin-out") arrangement (HL92)

- 50EH5 – Beam Power tube, (Similar to 50C5 but with higher gain, some radios that use this tube do not have an audio amplifier section.)

- 50L6 – Beam power tube (see also 25L6)

- 50HK6 – Power pentode (Filament is tapped for use with a dial lamp)

- 50A5 – Beam Power Tube (Loctal, similar to 50L6)

- 50X6 – Dual Diode (Loctal, commonly used as a rectifier-doubler)

- 50DC4 – Rectifier diode (Similar to 35W4 except for filament)

- 117 volt heater tubes

All of the following tubes are designed to operate with their heaters connected directly to the 117 volt (now 120 volt) electrical mains of North America. All of them use indirectly heated cathodes. All of them incorporate at least one rectifier diode.

- Rectifier diode – Beam power pentode combinations

- 117L7GT

- 117M7GT

- 117N7GT

- 117P7GT

- Rectifier tubes

- 117Z3 – Single diode, 7-pin miniature version of 117Z4GT

- 117Z4GT

- 117Z6GT – Dual diode, can be used as a voltage doubler

- Other tubes with nonstandard filament voltages

The tubes in this list are most commonly used in series-wired circuits.

- 4CB6 – Sharp-cutoff pentode

- 7AU7 – Medium-mu Dual triode with a tapped filament like its more common brother, the 12AU7. 7.0/3.5 V

- 7KY6 – Sharp-cutoff frame-grid pentode with a 7.3 volt nominal heater voltage for use as video output tube in TV receivers, 9-pin miniature socket[41]

- 8AC10 – Compactron High-mu triple triode for use as NTSC chroma signal demodulator matrix in analog color TV receivers, 12-pin base

- 8B10 – Compactron Dual triode and dual diode

- 8FQ7/8CG7 – Dual triode (8 V version of the common 6CG7)

- 10DE7 – Dual triode (dissimilar triode sections)

- 11DS5 – Beam Power tube (11 V filament version of the 50B5/35B5)

- 13CW4 – Nuvistor used as a preamplifier in Neumann condenser microphones U-47 and U-48 after the production of the VF14 ceased

- 18FX6 – Pentagrid converter (18 V version of the 12BE6)

- 18FY6 – Dual diode/triode (18 V version of the 12AV6)

- 34GD5 – Beam power tube (34 V version of the 35C5/50C5)

- 36AM3 – Half-wave rectifier (36 V version of the 35W4)

- 38HE7 – Compactron Diode and beam power tube

- 38HK7 – Compactron Diode and beam power tube

- List of EIA professional tubes

Note: Most of these are special quality versions of the equivalents given. Some manufacturers preceded the EIA number with a manufacturer's code, as explained above.

- 4000s

- 4042 – Ceramic/metal pencil-type disk-seal UHF power triode for pulsed operation up to 425 W

- 4062A – Ceramic/metal pencil-type disk-seal SHF power triode up to 4 GHz, mu = 100, Panode = 10 W

- 4065 – Directly heated electrometer triode, grid current ≤125 fA, 4-pin all-glass pigtail, for probe amplifiers

- 4205 – Directly heated power triode, 4-pin bayonet base with offset pin

- 4270A (3C/350E) – Directly heated power triode, 4-pin base

- 4275 – Directly heated power triode, 4-pin base

- 4300 – Directly heated power triode, 4-pin base

- 4307A – Power pentode similar to the output beam-tetrode type 807. It differs from an 807 by being a directly heated pentode rather than an indirectly heated beam-tetrode. Both types are contained in an ST-16 bulb with an anode cap and 5-pin "American" UY base

The SY4307A made by Standard Telephones and Cables/Brimar is historically notable because a pair of them in parallel Class-C was used as the output stage in a transmitter built in secret by Australian soldiers in Japanese-occupied Portuguese Timor during World War II in 1942. This transmitter, now reconstructed and on display at the Australian War Memorial in Canberra, was called "Winnie the War Winner".[42]

- 4307AF – 4307A qualified for use in standard aircraft radio

- 4560 – Character generator monoscope for text mode video rendering in early computer monitors, with a square target having letters, digits and symbols stenciled into it in a customer-supplied 8x8 array. An electron beam selects and scans a character, both by appropriate electrostatic deflection, and generates an analog video signal;[43] cf. CK1414, TH9503

- 4598, 7539, 7828, 8087, 8098 – Graphechon dual-electron gun scan conversion tubes, analog video transcoders with simultaneous R/W capability for realtime resolution and frame rate transcoding between different analog video standards. This was achieved by a CRT/camera tube combination; the CRT part writes onto a thin, dielectric target; the camera part reads the generated charge pattern at a different scan rate from the back side of this target.[44][45][46][47][48] The setup could also be used as a genlock

- 4613 – Directly heated power triode, 4-pin base

- 4614 – Indirectly heated power triode, 5-pin base

- 4641 – Directly heated power triode, 4-pin base

- 4671/E1C (955) – Indirectly heated Acorn triode

- 4672/E1F (954) – Indirectly heated Acorn pentode

- 4674 – Indirectly heated Acorn diode

- 4675 – 4671/E1C with a 4 Volts heater

- 4676 – 4672/E1F with a 4 Volts heater

- 4678 (EM1) – Indirectly heated tuning indicator

- 4683 – Directly heated power triode, 8-pin base

- 4695/E2F (956) – Indirectly heated Acorn pentode

- 5000s

5651

- 5331, 5332, 5514 – Directly heated power triodes, 4-pin base with anode top cap

- 5556 – Directly heated power triode, 4-pin base

- 5593 – Phasitron, a magnetically controlled beam-deflection phase modulator tube[49] similar to the 2H21, used in early FM broadcast transmitters[3][4][5]

- 5608 – Double power triode, designed for use with AC anode voltage and critical grid leak requirements

- 5651 – 86-volts, cold-cathode, glow-discharge voltage reference, 7-pin miniature base

- 5678 (CK5678 Raytheon) – 5 leads subminiature shielded pentode for RF applications

- 5691 – Special Red ruggedized long-life high-mu triode for industrial applications

- 5692 – Special Red ruggedized long-life medium-mu triode for industrial applications

- 5693 – Special Red ruggedized long-life sharp-cutoff pentode for industrial applications

- 5654, 6AK5, EF95, CV4010, 62H1P, 408A – VHF pentode; common in vintage radar IF amplifiers.

- 5704 – Subminiature diode, all-glass pigtailed

- 5727 – 650 V, 100 mAavg, 500 mApeak, 10 Asurge Indirectly heated tetrode thyratron, positive starter voltage, miniature 7-pin base

- 5729 – Beam-deflection, 30-channel analog multiplexer for telecomms transmitting channel banks, internal electrostatic focusing and deflection to determine through which one out of 30 grids the electron beam passes to the common anode.[50] Cf. 5738, 6090, 6091, 6170, 6324

- 5731 – Narrow-tolerance selected 955 Acorn triode for use in Radiosonde weather balloon transmitters

- 5734 – Mechano-electronic displacement sensor; a vacuum triode with its anode mounted on a shaft that extends through a thin, flexible metal diaphragm; shaft movement is reflected in anode current; Fres = 12 kHz[51][52]

- 5738 – Beam-deflection, secondary emission, 25-channel analog multiplexer, internal electrostatic focusing and deflection to determine which one out of 25 individually controllable dynodes receives the electron beam controlled by a common grid.[53] Cf. 5729, 6090, 6091, 6170, 6324

- 5749, 6BA6, EF93, W727 – RF pentode

- 5750, 6BE6, EK90, X727 – Heptode mixer

- 5751 – Low-voltage, low-noise avionics dual triode with separate cathodes

- 5814A – Industrial, computer-rated version of 12AU7/ECC82

- 5836, 6BL6

- Raytheon RK5836

– Sutton tube, a reflex klystron used as a 250 mW CW microwave source, 1.6 to 6.5 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

- 5837, 6BM6 – Sutton tube used as a 150 mW CW microwave source, 550 MHz to 3.8 GHz depending upon an external cavity. 4-pin peewee base with cavity contact rings and top cap

- 5845 – Dual directly heated saturated-emission diode. Acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits.

- 5876A – Glass pencil-type disk-seal UHF power triode up to 2 GHz

- 5930 – Ruggedized, directly heated power triode, 4-pin base

- 5962 – 700 V/2...55 µA Corona voltage reference, miniature 7-pin base with anode top cap

- 5963, 5964, 5965 – Dual triode, designed for high speed digital computers, has a high zero-bias anode current, identical to 12AV7

- 5998, 6336A, 6394, 6520, 6528, 7802 – Double power triodes, designed for series voltage regulator applications

- 6000s

- 6047 – Additron, a triple-control grid, split-anode tetrode for use as a single-bit digital full adder[54] (technically a hexode)

- 6057, 12AX7, ECC83, M8137, B339 – High-mu double triode

- 6059, 6BR7 – Low-microphonics pentode

- 6060, 12AT7, ECC81, M8162, B309 – High-mu double triode

- 6064, 6AM6, EF91, M8083, Z77 – R.F. pentode

- 6067, 12AU7, ECC82, M8136, B329 – Medium-mu double triode

- 6080, 6AS7 – Very-low impedance double power triode, designed for series voltage regulator applications, now popular for output transformerless audio amplifiers

- 6082 – Ruggedized, indirectly heated power triode, octal base

- 6090 – Beam-deflection, 18-channel analog demultiplexer for telecomms receiving channel banks, internal electrostatic focusing and deflection to determine which one out of 18 anodes receives the electron beam controlled by a common grid.[55] Cf. 5729, 5738, 6091, 6170, 6324

- 6091 – Beam-deflection, 25-channel analog multiplexer for telecomms transmitting channel banks, internal electrostatic focusing and deflection to determine through which one out of 25 grids the electron beam passes to the common anode.[56] Cf. 5729, 5738, 6090, 6170, 6324

- 6146 – 60 MHz, 120 W Power AF/RF/VHF beam pentode

- 6146B (8298A) – Improved version of 6146, 6146A and 8298.

- 6170 and 6324 – Beam-deflection, 25-channel analog multiplexer for telecomms transmitting channel banks, external focusing and deflection by a multiphase, rotating magnetic field to determine through which one out of 25 grids the electron beam passes to the common anode.[57] Cf. 5729, 5738, 6090, 6091

- 6173 – Pencil-type disk-seal UHF diode up to 3.3 GHz

- 6196 – Directly-heated dual, compensating electrometer tetrode with space charge grids for use in the 2 branches of a differential-in, differential-out bridge circuit[58]

- 6218/E80T (CV5724) – Modulated, single-anode beam deflection tube for pulse generation up to 375 MHz; shock resistant up to 500 g[59][60]

- 6351 – Secondary emission pentode for wide band RF amplifiers

- 6353 – 19.3 kV/25...1000 µA Corona voltage reference, miniature 7-pin base with anode top cap

- 6263 – Pencil-type disk-seal UHF power triode up to 500 MHz, Panode = 8 W

- 6361 – Convectron, an inclinometer tube that senses tilt from the vertical by means of different gas convections around a heating wire in a glass envelope, of two 6361s aligned in a 90° V-shaped position to each other and the heating wires connected in a bridge circuit

- 6391 – Subminiature low-microphonics pentode, 8-pin all-glass pigtailed

- 6441 – 650 V, 100 mAavg, 300 mAsurge Tacitron, a grid turn-off hydrogen thyratron with a grid that forms a shield around both the cathode and anode and separates the two by a wire mesh, so the arc discharge can be extinguished by a negative grid that surrounds the positive anode with a field of opposing polarity and inhibits conduction, taking over part of the anode current during deionisation – similar to today's GTOs; Octal base[61]

- 6462 – Magnetic pickup tube, a 1-axis beam-deflection magnetometer with approx. 1 G (100 μT) resolution; an electron beam is electrostatically centered between two anodes while no magnetic field is present; the magnetic field to be detected will then deflect the beam more towards one of the anodes, resulting in an imbalance between the two anode currents[62]

- 6550 – 20 W AF beam tetrode for high fidelity amplifiers

- 6571 – Williams-type computer memory tube[63]

- 6577 – Typotron, a charactron for text mode video rendering in early computer monitors[64]

- 6700 – 200 ns Decade counter Magnetron Beam Switching Tube, 6.3 V, 300 mA heater[65]

- 6701 – Low-voltage 500 ns decade counter Magnetron Beam Switching Tube, 6.3 V, 300 mA heater[66]

- 6703 – 500 ns Decade counter Magnetron Beam Switching Tube, 6.3 V, 300 mA heater

- 6704 – 100 ns Decade counter Magnetron Beam Switching Tube with internal spade load resistors, 6.3 V, 300 mA heater

- 6710, 6711, 6712 (High current), 6714 (Low voltage) – 2 MHz Decade counter Beam-X Switch, 6.3 V heater

- 6762 – Wamoscope, a TWT/CRT combination used to directly visualize an incoming microwave signal by electron velocity-sorting[67]

- 6835, 7570, 7571 – Single-electron gun recording storage tube, an analog video frame freezer tube. This was achieved by a CRT that writes the video image onto a thin, dielectric target and subsequently can read the generated charge pattern up to 30000 times from that target, producing a video signal containing a static shot that resembles a still photograph[68][69][70]

- 6846 – Gas-filled, three-cathode 1-bit binary counter or switching tube, 7-pin base[71]

- 6877, 7233 – Power triodes, designed for series voltage regulator applications

- 6900 – Dual power triode for pulse applications in missiles, avionics and industrial systems; noval base

- 6922 (E88CC, industrial version of 6DJ8/ECC88)

- 6973 – Power pentode similar in shape, size, and pinbase to the EL84/6BQ5, but with a high gain for more than double the output range. Popular in some makes of 1960s era guitar amplifiers, though rarely implemented in modern times.

- 7000s

- 7077 – Miniature all-ceramic disk-seal UHF triode

- 7105 – 12.6-volts version of 6080

- 7189/6BQ5/EL84 – AF Power pentode

- 7189A – Improved 7189

- 7199 – Split triode-pentode, noval pinbase. Similar to 6U8.

- 7229,[72] 7230,[73] 7231,[74] 7232,[75] 7439, 7440, 7441, 7595, 7596, 7597, 7598, 7599, 7600, 7602 – Krytrons, cold-cathode gas-filled trigger tubes with a primer electrode for use as a very high-speed, high-surge current switch[76] – second source to EG&G

- 7236 – Double power triode for use as long-life power amplifier in computer applications

- 7241, 7242 – Triple-grid/cathode power triodes, designed for hi-rel series voltage regulator applications

- 7266 – Miniature all-ceramic disk-seal UHF diode

- 7289 – 3 GHz, 40 W UHF planar power triode

- 7308/E188CC – Premium version of 6922

- 7360 – Beam deflection tube, used as balanced modulator/mixer up to 100 MHz[77][78][79]

- 7414 – Time Totalizer, a metal-vapor coulometer, a cold-cathode gas-discharge tube where metal is constantly sputtered off the cathode and deposited on a collector element whose resistance therefore decreases with elapsed time[80]

- 7430 – Flat-envelope version of the 6AK5/EF95 sharp-cutoff pentode for use on PCBs in Radiosonde weather balloon transmitters

- 7548 – Secondary emission hexode for pulse generator and pulse amplifier applications

- 7551 - Noval-base beam power pentode with 12-15 volt filament. 6.3 volt filament version was 7558. Used in telephony, RF amplification, and more rarely AF amplification.

- 7554 – Ceramic/metal pencil-type disk-seal SHF power triode up to 5 GHz

- 7572, 7575, 7702 – Dual-electron gun recording storage tube, a realtime analog video frame freezer tube with simultaneous R/W, and storing capability. This was achieved by a CRT/camera tube combination; the CRT part writes the video signal onto a thin, dielectric target, which can hold the generated charge pattern for many hours; the camera part reads the charge pattern from the back side of this target, producing a video signal containing a static shot that resembles a still photograph[81][82][83]

- 7586 – First Nuvistor available on the market, medium-mu triode

- 7587 – Nuvistor Sharp cutoff tetrode

- 7591 – Beam power pentode, octal pinbase. Found in many guitar amps made by Gibson and Ampeg.

- 7688, 7690 (Medium-mu), 7689 (high-mu) – triple triodes

- 7699 – Dual tetrode for wide band push-pull amplifiers

- 7762 – Shock-proof avionics AF beam power pentode

- 7763 – Beam deflection tube, used as IF amplifier/limiter where a constant phase shift over a wide range of input signal amplitudes is required[84]

- 7768 – Miniature all-ceramic disk-seal SHF triode up to 4 GHz

- 7868 – Beam power pentode, magnoval pinbase version of 7591. Found in many of the once popular Challenger series PA amps made by Bogen Communications, also found in some guitar amplifiers made by Ampeg.

- 7895 – Improved 7586 Nuvistor with higher mu

- 8000s

- 8011 – Micropup-type UHF power triode up to 600 MHz

- 8056 – Nuvistor triode for low supply voltage

- 8058 – Nuvistor triode with grid on envelope and an anode cap, for grounded-grid UHF circuits

- 8069 – 8 kV/23...1000 µA Corona voltage reference, cathode cylinder and anode top cap

- 8089 – 1.6 kV/20...800 µA Corona voltage reference, 2-pin all-glass pigtailed

- 8090 – 3.5 kV/50...1000 µA Corona voltage reference, 9-pin with anode top cap

- 8091 – 4 kV/50...1000 µA Corona voltage reference, 9-pin with anode top cap

- 8122 – Forced-air cooled, 300 W@470 MHz beam power tetrode

- 8256 – 3.5 kV/35...1900 µA Corona voltage reference, 2-pin all-glass pigtailed

- 8257 – 1.2 kV/15...750 µA Corona voltage reference, 2-pin all-glass pigtailed

- 8393 – Nuvistor Medium-mu triode, used in Tektronix oscilloscopes, 12.6 Volt heater

- 8469 – 400 V/5...400 µA Corona voltage reference, 2-pin all-glass pigtailed

- 8506 – Miniature all-ceramic disk-seal UHF triode

- 8514 – 1 kV/10...800 µA Corona voltage reference, 7-pin with anode top cap

- 8515 – 1.6 kV/20...950 µA Corona voltage reference, 7-pin with anode top cap

- 8526 – Nuvistor-type medium-mu dual triode

- 8873 – 500 MHz, 200 W anode dissipation power triode

- 8874 – 500 MHz, 400 W anode dissipation power triode

- 8875 – 500 MHz, 300 W anode dissipation power triode

- 8877 = 3CX1500A7 – Ceramic, forced air cooled, 1.5 kW power triode

- 8974 (X-2159) – Giant water-cooled megawatt-class tetrode used for very high-power broadcast and industrial service; possibly the most powerful tube ever commercially produced

- List of European Mullard–Philips tubes, with American equivalents

Main article: List of Mullard–Philips vacuum tubes

- List of Pro Electron professional tubes

Note: Typecode explained above.

- X - Electro-optical devices

- XA

- XA1003 – Phototube, caesium-on-oxydated-silver cathode, 2-pin all-glass pigtailed

- XG

- XG2000 – Image converter for x-ray diagnostics

- XL

- XL7900 – Vibrating-capacitor chopper front end for dosimeters, electrometers, pH meters etc., Magnoval base with gold-plated pins[85]

- XM

- XM1000 – Nimo tube, directly heated cathode-ray 1-digit numeric display tube, decimal points on both sides, hence 12 stenciled electron guns, top-viewing, green, 15 mm high Futura Medium font, oval envelope for easy horizontal stacking, 14-pin base

- XP

- XP1000 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal (14-pin) base

- XP1001 – 10-stage photomultiplier for gamma ray scintillation spectrometry, Sb-Cs cathode, Ag-Mg-O-Cs dynodes

- XP1002 – 10-stage photomultiplier, blue/green/yellow/orange-sensitive Sb-Na-K-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

- XP1003 – 10-stage photomultiplier with quartz window, UV/blue/green/yellow/orange-sensitive Sb-Na-K-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

- XP1004 – 10-stage photomultiplier with quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

- XP1005 – 10-stage Ag-O-Cs (800±100 nm) photomultiplier, IR/red-sensitive Ag-O-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

- XP1010 – 10-stage photomultiplier for r-ray and gamma ray scintillation spectrometry, selected 150AVP for low noise and resolution, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, duodecal (12-pin) base

- XP1011 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, shock and vibration-proof, duodecal base

- XP1020 – 12-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, 100 O output, duodecal (20-pin) base

- XP1021 – 12-stage photomultiplier, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, 50 O output, duodecal base

- XP1023 – 12-stage photomultiplier with quartz window Sb-Cs cathode, Ag-Mg-O-Cs dynodes, UV/blue-sensitive, 50 O output, duodecal base

- XP1030 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal (14-pin) base

- XP1031 – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, for gamma ray scintillation spectrometry

- XP1032 – 10-stage photomultiplier with 3 mm quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

- XP1033 – 10-stage photomultiplier with 10 mm quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, diheptal base

- XP1040 – 14-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, concave window, duodecal base

- XP1110 – Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

- XP1111 – Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, pigtails

- XP1113 – 6-stage Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

- XP1114 – 4-stage Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

- XP1115 – Photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, pigtails, shock and vibration-proof

- XP1116 – Photomultiplier, red-sensitive Ag-O-Cs cathode, Ag-Mg-O-Cs dynodes, shock and vibration-proof

- XP1117 – 9-stage photomultiplier, blue/green/yellow/orange-sensitive Sb-Na-K-Cs cathode, Ag-Mg-O-Cs dynodes

- XP1118 – Photomultiplier with quartz window, UV/blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes

- XP1120 – 17-stage photomultiplier for x-ray (λ > 200 pm) or UV (λ < 150 nm) photon counting in a high-vacuum environment, Nickel cathode, Cu-Be-O dynodes, coaxial outputs, built-in resistor ladder

- XP1121 – 17-stage photomultiplier for ion (> 10 keV) or electron (0.1...10 keV) photon counting in a high-vacuum environment, Cu-Be-O cathode and dynodes, coaxial outputs, built-in resistor ladder

- XP1122 – 17-stage photomultiplier for x-ray (λ > 200 pm) or UV (λ < 150 nm) photon counting in a high-vacuum environment, Nickel cathode, Cu-Be-O dynodes, coaxial outputs, built-in resistor ladder

- XP1123 – 17-stage photomultiplier for ion (> 10 keV) or electron (0.1...10 keV) photon counting in a high-vacuum environment, Cu-Be-O cathode and dynodes, coaxial outputs, built-in resistor ladder

- XP1130 – 17-stage photomultiplier for x-ray (λ > 200 pm) or UV (λ < 150 nm) photon counting in a high-vacuum environment, Nickel cathode, Cu-Be-O dynodes, coaxial outputs, built-in resistor ladder

- XP1131 – 17-stage photomultiplier for ion (> 10 keV) or electron (0.1...10 keV) photon counting in a high-vacuum environment, Cu-Be-O cathode and dynodes, coaxial outputs, built-in resistor ladder

- XP1140 – 6-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, fast, diheptal base

- XP1141 – 7-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, fast, diheptal base

- XP1180/52AVP – 10-stage photomultiplier, blue-sensitive Sb-Cs cathode, Ag-Mg-O-Cs dynodes, fast, diheptal base

- XP1240 – Photomultiplier

- XQ

- XQ1023 – Camera tube

- XQ1029R – Camera tube

- XQ1032 – 1" Vidicon; magnetic focusing and deflection

- XQ1053 – Camera tube

- XQ1072 – 1" Plumbicon

- XQ1073 – XQ1072 with higher resolution and improved low level contrast

- XQ1200 – Vidicon, silicon target

- XQ1270 – ⅔" Vidicon; Overall length 108mm (4¼")

- XQ1272 – Camera tube

- XQ1274 – ⅔" Newvicon, magnetic focussing and deflection, zinc telluride, zinc selenide target

- XQ1275 – Vidicon, silicon target

- XQ1276 – XQ1274 with high sensitivity into the near infrared

- XQ1277 – XQ1275 with high sensitivity into the near infrared

- XQ1278 – XQ1275 with better geometry and uniform signal

- XQ1285 – 1" Vidicon; magnetic focusing and deflection, precision electron gun

- XQ1290 – Camera tube

- XQ1293 – Camera tube

- XQ1300 – Saticon Camera Tube

- XQ1340 – Low-light vidicon

- XQ1371 – Resistron camera tube

- XQ1380 – XQ1274 with radiation resistant (anti-browning) faceplate

- XQ1381 – ⅔" Newvicon; electrostatioc focusing and magnetic deflection with radiation-resistant (anti-browning) faceplate

- XQ1395 – High-resolution Resistron camera tube

- XQ1410B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1412 – 6/5" Plumbicon; low lag, unity gamma matched to P20 phosphor

- XQ1413B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1415B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1427 – ⅔" Plumbicon; low lag

- XQ1427B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1430B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1435B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1440 – 1" Newvicon, separate mesh, zinc telluride, zinc selenide target

- XQ1500B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1505B/G/R – Plumbicon with color filter for color TV broadcast

- XQ1560 – 1" Saticon

- XQ1565 – 1" Saticon

- XQ1570 – 1" Saticon

- XQ1575 – 1" Saticon

- XQ1585 – 1" Saticon

- XQ1600 – ½" Vidicon; separate mesh, electrostatic focusing and magnetic deflection

- XQ1601 – ½" Newvicon; separate mesh, electrostatic focusing and magnetic deflection

- XQ2070/02B/G/R – Plumbicon with color filter for color TV broadcast

- XQ2070/05B/G/R – Plumbicon with color filter for color TV broadcast

- XQ2075/02B/G/R – Plumbicon with color filter for color TV broadcast

- XQ2075/05B/G/R – Plumbicon with color filter for color TV broadcast

- XQ2172 – 1" Plumbicon; wide dynamic range matched to digital radiography applications

- XQ2182 – 1" Plumbicon; wide dynamic range matched to digital radiography applications

- XQ2427B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3070/02B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3070/05B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3075/02B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3075/05B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3427B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3430B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3435B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3440B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3445B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3457B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3467B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3477B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3487B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3550B/G/R – Plumbicon with color filter for color TV broadcast

- XQ3555B/G/R – Plumbicon with color filter for color TV broadcast

- XQ4187B/G/R – Plumbicon with color filter for color TV broadcast

- XQ4502 – 2" Plumbicon; Highest resolution, low lag

- XQ5002 – 2" Plumbicon; Electrostsatic deflection for improved corner resolution, low output capacitance

- XQ7002 – 1" Plumbicon; Low output capacitance

- XQ8002 – 1" Plumbicon

- XQ9002 – 1" Plumbicon

- XR

- XR1000 – Monoscope, test pattern specified by suffix

- XX

- XX1000 – 2-stage image intensifier

- XX1010 – Image intensifier

- XX1020 – Image intensifier

- XX1030 – Image intensifier

- XX1050 – Image intensifier

- XX1060 – Image intensifier

- XX1066 – 1. Gen. 3-stage image intensifier

- XX1140 – 1. Gen. 3-stage image intensifier

- XX1190 – 1. Gen. inverter, 1-stage image intensifier

- XX1192 – 1. Gen. inverter, 1-stage image intensifier

- XX1200 – 1. Gen. inverter, 1-stage image intensifier

- XX1211 – 1. Gen. inverter, 3-stage image intensifier

- XX1270 – 1. Gen. inverter, 2-stage image intensifier

- XX1400 – 2. Gen. inverter, 1-stage image intensifier

- XX1430 – 1. Gen. inverter, 1-stage image intensifier

- XX1510 – 1. Gen. 3-stage image intensifier

- XX1610 – 2. Gen. image intensifier

- XX1800 – 2. Gen. proximity focused, 1-stage image intensifier

- Y - Vacuum tubes

- YA

- YA1000 – 5 kV, 5mA, Directly heated saturated-emission diode with pure-metal cathode for use in RMS converters of AC voltage/current stabilizer circuits, noval base[86]

- YD

- YD1000 – 120 kW, Water-cooled RF power triode

- YD1001 – 120 kW, Air-cooled RF power triode

- YD1012 – 360 kW, Vapor-cooled RF power triode

- YD1130 – 580 W, Air-cooled, linear RF/AF power triode

- YD1252 (RS 2051 V) – 420 kW, Water-cooled, modulator power triode

- YD1300 – 35 W, Air-cooled, UHF power triode

- YD1301 – 50 W, Air-cooled, UHF power triode

- YD1302 – 55 W, Air-cooled, UHF power triode

- YD1332 – 250 W, Air-cooled, UHF power triode

- YD1333 – 100 W, Air-cooled, UHF power triode

- YD1334 – 110 W, Air-cooled, UHF power triode

- YD1335 – 550 W, Air-cooled, UHF power triode

- YD1336 – 220 W, Air-cooled, UHF power triode

- YD1342 – 30 MHz, 530 kW, Water-cooled RF power triode

- YD1352S (8867, DX334) – Neotron, a field-effect tube, 5 MHz, 3 kW, water-cooled, magnetically beamed RF power pulse generator triode

- YG

- YG1000 – Directly heated electrometer tetrode with an oxide cathode and a space charge grid, grid current ≤600 fA, magnoval base with input grid on top cap

- YH

- YH1000 – Traveling-wave tube

- YH1050 – Traveling-wave tube

- YH1110 – Traveling-wave tube

- YH1120 – Traveling-wave tube, >3 GHz

- YH1131 – Traveling-wave tube, >3 GHz

- YH1150 – Traveling-wave tube

- YH1160 – Traveling-wave tube, >3 GHz

- YH1181 – Traveling-wave tube, >3 GHz

- YH1190 – Traveling-wave tube, >3 GHz

- YH1200 – Traveling-wave tube, >3 GHz

- YJ

- YJ1000 – Indirectly heated, 2.5 kW magnetron for use as a pulsed X-band oscillator between 9.19 and 9.32 GHz

- YJ1462 – Indirectly heated, 28 kW coaxial magnetron for use as a pulsed X-band oscillator at 9.375 GHz

- YK

- YK1000 – Water-cooled, permanent-magnet 11 kW UHF linear-beam Klystron for use in TV transmitters between 400 and 620 MHz

- YK1004 – Water-cooled, permanent-magnet 11 kW UHF linear-beam Klystron for use in TV transmitters between 610 and 790 MHz

- YK1005 – Water-cooled, permanent-magnet 11 kW UHF linear-beam Klystron for use in TV transmitters between 470 and 860 MHz

- YK1046 – 35 mW X-band Klystron, 9.16 to 9.34 GHz

- YK1151 – Forced-air cooled, permanent-magnet 25 kW UHF linear-beam Klystron for use in TV transmitters between 470 and 860 MHz

- YK1190 – Water-cooled 40 kW UHF linear-beam Klystron for use in TV transmitters between 470 and 610 MHz

- YK1191 – Water-cooled 40 kW UHF linear-beam Klystron for use in TV transmitters between 590 and 720 MHz

- YK1192 – Water-cooled 40 kW UHF linear-beam Klystron for use in TV transmitters between 710 and 860 MHz

- YL

- YL1000/8463 – RF power pentode

- YL1020/8118 – See QQZ03/20

- YL1030 – See QQZ06/40

- YL1050 – RF power tetrode

- YL1060/7854 – See QQE06/40

- YL1070/8117 – RF power pentode

- YL1071 – YL1070 with a different heater

- YL1080/8348 – RF power pentode

- YL1120 – RF power tetrode

- YL1130/8408 – RF power pentode

- YL1150/8579 – RF power pentode

- YL1190/8580 – RF power pentode

- YL1200 – See PE1/100

- YL1210 – QQE03/12 with a different heater

- YL1220 – QQE02/5 with a different heater

- YL1240/8458 – RF power pentode

- YL1250/8505 – RF power pentode

- YL1270/8581 – RF power pentode

- YL1290 – QE08/200 with a different heater

- YL1310/8603 – RF power pentode

- YL1360 – QQE04/5 with a different heater

- YL1570 (RS 1084 CJ) – RF power tetrode

- Z - Gas-filled tubes

Note: See also standard M-P tubes under Z

- ZA

- ZA1000 – Neon-filled, coaxial, tritium-primed (half-life: 12.32 years), sputtered-molybdenum cold-cathode switching diode, meshed cylinder anode, all-glass pigtailed[87]

- ZA1001 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode with traces of heavy gas (krypton/xenon) for slow de-ionization, e.g. for low-frequency relaxation oscillators; meshed cylinder anode, all-glass pigtailed

- ZA1002 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode, large difference between burning and ignition voltage, meshed cylinder anode, 3-pin all-glass pigtailed

- ZA1003 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode for use as indicator tube in transistorized circuits, meshed cylinder anode, 3-pin all-glass pigtailed

- ZA1004 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode, small difference between burning and ignition voltage, for use as indicator tube in transistorized circuits or as 86.4 V Voltage reference, meshed cylinder anode, 3-pin all-glass pigtailed

- ZA1005 – Neon-filled, coaxial, tritium-primed, sputtered-molybdenum cold-cathode switching diode for use like a DIAC in thyristor circuits, meshed cylinder anode, 2-pin all-glass pigtailed

- ZC

- ZC1010 (Z661W) – 8 mAavg, 50 mApeak, Gas-filled, cold-cathode AC trigger pentode, two starters and a primer electrode, positive starter voltage, 5-pin all-glass pigtailed, envelope inside radioactively coated for a constant ignition voltage, for use in bidirectional counters

- ZC1040 – 25 mA, Gas-filled, cold-cathode AC trigger tetrode, one starter and a primer electrode, positive starter voltage, noval base

- ZC1050 – 2 mA, Gas-filled, cold-cathode, luminiscent trigger tetrode, one starter and a primer, 300 mlm light output[88] for use as self-displaying shift register cells in large-format, crawling-text dot-matrix displays;[89] all-glass pigtailed

- ZC1060 – 20 mAavg, 5 kApeak, Gas-filled, cold-cathode, high-current trigger triode for e.g. capacitor discharge circuits. One external (capacitive) starter electrode

- ZM

- ZM1000 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14 mm character height side-viewing, left decimal point

- ZM1000R – ZM1000 with a red contrast filter coating

- ZM1001 –  + - ~ X Y Z  Neon-filled digital indicator tube, 14mmCH side-viewing, for use with ZM1000

- ZM1001R – ZM1001 with a red contrast filter coating, for use with ZM1000R

- ZM1002 –  ns μs ms s Hz kHz MHz  Neon-filled digital indicator tube, 13mmCH side viewing, for use with ZM1000 in digital frequency counters

- ZM1003 –  1 - +  Neon-filled digital indicator tube, 14mmCH side-viewing, for use with ZM1000

- ZM1005 –  0 1 2 3 4 5 6 7 8 9  Long-life neon-filled digital indicator tube, 14mmCH side-viewing, left decimal point, multiplex-capable

- ZM1005R – ZM1005 with a red contrast filter coating

- ZM1006 –  1 2 3 4 5 6  Neon-filled digital indicator tube, side-viewing, left and right decimal point, for use in TV receivers

- ZM1008 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14mmCH side-viewing

- ZM1010 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14mmCH side-viewing, left decimal point

- ZM1012 –  0 1 2 3 4 5 6 7 8  Neon-filled digital indicator tube, 14mmCH side-viewing

- ZM1015 –  0 1 2 3 4 5 6 7 8  Neon-filled digital indicator tube, 14mmCH side-viewing

- ZM1020 (Z520M) – ZM1022 with a red contrast filter coating

- ZM1021 (Z521M) – ZM1023 with a red contrast filter coating, for use with ZM1020

- ZM1022 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH top-viewing, no decimal point

- ZM1023 –  A V Ω % + - ~  Neon-filled digital indicator tube, 15.5mmCH top-viewing, for use with ZM1022 in digital multimeters

- ZM1024 – ZM1025 with a red contrast filter coating, for use with ZM1020

- ZM1025 –  c/s Kc/s Mc/s μs ms ns s  Neon-filled digital indicator tube, 15.5mmCH top-viewing, for use with ZM1022 in digital frequency counters

- ZM1030 – ZM1032 with a red contrast filter coating

- ZM1031 – ZM1031/01 without the  ~

- ZM1031/01 – ZM1033/01 with a red contrast filter coating, for use with ZM1030

- ZM1032 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, no decimal point, 5 dual cathodes and separate odd/even anode compartments for biquinary multiplexing

- ZM1033/01 –  + - ~  Neon-filled digital indicator tube, 15.5mmCH side-viewing, separate anode compartment for  + , for use with ZM1032

- ZM1040 (Z522M) – ZM1042 with a red contrast filter coating

- ZM1041 – ZM1043 with a red contrast filter coating, for use with ZM1040

- ZM1041S – ZM1043S with a red contrast filter coating, for use with ZM1040

- ZM1042 (Z5220M) –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 30mmCH side-viewing, no decimal point

- ZM1043 –  + -  Neon-filled digital indicator tube, 30mmCH side-viewing, for use with ZM1042

- ZM1043S –  Y X + W U Z -  Neon-filled digital indicator tube, 30mmCH side-viewing, for use with ZM1042

- ZM1047 – ZM1049 with a red contrast filter coating, for use with ZM1040

- ZM1049 –  T F S N Z Y G H M X  Neon-filled digital indicator tube, side-viewing, for use with ZM1042 in numerical control systems

- ZM1050

  ZM1070

(Z550M, 8453) – Neon-filled digital indicator tube, top-viewing, dekatron-type readout with common anode and common cathodes, pulsating anode voltage, controlled by 5-volts sensitive starter electrodes, for transistorized circuits

  ZM1060 (Z505S) – Argon-filled, 50 kHz decade Counter/Selector Dekatron

  ZM1070 (Z504S, 8433) – Neon-filled, 5 kHz decade Counter/Selector Dekatron

  ZM1080 – ZM1082 with a red contrast filter coating

  ZM1081 – ZM1083 with a red contrast filter coating, for use with ZM1080

  ZM1082 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 14mmCH side-viewing, no decimal point, probe electrode

  ZM1083 –  + - ~  Neon-filled digital indicator tube, 14mmCH side-viewing, for use with ZM1082

  ZM1100 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH top-viewing

  ZM1120 – ZM1122 with a red contrast filter coating

  ZM1122 –  0 1 2 3 4 5 6 7 8 9  Miniature neon-filled digital indicator tube, 7.8mmCH top-viewing

  ZM1130 – ZM1132 with a red contrast filter coating

  ZM1131 – ZM1133 with a red contrast filter coating, for use with ZM1080

  ZM1132 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, side-viewing, left and right decimal point

  ZM1133 –  + - ~  Neon-filled digital indicator tube, side-viewing, for use with ZM1132

  ZM1136L/R – ZM1138L/R with a red contrast filter coating

  ZM1137 – ZM1139 with a red contrast filter coating, for use with ZM1136L/R

  ZM1138L/R –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 13mmCH side-viewing, left or right decimal points (specify)

  ZM1139 –  + - ~ Ω  Neon-filled digital indicator tube, 13mmCH side-viewing, for use with ZM1138 in digital multimeters

  ZM1162 –  0 1 2 3 4 5 6 7 8 9  Long-life neon-filled digital indicator tube, 15.5mmCH top-viewing, no decimal point, rectangular envelope for close stacking in both axes

  ZM1170 – ZM1172 with a red contrast filter coating

  ZM1172 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, no decimal point

  ZM1174 – ZM1175 with a red contrast filter coating

  ZM1175 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, left decimal point

  ZM1176 – ZM1177 with a red contrast filter coating

  ZM1177 – ZM1175, but right decimal point

  ZM1180 – ZM1182 with a red contrast filter coating

  ZM1181 – ZM1183 with a red contrast filter coating, for use with ZM1180

  ZM1182 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16mmCH top-viewing, no decimal point, semi-rectangular envelope for close horizontal stacking

  ZM1183 –  + - ~ Ω  Neon-filled digital indicator tube, top-viewing, 13mmCH for use with ZM1182 in digital multimeters

  ZM1184D – ZM1185D with a red contrast filter coating

  ZM1185A (GR1420) –  1 2 3 4 5 6 U K E R  Neon-filled digital indicator tube, 16mmCH top-viewing

  ZM1185D (GR1430) –  ∇ Δ  Neon-filled digital indicator tube, 16mmCH top-viewing, for use in elevators

  ZM1185E (GR1472) –  0 1 2 3 4 5 - t kg +  Neon-filled digital indicator tube, 16mmCH top-viewing

  ZM1200 – Pandicon, multiplexed 14-digit display tube with decimal points and punctuation marks, pin connections on both ends

  ZM1202 – 12-Digit Pandicon

  ZM1204 – 10-Digit Pandicon

  ZM1206 – 8-Digit Pandicon

  ZM1210

- ZM1212

– ZM1212 with a red contrast filter coating

- ZM1212 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH side-viewing, left decimal point, all-glass pigtailed

- ZM1220 – ZM1222 with a red contrast filter coating

- ZM1222 –  0 1 2 3 4 5 6 7 8 9  Large neon-filled digital indicator tube, 40mmCH side-viewing

- ZM1230 – ZM1232 with a red contrast filter coating

- ZM1232 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 15.5mmCH upside-down side-viewing, no decimal point

- ZM1240 – ZM1242 with a red contrast filter coating

- ZM1241 – ZM1243 with a red contrast filter coating, for use with ZM1240

- ZM1242 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16mmCH side-viewing, right decimal point

- ZM1243 –  + - ~ Ω  Neon-filled digital indicator tube, 16mmCH side-viewing, for use with ZM1242 in digital multimeters

- ZM1263 –  ~ ⚫  Neon-filled digital indicator tube, 10mmCH side-viewing

- ZM1290 – ZM1292 with a red contrast filter coating

- ZM1292 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 10mmCH side-viewing

- ZM1330 – ZM1332 with a red contrast filter coating

- ZM1331 – ZM1333 with a red contrast filter coating, for use with ZM1330

- ZM1332 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 13.1mmCH side-viewing, left and right decimal points, all-glass pigtailed

- ZM1333 –  + - ~ Ω  Neon-filled digital indicator tube, 13.1mmCH side-viewing, all-glass pigtailed, for use with ZM1332 in digital multimeters

- ZM1334 – ZM1336 with a red contrast filter coating

- ZM1335 – ZM1337 with a red contrast filter coating, for use with ZM1334

- ZM1336 –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 13.1mmCH side-viewing, left and right decimal points, multiplex-capable

- ZM1337 –  + - ~ Ω  Neon-filled digital indicator tube, 13.0mmCH side-viewing, right decimal point (!), all-glass pigtailed, red contrast filter coating, for use with ZM1336 in digital multimeters

- ZM1350 – Varisymbol, planar neon-filled digital 40mm x 27mm fourteen-segment display tube, right decimal point, separate underscore text cursor, keep-alive cathode, multiplex-capable, viewing angle 160°

- ZM1360 – ZM1350 with 60mm x 40mm characters

- ZM1370 – ZM1350 with 20mm x 13mm characters

- ZM1410 – ZM1412 with a red contrast filter coating

- ZM1412 – Neon-filled digital seven-segment display tube, 8.6mmCH side-viewing, right decimal point and left punctuation mark, all-glass pigtailed

- ZM1500 – Pandicon, multiplexed 12-digit, 7-segment display tube

- ZM1550 – Planar neon-filled digital two-digit seven-segment display tube, right decimal points

- ZM1551 – Planar neon-filled digital 1½-digit seven-segment display tube with  +  and  -  signs, right decimal points

Note: More Nixie tubes under standard - ZM and other letter - GR

- ZP

- ZP1000 – Borium tri-fluoride-filled Geiger-Müller tube, thermal neutrons

- ZP1010 – Borium tri-fluoride-filled Geiger-Müller tube, thermal neutrons

- ZP1020 – Borium tri-fluoride-filled Geiger-Müller tube, thermal neutrons

- ZP1070 – Subminiature Geiger-Müller tube, all-glass pigtailed

- ZP1080 – Halogen-quenched Geiger-Müller tube, β and γ

- ZP1100 – Halogen-quenched Geiger-Müller tube, γ; pigtailed

- ZP1200 – Halogen-quenched Geiger-Müller tube, γ

- ZP1300 – Halogen-quenched Geiger-Müller tube, γ and high-energy β

- ZP1330 – Halogen-quenched Geiger-Müller tube, for use in damp and/or saline atmosphere, β and γ

- ZP1400 – Halogen-quenched Geiger-Müller tube, 9mm diameter mica window, β and γ

- ZP1430 – Halogen-quenched Geiger-Müller tube, 27.8mm diameter mica window, α, β, γ

- ZP1490 – Halogen-quenched Geiger-Müller tube, 28mm diameter mica window, low-level α, β and γ

- ZP1600 – Halogen-quenched Geiger-Müller tube, 19.8 mm diameter mica window, X-rays, 6.0 to 20 keV energy, 60 to 200 pm wavelength range

- ZP1610 – Side window, organically quenched Geiger-Müller tube. 7 x 18 mm mica window; X-rays, 2.5 to 40 keV energy, 30 to 500 pm wavelength range

- ZP1700 – Halogen-quenched, cosmic-ray guard counter tube for low-background measurements; to be used with another radiation counter tube in an anticoincidence circuit

- ZP1800 – Halogen-quenched Geiger-Müller tube for use at temperatures up to 200 °C, γ

- ZP1810 – Halogen-quenched Geiger-Müller tube for use at temperatures up to 200 °C, γ, low sensitivity, up to 40 mGy/h

- ZP1860 – Halogen-quenched Geiger-Müller tube, β and γ

- ZT

- ZT1000 – 21 kV, 10 A Mercury vapor triode thyratron

- ZX

- ZX1000 – 800 V, 1140 Apk, 13Aavg Ignitron

- ZX1051 – Water-cooled, 56 Aavg Ignitron

- ZX1052 – Water-cooled, 140 Aavg Ignitron

- ZX1053 – Water-cooled, 355 Aavg Ignitron

- ZX1060 – Water-cooled, 10 Aavg Ignitron

- ZX1061 – Water-cooled, 10 Aavg Ignitron

- ZX1062 – Water-cooled, 10 Aavg Ignitron

- ZX1063 – Water-cooled, 10 Aavg Ignitron

- ZY

- ZY1000 (872B) – High voltage, half-wave mercury-vapor rectifier

- ZY1001/8008A – High voltage, half-wave mercury-vapor rectifier, 4-pin base with anode top cap

- ZY1002 – High voltage, half-wave mercury-vapor rectifier, E40 (Goliath) Edison screw lamp base with anode top cap

- ZZ

- ZZ1000 – 81 V Voltage reference, 2-pin all-glass pigtailed

- ZZ1010 – 85 V Voltage reference

- ZZ1020 (STV85-8) – 82 V Voltage reference with primer electrode, 3-pin all-glass pigtailed

- ZZ1030 (STV500-0,1) – Quad 125 V Voltage references, noval base

- ZZ1031 – Quad Voltage reference, noval base

- ZZ1040 (STV100-60Z) – 100 V Voltage reference with primer electrode

- ZZ1050 – 82 V Voltage reference, 2-pin all-glass pigtailed

- List of European transmitting tubes

Note: Typecode explained above.

- B - Backward-wave amplifier

- BA

- BA9/20 – X-band, 20 mW, Forced-air cooled backward-wave oscillator

- D - Rectifier incl. grid-controlled

- DA

- DA1.5/75 – 1.5 kV, 75 W Half-wave power rectifier, triode TA1.5/75 without grid

- DA12/24000 – 12 kV, 24 kW Water-cooled half-wave power rectifier

- DC

- DC1/50 – 1 kV, 50 mA Full-wave power rectifier, DC1/60 with dual anode top cap

- DC1/60 – 1 kV, 60 mA Full-wave power rectifier

- DC2/200 – 2 kV, 200 mA Full-wave power rectifier with dual anode top cap

- DCG

- DCG1/125 – 1 kV, 125 mA Half-wave mercury-vapor rectifier with Edison screw lamp base and anode top cap

- DCG12/30 – 12 kV, 30 A Grid-controlled, half-wave mercury-vapor rectifier with anode top cap

- DCX

- DCX4/1000 – 4 kV, 1 kW Half-wave xenon rectifier with anode top cap

- DCX4/5000 – 4 kV, 5 kW Half-wave xenon rectifier with anode top cap

- DE

- DE2/200 – 2 kV, 200 W Full-wave power rectifier with dual anode top cap

- J - Magnetron

- JP

- JP8/02B – 8.8 GHz, 25 W Magnetron

- JP9/15 – 9.345 to 9.405 GHz, 15 kW Forced-air cooled magnetron for pulsed service

- JPT

- JPT9/01 – 9.15 to 9.60 GHz, 5 W Magnetron

- K - Klystron

- KB

- KB9/150W – X-band, 150 W Water-cooled, dual-resonator klystron

- KS

- KS7/85 – 6.5 to 7.5 GHz, 85 mW Reflex klystron

- L - Traveling-wave tube

- LA

- LA9/3 – 7 to 11.5 GHz Forward-wave amplifier

- LA16/2 – 11.5 to 18 GHz Forward-wave amplifier

- M - AF modulator Triode

- MA

- MA4/600 – 4 kV, 600 W Radiation-cooled triode

- MB

- MB1/50 – 1 kV, 50 W Radiation-cooled triode

- MB2/200 – 2 kV, 200 W Radiation-cooled triode

- MY

- MY3/275 – 3 kV, 275 W Radiation-cooled triode

- MZ

- MZ2/200 – 2 kV, 200 W Radiation-cooled triode

- P - Pentode

- PA

- PA12/15 – 15 kW Water-cooled shortwave pentode

- PA12/20 – 20 kW Water-cooled pentode made by Philips and used in the 1930s and 1940s

- PAL

- PAL12/15 – Air-cooled version of PAW12/15

- PAW

- PAW12/15 – 15 kW Water-cooled shortwave pentode

- PB

- PB2/200 – 200 W Shortwave pentode

- PB3/1000 – 1 kW Shortwave pentode

- PC

- PC03/3 – 3 W Shortwave pentode

- PC3/1000 – 1 kW Shortwave pentode

- PE

- PE04/10 – 10 W Shortwave pentode

- PE1/100 (YL1200) – 100 W Shortwave pentode

- Q - Tetrode

- QB

- QB2/75 – 75 W Beam-tetrode

- QB5/2000 – 2 kW Beam-tetrode

- QBL

- QBL4/800 – Air-cooled 800 W beam-tetrode

- QBL5/3500 – Air-cooled 3500 W beam-tetrode

- QBW

- QBW5/3500 – Water-cooled 3500 W beam-tetrode

- QC

- QC05/15 – 15 W Beam-tetrode

- QC05/35 – 35 kW Beam-tetrode

- QE

- QE04/10 – 10 W Beam-tetrode

- QE05/40 (6146) – 40 W Radiation-cooled output beam-tetrode, popular amongst radio amateurs as a final RF amplifier

- QE08/200 – 200 W Beam-tetrode

- QEL

- QEL1/150 – Air-cooled 150 W beam-tetrode

- QEL1/250 – Air-cooled 250 W beam-tetrode

- QEP

- QEP20/18 – 18 W Beam-tetrode for use as a pulse modulator

- QQC

- QQC03/14 – 14 W Dual beam-tetrode

- QQE

- QQE02/5 (6939) – 5 W Dual beam-tetrode

- QQE03/12 (6360) – 12 W Dual beam-tetrode

- QQE03/20 (6252) – 20 W Dual beam-tetrode

- QQE04/5 (7377) – 5 W Dual beam-tetrode

- QQE06/40 (5894, YL1060) – 40 W dual beam-tetrode, internally neutralized, Septar base with dual anode top cap

- QQV

- QQV02/6 – 6 W dual beam-tetrode

- QQV03/20A – 20 W Radiation-cooled split-anode tetrode made by Mullard and used in the 1940s, 1950s and 1960s as a VHF frequency-doubling output stage with balanced output.

- QQV07/50 – 50 W Dual beam-tetrode

- QQZ

- QQZ03/20 (8118, YL1020) – 20 W Dual beam-tetrode

- QQZ06/40 (YL1030) – 40 W Dual beam-tetrode

- QV

- QV04/7 – 7 W Beam-tetrode

- QV05/25 (807) – 25 W Radiation-cooled output beam-tetrode made by Mullard.

- QV2/250C – 250 W Beam-tetrode

- QY

- QY3/65 – 65 W Beam-tetrode

- QY5/3000A – 3 kW Beam-tetrode

- QY5/3000W – Water-cooled version of QY5-3000A

- QYS

- QYS50/P40 – Pulsed power tetrode, Silica envelope, 50 kV anode voltage, considerable x-radiation, 810 °C anode temperature at 700 W anode dissipation, 40 A anode current at duty factor 0.0005, Vg1Cut-off (IA=1 mA@VA=55 kV): > -3.4 kV, gm: 38 mS

- QZ

- QZ06/20 – 25 W VHF Power tetrode up to 175 MHz

- R - Rectifier

- RG

- RG1000/3000 – 1 kV, 3 A Half-wave mercury-vapor rectifier with anode top cap

- RGQ

- RGQ7.5/0.6 – 7.5 kV, 600 mA Half-wave mercury-vapor rectifier with anode top cap

- RGQ20/5 – 20 kV, 5 A Half-wave mercury-vapor rectifier with anode top cap

- T - AF/RF/oscillator Triode

- TA

- TA04/5 – 400 V, 50 W Radiation-cooled power triode

- TA1.5/75 – 1.5 kV, 75 W Radiation-cooled power triode

- TA4/2000K – 4 kV, 2 kW Air-cooled power triode made by Philips in the 1930s

- TA18/100000 – 18 kV, 100 kW Water-cooled power triode

- TB

- TB04/8 – Directly heated Doorknob VHF power triode up to 600 MHz

- TB2.5/400 – 2.5 kV, 300 W Radiation-cooled power triode

- TB5/2500 – 5 kV, 2.5 kW Radiation-cooled power triode

- TBL

- TBL2/300 – 2 kV, 300 W Forced air-cooled power triode

- TBL15/125 – 15 kV, 125 kW Forced air-cooled power triode

- TBW

- TBW6/14 – 6 kV, 14 kW Water-cooled power triode

- TBW15/125 – 15 kV, 125 kW Water-cooled power triode

- TC

- TC03/5 – RF power triode up to 85 MHz, 5 W

- TC2/250 – RF power triode up to 20 MHz, 250 W

- TD

- TD03/5 – Indirectly heated disk-seal UHF power triode up to 2 GHz

- TD03/10 – Indirectly heated disk-seal UHF power triode up to 2.8 W, 3.75 GHz

- TD03/10F – TD03/10 with internal feedback for use as an oscillator

- TD04/20 – Indirectly heated disk-seal UHF power triode up to 13.5 W, 1 GHz

- TD1/100C = 2C39BA – Indirectly heated, ceramic disk-seal UHF power triode up to 24 W, 3.5 GHz[90]

- TD2/400 – Directly heated, ceramic disk-seal UHF power triode up to 600 W, 900 MHz

- TD2/500 – Directly heated, ceramic disk-seal UHF power triode up to 500 W, 940 MHz

- TE

- TE05/10 – RF power triode up to 150 MHz

- TX

- TX12/12W – Water-cooled RF power triode

- TX12/20W – Water-cooled RF power triode

- TX10/4000 – Power triode, Silica envelope, 12 kV anode voltage, 4 kW anode dissipation, 1.6 A cathode current, gm: 4.5 mS, for use as self-excited high-power oscillator in induction heating equipment.

- TY

- TY2/125 – 135 W VHF power triode up to 200 MHz

- TY12/50A – Forced-air cooled 45 kW RF power triode up to 30 MHz

- TY12/50W – Water-cooled 50 kW RF power triode up to 30 MHz

- TYS

- TYS2/250 – Power triode, Silica envelope, 2.5 kV anode voltage, 250 W anode dissipation

- TYS4/500 – Power triode, Silica envelope

- TYS5/1000 – Power triode, Silica envelope

- TYS5/2000 – Power triode, Silica envelope

- TYS5/3000 – Power triode, Silica envelope, 6 kV anode voltage, 950 °C anode temperature at 3.5 kW anode dissipation, 2.8 A cathode current, gm: 15 mS. Used in RF generators for induction hardening.

- X - Thyratron

- XGQ

- XGQ2/6400 – 2 kV, 6.4 kW Mercury-vapor tetrode thyratron with anode and grid1 top caps

- XR

- XR1/1600 (5545) – 1 kV, 1.6 kW Inert gas-filled triode thyratron with anode top cap

- XR1/6400 – 1 kV, 6.4 kW Inert gas-filled triode thyratron with anode top cap

- List of other letter tubes

- A

Edison and Swan Electric Light Company (British Mazda/EdiSwan):

- A40 – Acorn UHF triode up to 600 MHz, 4 Volts heater

- A41 – Acorn UHF pentode up to 600 MHz, 4 Volts heater

- AC*/

Mazda/EdiSwan 4-volts AC, indirectly heated receiver tubes:

- AC/HL – AF triode, British 5-pin base

- AC/HLDD = TDD4 = MHD4 – Dual diode and AF triode, British 7-pin base

- AC/ME – Tuning indicator, British 7-pin base

- AC/P, AC/P1 – AF triode, British 5-pin base

- AC/P4 – CRT electrostatic-deflection output power triode, British 5-pin base

- AC/PEN – AF power pentode, British 7-pin base

- AC/S2PEN – RF pentode, British 7-pin base

- AC/SP1 – RF pentode for use in squelch circuits or, as the reactance tube, in AFC circuits, British 7-pin base

- AC/SP3 – RF pentode for shortwave and TV receivers, British 7-pin base

- AC/SP3/RH – Low-noise, low-microphonics RF pentode for shortwave and TV receivers, British 7-pin base

- AC/TH1 – Triode/hexode oscillator/mixer, British 9-pin base

- AC/TP = TP4 – Triode/pentode oscillator/mixer, British 7-pin base

- AC/VP1, AC/VP2 – RF pentode, British 7-pin base

- AC2/HL – High-mu triode

- AC2/PEN – AF Power pentode

- AC2/PEN.DD – Dual diode and AF Power pentode

- AC4/PEN – AF Beam power pentode

- AC5/PEN – AF Beam power pentode

- AC5/PEN.DD – Dual diode and AF Beam power pentode

- AC6/PEN – Beam power pentode for use as a magnetic horizontal-deflection output amplifier

- ACT

Marconi-Osram Valve Company:

- ACT9 – 800 W Air cooled transmitting triode up to 15 MHz, with derating up to 80 MHz

- AP

Mullard:

- AP4 = 4676 – Acorn UHF pentode up to 430 MHz, 4 Volts heater

- AT

Mullard:

- AT4 = 4675 – Acorn UHF triode up to 430 MHz, 4 Volts heater

- B

- BA

Industrial Electronics Engineers:

- BA-0000-P31 – Nimo tube, cathode-ray 1-digit numeric display tube, 10 stenciled electron guns aiming at a P31-phosphor (green, medium-persistence) fluorescent screen, top-viewing, Futura Medium font, 2.5 kV anode voltage, 12-pin base

- BG

Burroughs:

- BG08220-K – Neon-filled, planar, base-24 display dekatron with four sets of guiding cathodes and a carry/borrow cathode, for use e.g. in direction-finding equipment

- BT

British Thomson-Houston (General Electric subsidiary):

- BT1 – Thyratron used in Wynn-Williams' binary prescaler for the alpha particle counter that Rutherford, Chadwick et al. used for their nuclear research at the Cavendish Laboratory in the 1930s[91]

- C

- CH

Tung-Sol:

- CH1027 – Curristor – Four types of nitrogen-filled, radioactive constant-current tubes with a current plateau from 25 to 500 V, all-glass pigtailed, active material is 226Ra with a half-life of 1601 years, for linear capacitor charging and draining in missile and ordnance mine timing circuits, instrumentation biasing, as current reference, etc.:

- CH1027-9 – 10−9 A, 18.75 μCi (694 kBq)

- CH1027-10 – 10−10 A, 1.875 μCi (69.4 kBq)

- CH1027-11 – 10−11 A, 187.5 nCi (6.94 kBq)

- CH1027-12 – 10−12 A, 18.75 nCi (694 Bq)

- CK

Raytheon:

| CK1414 EBCDIC target |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H | I | ? | . | ¤ | [ | < | ⊞ |
| + | A | B | C | D | E | F | G |
| Q | R | ! | $ | * | ] | ; | △ |
| - | J | K | L | M | N | O | P |
| Y | Z | = | , | % | ≶ | ' | ⊠ |
|  | / | S | T | U | V | W | X |
| 8 | 9 | 0 | # | @ | : | > | ■ |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

- CK1022 – 1 kV/5...55 µA Corona voltage reference, miniature 7-pin base with anode top cap

- CK1037 = 6437 – 700 V/5...125 µA Corona voltage reference, 3-pin all-glass pigtailed

- CK1038 – 900 V/5...55 µA Corona voltage reference, 3-pin all-glass pigtailed

- CK1039 = 6438 – 1.2 kV/5...125 µA Corona voltage reference, 3-pin all-glass pigtailed

- CK1366, CK1367, CK1368, CK1369 – CRTs with an unphosphored front glass but with fine wires embedded in it for use as electrostatic print heads; the wires would pass the electron beam current through the glass onto a sheet of paper where the desired content was therefore deposited as an electrical charge pattern. The paper was then passed near a pool of liquid ink with the opposite charge. The charged areas of the paper attract the ink and thus form the image.[92][93]

- CK1383 – Dual-electron gun recording storage tube, a realtime polar, radar PPI-to-rectangular, TV-type analog video transcoder similar to the 7702, with simultaneous R/W, and storing capability. This was achieved by a CRT/camera tube combination; the CRT part writes the PPI-format image onto a thin, dielectric target; the camera part reads the generated charge pattern in TV format from the back side of this target.[94]

- CK1414 – Symbolray character generator monoscope for text mode video rendering in early computer monitors, with a square target having letters, digits and symbols patterned on it in a customer-supplied 8x8 or 8x12 array. An electron beam selects and scans a character, both by appropriate electrostatic deflection, and generates an analog video signal;[95][96] cf. 4560, TH9503

- CL

Ferranti:

- CL40 and CL41 – Indirectly heated, linear light source (glow modulator tube), mercury/argon-filled gas diode with primer electrode, 8-pin base, for rotating-drum FAX receivers, film soundtrack recording, etc.

- CL42 and CL43 – Indirectly heated, low-noise linear light source, helium-filled gas diode with primer electrode, 8-pin base, for film soundtrack recording, interferometers, etc.

- CL44 – Indirectly heated, low-noise linear light source, neon-filled gas diode with primer electrode, 8-pin base

- CL50 and CL52 – Indirectly heated, linear light source, gas-filled diode with primer electrode, 7-pin base, for rotating-drum FAX receivers, film soundtrack recording, etc.

- CL55 – Indirectly heated, spectrally pure light source, helium-filled gas diode with primer electrode, 7-pin base with anode top cap

- CL56 – Indirectly heated, spectrally pure light source, krypton-filled gas diode with primer electrode, 7-pin base with anode top cap

- CL57 – Indirectly heated, spectrally pure light source, neon-filled gas diode with primer electrode, 7-pin base with anode top cap

- CL58 – Indirectly heated, spectrally pure light source, xenon-filled gas diode with primer electrode, 7-pin base with anode top cap

- CL60 – Indirectly heated triode flood beam CRT-type stroboscope lamp with a green A-type phosphor with <1 μs decay time and 10 kCd light output, 20 kV anode voltage, 7-pin duodecal base

- CL61 – CL60 with a blue P-type phosphor with 5 μs decay time and 16 kCd light output

- CL62 – CL60 with an UV Q-type phosphor with 100 ns decay time and 240 Cd light output

- CL63 – CL60 with a yellow-green C-type phosphor with 6 μs decay time and 24 kCd light output

- CL64 – CL60 with a yellow V-type phosphor with 5 μs decay time and 12 kCd light output

- CL65 – CL60 with a red R-type phosphor with 2 μs decay time and 14 kCd light output

- CL66 – CL60 with a white T-type phosphor with 5 μs decay time and 12 kCd light output

- D

Philips:

- D1 – Early directly heated triode used in 1920s TRF and regenerative radios

- DDR

Mullard:

- DDR100 – 100 g max., 250 Hz max., 1-axis accelerometer double diode with elastically supported anodes, 6.3V/600mA indirect heater, fres = 1 kHz, B8G base[97]

- DZ

Cerberus:

- DZ10 – 3 kHz max. Decade Counter/Selector Dekatron, 14-pin diheptal base

- E

- EN

Ferranti:

- EN10 – Neostron, 400 Apk Gas-filled, cold-cathode tetrode thyratron, differential trigger electrodes, 8-pin base, for use as a relay or as a reddish 700 Cd stroboscope lamp

- EN15 – 80 Aavg Neon-filled, cold-cathode tetrode thyratron, differential trigger electrodes, miniature 9-pin base, for use as a stroboscope lamp

- EN30 – 250 Apk Gas-filled, arc-discharge cold-cathode tetrode thyratron, differential trigger electrodes, miniature 7-pin base with anode cap, for use as a relay or as a stroboscope lamp

- EN40 – 250 Apk Gas-filled, cold-cathode tetrode thyratron, differential trigger electrodes, 8-pin base, for use as a whitish stroboscope lamp with a high actinism for photographic film

- EN55 Single, EDN10 dual xenon-filled, arc-discharge cold-cathode tetrode thyratron, external (capacitive) trigger, 12-pin base, for use as a white 140 kCd stroboscope lamp

- EN60 – Gas-filled, arc-discharge cold-cathode tetrode thyratron, external (capacitive) trigger, Edison screw lamp base with anode cap, for use as a white 900 klm@10µF@800V stroboscope lamp

- G

Standard Telephones and Cables/Brimar:

- G10/241E – Nomotron, an unidirectional Dekatron with multi-alloy cathodes

Cerberus:

- G11 – 5 mA Gas-filled, cold-cathode switching diode e.g. for relaxation oscillators, 2-pin all-glass pigtailed

- G42 – 35 mApeak Gas-filled switching diode e.g. for relaxation oscillators, 2-pin all-glass pigtailed

- GC

Ericsson Telephones Limited:

- GC10/2P – Neon-filled, 1 kHz Miniature decade Counter Dekatron, a gas-filled, bidirecional decade counter tube

- GC10A – Helium-filled, decade Counter Dekatron

- GC10B – Neon-filled, 4 kHz Long life, decade Counter Dekatron

- GC10/4B – 4 kHz Decade Computing Counter Dekatron with carry/borrow cathodes "0" and "9" and intermediate cathodes "3" and "5" wired to separate pins

- GC10D – 20 kHz Decade Counter Dekatron, for single-pulse operation

- GC12/4B – 4 kHz Duodecimal Counter Dekatron with carry/borrow cathodes 11 and 12 and intermediate cathodes 6 and 8 wired to separate pins

- GCA

Ericsson Telephones Limited:

- GCA10G – 10 kHz max. Decade Counter Dekatron with routing guides and aux anodes to directly drive Nixie tubes, B27A base without the inner pin ring

- GD

Ericsson Telephones Limited:

- GD2V – 2 kV, 16 J discharge tube, all-glass studded

- GD75P – 75 V Voltage reference, miniature 7-pin base

- GD90M – 90 V Voltage reference, miniature 7-pin base

- GD340X – 345 V/3...200 µA Corona voltage reference, all-glass pigtailed

- GD350X, GD350Y – 350 V/3...200 µA Corona voltage reference, all-glass pigtailed

- GD550W – 550 V, 1.5 J Discharge tube, e.g. for power relaxation oscillators, all-glass pigtailed

- GDT

Ericsson Telephones Limited:

- GDT120M – 9 mA Gas-filled cold-cathode DC triode, one starter and a separate glow diode acting as an optical primer, miniature 7-pin base

- GE

Ferranti:

- GE10 – Directly heated saturated-emission diode. Acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits. It has two shorted pins that can be used to disable the circuit if the tube is removed from its socket

- GK

Cerberus:

- GK11 – Touch button tube, an illuminated capacitance touch switch; a cold-cathode DC relay tube, external (capacitive) starter activated by touching; then the cathode glow is visible as an orange ring. 2-pin all-glass pigtailed

- GN

Ferranti:

- GN10 – 250 Amps pulse-current, cold-cathode tetrode thyratron. Octal base

- GR

Ericsson Telephones Limited:

- GR2G –  + -  Neon-filled digital indicator tube, 18 x 18 mm characters, side-viewing

- GR2H –  + -  Neon-filled digital indicator tube, 20 x 20 mm characters, top-viewing

- GR4G –  ¼ ½ ¾ 1  Neon-filled digital indicator tube, 18 x 30 mm characters, side-viewing

- GR7M –  + - V A Ω % ~  Neon-filled digital indicator tube, 15.5 mm character height, top-viewing

- GR10A – Gas-filled digital indicator tube with a dekatron-type readout

- GR10G –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16.86 x 30 mm characters, side-viewing

- GR10H –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 12 x 19 mm characters, top-viewing

- GR10J –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 16.86 x 30 mm characters, side-viewing

- GR10K –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 12 x 19 mm characters, top-viewing

- GR10M –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 10 x 15.5 mm characters, top-viewing

- GR10W –  0 1 2 3 4 5 6 7 8 9  Neon-filled digital indicator tube, 8.42 x 15 mm characters, side-viewing, all-glass pigtailed

- GR12G –  A B C D E F G H I J K L  Neon-filled digital indicator tube, 16 x 30 mm characters, side-viewing

- GR12H –  E L M N P R S T U V W X  Neon-filled digital indicator tube, 16 x 30 mm characters, side-viewing

Note: More Nixie tubes under standard - ZM and professional - ZM

Cerberus:

- GR15 – 15 mA Gas-filled cold-cathode DC tetrode, one starter and one electrical primer and tritium-primed (half-life: 12.32 years), noval base, for voltage triggers, RC timers etc.

- GR16 – 20 mA Gas-filled, cold-cathode, tritium-primed AC/DC triode, one starter and an EM shield, noval base, for voltage triggers, RC timers etc.

- GR17 – 15 mA Gas-filled cold-cathode AC triode, one starter and an EM shield, noval base, for voltage triggers, RC timers etc.

- GR31 – 15 mA Gas-filled cold-cathode DC tetrode, one starter and one electrical primer plus a tritium primer, noval base

- GR44 – 12 mA Subminiature gas-filled cold-cathode DC pentode, two starters and one primer electrode plus a tritium primer, 5-pin all-glass pigtailed

- GR46 – 12 mA Subminiature gas-filled cold-cathode DC tetrode, one starter and one primer electrode, 4-pin all-glass pigtailed

- GRD

Ferranti:

- GRD7 – Educational, directly heated saturated-emission guard ring diode

- GS

Ericsson Telephones Limited:

- GS10C – 4 kHz max. Decade Counter/Selector Dekatron, top-viewing, duodecal base

- GS10D – Hydrogen-filled, 20 kHz max. Decade Counter/Selector Dekatron, duodecal base

- GS10H – 4 kHz max. Decade Counter/Selector Dekatron with routing guides, B17A base

- GS12C – 4 kHz max. Duodecimal Counter/Selector Dekatron, with solder lugs

- GS12D – Neon-filled, 4 kHz max. duodecimal Counter/Selector Dekatron, duodecal base with two additional pigtails for the guide electrodes

- GSA

Ericsson Telephones Limited:

- GSA10G – 10 kHz max. Decade Counter/Selector Dekatron with routing guides and aux anodes to directly drive Nixie tubes, B27A base

- GTE

Ericsson Telephones Limited:

- GTE120Y – 5 mA Subminiature DC trigger tetrode, one starter and one primer, all-glass pigtailed

- GTE130T – 8 mApeak DC trigger tetrode, one starter and one primer, close tolerance, low aging, quadrant I operation only, noval base

- GTE175M – 3.5 mAavg, 50 mApeak DC Trigger tetrode, one starter and one primer, miniature 7-pin base, for Dekatron coupling circuits

- GTR

Ericsson Telephones Limited:

- GTR120W – 9 mA Subminiature DC trigger triode, 3-pin all-glass pigtailed, for computer applications

- GTR75M – 75 V Voltage reference, Miniature 7-pin

- GTR95M/S – 95 V Voltage reference, Miniature 7-pin

- GTR150 – Subminiature, primed 150 V voltage reference, all-glass pigtailed

- K

- KN

KN2

Edgerton, Germeshausen, and Grier:

- KN2 – 4 kV, 500 Asurge Krytron, a cold-cathode gas-filled tube with a primer electrode, for use as a very high-speed, high-surge current switch; similar to a thyratron, lifespan 107 shots, 4-pin all-glass pigtailed[98]

- KN4 – 5 kV, 2.5 kAsurge Krytron with a primer electrode, lifespan 25000 shots, 4-pin all-glass pigtailed

- KN6 – 5 kV, 3 kAsurge Krytron with a primer electrode, lifespan 35000 shots, 4-pin all-glass pigtailed

- KN6B – 8 kV, 3 kAsurge Krytron with a primer electrode, lifespan 35000 shots, 4-pin all-glass pigtailed

- KN9 – 4 kV, 500 Asurge Krytron with a primer electrode, lifespan 1.5⋅107 shots, 4-pin all-glass pigtailed

- KN11B – 2.5 kV, 1.5 kAsurge Sprytron, lifespan 2000 shots, 3-pin all-glass pigtailed

- KN12 – 5 kV, 3 kAsurge Sprytron, lifespan 500 shots, 3-pin all-glass pigtailed

- KN22 – 5 kV, 100 Asurge Krytron with a primer electrode, lifespan 2⋅107 shots, 4-pin all-glass pigtailed, for laser pumping, to drive Pockels cells, also for educational purposes[99]

- KN26 – 5 kV, 3 kAsurge Krytron with a primer electrode, lifespan 75000 shots, 4-pin all-glass pigtailed

- KT

Marconi-Osram Kinkless Tetrode beam power tubes

- KT32 (25L6, 25L6G, 25L6GT and 25W6GT)

- KT33 (25A6GT)

- KT41

- KT61 (6M6G) in parallel filament circuits

- KT63 (6F6, 6F6G, 6F6GT)

- KT66 (6L6GC)

- KT67 – Small transmitting valve

- KT71 (50L6GT)

- KT77 – Similar to EL34, 6CA7

- KT81

- KT88 = 6550A = CV5220 (12E13, 7D11) – AF beam power tube, two tubes are capable of providing 100W output, Class-AB1, octal base

- KT90

"Tung-Sol":

- KT120 – New production tube

- KT150 – New production tube

- M

- M8000s

UK Military developed:

- M8083 – Sharp-cutoff pentode, miniature 7-pin base (SQ version of EF91 = 6AM6 = Z77)

- M8162 = 6060 – High-mu dual triode, for use as RF amplifier/mixer in VHF circuits, noval base (SQ version of ECC81 = 12AT7 = B309)

- MC

Philips:

- MC6-16, MC13-16 – CRT-type flying-spot scanners, P16-type phosphor (violet/near-ultraviolet, very short persistence), for use in a telecine

- ME

Edison and Swan Electric Light Company (British Mazda/EdiSwan):

- ME91 – AC/DC mains tuning indicator

- P

- PBG

Dale:

- PBG12201 – Neon-filled, planar, dual 200-segment linear glow-transfer bar graph display tube with three cathode strings,[100] for use in VU meters etc.; cf. BG08220-K

- PBG12203 – Neon-filled, planar, dual 201-segment linear glow-transfer bar graph display tube with three cathode strings, for use in VU meters etc.

- PBG12205 – Neon-filled, planar, dual 200-segment linear glow-transfer bar graph display tube with five cathode strings, for use in VU meters etc.

- PBG16101 – Neon-filled, planar, dual 101-segment linear glow-transfer bar graph display tube with three cathode strings, for use in VU meters etc.; similar to ИН-33

- PD

Edison and Swan Electric Light Company (British Mazda/EdiSwan):

- PD220 – Dual AF power triode for battery-supplied equipment (1939)

- PL

Philips:

- PL21 = 2D21 = EN91 – 100 mAavg, 500 mApeak, 10 Asurge, Gas-filled, indirectly heated tetrode thyratron, negative starter voltage, miniature 7-pin base, for relay and grid-controlled rectifier service

- PL323 = 3C23 – 1.5 Aavg, 6 Apeak, Mercury-vapor triode thyratron, 4-pin base with anode top cap

- PL5727 = 5727 – 100 mAavg, 500 mApeak, 10 Asurge, Tetrode thyratron, 7-pin miniature base

- Q

Philips:

- Q13-110GU – CRT-type flying-spot scanner, white phosphor, for use in a telecine

- QK

Raytheon:

- QK329 – Beam-deflection square-law tube for use as a function generator in analog computers. A flat sheet beam is electrostatically deflected across the anode which is partially covered by a parabolically stenciled screen "grid" that acts as the tube's output. Two tubes may be combined to form a 1-quadrant analog multiplier using the equation a⋅b = (a+b)2 − (a−b)2/4 where the deflection electrode signals a+b and a−b can be obtained directly from a fully balanced resistor bridge[101]

- R

Marconi-Osram Valve Company:

- R – Early directly heated triode derived from the French TM tube and used by many amateurs in the 1920s[102][103][104]

- RK

Raytheon:

- RK61 – Miniature, gas-filled, directly heated thyratron designed specifically to operate like a vacuum triode below its ignition voltage, allowing it to both amplify analog signals and work as a relaxation oscillator, for use as a self-quenching superregenreative detector up tp 100 MHz in radio control receivers, activating a relay in its anode circuit when a carrier wave is received; 4-pin all-glass pigtailed, 1.4 V, 45 mA filament, Ua=45 V, Ia=1.5 mA.[105]

- RK62 – RK61's predecessor, marketed since 1938;[106] this was the major technical development which led to the wartime development of radio-controlled weapons and the parallel development of radio controlled modelling as a hobby.[107]

- S

- SB

Radio Corporation of America:

- SB256 – 256-bit Selectron tube, an early form of digital computer memory[108][109]

- SU

Cossor:

- SU25 – EHT rectifier

- SU2150 (CV1120) – High-voltage vacuum half-wave rectifier for use in CRT power supplies

- T

British General Electric Company:

- TuneOn – Early neon-filled bar graph tuning indicator, a glass tube with a short wire anode and a long wire cathode that glows partially; the glow length is proportional to the tube current[110]

- TuneOn Button – Early glow modulator used as a budget-priced tuning indicator – a neon lamp whose brightness is proportional to the tube current[111]

Standard Telephones and Cables/Brimar:

- Tunograph – Precursor of the "Magic Eye" tuning indicator first introduced in 1933; a tiny CRT with 1-axis electrostatic deflection and a phosphored target at 45° to the electron beam, so the projected green dot can be observed from the side[112]

- TH

Compagnie Française Thomson-Houston:

- TH9503 – Scripticon, a character generator monoscope for text mode video rendering in early computer monitors, with a square target having letters, digits and symbols patterned on it in an (optionally customer-supplied) 8x8 array. An electron beam selects and scans a character, both by appropriate magnetic deflection, and generates an analog video signal;[113] cf. 4560, CK1414

- TM

"Loupiote" – a TM tube

E.C.&A. Grammont and Compagnie des Lampes (1888):

- TM – Vacuum triode for amplification and detection of radio signals, developed in France and made since 1915.[114][115][116][117] It became the standard receiving and amplifying tube of the Entente countries during World War I, and the first mass-produced radio tube. TM's production volume in France alone is estimated at 1.1 million units; in addition, the production of TM and/or improved versions was started in the UK (Marconi–Osram R tube), the Netherlands (Philips E tube), the United States and the Soviet Union (R-5, Russian: Р-5).(ru)

The TM was developed in 1914–15 by the French military telecommunications service Télégraphie Militaire on the initiative of their technical director Gustave-Auguste Ferrié. He and his assistant, physicist Henri Abraham, visited the American laboratories on a number of occasions and were aware of the works of Lee de Forest, Reginald A. Fessenden and Irving Langmuir. They knew that de Forest's Audion and Henry Round's British tube were unreliable and imperfect, and Langmuir's Pliotron was too complex for mass production. They also knew about the latest German developments: Soon after the outbreak of the war, Ferrié received extensive information from a former Telefunken employee, the Frenchman Paul Pichon, who, upon return from a mission from his German employer to gather samples of the latest triodes from the USA, had to surrender himself and the samples to the French. The samples Pichon brought performed poorly due to insufficient vacuum. Following the ideas of Langmuir, Ferrié required the industry to guarantee a high vacuum in series production.

In October 1914, Ferrié, Abraham and François Péri from the radiotelegraph centre in Lyon/La-Doua(fr) went to the light bulb department of Société des Téléphones E.C.&Alexandre Grammont in Lyon to develop with them a triode suitable for mass production. The first prototypes, mere copies of de Forest's Audion, proved to be unreliable and unstable; the next ones were rejected for being too complex. Only the fourth prototype developed in December 1914, with a vertical coaxial system, an Edison screw lamp base for the filament and additional side terminals for anode and grid, was deemed suitable for series production, which started in February 1915 and stopped in October 1915 when it became clear that the vertical structure of "Abraham's Lamp" was too fragile and too many tubes were damaged during transport. Ferrié asked Péri to resolve the problem, and two days later Péri and Jacques Biguet came up with a horizontal coaxial system on the latest four-pin type "A" base. The series production of the Péri/Biguet tubes, named TM after Ferrié's service unit, began in November 1915 under Grammont's Radio Fotos brand; this variant became highly successful, and when demand started to exceed Grammont's production capacity, Compagnie des Lampes (1888) in Ivry-sur-Seine also started production under their Métal brand. Ferrié and Abraham were nominated for the 1916 Nobel Prize in Physics for their work in the field of radio communications.

The TM is a cylindrical/coaxial triode; the directly-heated cathode is a filament made of pure tungsten with a diameter of 60 μm, the anode is a nickel cylinder with a diameter of 10 mm and a length of 15 mm. The dimensions and material of the grid depend on the place of production – the Grammont plant in Lyon used molybdenum wire, the CdL plant in Ivry-sur-Seine used nickel. The diameter of the grid spiral is 4 resp. 4.5 mm. The filament required 4 V and 700 mA to bring it up to white heat; the bright glow prompted Grammont in 1923 to start producing TM tubes with dark blue glass envelopes to protect the eyes of radio operators from the blinding glare, and hide the harmless, but unsightly plaque of metal particles inevitably deposited on the inner wall of the bulb while evacuating during production – but also prevented the triodes' previous, secondary use as light sources, which had earned them their nickname Loupiote ("little lamp").

The TM could be used for their intended purpose, amplifying and detecting signals in radio receivers, or as power oscillators in low-power radio transmitters, and also, by paralleling of several tubes, as AF power amplifiers. The Soviet analogue of the TM, the triode R-5, could withstand anode voltages of up to 500...800 V, and was able to deliver a power of up to 1 W in Class-C mode, but only 40 mW in Class-A mode. A typical single-TM radio receiver of World War I ran at Ua=40 V, Ug=0 V, Ia≈2 mA, gm=400 μS, Ri=25 kΩ, μ=10. With an anode voltage of 160 V and a grid bias of -2 V, the anode current was 3...6 mA, while the reverse grid current reached 1 μA.[118]

The problem of TM tubes was their short service life of 100 hours maximum – if the tube was manufactured in strict accordance with the specifications. In wartime, this was not always possible; due to raw materials supply problems, plants sometimes had to use substandard materials. Such tubes were marked with a cross; they differed from the standard by a higher noise level and were prone to catastrophic failures due to cracks in the glass envelope.

Media related to French triode "Loupiote" at Wikimedia Commons

- TT

Bendix:

- TT8, TT9, TT13, TT15, TT17, TT18, TT20, TT21, TT22 – Chronotron, integrating, balanced-bridge hot-wire/PTC time delay devices

Marconi-Osram Valve Company:

- TT11 – Low power VHF transmitting beam tetrode

- TT21 – RF power beam-tetrode derived from KT88

- TT100 – RF power beam-tetrode

- V

- VHT

Ferranti:

- VHT1 – Pentagrid converter, 1933

- VS

Ericsson Telephones Limited:

- VS10G – Trochotron, an electron-beam decade counter tube

- VS10G-M – VS10G with a magnetic shield

- VS10H – High-current trochotron

- VS10K – Low-voltage trochotron

- List of other number tubes

- 1

- 175HQ – Ultra high reliability pentode for use in long-haul submarine communications cable repeaters[119]

- 1600s

- 1602 – Directly heated triode used for A.F. amplification with low microphonics. 7.5 volt heater/filament. 12 watts of A.F. operating in Class-A. 15 watts of low R.F. operating in Class-C. Similar to type 10.

- 1603 – Indirectly heated pentode used for A.F. amplification with low microphonics. Similar to types 6U7, 57, 6D6 and 6C6. UX6 Base.

- 1608 – Directly heated triode giving 20 watts at up to 45 MHz. 2.5 volt heater/filament. UX base.

- 1609 – Directly heated pentode used for A.F. amplification with low microphonics. American 5-Pin(UY)base.

- 1610 – Directly heated pentode specially designed for use as a crystal oscillator. 2.5 volt heater/filament, American 5-Pin base.

- 1612 – Pentagrid converter; low-microphonics version of type 6L7. Both control grids (1 and 3) are sharp-cutoff.

- 1619 – Beam Power Tetrode, similar to 6L6 with directly heated filament, common in World War II battle tank transmitters.

- 1624, 1625 – Very similar to the 807, but with different heater voltage

- 1626 – RF triode, very similar to 6J5 but with 12.6 volt filament

- 1629 – Tuning indicator tube with DC amplifier triode unit

- 1630 – Indirectly heated, orbital-beam, secondary-emission, 12-pin Jumbo Acorn-type UHF hexode

- 1633 – Dual triode, equivalent to 6SN7 with 26 volt filament (World War II aircraft use)

- 1635 – Indirectly heated, 10.4 W dual AF power triode, 8-pin base

- 1636 – Secondary emission UHF beam deflection tube, used as a balanced mixer up to 600 MHz[120]

- 1650 – High-altitude version of the 955 Acorn-type triode

- 1680 – Dual-control heptode for use as a NAND gate in a coincidence circuit in IBM computers, 6BE6/EK90 with a sharp-cutoff grid no.3

- 2

- 24B1 – Trigatron

- 24B9 – Trigatron

- 29C1 – Directly heated saturated-emission diode; acts as a heating current-controlled, variable series resistor in voltage/current stabilizer circuits.

- 200s

- 203A – 40 W, Directly heated RF transmitter power triode, 4-pin base, anode on top cap

- 204A – 450 W, Directly heated RF transmitter power triode, 3-pin base, anode on top cap

- 205D – 1 W, Directly heated AF or modulator power triode, 4-pin base

- 207 – 22.5 kW, Water-cooled, directly heated RF transmitter power triode

- 210T – Directly heated RF transmitter power triode, 4-pin base, similar to type 10 triode with an isolantite base

- 210DET – Cossor directly heated, 2 volts, special detector

- 210HF – Cossor, directly heated, 2 volts, triode

- 210HL – Cossor, directly heated, 2 volts, triode

- 210LF – Cossor, directly heated, 2 volts, triode

- 210PG – Cossor, directly heated, 2 volts, variable-mu pentagrid

- 210RC – Cossor, directly heated, 2 volts, very high impedance triode

- 210SPT – Cossor, directly heated, 2 volts, HF pentode

- 210VPT – Cossor, directly heated, 2 volts, HF variable-mu shielded pentode

- 211 – 260 W, Directly heated AF or modulator power triode now favored by audiophiles; 4-pin base

- 212E – 250 W, Directly heated RF transmitter power triode, 4-pin base

- 215P – Directly heated AF power triode

- 220B – 5 kW, Water-cooled, directly heated AF/modulator power triode

- 228 – 2.5 kW, Directly heated RF/AF power triode

- 230XP – Directly heated power triode

- 232C – 8.5 kW, Water-cooled, directly heated RF transmitter power triode

- 236A – 12 kW, Water-cooled, directly heated RF transmitter power triode

- 240B – Directly heated dual AF power triode

- 241B – 300 W, Directly heated AF/modulator power triode, 3-pin base, anode on top cap

- 242A – Directly heated AF/modulator power triode, 4-pin base

- 250TH – 1.1 kW, Directly heated AF/modulator power triode, 4-pin base, anode on top cap

- 254A – 14 W, Directly heated RF transmitter power triode, 4-pin base, anode on top cap

- 261A – 170 W, Directly heated AF/modulator power triode, 4-pin base

- 268A – 20 W, Directly heated power triode, 4-pin base, anode on top cap

- 270A – 585 W, Directly heated AF/RF power triode, 4-pin base, anode on top cap

- 275A – 3 W, Directly heated AF power triode, 4-pin base

- 276A – 170 W, Directly heated AF/RF power triode, 4-pin base

- 279A – 1.75 kW, Directly heated AF/RF power triode

- 295A – 125 W, Directly heated AF/RF power triode, 4-pin base

- 298A – 5 kW, Water-cooled, directly heated power triode

- 3

- 300s

- 300B – 40 watt directly heated power triode, 4-pin base

- 316A = VT191 – Directly heated Doorknob-type UHF power triode up to 750 MHz

- 322 – Oil can-type disk-seal UHF clipper power diode, 800 VPIV, 15 W, 1500 MHz

- 328 – Tungar bulb, a low-voltage, gas-filled, full wave rectifier for charging 12V lead-acid batteries at 1.3 A

- 368A – Directly heated Doorknob UHF power triode, graphite anode, up to 1.7 GHz[121]

- 388A – Directly heated Doorknob UHF power triode, graphite anode, up to 1.7 GHz[121]

- 4

- 4XP – Directly heated power triode

- 41MP – Indirectly heated power triode

- 45 (CX345) – Directly heated power triode

- 400s

- 402P – Indirectly heated power triode, 7-pin base

- 416B – Planar SHF power triode, 500 mW output at 4 GHz

- 416D – Planar SHF power triode with BeO spacers, 5 W output at 4 GHz

- 446A – Early Lighthouse UHF triode, 10 dB noise figure at 1 GHz

- 450TH – Early Eimac high-mu power triode, 450 watt plate dissipation to 40 MHz[122]

- 455A – Ultra high reliability pentode for use in submarine communications cable repeaters[123]

- 5

- 5BP4 – Five-inch CRT used in pre-World War II television receivers, such as the RCA TRK-5 and in early radars such as the SCR-268 and SCR-270.[124]

- 5CEP11 (blue, short persistence); 10VP15, 5AKP15, 5DKP15, 5ZP15 (blue-green, extremely short); 5BNP16, 5CEP16, 5DKP16, 5ZP16 (violet/near-ultraviolet, very short); 5AKP24, 5AUP24, 5DKP24, 5ZP24 (green, short); 131QP55 (blue-green, very short); 131QP56 (blue-violet, very short) – CRT-type flying-spot scanners for use in a telecine

- 500s

- 527 – High-mu power triode up to 900 W

- 559 – Lighthouse-type disk-seal UHF diode

- 592 = 3-200A3 – Medium-mu power triode up to 200 W, 150 MHz

- 6

- 6P10 – Ultra high reliability pentode for use in short-haul submarine communications cable repeaters[119]

- 6P12 – Ultra high reliability pentode for use in long-haul submarine communications cable repeaters[119]

- 7

- 7JP1 – Monochrome cathode ray tube for use in early postwar oscilloscopes. Electrostatic deflection, P1 green, short-persistence phosphor, 7" screen.

- 7JP4 – Monochrome cathode ray tube common in early postwar TV receivers. Electrostatic deflection, P4 white, medium-persistence phosphor, 7" screen.

- 7JP7 – Monochrome cathode ray tube for use in early postwar radar displays. Electrostatic deflection, P7 blue-white, long-persistence phosphor, 5½" screen.

- 700s

- 703A – Directly heated Doorknob UHF power triode up to 1.5 GHz

- 713A – Indirectly heated Doorknob UHF power pentode

- 717A (CV3594, VT269) – Indirectly heated Doorknob UHF power pentode

- 8

- 800s

- 800 – Directly heated V.H.F. power triode, giving 35 watts up to 60 MHz and 18 watts at 180 MHz. American 4-Pin(UX)base with side locating pin.

- 801 – Directly heated power triode, used in pairs in Class-B in A.M. modulation sections of transmitters giving up to 45 watts of power at 60 MHz and 22 watts at 120 MHz.

- 802 – Indirectly heated H.F. power pentode, giving 8 watts up to 30 MHz and 4 watts at 110 MHz.

- 803 – Directly heated H.F. power pentode, giving 50 watts up to 20 MHz and 25 watts at 70 MHz.

- 804 – Directly heated H.F. power pentode, giving 20 watts up to 15 MHz and 10 watts at 10 MHz.

- 805 – Directly heated H.F. high-mu triode, giving 140 watts up to 30 MHz and 70 watts at 85 MHz.

- 806 – Directly heated H.F. high-mu triode, giving 390 watts up to 30 MHz 195 watts at 100 MHz.

- 807 – Indirectly heated H.F. beam power tetrode, giving 25 watts up to 30 MHz and 12 watts at 125 MHz. A variation of type 6L6 originally designed as a Class-C transmitter tube. Later used in pairs as push-pull outputs for high-wattage Class-AB2 audio amplifiers. Also used as a horizontal output tube in early TV receivers. One of the first commercial tubes that used the top cap to connect the anode (instead of the control grid) to the circuit.

- 808 – Directly heated H.F. high-mu triode, giving 140 watts up to 30 MHz and 70 watts at 130 MHz.

- 809 – Directly heated H.F. high-mu triode, giving 55 watts up to 27 MHz and 30 watts at 100 MHz.

- 810 – Directly heated H.F. triode, 10 volt filament and Zirconium Carbide anode. Base fits R.C.A. UT-541A Socket.

- 811A – Directly heated H.F. triode, 6.3 volt filament, 88 watts

- 813 – Beam Power Tetrode possessing about 5 times the Anode dissipation of an 807.

- 814 – A directly heated Beam Power Tetrode giving about 130 watts at 30 MHz and 65 watts at 100 MHz operating in Class-C.

- 815 – An indirectly heated dual Pentode. International Octal, (IO), base.

- 825 – First commercially available klystrode, a VHF/UHF linear-beam transmitting tube, similar to a klystron

- 829 – A dual indirectly heated beam power tetrode. Two 6.3 volt heaters sharing a common tap.

- 830 – A directly heated triode giving about 50 watts at 15 MHz and 7.5 watts at 60 MHz operating in Class-C.

- 831 – A directly heated triode giving about 400 watts at 20 MHz and 200 watts at 60 MHz operating in Class-C. 11 volt heater/filament.

- 833 – A larger directly heated high-mu triode giving about 1 kW at 30 MHz and 500 watts at 45 MHz operating in Class-C. Usable up to 100 MHz at reduced power, (400 W). 10 volt heater/filament drawing 10 A. The anode of this device is fabricated from tantalum. Anode current of 800 mA with an anode voltage of 3 kV and grid voltage of zero. Anode current of 4.3 A at a voltage of 750 with 350 volt on the grid. Uses two-part R.C.A socket assembly UT-103.[125]

- 833A – Improved 833.

- 834 – A directly heated triode giving 58 watts at 100 MHz and 25 watts at 350 MHz operating in Class-C. 7.5 volt heater/filament. Fitted with an American 4-Pin, (UX4), base with side locating pin.

- 836 – An indirectly heated high vacuum rectifier with a peak inverse voltage of 5 kV and peak anode current of 1 ampere. 2.5 volt heater.

- 837 – An indirectly heated pentode giving 11 watts at 20 MHz and 5 watts at 80 MHz. operating in Class-C. 12.6 volt heater.

- 838 – A directly heated triode giving about 100 watts at 30 MHz operating in Class-C. 10 volt heater/filament.

- 841 – A directly heated high-mu triode giving about 10 watts at 6 MHz and 5 watts at 170 MHz operating in Class-C. 7.5 volt heater/filament.

- 842 – A directly heated triode giving about 3 watts at 6 MHz operating in Class-C. 7.5 volt heater/filament.

- 843 – An indirectly heated tetrode giving gain at 6 MHz and usable up to 200 MHz operating in Class-C. 2.5 volt heater/filament.

- 844 – A directly heated triode giving gain at 6 MHz and usable up to 155 MHz operating in Class-C. 2.5 volt heater/filament.

- 845 – A directly heated triode giving up to 24 watts if undistorted power in Class-A at audio frequency with an anode voltage of 1250. 11 volt heater/filament.

- 849 – A directly heated triode giving gain at 3 MHz operating in Class-C. Two 849s, working in push-pull Class-B are capable of delivering 1.1 kW of audio output with an anode voltage of 3 kV. Usable up to 30 MHz. 11 volt filament/heater.

- 850 – A directly heated tetrode giving 120 watts of power gain up to 13 MHz and 50 watts at 100 MHz, operating in Class-C. 10 volt heater/filament.

- 851 – A directly heated triode giving 1.5 kW of power up to 3 MHz operating in Class-C. 11 volt heater/filament.

- 852 – A directly heated triode giving 75 W of power up to 30 MHz operating in Class-C. 10 volt heater/filament.

- 857B – Large mercury-vapor rectifier used in 50 kW class broadcast transmitters. 22 kV anode voltage, 10 A anode current. Filament 5 V @ 30 A

- 860 – A directly heated tetrode giving 105 W of power up to 30 MHz and 50 watts at 120 MHz operating in Class-C. 10 volt heater/filament.

- 861 – A directly heated triode giving 400 W of power up to 20 MHz and 200 watts at 60 MHz operating in Class-C. 11 volt heater/filament.

- 862 – Large water-cooled triode for broadcast/industrial applications. Used in experimental 500 kW transmitter at WLW.

- 864 – A directly heated general-purpose, low-microphonics triode with a maximum anode voltage of 135 volts and anode current of 3.5 mA. 1.1 volt heater/filament.

- 865 – A directly heated tetrode giving 30 W of power up to 15 MHz 15 watts at 70 MHz operating in Class-C. 11 volt heater/filament.

- 866

- 866A

– A mercury-vapor rectifier with a peak inverse voltage of 5 kV and peak anode current of 1 ampere. Average anode current, 250 mA, forward drop, 15 volt. Heater voltage and current, 2.5 at 5 A. American 4-Pin(UX) base.

- 866A – Improved 866 with a peak inverse voltage of 10 kV and a forward drop of 10 volt.

- 872 – A mercury-vapor rectifier with a peak inverse voltage of 5 kV and peak anode current of 5 amperes. Average anode current, 1250 mA, forward drop, 15 volt. Heater voltage, 5.0 at 10 A. Base fits R.C.A. UT-541A Socket.

- 872A – Improved 872 with a peak inverse voltage of 10 kV, a forward drop of 10 volt and a heater current of 6.25 A.

- 879 – A high vacuum rectifier with a peak inverse voltage of ca. 15 kV and peak anode current of ca. 5 mA. 2.5 volt heater and American 4-Pin, (UX) base. Used as half wave rectifier for high voltage cathode ray tube supplies. Similar to type 2X2.

- 884 – An indirectly heated triode thyratron. 6.3 volt heater/filament, International Octal, (IO), base. Electrically similar to type 885. Once commonly used as a sawtooth horizontal sweep waveform generator in recurrent-sweep oscilloscopes. Marketed by DuMont under the type number 6Q5.

- 885 – An indirectly heated triode thyratron. 2.5 volt heater/filament, American 5-Pin (UY) base. Otherwise similar to type 884.

- 898 – Large water-cooled triode for broadcast/industrial applications. Updated version of 862, with 3 phase filament structure.

- 9

- 900s

- 934 – Vacuum Phototube, spectral S4 response (maximum sensitivity at 400±50 nm), 3-pin Small-Shell Peewee base

- 935 – Vacuum Phototube, spectral S5 response (maximum sensitivity at 340±50 nm), 4-pin octal base

- 950 – Power pentode with directly heated cathode, used in storage battery home radios with 2.0 volt filament supply. Similar to types 1F4 and 1J5G

- 951 – Sharp-cutoff pentode with directly heated cathode, used in storage battery home radios with 2.0 volt filament supply. Similar to type 1B4P

- 953 – Acorn-type UHF diode; 6.3 V heater

- 954 (4672/E1F) – Indirectly heated Acorn-type sharp-cutoff pentode giving gains of 2...3 up to 300 MHz operating in Class-A and usable up to 600 MHz with careful stage design; 6.3 V heater

- 955 (4671/E1C) – Indirectly heated Acorn-type triode giving a power of 135 mW up to 600 MHz operating in Class-A and 500 mW in Class-C with careful stage design; 6.3 V heater

- 956 (4695/E2F) – Indirectly heated Acorn-type remote-cutoff pentode giving gains of 3...4 up to 600 MHz operating in Class-A with careful stage design; 6.3 V heater

- 957 (D1C) – Directly heated Acorn-type UHF receiving triode; 1.25 V filament for portable equipment

- 958 (D2C) – Directly heated Acorn-type UHF transmitting triode with dual, paralleled 1.25 V filaments for increased emission, for portable equipment

- 958A – 958 with tightened emission specs

- 959 (D3F) – Directly heated Acorn-type sharp-cutoff UHF pentode; 1.25 V filament for portable equipment

- 991 – 60-Volts Voltage reference, T4½ lightbulb with dual-contact, bayonet candelabra mount

- 9000s

- 9001 – 954 with a miniature 7-pin base

- 9002 – 955 with a miniature 7-pin base

- 9003 – 956 with a miniature 7-pin base

- 9004 – Acorn UHF diode

- 9005 – Acorn UHF diode with a 3.6 V heater

- 9006 – Detector diode with a miniature 7-pin base

- List of tubes used in 1920s and 1930s radio receivers

[126]

- Tubes with directly heated cathodes

Used with AC, DC or home-based storage battery power supplies (1927–31)

- With 1.1 Volt DC filaments

Used in 1920s home radios. Filaments powered by 1.5 volt dry cells, anodes powered by storage batteries.

- WD-11 – triode/detector

- With 2.0 Volts DC filaments

Used in 1930s home radios powered by storage batteries.

- 19 – Dual power triode – also used in farm radios with 6-volt vibrator power supplies. Early version of octal type 1J6G.

- 20 – Power triode – Early versions numbered UX-120.

- 22 – Sharp-cutoff tetrode – Early versions numbered UX-222 or CX-322.

- 25S – Dual detector diode, medium-mu triode. Identical to type 1B5. Usually numbered 1B5/25S.

- 30 – Medium-mu triode, An upgraded version of type 01-A – Early versions numbered RCA-230 or CX-330. Can also be used as a power triode. The type 30 was popular amongst amateurs of the day. Early UX4 based version of octal type 1H4G.

- 31 – Power triode, UX4 based – Early versions numbered RCA-231 or CX-331.

- 32 – Sharp-cutoff tetrode – Early versions numbered RCA-232 or CX-332.

- 33 – Power pentode – Early versions numbered RCA-233 or C-333.

- 34 – Remote-cutoff tetrode – Early versions numbered RCA-234 or CX-334.

- 49 – Dual-grid power triode, similar to type 46

- With 3.3 Volts DC filaments

Used in 1920s home radios powered by dry cells (filaments) and storage batteries (B-plus voltage).

- V99 – Low-mu triode. Except for stub-pin bayonet base and pinout, electronically similar to X99

- X99 – Similar to V99, but with standard pins and different basing arrangement (pinout).

- With 5.0 Volts DC filaments

Used in 1920s home radios powered by storage batteries.

- 00-A – Detector triode with a trace of argon. "00-A" is the number used in most tube manuals. Numbers for earlier versions include UX-200-A and CX-300-A (long pins, push-in socket) and UV-200-A (stub pins, bayonet socket).

- 01-A – All-purpose low-mu triode, used as RF amplifier, detector, AF amplifier and power triode. The most popular tube of the 1920s. "01-A" is the number used for replacements manufactured after 1930 and in tube manuals. Numbers for early versions include UX-201-A and CX-301-A (long pins, push in socket) and UV-201-A (stub pins, bayonet socket).

Note: There were four tubes in the "01" series, each with different current ratings for their filaments. Type 01-A was the most commonly used.

Types UV 201 and UX 201 – 1.0 ampere

Type 01-A (UV 201-A, UX 201-A, etc.) – 250 milliampere

Type UX 201-B – 125 milliampere

Type UX 201-C – 60 milliampere

- 12-A – Medium-mu triode, often used as detector, audio driver or audio output, but not as an RF amplifier. This type is listed in tube manuals after 1930 for replacements purposes. Also referred to as type 112-A. Many early versions are marked UX-112-A or CX-112-A.

- 40 – Medium-mu triode – Early versions numbered UX-240. Introduced in 1927, this was an upgraded version of the "01" series. The "01" series had an amplification factor of 8, while type 40 had an amplification factor of 30. (By comparison, the two AC triodes introduced in the same time period – types 26 and 27 – had amplification factors of 8.3 and 9, respectively.) Because this was the highest-amplification triode available, advertising literature of the time lists it as a high-mu triode, although it is now classified as a medium-mu triode. Type 40 had the highest amplification factor of any triode until the introduction in 1932 of diode/triode complex type 2A6, which had an amplification factor of 100. It also had the highest amplification factor of any DC filament triode until the introduction in 1939 of complementary diode/triode complex types 1H5GT (octal) and 1LH4 (loctal), which both had amplification factors of 65.

- Other directly DC-heated tubes

- 2HF – Tube-based "integrated circuit" with 2 tetrodes and passive components in the same envelope

- 3NF – Tube-based "integrated circuit" with 3 triodes and passive components in the same envelope. 4V heater

- WG38 – Tube-based "integrated circuit" with 2 pentodes, a triode and passive components in the same envelope

- Directly AC-heated power tubes

- 10 – Power triode – Early versions numbered UX-210 or CX-310.

- 26 – Medium-mu triode, used in early AC radio receivers manufactured in the late 1920s. Used as an RF or AF amplifier, but not as an dectetor or power output tube – Early versions numbered UX-226 or CX-326.

- 45 – Power Triode – Early versions numbered UX-245 or CX-345.

- 46 – Dual grid power triode – Grids 1 and 2 connected together for use as push-pull Class-B outputs, Grid 2 and anode connected together for use as single-tube audio driver.

- 47 – Power pentode – Early versions numbered RCA-247 or C-347.

- 50 – Power triode – Early versions numbered UX-250 or CX-350.

- 71-A – Power triode – This type listed in tube manuals after 1930 for replacements purposes. Also referred to as 171-A. Many early versions numbered as UX-171-A or CX-371-A.

- Directly AC-heated rectifier tubes

- 80 – Full-wave rectifier used in early power supplies or battery eliminators (electronically similar to 5Y3G) – Early versions numbered UX-280 or CX-380

- 81 – Half-wave rectifier – Early versions numbered UX-281 or CX-381.

- 82 – Full-wave mercury-vapor rectifier

- 83 – Full-wave mercury-vapor rectifier

- 83-V – High-vacuum version of type 83, Early UX4 based version of octal type 5V4G.

- Tubes with indirectly heated cathodes

- With DC heaters

- 15 – Sharp-cutoff pentode, used in farm radios, in autodyne circuits requiring a separate cathode.

- 48 – Power tetrode, used in 32-volt farm radios. When two are parallel-connected, they can operate with anode and screen voltages as low as 28 volt.

- For use with an AC heating transformer

Note: All have 2.5 volt heaters.

- 24 – Sharp-cutoff tetrode, UX5 based, Early versions numbered UY-224 and C-324

- 24-A – an upgraded version of type 24, see type 24 above. Early versions numbered UY-224A and C-324A

- 27 – Medium-mu triode, UX5 based, Early versions numbered UY-227 and C-327. The first North American tube with an indirectly heated cathode, which is necessary for detector circuits in AC powered tube radios.

- 35 – Remote-cutoff tetrode, UX5 based, (Commonly branded as 35/51). Early versions numbered UY-235 or C-335

- 51 – Similar to 35, see type 35 above. (Commonly branded as 35/51)

- 53 – Dual power triodes, Class-B, UX7 based, (Except for heater, electronically similar to 6A6 and octal based 6N7)

- 55 – Dual-diode, medium-mu triode, UX6 based, (Except for heater, electronically similar to type 85, and octal based 6V7G, but not to 75)

- 56 – Medium-mu triode, UX5 based, (Except for heater, electronically similar to 76, and octal based 6P5G)

- 57 – Sharp-cutoff pentode used in cabinet and mantel radio receivers, UX6 based, (Except for heater, electronically similar to 6C6 and octal based 6J7G, and somewhat similar to type 77)

- 58 – Remote-cutoff pentode, UX6 based, (Except for heater, electronically similar to 6D6 and octal based 6U7G, but not to 78)

- 59 – Power pentode, UX7 based.

- For use with AC/DC or vehicle-based storage-battery power supplies

Note: All have 6.3 volt heaters except type 43

- 1-V – Half-wave rectifier with indirectly heated cathode, UX4 based, (often branded as type 1V/6Z3). Early version was KR-1.

- 36 – Sharp-cutoff tetrode, UX5 based. Early versions numbered RCA-236 or C-336

- 37 – Medium-mu triode, UX5 based. Early versions numbered RCA-237 or C-337

- 38 – Power pentode, UX5 based. Early versions numbered RCA-238

- 39 – Remote-cutoff pentode, UX5 based (Commonly branded as 39/44).

- 41 – Power pentode, Early UX6 based version of octal type 6K6G, and loctal type 7B5.

- 42 – Power pentode, Early UX6 based version of octal type 6F6G, Except for heater, similar to types 2A5 and 18.

- 43 – Power pentode, Early UX6 based version of octal type 25A6G, Commonly used in AC/DC radios.

- 44 – Similar to type 39, see type 39 above. (Commonly branded as 39/44).

- 75 – Dual-diode, high-mu triode. Early UX6 based version of octal types 6B6G & 6SQ7GT, and loctal type 7B6, and 7-pin miniature type 6AV6. Also except for heater, electronically similar to 2A6.

- 76 – Medium-mu triode, Early UX5 based version of octal type 6P5G.

- 77 – Sharp-cutoff pentode, Early UX6 based version of octal type 6J7G.

- 78 – Remote-cutoff pentode, Early UX6 based version of octal type 6K7G.

- 79 – Dual power triode, Early UX6 based version of octal type 6Y7G.

- 84 – Full-wave rectifier with indirectly heated cathode, (often branded as type 84/6Z4). Early UX5 based version of octal type 6X5GT, and loctal 7Y4, and 7-pin miniature 6X4.

- 85 – Dual-diode, medium-mu triode. Early UX6 based version of octal type 6V7G, except for heater voltage similar to type 55. Also somewhat similar to octal type 6SR7GT and 7-pin miniature types 6BF6.

- 89 – Power pentode, UX6 based.

- Shielded tubes for Majestic radios

In the early 1930s, the Grigsby-Grunow Company – makers of Majestic brand radios – introduced the first American-made tubes to incorporate metal shields. These tubes had metal particles sprayed onto the glass envelope, copying a design common to European tubes of the time. Early types were shielded versions of tube types already in use. (The shield was connected to the cathode.) The Majestic numbers of these tube types, which are usually etched on the tube's base, have a "G" prefix (for Grigsby-Grunow) and an "S" suffix (for shielded). Later types incorporated an extra pin in the base so that the shield could be connected directly to the chassis.

Replacement versions from other manufacturers, such as Sylvania or General Electric, tend to incorporate the less expensive, form-fitting Goat brand shields that are cemented to the glass envelope.

Grigsby-Grunow did not shield rectifier tubes (except for type 6Y5 listed below) or power output tubes.

- Early types based on existing tubes. (Non-shielded versions may be used, but add-on shielding is recommended.)

- G-2A7-S – Pentagrid converter

- G-2B7-S – Semiremote-cutoff pentode, dual detector diode

- G-6A7-S – Pentagrid converter

- G-6B7-S – Semiremote-cutoff pentode, dual detector diode

- G-6F7-S – Remote-cutoff pentode, medium-mu triode

- G-25-S – Medium-mu triode, dual detector diode for 2.0 volt storage battery radios. Glass type 1B5/25S used for replacement.

- G-51-S – Remote-cutoff tetrode

- G-55-S – Medium-mu triode, dual detector diode

- G-56-S – Medium-mu triode

- G-56A-S – Medium-mu triode, original version of type 76, but with 400 milliampere heater. (Not to be confused with types 56 or G-56-S, which has a 2.5 volt, 1.0 ampere heater.)

- G-57-S – Sharp-cutoff pentode

- G-57A-S – Sharp-cutoff pentode, original version of type 6C6, but with 400 milliampere heater. (Not to be confused with types 57 or G-57-S, which has a 2.5 volt, 1.0 ampere heater.)

- G-58-S – Remote-cutoff pentode

- G-58A-S – Remote-cutoff pentode, original version of type 6D6, but with 400 milliampere heater. (Not to be confused with types 58 or G-58-S, which has a 2.5 volt, 1.0 ampere heater.)

- G-85-S – Similar to G-55-S, but with 6.3 volt heater.

- Later types

- 6C7 – Medium-mu triode, dual detector diode, similar to later octal types 6R7 and 6SR7. Seven pin base. (Shield to pin 3.)

- 6D7 – Sharp-cutoff pentode, identical to type 6C6, but with 7-pin base. (Shield to pin 5.)

- 6E7 – Remote-cutoff pentode, identical to type 6D6, but with 7-pin base. (Shield to pin 5.)

- 6Y5 – Dual rectifier diode, similar to type 84/6Z4, but with 6-pin base. (Shield to pin 2.)

- Other tubes unique to Majestic radios

- G-2-S and G-4-S – Dual detector diodes with common cathodes. The first detector diodes packaged in a separate tube. Forerunners of octal type 6H6. Spray-shielded. Both tubes have 2.5 volt heaters. G-2-S is larger and has a 1.75 ampere heater. Type G-4-S has a 1.0 ampere heater. Later Sylvania replacement type 2S/4S has a 1.35 ampere heater.

- 2Z2/G-84 – Half-wave rectifier diode with 2.5 volt indirectly heated cathode. A lower-voltage version of type 81. Not interchangeable with type 6Z4/84.

- 6Z5 – Full-wave rectifier, similar to types 6Z4/84 and 6X5, but with 12.6 volt center-tapped heater.

- Rarely used tubes

- 14 – Similar to 24-A but with a 14 volt, 300 milliampere heater. Used in Philco models 46 and 46E

- 17 – Similar to 27 but with a 14 volt, 300 milliampere heater. Used in Philco models 46 and 46E

- 18 – Similar to 2A5 and 42 but with a 14 volt, 300 milliampere heater. No known commercial use.

- 29 – Wunderlich detector. Known to have been manufactured by Sylvania.

- 52 – Dual grid power triode similar to types 46 and 49. Has 6.3 volt filament. Most commonly used in early car radios.

- 64 – Sharp-cutoff tetrode (Except for 400 milliampere heater, similar to 36)

- 65 – Remote-cutoff pentode (Except for 400 milliampere heater, similar to 39)

- 67 – Medium-mu triode (Except for 400 milliampere heater, similar 37)

- 68 – Power pentode (Except for 400 milliampere heater, similar to 38)

- 69 – Wunderlich detector

- 70 – Wunderlich detector used in Mission Bell model 19 car radio. Listed in early Philco tube lists.

- 90 – Wunderlich detector

- 92 – Wunderlich detector

- 95 – Original number of type 2A5

- 181 – Power triode

- 182-B – Similar to 482-B below.

- 183 – Similar to 483 below.

- 213 – Early version of type 80 – Often numbered UX-213

- 216 – Early version of type 81 – Often numbered as UX-216-B

- 482-B – Power triode with directly heated cathode. Used in Sparton AC radios, circa 1929. Replacements often numbered 182-B/482-B. Similar to type 71-A, but with higher anode voltage.

- 483 – Power triode with directly heated cathode. Used in Sparton AC radios, circa 1929. Replacements often numbered 183/483. Similar to type 45, but with a 5.0 volt, 1.25 ampere heater.

- 485 – Medium-mu triode with indirectly heated cathode. Used in Sparton AC radios, circa 1929. Similar to types 56 and 76, but with a 3.0 volt, 1.25 ampere heater, and lower anode voltage.

- Lettered loctal tubes used in Philco radios

- FM1000 – Unusual pentagrid for use as oscillator and coincidence-type phase detector in a PLL FM quadrature detector. The anode signal is loosely coupled into the oscillator tank and pulls it to stay quadrature-phase-locked with the IF; manufactured by Sylvania and used in Philco AM/FM radios of the late 1940s and early 1950s. Predecessor of the nonode approach

- XXB – Medium-mu twin triode, also numbered 3C6/XXB

- XXD – Medium-mu twin triode, also numbered 14AF7/XXD

- XXFM – High-mu triode, twin diode (one shares its cathode with the triode, one with separate cathode), also numbered 7X7/XXFM

- XXL – Medium-mu triode, also numbered 7A4/XXL

- Russian tubes

Media related to Soviet vacuum tubes at Wikimedia Commons

Media related to Soviet gas discharge tubes at Wikimedia Commons

- List of standard tubes, with American equivalents

Note: Typecode explained above.

- 6J1J 6Ж1Ж (954) – Indirectly heated Acorn-type sharp-cutoff pentode, 6.3 V heater

- 6K1J 6К1Ж (956) – Indirectly heated Acorn-type remote-cutoff pentode, 6.3 V heater

- 6L1P 6Л1П – Nonode for FM quadrature detection

- 6N1P 6Н1П – Dual triode, similar to 6DJ8/ECC88

- 6N2P 6Н2П – Dual triode, similar to 12AX7/ECC83

- 6N3P 6Н3П (2C51) – Dual triode

- 6N8S 6Н8С (6SN7/ECC33) – Dual triode

- 6N9S 6Н9С (6SL7) – Dual triode

- 6N13S 6Н13С (6AS7G) – Dual power triode

- 6N14P 6Н14П – Dual RF/VHF triode, similar to ECC84/6CW7

- 6N23P 6Н23П (6DJ8/ECC88) – Dual triode

- 6N24P 6Н24П (ECC89/6FC7/6ES8) – Dual RF/VHF triode for cascode amps

- 6P1P 6П1П – Power pentode, similar to 6AQ5/EL90

- 6P3S 6П3С – Beam-power tetrode, similar to 6L6GB

- 6P3S-E 6П3С-Е – Beam-power tetrode, similar to 5881/6L6WGB

- 6P6S 6П6С (6V6) – Beam-power tetrode

- 6P14P 6П14П (6BQ5/EL84) – Power pentode

- 6S1J 6С1Ж (955) – Indirectly heated Acorn-type triode, 6.3 V heater

- 6S19P 6С19П – Output triode

- List of professional tubes

Note: Typecode explained above.

- V1-0.15/55 В1-0.15/55 – 55 kV, 150 mA Half-wave rectifier

- VI1-5/20 ВИ1-5/20 – 20 kV, 5 A Half-wave pulse rectifier

- G-807 Г-807 – Shortwave transmitter tube (the Russian 807 analogue).

- GI-7B ГИ-7Б – Impulse tube

- GM-70 ГМ-70 – Modulator tube

- GS-31B ГС-31Б – UHF transmitter tube

- GU-29 ГУ-29 – VHF transmitter tube

- GU-50 ГУ-50 – VHF transmitter pentode, similar to the German LS-50 (no direct U.S. equivalent)

- GU-78B ГУ-78Б – VHF transmitter tetrode

- I3-70-0.8A И3-70-0.8 – 800 V, 70 A Ignitron

- I3-200-1.5A И3-200-1.5 – 1.5 kV, 200 A Ignitron

- SG203K СГ203К – 82 V Voltage reference

- SG204K СГ204К – 164 V Voltage reference

- TGI1-270/12 ТГИ1-270/12 – 12 kV, 270 A Hydrogen thyratron

- List of indicator tubes

- IN-33 ИН-33 – Neon-filled, planar, dual 105-segment linear glow-transfer bar graph display tube with three cathode strings, for use in VU meters etc.; similar to PBG16101

| ⚫ | ⚫ | ⚫ | ⚫ |
| --- | --- | --- | --- |
| ⚫ | ⚫ | ⚫ | ⚫ |
| ⚫ | ⚫ | ⚫ | ⚫ |
| ⚫ | ⚫ | ⚫ | ⚫ |

- ITM2-M ИТМ2-М – Four-color phosphored-thyratron latching pixel; 4x4 array of 4 subminiature dual-starter luminiscent thyratrons each for the colors red, yellow, green and blue (thus, 5 intensities per color yields 54 = 625 colors), 4x4 matrix of 10-volts sensitive starter electrodes, cubic envelope for easy stacking in both axes, 12-pin all-glass pigtaled,[127] similar to today's RGBA LEDs

- ITS1A ИТС1А – Green phosphored-thyratron latching seven-segment display, no decimal point, 5-volts sensitive starter electrodes, all-glass pigtaled, rectangular envelope for easy stacking in both axes

- MTX-90 МТХ-90 – Small neon-filled thyratron for use as a latching single-dot indicator, top-viewing, top of envelope acts as a magnifier, all-glass pigtaled, comes with a blob of solder on the end of each wire for rapid installing, like today's ball grid arrays

- List of other tubes

- LP-4 ЛП-4 – Linear trochotron, 26-pin Acorn-type all-glass pigtaled,[128]

- Compagnie des Lampes (1921, "French Mazda") and Mazda-Belvu

Not to be confused with Compagnie des Lampes (1888, see above) nor with British Mazda (see above).

The 1921 incarnation of La Compagnie des Lampes (since 1953 as Lampe Mazda) made light bulbs and electronic tubes under the French Mazda brand. Many of their tubes were also available from Compagnie Industrielle Française des Tubes Electroniques (CIFTE)[129] under their Mazda-Belvu brand, which otherwise used mostly EIA, RETMA and Mullard–Philips tube designations.

Examples:

Before 1949:[130]

- 1883 – Indirectly-heated, 350 V/125 mA full-wave rectifier, 5 V/1.6 A heater

- 2XM600 – Directly-heated, 10 kV/250 mA half-wave mercury-vapor rectifier, 2.5 V/5 A heater

- 4Y25 = 807 – Indirectly-heated beam tetrode

- RETMA tube 6H8G

- RCA-800 tubes 879, 884

Since 1949 with a fire pot logo:[131]

- RMA tube 2E30

- 3T20 – Directly-heated power triode, graphite anode

- 3T100 – Directly-heated power triode, graphite anode

- 4Y50 – Indirectly-heated beam tetrode

- E1 – Electrometer tetrode

- E2 – Dual electrometer tetrode

- ST130 – 130 V Neon-filled voltage reference

Since 1953 as LAMPE MAZDA:[132]

- RMA tube 2G21

- 4Y100 = 7745 – Dual beam tetrode

- RCA-800 tubes 829, 832

- 927 – Gas-filled phototube

- 929 – Vacuum phototube

- EIA tubes 6196, 6250

- E5 – Subminiature electrometer tetrode, all-glass pigtailed

Since 1959 with a Faravahar logo related to Ahura Mazda:[133]

- 3T50 – Directly-heated power triode, thoriated-tungsten filament, graphite anode

- 4Y75 – Directly-heated power triode

- RETMA tube 6K8

- 78A – Directly-heated, educational diode

- EIA tubes 7233, 7242, 7377, 8418

- E6 – Subminiature dual electrometer tetrode, all-glass pigtailed

- E7, E9 – Subminiature electrometer pentodes, all-glass pigtailed

- Mullard–Philips tubes ECF202, ECL802, ED501, EF816, EL503, EY81F, EY802, GY86, GY802, PY81F

- F7024A (Diode), F7024C (Triode), F7024E (Tetrode), F7024L (Pentode) – Set of 4 educational tubes

- F9116 – Electrometer tetrode

- K25000A1 – Directly-heated, 25 kV/70 mA half-wave rectifier, 2.5 V/9 A heater

- References and footnotes

- Specific items

  See "Decoding type numbers" ff.

    "GL-2H21 Phasitron data sheet" (PDF). General Electric. September 1945. Retrieved 25 December 2016. (as JPGs) • RMA Release #486, 25 April 1946

    Adler, Robert (January 1947). "A New System of Frequency Modulation" (PDF). Institute of Radio Engineers. Retrieved 25 December 2016.

    Rider, John. F.; Seymour D. Uslan (1948). "FM Transmission and Reception" (PDF). John F. Rider Publisher, Inc. pp. 130–135. Retrieved 25 December 2016.

    Dave Hershberger (W9GR): PHASITRON vacuum tube web page

    "Tungar bulb data manual" (PDF). General Electric. Retrieved 12 February 2016.

    "Eimac power grid tubes - Quick Reference Catalog 175" (PDF). Eitel McCullough. 1975. Retrieved 1 May 2017.

    "Preferred Types of Electron Tubes 1967" (PDF). Retrieved 17 May 2013.

    "European Type Designation Code System for Electronic Components" (PDF) (15 ed.). Pro-Electron, Brussels, Belgium. June 2008. Archived from the original (PDF) on 29 December 2013. Retrieved 25 December 2013.

    "Akumed Berlin" hearing-aid eyeglasses schematic

    "Philips Miniwatt 1938" (PDF). Retrieved 31 January 2016.

    "Tabelle der Heeres-Batterie-Spezialröhren" (PDF) (in German). Lorenz. Retrieved 21 December 2015.

    Miniwatt Technical Data, 6th Edition; 1958; Published by the "Miniwatt" Electronics Division of Philips Electrical Industries Pty. Limited, 20 Herbert Street, Artarmon, N,S,W., Australia

    "Miniwatt" Premium Quality and Special Purpose Tubes, Philips Electrical Industries Pty. Ltd., Australia, November 1957.

    "Cold cathode tubes ZnnnA". Retrieved 17 May 2013.

    "Les lampes" (PDF) (in French). Retrieved 1 May 2017.

    BW604 data sheet

    BW1010 data sheet

    "Belvu tubes electroniques, Licence R.C.A" (PDF) (in French). Retrieved 25 April 2017.

    "Vade-mecum ADZAM" (PDF) (in French). 1957. Retrieved 1 May 2017.

    "EEV Valve data book" (PDF). March 1966. Retrieved 1 May 2017.

    "Master Valve Guide" (PDF). Mullard. 1935. Retrieved 12 February 2016.

    "Philips radio-artikelen 1927" (PDF) (in Dutch). Retrieved 1 May 2017.p. 15

    Дроздов, К. И. (1948). "Справочник по западно-европейским приёмным лампам (West-European receiving tubes)" (PDF) (in Russian). Retrieved 1 May 2017.

    Georgescu, Aurel; Golea, Ion (1956). "Catalog De Tuburi Electronice" (PDF) (in Romanian). Editura Tehnikă Bukurești. Retrieved 10 September 2017.

    "TUNGSRAM ELECTRON TUBE NUMBERING SYSTEM" (PDF). 2004. Retrieved 1 May 2017.

    МЕТАЛЛИЧЕСКИЕ ЛАМПЫ (METAL TUBES)

    ЭНЦИКЛОПЕДИЯ ЛАМПОВОЙ РАДИОАППАРАТУРЫ (Encyclopedia of tubes for radio equipment)

    А.Л. Булыев; В.И. Галкин; В.А. Прохоренко (1982). "СПРАВОЧНИК ПО ЗЛЕКТРОВАКЧЧМНЫМ ПРИБОРАМ (HANDBOOK ON ELECTRONIC DEVICES)" (PDF) (in Russian). БЕЛАРУСЬ. pp. 10ff. Retrieved 1 May 2017.

    Ewert, Jürgen. "Vacuum Tube Numbering Schemes, Bases & Bulbs".

    HAYASHI, Koji; JAPAN, Ibaraki. "日本の真空管名称制度 (Tube Naming System Japan)" (in Japanese).

    HAYASHI, Koji; JAPAN, Ibaraki. "Gallery on Tubes/真空管展示室" (in Japanese).

    5J6 data sheet - this particular Tung-Sol datasheet contains a copy/paste error in the description where it cites 6J6's 450 mA heater current when it should read 5J6's 600 mA.

    Schematic for General Electric model F-40, a 1938 reflex radio using a 6B7.

    http://www.mif.pg.gda.pl/homepages/frank/sheets/191/1/12AB5.pdf

    "12AU7 data sheet" (PDF). STC. August 1950. Retrieved 1 May 2017.

    http://www.mif.pg.gda.pl/homepages/frank/sheets/127/1/12AV7.pdf

    http://www.mif.pg.gda.pl/homepages/frank/sheets/127/1/12AZ7.pdf

    http://www.mif.pg.gda.pl/homepages/frank/sheets/049/1/12BH7A.pdf

    RCA: Receiving Tube Manual RC21, p.360

    RCA: Receiving Tube Manual RC30, p.397

    http://www.ww2australia.gov.au/japadvance/boots.html

    "4560 Custom-built, 2" diameter, Electrostatic-Focus, Electrostatic-Deflection Monoscope Tubes For Use As Alphanumeric Character Generators data sheet" (PDF). RCA Electronic Components. May 1969. Retrieved 27 September 2017.

    "4598 Graphechon Tube data sheet" (PDF). RCA Electronic Components. February 1971. Retrieved 1 May 2017.

    "7539 Graphechon Tube data sheet" (PDF). RCA Electronic Components. March 1960. Retrieved 1 May 2017.

    "GEC 7828 Scan conversion tube data sheet" (PDF). General Electric Corporation. 10 April 1961. Retrieved 1 May 2017.

    "8087 Scan-Conversion Storage Tube data sheet" (PDF). Machlett Laboratories, Inc. 16 September 1963. Retrieved 1 May 2017.

    "Rauland 8098 Signal Storage Tube data sheet" (PDF). Rauland Corporation. 8 January 1962. Retrieved 1 May 2017.

    RMA Release #600, 2 September 1947

    "5729 30 channel radial beam tube - collector type data sheet" (PDF). National Union Electric Corporation. 9 April 1951. Retrieved 1 May 2017.

    "5734 Mechano-electronic transducer, triode type data sheet" (PDF). R.C.A. Manufacturing Company. November 15, 1948. Retrieved 1 May 2017.

    "5734A Mechano-electronic transducer data sheet" (PDF). Toshiba Corp. March 14, 1964. Retrieved 1 May 2017.

    "5738 Commutator tube data sheet" (PDF). Federal communication laboratories, Inc., Nutley, New Jersey, USA. 6 October 1948. Retrieved 1 May 2017.

    "6047 Additron data sheet, RTMA Engineering Dept. Release #954" (PDF). Rogers Majestic Corp. March 20, 1951. Retrieved 14 August 2016.

    "6090 18 channel radial beam tube - multiple anode type data sheet" (PDF). National Union Electric Corporation. January 1956. Retrieved 15 June 2013.

    "6091 25 channel radial beam tube - multiple grid type data sheet" (PDF). National Union Electric Corporation. January 1956. Retrieved 1 May 2017.

    "6170 & 6324 25 channel radial beam tube - multiple grid type data sheet" (PDF). National Union Electric Corporation. December 1955. Retrieved 15 June 2013.

    "Tube Electrometre Double Tétrode à 2 Grilles de Charge d'Espace data sheet" (PDF) (in French). Compagnie Industrielle Française des Tubes Electroniques (CIFTE). January 1968. Retrieved 1 May 2017.

    "6218 data sheet, RTMA Engineering Dept. Release #1115" (PDF). Rogers Majestic Corp. 25 August 1952. Retrieved 1 May 2017.

    "E80T data sheet" (PDF). Philips. 4 April 1956. Retrieved 1 May 2017.

    Richard G. Cumings (8 June 1956). "NRL Memorandum Report 606: Application of Tacitron Type RCA 6441 to Pulse Circuitry" (PDF). United States Naval Research Laboratory. Retrieved 19 November 2017.

    "6462 Magnetic pick-up tube data sheet" (PDF). National Union Electric Corporation. 9 May 1956. Retrieved 1 May 2017.

    "6571 Computer storage tube data sheet" (PDF). RCA Electron Tube Division. 21 March 1955. Retrieved 1 May 2017.

    "6577 Typotron, 5" character-writing CRT-type storage tube data sheet" (PDF). Hughes Aircraft Corporation. 24 November 1954. Retrieved 29 August 2017.

    "6700 Magnetron Beam Switching Tube data sheet" (PDF). Burroughs Corporation. August 1956. Archived from the original (PDF) on 4 March 2014. Retrieved 4 March 2014.

    "6701 Magnetron Beam Switching Tube data sheet" (PDF). Burroughs Corporation. August 1956. Archived from the original (PDF) on 4 March 2014. Retrieved 4 March 2014.

    "6762 Wamoscope data sheet" (PDF). Sylvania Electric Products. 17 January 1957. Retrieved 1 May 2017.

    "CK6835 Recording storage tube data sheet" (PDF). Raytheon Company. 1 November 1959. Retrieved 1 May 2017.

    "CK7570 Recording storage tube data sheet" (PDF). Raytheon Company. 1 November 1959. Retrieved 1 May 2017.

    "CK7571 Recording storage tube data sheet" (PDF). Raytheon Company. 1 November 1959. Retrieved 1 May 2017.

    "6846 Binary tube data sheet" (PDF). Sylvania Electric Products. July 1956. Retrieved 1 May 2017.

    "7229 Cold-Cathode Trigger Tube data sheet E287B" (PDF). CBS/Hytron. 22 June 1958. Retrieved 11 September 2017.

    "7230 Reliable Cold-Cathode Trigger Tube data sheet E287C" (PDF). CBS/Hytron. 25 August 1958. Retrieved 11 September 2017.

    "7231 Subminiature Cold-Cathode Trigger Tube data sheet E287D" (PDF). CBS/Hytron. 22 June 1958. Retrieved 11 September 2017.

    "7232 Reliable Subminiature Cold-Cathode Trigger Tube data sheet E287E" (PDF). CBS/Hytron. 22 June 1958. Retrieved 11 September 2017.

    CBS/Hytron "Krytron Trigger Tubes" spec sheets E-337 (30 March 1959), E-337A-1 (20 June 1960), E-337A-2 (20 June 1960)

    "7360 Beam Deflection Tube data sheet" (PDF). R.C.A. Manufacturing Company. March 1961. Retrieved 1 May 2017.

    M. B. Knight (1960). "A new miniature beam deflection tube" (PDF). RCA Electron Tube Division. Retrieved January 22, 2017.

    H. C. Vance K2FF (1960). "SSB Exciter Circuits Using a New Beam-Deflection Tube" (PDF). QST. Retrieved May 30, 2013.

    "7414 Subminiature Time Totalizer data sheet" (PDF). Bendix Corporation. 14 March 1959. Retrieved 1 May 2017.

    "CK7572 Recording storage tube data sheet" (PDF). Raytheon Company. 15 December 1959. Retrieved 1 May 2017.

    "CK7575 Recording storage tube data sheet" (PDF). Raytheon Company. 15 December 1959. Retrieved 1 May 2017.

    "CK7702 Recording storage tube data sheet" (PDF). Raytheon Company. 15 March 1960. Retrieved 1 May 2017.

    "7763 Sheet Beam Tube data sheet" (PDF). General Electric. 5 March 1962. Retrieved 1 May 2017.

    This tube's designation is inconsistent with the scheme

    Wechselspannungs- und Wechselstrom-Stabilisierungsschaltungen mit der Diode YA1000. Telefunken Laborbuch (in German). IV. Ulm: AEG-Telefunken. 1967. pp. 189–195.

    "The ZA100x series switching tubes from Philips". Retrieved 19 August 2013.

    "ZC1050 data sheet" (PDF). Philips. February 1968. Retrieved 21 December 2013.

    Thaens, J. G. M.; van Vlodrop, P. H. G. "Electronic Applications Vol. 27 No. 3: Running Text Display with Cold-Cathode Trigger Tubes" (PDF). Philips Elcoma Division, Central Application Laboratory, Eindhoven, The Nederlands. Retrieved 21 December 2013.

    "Disc Seal Triodes" (PDF). Mullard. 1965. Retrieved 12 February 2016.

    C. E. Wynn-Williams (2 May 1932). "Proceedings of the Royal Society of London: A Thyratron Scale of Two Automatic counter" (PDF). A, Containing Papers of a Mathematical and Physical Character. Royal Society. pp. 312–324. Retrieved 1 May 2017.

    "CK1366 CK1367 Printer-type cathode ray tube data sheet" (PDF). Raytheon Company. 1 November 1960. Retrieved 1 May 2017.

    "CK1368 CK1369 Printer-type cathode ray tube data sheet" (PDF). Raytheon Company. 1 November 1960. Retrieved 1 May 2017.

    "CK1383 Recording storage tube data sheet" (PDF). Raytheon Company. 15 February 1963. Retrieved 1 May 2017.

    "CK1414 Symbolray character generating cathode ray tube data sheet" (PDF). Raytheon Company components division, industrial components operation. 15 April 1966. Retrieved 1 May 2017.

    "Symbolray™ application note" (PDF). Raytheon Company components division, industrial components operation. Retrieved 1 May 2017.

    "DDR100 Accelerometer double diode data sheet" (PDF). Mullard. Retrieved 1 May 2017.

    "Krytrons - Cold Cathode Switch Tubes data sheet K5500B-1" (PDF). EG&G Electro-Optics Division, Salem, Massachusetts, USA. September 1973. Retrieved 11 September 2016.

    Wahl, Günter. "Hightech-Elektronik-Experimente" (PDF) (in German). Franzis Verlag. Retrieved 26 Dec 2014.

    "Plasma Panel Displays - Dual Linear Bar Graph" (PDF). Vishay Dale, Columbus, Nebraska, USA. November 2000. Retrieved 8 March 2014.

    Miller, Joseph A.; Soltes, Aaron S.; Scott, Ronald E. (February 1955). "Wide-band Analog Function Multiplier" (PDF). Electronics. Retrieved 15 June 2013.

    Wyse, Barry (2000). "Extracts from "The Saga of Marconi Osram Valves", part 1" (PDF). The British Vintage Wireless Society. p. 12ff. Retrieved 1 May 2017.

    R-type tube on The National Valve Museum

    Lankshear, Peter (July 1996). "Valve filament/heater voltages" (PDF). Electronics Australia. Retrieved 1 May 2017.

    "Subminiature gas triode type RK61 data sheet" (PDF). Raytheon Company. Retrieved 1 May 2017.

    "Ed Lorenz Mystery Tube". Retrieved 1 May 2017.

    George Honnest-Redlich Radio Control for Models (1950) p. 7

    "SB256 Selective Electrostatic Storage Tube data sheet" (PDF). RCA Electron Tube Division. November 1951. Retrieved 4 November 2017.

    Charles S. Osborne • lampes-et-tubes.info

    TuneOn data sheet

    TuneOn Button data sheet

    BRIMAR (STC) Tunograph, Visual Tuning Indicator on lampes-et-tubes • Tunograph data sheet

    "TH9503 Scripticon character generating cathode ray tube data sheet" (PDF). Compagnie Française Thomson-Houston, division tubes electroniques, Paris (France). January 1968. Retrieved 27 September 2017.

    Van Bergen, Fons (2000). "About the French TM valve" (PDF). The British Vintage Wireless Society. p. 20ff. Retrieved 1 May 2017.

    Champeix, Robert. "Grande et Petite Histoire de la Lampe TM" (in French). Les Anciens de la Radio et de l'Électronique. Retrieved 1 May 2017.

    TM tube; Horned tube on The National Valve Museum

    Gerald Garratt G5CS. "Why the French R valve?". Retrieved 1 May 2017.

    Grid-anode curves for the Soviet R-5 triode, a licensed clone of the French TM triode made by La Compagnie des Lampes (1888)

    McNally, J.O.; Metson, G.H.; Veazie, E.A.; Holmes, M.F. (January 1957). "Electron tubes for the transatlantic cable system" (PDF). The Bell system technical journal. pp. 163ff. Retrieved 9 February 2016.

    "1636 U-H-F Beam deflection mixer" (PDF). RCA Electron Tube Division. 3 November 1944. Retrieved 1 May 2017.

    "368A, 368AS and 388A data sheet" (PDF). Western Electric. Retrieved 19 January 2016.

    http://www.tubecollectors.org/eimac/archives/450th(50).pdf

    Holdaway, V.L.; Van Haste, W.; Walsh, E.J. (July 1964). "Electron tubes for the SD submarine cable system" (PDF). The Bell system technical journal. pp. 1311ff. Retrieved 9 Feb 2016.

    https://www.crtsite.com/tv-crt.html

    "RCA Air-Cooled Transmitting Tube Manual TT3" (PDF). R.C.A. Manufacturing Company, Harrison, New Jersey, USA. 1938. Retrieved 17 October 2013.

    Lankshear, Peter (August 1988). "The Methuselah of valves" (PDF). Electronics Australia. Retrieved 1 May 2017.

    "ИНДИКАТОР ИТМ2-М data sheet" (PDF) (in Russian). Moscow Electric Lamp Plant (МЭЛЗ/MELZ). 1944. Archived from the original (PDF) on 12 October 2013. Retrieved 9 May 2013.

    "Линейный Трохотрон Типа ЛП-4 data sheet" (PDF) (in Russian). Moscow Electric Lamp Plant (МЭЛЗ/MELZ). Archived from the original (PDF) on 9 March 2014. Retrieved 8 March 2014.

    La Compagnie des Lampes on radiomuseum.org

    French Mazda datasheets before 1949: 18MA4 by CdL • 1883 (July 1948) by CdL, BELVU • 2XM400 (September 1947) by CdL • 2XM600 (September 1947) by CdL • 4Y25 (February 1949) by CdL, BELVU • 5Y35 (July 1948) by CdL • 6H8G (September 1947) by CdL • 879 (September 1947) by CdL • 884 (January 1949) by CdL • 8SAx by CdL • C75S (June 1947) by CdL • C95S (June 1947) by CdL

    French Mazda datasheets 1949–53: 2E30 (November 1949) by CdL • 31MA4 (February 1950) by CdL • 3T20 (July 1949) by CdL • 3T100 (July 1949) by CdL, BELVU • 4Y50 (November 1950) by CdL, BELVU • C30S (January 1950) by CdL • C127S (January 1950) by CdL • C220MW1 (January 1950) by CdL • E1 (April 1950) by CdL • E2 (April 1950) by CdL • ST130 (September 1949) by CdL

    French Mazda datasheets 1953–59: 2G21 (October 1953) by CdL, BELVU • 4Y100 (September 1960) by CdL, BELVU • 43MG4 (December 1954) by CdL • 43MH4 (March 1954) by CdL • 43MR4 (December 1954) by CdL • 54MS4 (June 1955) by CdL • 829 (June 1955) by CdL • 832 (June 1955) by CdL • 927 (July 1954) by CdL • 929 (June 1957) by CdL • 6196 (November 1959) by CdL • 6250 (November 1959) by CdL, BELVU • E5 (September 1960) by CdL • JA10 (September 1960) by CdL, BELVU

  -   French Mazda datasheets since 1959: 3T50 (February 1966) by CdL, BELVU • 4Y75 (February 1964) by CdL, BELVU • 6K8 (June 1964) by CdL • 78A (September 1966) by CdL, BELVU • 7233 (April 1962) by CdL • 7242 (April 1965) by CdL • 7377 (April 1962) by CdL • 8418 (February 1963) by CdL, BELVU • E6 (February 1964) by CdL • E7 (June 1965) by CdL • E9 (September 1965) by CdL • ECF202 (April 1967) by CdL, BELVU • ECL802 (December 1966) by CdL, BELVU • ED501 (February 1966) by CdL • EF816 (April 1967) by CdL • EL183 (June 1959) by CdL, BELVU • EL503 (June 1966) by CdL, BELVU • EY81F (April 1967) by CdL, BELVU • EY802 (April 1967) by CdL, BELVU • F7024x (April 1967) by CdL, BELVU • F9102 (April 1965) by CdL, BELVU • F9116 (December 1965) by CdL • GY86 (June 1966) by CdL • GY802 (April 1967) by CdL • K25000A1 (June 1961) by CdL • PY81F (April 1967) by CdL, BELVU

- General literature and data sheets

- Frank Philipse's Tube Datasheet Archive

- Mirrors in Brazil • Brazil searchable • Germany • Germany • Hong Kong • Hong Kong • Hungary • Poland[fast] • Poland searchable • Romania • Romania searchable • Sweden • USA • USA

- Tubebooks.org datasheet collection

- Roy J. Tellason's tube datasheet collection

- Klausmobile Russian tube directory

- General Electric Essential Characteristics, 1970

- RCA Receiving Tube Manuals R10 (1932) • RC11 (1933) • RC12 (1934) • RC13 (1937) • RC14 (1942) • RC15 (1948) • RC16 (1951) • RC17 (1954) • RC18 (1956) • RC19 (1959) • RC20 (1960) • RC21 (1961) • RC22 (1963) • RC23 (1964) • RC24 (1965) • RC25 (1966) • RC26 (1968) • RC27 (1970) • RC28 (1971) • RC29 (1973) • RC30 (1975)

- Scanned tube documentation (PDFs): Tubebooks • Frank Philipse • 4tubes

- Sylvania Technical Manual, 1958

- J. P. Hawker (ed), Radio and television servicing, Newnes, London, 1964

- Camera tube datasheets

- [1] • [2] Decoding type numbers

- Decoding Valve, Transistor and CRT Numbers

- Vacuum Tube Numbering Schemes, Bases & Bulbs

- European tube designation systems: [3] • [4] • [5]

- See also

- Electronics portal

- External links

|  | Wikimedia Commons has media related to Vacuum tubes and Gas discharge tube. |
| --- | --- |

- Vacuum Tube Data Sheet Locator

- Tube Substitution and Characteristics Guide

- British virtual thermionic valve museum with good quality pictures and data

- Belgian virtual thermionic valve museum with good quality pictures and data

- Radio museum

- Virtual Valve Museum

| - v - t - e Thermionic valves |  |
| --- | --- |
| Theoretical principles | - Thermionic emission - Work function - Hot cathode - Space charge - Control grid - Suppressor grid - Anode - Glowing anode - Getter |
| Types | - Diode - Audion - Triode - Acorn tube - Nuvistor - Tetrode - Beam tetrode - Pentode - Pentagrid (Hexode, Heptode, Octode) - Nonode - Cathode ray tube - Additron - Backward-wave oscillator - Beam deflection tube - Charactron - Compactron - Eidophor - Iconoscope - Inductive output tube - Kinescope - Klystron - Magic eye - Magnetron - Monoscope - Phototube - Photomultiplier - Selectron tube - Storage tube - Sutton tube - Talaria projector - Traveling-wave tube - Trochotron - Video camera tube - Williams tube - Fleming valve |
| Numbering systems | - RMA - RETMA - Marconi-Osram - Mullard–Philips - JIS - Russian |
| Examples | - List of vacuum tubes - List of tube sockets |

Categories:

- Vacuum tubes

- Electronics lists

- Gas-filled tubes

- Navigation menu

- Not logged in

- Talk

- Contributions

- Create account

- Log in

- Article

- Talk

- Read

- Edit

- View history

- Search

Top of Form

Bottom of Form

Top of Form

Bottom of Form

- Main page

- Contents

- Featured content

- Current events

- Random article

- Donate to Wikipedia

- Wikipedia store

- Interaction

- Help

- About Wikipedia

- Community portal

- Recent changes

- Contact page

- Tools

- What links here

- Related changes

- Upload file

- Special pages

- Permanent link

- Page information

- Wikidata item

- Cite this page

- Print/export

- Create a book

- Download as PDF

- Printable version

- Languages

- Български

- Polski

- Русский

Edit links

- This page was last edited on 4 August 2019, at 15:14 (UTC).

- Text is available under the Creative Commons Attribution-ShareAlike License; additional terms may apply. By using this site, you agree to the Terms of Use and Privacy Policy. Wikipedia® is a registered trademark of the Wikimedia Foundation, Inc., a non-profit organization.

- Privacy policy

- About Wikipedia

- Disclaimers

- Contact Wikipedia

- Developers

- Cookie statement

- Mobile view