USA (RETMA) Tube Numbering System

First number: filament voltage

0	Cold cathode (i.e. voltage regulator tube).

1	0.1 - 2.0

2	2.1 - 2.9

3	3.0 - 3.9

N	n.0 - n.9

Letter or letters: sequence code

One or two letters.  When the single character codes were used up, double character codes were introduced.

Note that U - Z as the only character or last character, are generally (but not always) used for rectifiers

Final number: Element count

The number of elements in the tube, the filament counting as 1.  For example, the 6SN7 has two cathodes, two grids, two plates, and a filament.

Additional letters:

| A | Controlled heater warm-up time, although can also be used to denote increased ratings or performance over and above the non-A model. |
| --- | --- |
| B | Improved ratings/performance |
| C | Ditto |
| G | Glass bulb |
| GT | Glass tubular |
| W | Ruggedized version |
| X | Low loss ceramic base |
| Y | Low loss phenolic base |

Examples:

| 6 | SN | 7 | GT |  |
| --- | --- | --- | --- | --- |
| 6 |  |  |  | Filament rating is between 6.0 - 6.9V |
|  | SN |  |  | SN=sequence code |
|  |  | 7 |  | 7 elements |
|  |  |  | GT | glass tubular envelope |

| 6 | V | 6 | GT | Y |  |
| --- | --- | --- | --- | --- | --- |
| 6 |  |  |  |  | Filament rating is between 6.0 - 6.9V |
|  | V |  |  |  | V=sequence code |
|  |  | 6 |  |  | 6 elements |
|  |  |  | GT |  | glass tubular envelope |
|  |  |  |  | Y | low loss phenolic base |

Examples of common Jukebox Tubes

| 12 | AY | 7 |  |
| --- | --- | --- | --- |
| 12 |  |  | Filament rating is between 12.0 - 12.9V |
|  | AY |  | AY=sequence code |
|  |  | 7 | 7 elements |

| 5 | U | 4 | G | B |  |
| --- | --- | --- | --- | --- | --- |
| 5 |  |  |  |  | Filament rating is between 5.0 - 5.9V |
|  | U |  |  |  | U=sequence code [In this case it is a rectifier tube] |
|  |  | 4 |  |  | 4 elements |
|  |  |  | G |  | Glass Bulb |
|  |  |  |  | B | Improved ratings/performance |

| 6 | EU | 7 |  |
| --- | --- | --- | --- |
| 6 |  |  | Filament rating is between 6.0 - 6.9V |
|  | EU |  | EU=sequence code |
|  |  | 7 | 7 elements |

| 6 | L | 6 | G | C |  |
| --- | --- | --- | --- | --- | --- |
| 6 |  |  |  |  | Filament rating is between 6.0 - 6.9V |
|  | L |  |  |  | L=sequence code |
|  |  | 6 |  |  | 6 elements |
|  |  |  | G |  | Glass Bulb |
|  |  |  |  | C | Ditto |

| 6 | X | 4 |  |
| --- | --- | --- | --- |
| 6 |  |  | Filament rating is between 6.0 - 6.9V |
|  | X |  | X=sequence code |
|  |  | 4 | 4 elements |

| 12 | BH | 7 | A |  |
| --- | --- | --- | --- | --- |
| 12 |  |  |  | Filament rating is between 12.0 - 12.9V |
|  | BH |  |  | BH=sequence code |
|  |  | 7 |  | 7 elements |
|  |  |  | A | Controlled heater warm-up time |

7199

6973

Copyright 2004 Megalos Pouli Inc.