ZIP SOFTWARE UPDATING USING FASHSTA.EXE

The  software for ZIP is composed  by  2 file called : xxx.id,  xxx.mot  .

These  file must be  stored into the same directory.

- Switch OFF CPU power .

- Connect by a serial cable the PC UART RS232 port to Flash Programmer DB9 female contacts  connector.

- Connect CPU Board DB9 female contacts connector  to Flash Programmer DB9 male contacts connector.

- Switch ON CPU power and verify that programmer bi-color led turns on with green light.

- Run Flashsta.exe program in the PC:

  - It will appear:

Select the COM port and then “OK”.

- It will appear:

Push  “Refer…”, select the file (file.mot) you want to load, push “Open” and wait the file loading.

- When loading is complete:

Verify the file path is correct and then confirm pushing “OK”.

- If the communication between CPU and PC is active, the next window will appear,

on the contrary, if there is a problem a message will explain it.

Choose “Setting…”, set Baud Rate = 57600, push “OK”.

Then choose “E.P.R…” (Erase+Program+Read).

- It will ask you:

Push “OK” to confirm.

- Flash programming will start :

wait the flash programming end…:

Push “OK” to confirm.

Push “Exit” on the M16C Flash Start  window to close the Flashsta.

- Switch OFF power and disconnect the Programmer from CPU board.