LAYOUT L02.

PRODUCT TABLE L02.

| POSIT. | PRODUCT | N° DOSING | SETT. DOS. (gram./sec.) | N° ELECTRO. | SETT. EV (cc/sec.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Whitener / Milk | 1 | 1,5 | 1 | 9 |
| 2 | Sugar | 2 | 2,3 | 1 | 9 |
| 3 | Fresh Brew 1 | 3 | 1,7 | 2 | 16 |
| 4 | Fresh Brew 1 |  |  |  |  |
| 5 | Coffee Ins. | 4 | 0,6 | 4 | 10 |
| 6 | Fresh Brew 2 | 6 | 1,7 | 5 | 14 |
| 7 | Fresh Brew 2 (C2) | 8 | 1,7 | 5 | 14 |
| 8 | Chocolate | 7 | 3,0 | 6 | 9 |
| / | Hot Water | / | / | 3 | 20 |

KEYBOARD SELECTION L02.

| Strength - |  | Strength + |  |
| --- | --- | --- | --- |
| ### Sugar |  | ### Extra Sugar |  |
| #### FB Coffee 1 Black |  | ### FB Coffee 2 Black |  |
| FB Coffee 1 White |  | FB Coffee 2 White |  |
| Instant Coffee Black |  | FB Tea Black |  |
| Instant Coffee White |  | FB Tea White |  |
| Cappuccino FB |  | ### Cappuccino Instant |  |
| Chocolate |  | EspressoChoc. Instant |  |
| Chocomilk |  | Espresso Instant |  |
| Espresso FB |  | Hot Water |  |

PRESELECTION L02.

| PRESELECTION (N.) | DOSE VARIAT. | DESCRIPTION |
| --- | --- | --- |
| ## Strength - | -10 % | Reduces the powder of Coffee/Tea (3 step). |
| Strength + | +10 % | Increase the powder of Coffee/Tea (3 step). |
| Sugar (1) | + 100% | Insert the dose of sugar of the selection |
| Extra Sugar (10) | + 166% | Increase the dose of sugar to 166% |

All selections are without sugar by default.

Default Sugar: 0 gr.

Sugar: 5.3 gr.

Extra Sugar: 8.8 gr.

Tutte le selezioni di default sono senza zucchero.

SELECTION L02.

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| FB Coffee 1 Black | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
| ( 2 ) | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Coffee 1 White | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
| ( 3 ) | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Inst. Coffee Black ( 4 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |
| Inst. Coffee White ( 5 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Cappuccino FB ( 6 ) | Coffee FB 80 cc |  | Coff. FB 6,5 gr. |  |  |  |
|  | Milk 55 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 30 cc (tot. 165 cc) |  | Chocolate 3,5 gr. |  |  |  |
| Chocolate ( 7 ) | Chocolate 80 cc |  | Chocolate 19,0 gr. |  |  |  |
|  | Hot Water 60 cc (tot. 140 cc) |  |  |  |  |  |
| Chocomilk ( 8 ) | Chocolate 70 cc |  | Chocolate 16,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 130 cc) |  | Milk 1,4 gr. |  |  |  |
| SELECTION (N.) | WATER DOSES | ### SUGAR (to door) | POWDER DOSES |  |  |  |
| Espresso FB ( 9 ) | Coffee FB 50 cc |  | Coff. FB 3,3 gr. |  |  |  |
|  | Sugar 60 cc (tot. 110 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Coffee 2 Black | Coffee2 FB 110 cc |  | Coff.2 FB 9,0 gr. |  |  |  |
| ( 11 ) | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Coffee 2 White | Coffee2 FB 110 cc |  | Coff.2 FB 9,0 gr. |  |  |  |
| ( 12 ) | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| FB Tea Black ( 13 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Tea White ( 14 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Cappuccino Inst. ( 15 ) | Coffee Instant 40 cc |  | Coffee 1,6 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 40 cc (tot. 140 cc) |  | Chocolate 9,6 gr. |  |  |  |
| EspressoChoc Ins. ( 16 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Espresso Inst. ( 17 ) | Coffee Instant 35 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 30 cc (tot. 65 cc) |  | Sugar 5,3 gr. |  |  |  |
| Hot Water ( 18 ) | Hot Water 150 cc |  |  |  |  |  |

INFORMAZIONI GENERALI.

TABELLA PESI VALIDATORE:

| Linea | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Peso | 1 | 2 | 10 | 20 | 50 | 5 |

PREZZO SELEZIONI: Tutte le selezioni a 10.00.

NPREZZO SELEZIONI: Tutte le selezioni puntano al prezzo 1.

POSIZIONE PUNTO DECIMALE: 2.

TEMPERATURA CALDAIA EXP: ---- °C.

TEMPERATURA CALDAIA SOLUBILI: 86.0 °C.

RITARDI ACQUE / POLVERI: Ritardo polvere rispetto all’acqua 0.6 sec.

TIPO DI TASTIERA: Tastiera Astro.

LINGUE: Italiano, Francaise, Espanol, English, Deutsch, Danska, Ungaria, Nederlands, Svenska, Czech, Polski e Finnish.

Per l’impostazione del sistema di pagamento si utilizza il menu 2.1.2.1. Nel caso in cui il distributore non abbia sistema di pagamento bisogna impostare OFF.

Gestione Astro-Sfera in batteria master-slave. (La Rev. Astro 0.4 SUK può lavorare solo con la Rev. Sfera 1.5 o superiori).

GENERAL INFORMATION.

PAYMENT SYSTEMS DETAILS:

| Line | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Weight | 1 | 2 | 10 | 20 | 50 | 5 |

SELECTION PRICE: All the selections to 10.00.

SELECTION NPRICE: All selections are set price line 1.

DECIMAL POINT POSITION: 2.

TEMPERATURE BOILER EXP: ---- °C.

TEMPERATURE BOILER INSTANT: 86.0 °C.

WATER/POWDER DELAYS: Powder delays to water 0.6 sec.

TYPE OF KEYBOARD: Astro keyboard.

LANGUAGE: Italiano, Francaise, Espanol, English, Deutsch, Danska, Ungaria, Nederlands, Svenska, Czech, Polski and Finnish.

For the payment system setting it is used the menu 2.1.2.1. In case of situation without payment system it is needed to set this parameter to OFF.

The Sw. Astro Rev. 0.4 SUK can only work with Sfera/Rondò Rev. 1.5 or upper).

MODIFICHE.

SW CPU / SCHEDA ATTUAZIONI REV. 0.2:

Aggiunto layout L03.

SW CPU / SCHEDA ATTUAZIONI REV. 0.3:

Modifica del parametro CUP SETTLING TIME.

SW CPU / SCHEDA ATTUAZIONI REV. 0.4:

Modificato valore contrasto LCD per gestione display BLU.

Inserito layout L04.

CHANGES.

SW CPU - I/O BOARD REV. 0.2:

Inserted layout L03

SW CPU - I/O BOARD REV. 0.3:

Modified CUP SETTLING TIME parameter.

SW CPU - I/O BOARD REV. 0.4 UK:

Changed LCD contrast value for blue display management.

Inserted layout L04.