LAYOUT L15.

PRODUCT TABLE.

| POSIT. | PRODUCT | N° DOSING | SETT. DOS. (gram./sec.) | N° ELECTRO. | SETT. EV (cc/sec.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Whitener / Milk | 1 | 1,5 | 1 | 9 |
| 2 | Sugar | 2 | 2,3 | 1 | 9 |
| 3 | Fresh Brew 1 | 3 | 1,7 | 2 | 16 |
| 4 | Fresh Brew 1 |  |  |  |  |
| 5 | Coffee Ins. / Soup * | 4 | 0,6 / 2,5 * | 4 | 10 / 15 * |
| 6 | Fresh Brew 2 | 6 | 1,7 | 5 | 14 |
| 7 | Fresh Brew 2 (C2) | 8 | 1,7 | 5 | 14 |
| 8 | Chocolate | 7 | 3,0 | 6 | 9 |
| / | Hot Water | / | / | 3 | 20 |
| / | Carbonated Water | / | / | 7 | 17 |
| / | Still Water | / | / | 8 | 16 |

KEYBOARD SELECTION L15.

| Strength - |  | Strength + |  |
| --- | --- | --- | --- |

| SELECTION | SUGAR N. | EX. SUGAR N. | NO SUGAR N. |
| --- | --- | --- | --- |
| ## Coffee Fresh Brew Black | 17 | 18 | 19 |
| Coffee Fresh Brew White | 21 | 22 | 23 |
| Coffee Fresh Brew Ex. White | 24 | 25 | 26 |
| Chocolate | -- | -- | 27 |
| Soup * | -- | -- | 28 * |
| Hot Water | -- | -- | 29 |
| Coffee2 Fresh Brew White | 31 | 32 | 33 |
| Coffee2 Fresh Brew Ex. White | 34 | 35 | 36 |
| Coffee Instant Black | 37 | 38 | 39 |
| Coffee Instant White | 41 | 42 | 43 |
| Coffee Instant Extra White | 44 | 45 | 46 |
| ## Coffee2 Fresh Brew Black | 47 | 48 | 49 |
| FB Tea White | 51 | 52 | 53 |
| FB Tea Extra White | 54 | 55 | 56 |
| Fresh Brew Tea Black | 57 | 58 | 59 |
| ### Coffee Fresh Brew Cappuccino | 61 | -- | 62 |
| Coffee Fresh Brew Espresso | 63 | -- | 64 |
| Coffee Fresh Brew EspressoChoc | 65 | -- | 66 |
| ChocoMilk | -- | -- | 67 |
| Instant Cappuccino | 71 | -- | 72 |
| Instant EspressoChoc | 73 | -- | 74 |
|  | STILL |  | CARBONATED |
| Chilled Water | 81 |  | 84 |
| Orange ( Syrop 1 ) | 82 |  | 85 |
| Lemon (Syrop 2 ) | 83 |  | 86 |

* When it takes the place of the Coffee Instant with the Soup, and vice versa, is necessary disable the selections of the powder to replace and train those of the new, therefore modify of the value of setting of the doseur and electrovalve. correspondent like suitable in the table product.

* Quando si sostituisce il Coffee Instant con il  Soup, e viceversa, e’ necessario  disabilitare le selezioni della polvere da sostituire ed abilitare quelle della nuova, quindi  modificare del valori di taratura del dosatore ed elettrov. corrispondente come indicato nella tabella prodotti.

PRESELECTION L15.

| PRESELECTION (N.) | DOSE VARIAT. | DESCRIPTION |
| --- | --- | --- |
| ## Strength - | -10 % | Reduces the powder of Coffee/Tea (3 step). |
| Strength + | +10 % | Increase the powder of Coffee/Tea (3 step). |
| Extra Sugar | + 66% | Increase the dose of sugar to 66% |
| No Sugar | - 100% | Reset the dose of sugar of the selection |

SELECTION L15.

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fresh Brew Black ( 17 – 18 – 19 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| Fresh Brew White ( 21 – 22 – 23 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 24 – 25 – 26 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Chocolate ( 27 ) | Chocolate 80 cc |  | Chocolate 19,0 gr. |  |  |  |
|  | Hot Water 60 cc (tot. 140 cc) |  |  |  |  |  |
| Soup * ( 28 ) | Soup 150 cc |  | Soup 8,0 gr. |  |  |  |
| Hot Water ( 29 ) | Hot Water 150 cc |  |  |  |  |  |
| Fresh Brew White ( 31 – 32 – 33 ) | Coffee2 FB 110 cc |  | Coff.2 FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 34 – 35 – 36 ) | Coffee2 FB 110 cc |  | Coff.2 FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Coffee Black ( 37 – 38 – 39 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coffee White ( 41 – 42 – 43 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Coffee Ex. White ( 44 – 45 – 46 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 3,0 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Fresh Brew Black ( 47 – 48 – 49 ) | Coffee2 FB 110 cc |  | Coff.2 FB 9,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| Fresh Brew White ( 51 – 52 – 53 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 54 – 55 – 56 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Fresh Brew Black ( 57 – 58 – 59 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Cappuccino ( 61 – 62 ) | Coffee FB 80 cc |  | Coff. FB 6,5 gr. |  |  |  |
|  | Milk 55 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 30 cc (tot. 165 cc) |  | Chocolate 3,5 gr. |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| FB Espresso ( 63 – 64 ) | Coffee FB 50 cc |  | Coff. FB 3,3 gr. |  |  |  |
|  | Sugar 60 cc (tot. 110 cc) |  | Sugar 5,3 gr. |  |  |  |
| EspressoChoc ( 65 – 66 ) | Coffee FB 55 cc |  | Coff. FB 3,8 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 140 cc) |  | Chocolate 13,0 gr. |  |  |  |
| Chocomilk ( 67 ) | Chocolate 70 cc |  | Chocolate 16,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 130 cc) |  | Milk 1,4 gr. |  |  |  |
| Inst. Cappuccino ( 71 – 72 ) | Coffee Instant 40 cc |  | Coffee 1,6 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 40 cc (tot. 140 cc) |  | Chocolate 9,6 gr. |  |  |  |
| Ins.EspressoChoc ( 73 – 74 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Chilled Water ( 81 ) | Still 160 cc |  |  |  |  |  |
| Chilled Water ( 84 ) | Carbonated 140 cc |  |  |  |  |  |
|  | Still 20 cc (tot. 160) |  |  |  |  |  |
| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| Orange ( 82 ) | Still 150 cc |  | Syrop 1 32 ds |  |  |  |
| Orange ( 85 ) | Still 35 cc |  | Syrop 1 0 ds |  |  |  |
|  | Carbonated 95 cc |  | Syrop 1 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |
| Lemon ( 83 ) | Still 150 cc |  | Syrop 2 32 ds |  |  |  |
| Lemon ( 86 ) | Still 35 cc |  | Syrop 2 0 ds |  |  |  |
|  | Carbonated 95 cc |  | Syrop 2 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |

INFORMAZIONI GENERALI.

TABELLA PESI VALIDATORE:

| Linea | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Peso | 1 | 2 | 10 | 20 | 50 | 5 |

PREZZO SELEZIONI: Tutte le selezioni a 10.00.

NPREZZO SELEZIONI: Tutte le selezioni puntano al prezzo 1.

POSIZIONE PUNTO DECIMALE: 2.

TEMPERATURA CALDAIA EXP: ---- °C.

TEMPERATURA CALDAIA SOLUBILI: 86.0 °C.

RITARDI ACQUE / POLVERI: Ritardo polvere rispetto all’acqua 0.6 sec.

TIPO DI TASTIERA: Tastiera numerica.

LINGUE: Italiano, Francaise, Espanol, English, Deutsch, Ungaria, Nederlands, Portugues, Czech, Polski, Slovensko, Русский.

Per l’impostazione del sistema di pagamento si utilizza il menu 2.1.2.1. Nel caso in cui il distributore non abbia sistema di pagamento bisogna impostare OFF.

Gestione Astro-Sfera in batteria master-slave. (La Rev. Astro 1.5 IFE può lavorare solo con la Rev. Sfera 1.5 o superiori).

GENERAL INFORMATION.

PAYMENT SYSTEMS DETAILS:

| Line | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Weight | 1 | 2 | 10 | 20 | 50 | 5 |

SELECTION PRICE: All the selections to 10.00.

SELECTION NPRICE: All selections are set price line 1.

DECIMAL POINT POSITION: 2.

TEMPERATURE BOILER EXP: ---- °C.

TEMPERATURE BOILER INSTANT: 86.0 °C.

WATER/POWDER DELAYS: Powder delays to water 0.6 sec.

TYPE OF KEYBOARD: Numerical keyboard.

LANGUAGE: Italiano, Francaise, Espanol, English, Deutsch, Ungaria, Nederlands, Portugues, Czech, Polski, Slovensko, Русский.

MODIFICHE.

SW CPU / SCHEDA ATTUAZIONI REV. 1.1:

Modifica alla struttura del database (l’upgrade necessita l’inizializzazione del d.a.).

SW CPU / SCHEDA ATTUAZIONI REV. 1.1 UK:

Introdotta una modifica della gestione del guasto air-break.

Aggiornamento stringhe in inglese.

Modifica del valore minimo del tempo di uscita ugelli per freddo e caldo (80).

Modifica delle attuazioni generate dal pulsante utility “SPOUTS” (uscita e rientro degli ugelli senza arresto)

SW CPU / SCHEDA ATTUAZIONI REV. 1.2:

Aggiunta delle lingue polacca e finlandese.

Modifica delle lingue svedese e danese.

Modifica della gestione delle statistiche (azzeramento) via GSM.

Programmazione tramite l’utilizzo di UP-KEY.

Modifica della gestione dei lavaggi e degli schizzi su distributori con acque fredde.

SW CPU / SCHEDA ATTUAZIONI REV. 1.3:

Eliminazione del controllo della presenza della caraffa in modalità Jug Facilities.

Gestione VENDITA LIBERA con solo sistemi CASHLESS e protocollo MDB.

SW CPU / SCHEDA ATTUAZIONI REV. 1.4:

Gestione Astro-Sfrera/Rondò in batteria master-slave. (La Rev. Astro 1.4 può lavorare solo con Sfera/Rondò Rev. 1.3 o superiori).

SW CPU / SCHEDA ATTUAZIONI REV. 1.41:

Corretta gestione chiave Jug.

CHANGES.

SW CPU - I/O BOARD REV. 1.1:

Modification of database structure (upgrade needs the d.a. initialisation)

SW CPU - I/O BOARD REV. 1.1 UK:

Introduction of air-break failure management modification.

Updating of english display messages.

Modification of minimal value for hot and cold spouts movement times (80).

Modification of  movements caused by SPOUTS utility button (spouts outgoing and incoming without stop)

SW CPU - I/O BOARD REV. 1.2:

Insertion of Finnish and Polski languages.

Modified Swedish and Danska languages.

Modified of statistical management by GSM.

D.A. programmation by use of UP-KEY.

Modified washing and spurt management in d.a. with cold water.

SW CPU - I/O BOARD REV. 1.3:

Eliminated jug check during Jug Facilities operation.

Management of free-vend operation with only cashless systems and MDB protocol.

SW CPU –I/O BOARD REV. 1.4:

Management of Astro – Sfera/Rondò set in configuration master-slave. (The Sw. Astro Rev. 1.4 can only work with Sfera/Rondò Rev. 1.3 or upper).

SW CPU - I/O BOARD REV. 1.41:

Corrected Jug Facility Management.

SW CPU / SCHEDA ATTUAZIONI REV. 1.5:

Gestione batterie con due sistemi di pagamento posti su due distributori.

Gestione up-key per leggere e scrivere setup e per leggere statistiche.