LAYOUT L01.

PRODUCT TABLE.

| POSIT. | PRODUCT | N° DOSING | SETT. DOS. (gram./sec.) | N° ELECTRO. | SETT. EV (cc/sec.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Whitener / Milk | 1 | 1,5 | 1 | 9 |
| 2 | Chocolate | 2 | 3,0 | 2 | 9 |
| 3 | Expresso |  |  |  |  |
| 4 | Expresso |  |  |  |  |
| 5 | Expresso |  |  |  |  |
| 6 | Fresh Brew 2 | 6 | 1,7 | 4 | 16 |
| 7 | Fresh Brew 2 |  |  |  |  |
| 8 | Coffee Inst. / Soup* | 7 | 0,6 / 2,5 * | 5 | 10 / 15 * |
| 9 | Sugar | 9 | 2,3 | 1 | 9 |
| / | Hot Water | / | / | 6 | 20 |
| / | Carbonated Water | / | / | 7 | 17 |
| / | Still Water | / | / | 8 | 16 |

KEYBOARD SELECTION L01.

| Strength - |  | Strength + |  |
| --- | --- | --- | --- |

| SELECTION | SUGAR N. | EX. SUGAR N. | NO SUGAR N. |
| --- | --- | --- | --- |
| ## Coffee Been Black | 17 | 18 | 19 |
| Coffee Been White | 21 | 22 | 23 |
| Coffee Been Ex. White | 24 | 25 | 26 |
| Chocolate | -- | -- | 27 |
| Soup * | -- | -- | 28 * |
| Hot Water | -- | -- | 29 |
| Coffee Instant Black | 37 | 38 | 39 |
| Coffee Instant White | 41 | 42 | 43 |
| Coffee Inst. Ex. White | 44 | 45 | 46 |
| FB Tea White | 51 | 52 | 53 |
| FB Tea Extra White | 54 | 55 | 56 |
| Fresh Brew Tea Black | 57 | 58 | 59 |
| ### Cappuccino | 61 | -- | 62 |
| Espresso | 63 | -- | 64 |
| EspressoChoc | 65 | -- | 66 |
| ChocoMilk | -- | -- | 67 |
| Instant Cappuccino | 71 | -- | 72 |
| Instant EspressoChoc | 73 | -- | 74 |
|  | STILL |  | CARBONATED |
| Chilled Water | 81 |  | 84 |
| Orange ( Syrop 1 ) | 82 |  | 85 |
| Lemon (Syrop 2 ) | 83 |  | 86 |

* When it takes the place of the Coffee Instant with the Soup, and vice versa, is necessary disable the selections of the powder to replace and train those of the new, therefore modify of the value of setting of the doseur and electrovalve. correspondent like suitable in the table product.

* Quando si sostituisce il Coffee Instant con il  Soup, e viceversa, e’ necessario  disabilitare le selezioni della polvere da sostituire ed abilitare quelle della nuova, quindi  modificare del valori di taratura del dosatore ed elettrov. corrispondente come indicato nella tabella prodotti.

PRESELECTION L01.

| PRESELECTION (N.) | DOSE VARIAT. | DESCRIPTION |
| --- | --- | --- |
| ## Strength - | -10 % | Reduces the powder of Coffee/Tea (3 step). |
| Strength + | +10 % | Increase the powder of Coffee/Tea (3 step). |
| Extra Sugar | + 66% | Increase the dose of sugar to 66% |
| No Sugar | - 100% | Reset the dose of sugar of the selection |

SELECTION L01.

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Espresso Black ( 17 – 18 – 19 ) | Coffee been 85 cdv ( 50 cc) |  |  |  |  |  |
|  | Sugar 60 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 25 cc (tot. 135 cc) |  |  |  |  |  |
| Espresso White ( 21 – 22 – 23 ) | Milk 45 cc |  | Milk 3,0 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee been 85 cdv (50 cc) |  |  |  |  |  |
|  | Hot Water 45 cc (tot. 140 cc) |  |  |  |  |  |
| Esp. Extra White ( 24 – 25 – 26 ) | Milk 45 cc |  | Milk 4,5 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee been 85 cdv (50 cc) |  |  |  |  |  |
|  | Hot Water 45 cc (tot. 140 cc) |  |  |  |  |  |
| Chocolate ( 27 ) | Chocolate 80 cc |  | Chocolate 19,0 gr. |  |  |  |
|  | Hot Water 60 cc (tot. 140 cc) |  |  |  |  |  |
| Soup * ( 28 ) | Soup 150 cc |  | Soup 8,0 gr. |  |  |  |
| Hot Water ( 29 ) | Hot Water 150 cc |  |  |  |  |  |
| Coffee Black ( 37 – 38 – 39 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coffee White ( 41 – 42 – 43 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Coff. Extra White ( 44 – 45 – 46 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 3,0 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Fresh Brew White ( 51 – 52 – 53 ) | Tea 110 cc |  | Tea 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 54 – 55 – 56 ) | Tea 110 cc |  | Tea 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Fresh Brew Black ( 57 – 58 – 59 ) | Tea 110 cc |  | Tea 3,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| Cappuccino ( 61 – 62 ) | Milk 50 cc |  | Milk 6,5 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee been 80 cdv (45 cc) |  |  |  |  |  |
|  | Chocolate 30 cc (tot. 125 cc) |  | Chocolate 7,2 gr. |  |  |  |
| Espresso ( 63 – 64 ) | Coffee been 85 cdv (50 cc) |  |  |  |  |  |
|  | Sugar 90 cc (140 cc) |  | Sugar 5,3 gr. |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EspressoChoc ( 65 – 66 ) | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee been 60 cdv (30 cc) |  |  |  |  |  |
|  | Chocolate 50 cc (tot. 115 cc) |  | Chocolate 13 gr. |  |  |  |
| Chocomilk ( 67 ) | Chocolate 70 cc |  | Chocolate 16,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 130 cc) |  | Milk 1,4 gr. |  |  |  |
| Inst. Cappuccino ( 71 – 72 ) | Coffee Instant 65 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 50 cc |  | Milk 1,4 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 30 cc (tot. 145 cc) |  | Chocolate 6,0 gr. |  |  |  |
| InstEspressoChoc ( 73 – 74 ) | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Chilled Water ( 81 ) | Still 160 cc |  |  |  |  |  |
| Chilled Water ( 84 ) | Carbonated 140 cc |  |  |  |  |  |
|  | Still 20 cc (tot. 160) |  |  |  |  |  |
| Orange ( 82 ) | Still 150 cc |  | Syrop 1 32 ds |  |  |  |
| Orange ( 85 ) | Still 35 cc |  | Syrop 1 0 ds |  |  |  |
|  | Carbonated 95 cc |  | Syrop 1 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lemon ( 83 ) | Still 150 cc |  | Syrop 2 32 ds |  |  |  |
| Lemon ( 86 ) | Still 35 cc |  | Syrop 2 0 ds |  |  |  |
|  | Carbonated 95 cc |  | Syrop 2 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |

INFORMAZIONI GENERALI.

TABELLA PESI VALIDATORE:

| Linea | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Peso | 1 | 2 | 10 | 20 | 50 | 5 |

PREZZO SELEZIONI: Tutte le selezioni a 10.00.

NPREZZO SELEZIONI: Tutte le selezioni puntano al prezzo 1.

POSIZIONE PUNTO DECIMALE: 2.

TEMPERATURA CALDAIA EXP: 90.0 °C.

TEMPERATURA CALDAIA SOLUBILI: 86.0 °C.

RITARDI ACQUE / POLVERI: Ritardo polvere rispetto all’acqua 0.6 sec.

TIPO DI TASTIERA: Tastiera numerica.

LINGUE: Italiano, Francaise, Espanol, English, Deutsch, Ungaria, Nederlands, Portugues, Czech, Polski, Slovensko, Русский.

Per l’impostazione del sistema di pagamento si utilizza il menu 2.1.2.1. Nel caso in cui il distributore non abbia sistema di pagamento bisogna impostare OFF.

Gestione Astro-Sfera in batteria master-slave. (La Rev. Astro 1.53 IFE può lavorare solo con la Rev. Sfera 1.5 o superiori).

GENERAL INFORMATION.

PAYMENT SYSTEMS DETAILS:

| Line | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Weight | 1 | 2 | 10 | 20 | 50 | 5 |

SELECTION PRICE: All the selections to 10.00.

SELECTION NPRICE: All selections are set price line 1.

DECIMAL POINT POSITION: 2.

TEMPERATURE BOILER EXP: 90.0 °C.

TEMPERATURE BOILER INSTANT: 86.0 °C.

WATER/POWDER DELAYS: Powder delays to water 0.6 sec.

TYPE OF KEYBOARD: Numerical keyboard.

LANGUAGE: Italiano, Francaise, Espanol, English, Deutsch, Ungaria, Nederlands, Portugues, Czech, Polski, Slovensko, Русский.

For the payment system setting it is used the menu 2.1.2.1. In case of situation without payment system it is needed to set this parameter to OFF.

The Sw. Astro Rev. 1.53 IFE can only work with Sfera/Rondò Rev. 1.5 or upper).

MODIFICHE.

SW CPU / SCHEDA ATTUAZIONI REV. 0.5:

Implementazione gestioni sportelli e taniche per autoalimentazione.

Aggiornamento dosi.

SW CPU / SCHEDA ATTUAZIONI REV. 0.7:

Pulsante lavaggi in programmazione come nella rev. 0.5.

Gestione funzionamento ventola sulla mensola del FB montata in posizione centrale.

SW CPU / SCHEDA ATTUAZIONI REV. 0.8 UK:

Modifica codice F960400 in F960402 con unità fredda.

Aggiunta dei menù d’impostazione dell’ID per l’unità fredda (Tecn. 5.3) e del tempo di movimentazione ugelli per le bevande fredde e selezione acqua calda (Tecn. 5.4).

Modifiche per l’Operator friendly (pulsanti aggiunti, gestione lavaggi ed acqua calda).

Inserimento delle lingue danese, svedese e norvegese

SW CPU / SCHEDA ATTUAZIONI REV. 0.9:

Eliminati i layout L03-L04-L05.

Sistemazione alcune stringhe in programmazione.

Inserimento del messaggio “Rinsing cycle” durante il 1° risciacquo del mixer zucchero.

Modifica ciclo di erogazione delle bevande carbonate con una coda di acqua fredda.

Aggiunta menù di abilitazione delle aggiunte di acqua calda (Tecn. 5.5).

Implementazione MDB, EVADTS, GSM e SETUP AUTOMATICO (Fill>6.x / Fill>7.x /

Tecn>5.6 / Tecn>6.x).

SW CPU / SCHEDA ATTUAZIONI REV. 1.0:

Tempi di sgocciolamento programmabili.

Menu mdb per impostazione banconote accettate sotto il livello.

SW CPU / SCHEDA ATTUAZIONI REV. 1.1:

Modifica alla struttura del database (l’upgrade necessita l’inizializzazione del d.a.).

SW CPU / SCHEDA ATTUAZIONI REV. 1.1 UK:

Introdotta una modifica della gestione del guasto air-break.

Aggiornamento stringhe in inglese.

Modifica del valore minimo del tempo di uscita ugelli per freddo e caldo (80).

Modifica delle attuazioni generate dal pulsante utility “SPOUTS” (uscita e rientro degli ugelli senza arresto)

SW CPU / SCHEDA ATTUAZIONI REV. 1.2:

Aggiunta delle lingue polacca e finlandese.

Modifica delle lingue svedese e danese.

Modifica della gestione delle statistiche (azzeramento) via GSM.

Programmazione tramite l’utilizzo di UP-KEY.

Modifica della gestione dei lavaggi e degli schizzi su distributori con acque fredde.

SW CPU / SCHEDA ATTUAZIONI REV. 1.3:

Eliminazione del controllo della presenza della caraffa in modalità Jug Facilities.

Gestione VENDITA LIBERA con solo sistemi CASHLESS e protocollo MDB.

SW CPU / SCHEDA ATTUAZIONI REV. 1.4:

Gestione Astro-Sfrera/Rondò in batteria master-slave. (La Rev. Astro 1.4 può lavorare solo con Sfera/Rondò Rev. 1.3 o superiori).

SW CPU / SCHEDA ATTUAZIONI REV. 1.41:

Corretta gestione chiave Jug.

SW CPU / SCHEDA ATTUAZIONI REV. 1.5:

Gestione batterie con due sistemi di pagamento posti su due distributori.

Gestione up-key per leggere e scrivere setup e per leggere statistiche.

SW CPU / SCHEDA ATTUAZIONI REV. 1.51 IFE:

Corretta la gestione dello start-up.

SW CPU / SCHEDA ATTUAZIONI REV. 1.52 IFE:

Modifica del range del parametro CUP SETTLING TIME.

SW CPU / SCHEDA ATTUAZIONI REV. 1.53 IFE:

Corretta la gestione della taratura motodosatori.

CHANGES.

SW CPU - I/O BOARD REV. 0.5:

Implementation of slider and tank management.

Update doses

SW CPU - I/O BOARD REV. 0.7:

Cleaning utility in programmation menu as in rev. 0.5

Management of fan operation mounted in central position of FB shelf.

SW CPU - I/O BOARD REV. 0.8 UK:

Change of  F96301 code to F96307 with cold unit.

Addition of Cold ID unit setting menu (Tech. 5.3) and spouts movements time for cold drink and hot water (Tech. 5.4).

Changes for Operator Friendly (addes button, cleaning management and hot water)

Insertion of Danska, Norge and Svenska languages.

SW CPU - I/O BOARD REV. 0.9:

L03-L04-L05 layuots deleted.

Modification of some programmation message display.

Insertion of display message “Rinsing cycle” during the first rinsing of zucker mixer.

Addition of cold water queue to carbonated drink dispensing.

Insertion of enabling menu for hot water addition. (Tech. 5.5).

## Implementation of MDB, EVADTS, GSM and AUTOMATIC SETUP (Fill>6.x / Fill>7.x /

Tecn>5.6 / Tecn>6.x).

SW CPU - I/O BOARD REV. 1.0:

Programmable dripping times.

MDB menu for accepted below level bill setting.

SW CPU - I/O BOARD REV. 1.1:

Modification of database structure (upgrade needs the d.a. initialisation)

SW CPU - I/O BOARD REV. 1.1 UK:

Introduction of air-break failure management modification.

Updating of english display messages.

Modification of minimal value for hot and cold spouts movement times. (80).

Modification of  movements caused by SPOUTS utility button (spouts outgoing and incoming without stop)

SW CPU - I/O BOARD REV. 1.2:

Insertion of Finnish and Polski languages.

Modified Swedish and Danska languages.

Modified of statistical management by GSM.

D.A. programmation by use of UP-KEY.

Modified washing and spurt management in d.a. with cold water.

SW CPU - I/O BOARD REV. 1.3:

Eliminated jug check during Jug Facilities operation.

Management of free-vend operation with only cashless systems and MDB protocol.

SW CPU –I/O BOARD REV. 1.4:

Management of Astro – Sfera/Rondò set in configuration master-slave. (The Sw. Astro Rev. 1.4 can only work with Sfera/Rondò Rev. 1.3 or upper).

SW CPU –I/O BOARD REV. 1.41:

Corrected Jug Facicily management.

SW CPU - I/O BOARD REV. 1.5:

Inserted management of two payment systems in case of Sfera/Rondò set configuration.

Inserted management for setup and statistics read/write using UPKEY.

SW CPU - I/O BOARD REV. 1.51 IFE:

Corrected start-up management.

SW CPU –I/O BOARD REV. 1.52 IFE:

Modified CUP SETTLING TIME parameter range.

SW CPU / SCHEDA ATTUAZIONI REV. 1.53 IFE:

Modified the management of powder doser parameter.