LAYOUT L13.

PRODUCT TABLE.

| POSIT. | PRODUCT | N° DOSING | SETT. DOS. (gram./sec.) | N° ELECTRO. | SETT. EV (cc/sec.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Whitener / Milk | 1 | 1,5 | 1 | 9 |
| 2 | Sugar | 2 | 2,3 | 1 | 9 |
| 3 | Fresh Brew 1 | 3 | 1,7 | 2 | 16 |
| 4 | Fresh Brew 1 |  |  |  |  |
| 5 | Coffee Ins. / Soup * | 4 | 0,6 / 2,5 * | 4 | 10 / 15 * |
| 6 | Fresh Brew 2 | 6 | 1,7 | 5 | 14 |
| 7 | Fresh Brew 2 |  |  |  |  |
| 8 | Chocolate | 7 | 3,0 | 6 | 9 |
| / | Hot Water | / | / | 3 | 20 |
| / | Carbonated Water | / | / | 7 | 17 |
| / | Still Water | / | / | 8 | 16 |

KEYBOARD SELECTION L13.

| Strength - | ¤ | Strength + | ¤ |
| --- | --- | --- | --- |

| SELECTION | SUGAR N. | EX. SUGAR N. | NO SUGAR N. |
| --- | --- | --- | --- |
| ## Coffee Fresh Brew Black | 17 | 18 | 19 |
| Coffee Fresh Brew White | 21 | 22 | 23 |
| Coffee Fresh Brew Ex. White | 24 | 25 | 26 |
| Chocolate | -- | -- | 27 |
| Soup * | -- | -- | 28 * |
| Hot Water | -- | -- | 29 |
| Coffee Instant Black | 37 | 38 | 39 |
| Coffee Instant White | 41 | 42 | 43 |
| Coffee Instant Extra White | 44 | 45 | 46 |
| FB Tea White | 51 | 52 | 53 |
| FB Tea Extra White | 54 | 55 | 56 |
| Fresh Brew Tea Black | 57 | 58 | 59 |
| ### Coffee Fresh Brew Cappuccino | 61 | -- | 62 |
| Coffee Fresh Brew Espresso | 63 | -- | 64 |
| Coffee Fresh Brew EspressoChoc | 65 | -- | 66 |
| ChocoMilk | -- | -- | 67 |
| Instant Cappuccino | 71 | -- | 72 |
| Instant EspressoChoc | 73 | -- | 74 |
|  | STILL |  | CARBONATED |
| Chilled Water | 81 |  | 84 |
| Orange ( Syrop 1 ) | 82 |  | 85 |
| Lemon (Syrop 2 ) | 83 |  | 86 |

* When it takes the place of the Coffee Instant with the Soup, and vice versa, is necessary disable the selections of the powder to replace and train those of the new, therefore modify of the value of setting of the doseur and electrovalve. correspondent like suitable in the table product.

* Quando si sostituisce il Coffee Instant con il  Soup, e viceversa, e’ necessario  disabilitare le selezioni della polvere da sostituire ed abilitare quelle della nuova, quindi  modificare i valori di taratura del dosatore ed elettrov. corrispondente come indicato nella tabella prodotti.

PRESELECTION L13.

| PRESELECTION (N.) | DOSE VARIAT. | DESCRIPTION |
| --- | --- | --- |
| ## Strength - | -10 % | Reduces the powder of Coffee/Tea (3 step). |
| Strength + | +10 % | Increase the powder of Coffee/Tea (3 step). |
| Extra Sugar | + 66% | Increase the dose of sugar to 66% |
| No Sugar | - 100% | Reset the dose of sugar of the selection |

SELECTION L13.

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fresh Brew Black ( 17 – 18 – 19 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| Fresh Brew White ( 21 – 22 – 23 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 24 – 25 – 26 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Chocolate ( 27 ) | Chocolate 80 cc |  | Chocolate 19,0 gr. |  |  |  |
|  | Hot Water 60 cc (tot. 140 cc) |  |  |  |  |  |
| Soup * ( 28 ) | Soup 150 cc |  | Soup 8,0 gr. |  |  |  |
| Hot Water ( 29 ) | Hot Water 150 cc |  |  |  |  |  |
| Coffee Black ( 37 – 38 – 39 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |
| Coffee White ( 41 – 42 – 43 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coffee Ex. White ( 44 – 45 – 46 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 3,0 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Fresh Brew White ( 51 – 52 – 53 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 54 – 55 – 56 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Fresh Brew Black ( 57 – 58 – 59 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Cappuccino ( 61 – 62 ) | Coffee FB 80 cc |  | Coff. FB 6,5 gr. |  |  |  |
|  | Milk 55 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 30 cc (tot. 165 cc) |  | Chocolate 3,5 gr. |  |  |  |
| FB Espresso ( 63 – 64 ) | Coffee FB 50 cc |  | Coff. FB 3,3 gr. |  |  |  |
|  | Sugar 60 cc (tot. 110 cc) |  | Sugar 5,3 gr. |  |  |  |
| EspressoChoc ( 65 – 66 ) | Coffee FB 55 cc |  | Coff. FB 3,8 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 140 cc) |  | Chocolate 13,0 gr. |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Chocomilk ( 67 ) | Chocolate 70 cc |  | Chocolate 16,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 130 cc) |  | Milk 1,4 gr. |  |  |  |
| Inst. Cappuccino ( 71 – 72 ) | Coffee Instant 40 cc |  | Coffee 1,6 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 40 cc (tot. 140 cc) |  | Chocolate 9,6 gr. |  |  |  |
| Ins.EspressoChoc ( 73 – 74 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Chilled Water ( 81 ) | Still 160 cc |  |  |  |  |  |
| Chilled Water ( 84 ) | Carbonated 140 cc |  |  |  |  |  |
|  | Still 20 cc (tot. 160) |  |  |  |  |  |
| Orange ( 82 ) | Still 150 cc |  | Syrop 1 32 ds |  |  |  |
| Orange ( 85 ) | Still 35 cc |  | Syrop 1 0 ds |  |  |  |
|  | Carbonated 95 cc |  | Syrop 1 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |
| Lemon ( 83 ) | Still 150 cc |  | Syrop 2 32 ds |  |  |  |

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lemon ( 86 ) | Still 35 cc |  | Syrop 2 0 ds |  |  |  |
|  | Carbonated 95 cc |  | Syrop 2 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |

GENERAL INFORMATION.

PAYMENT SYSTEMS DETAILS:

| Line | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Weight | 1 | 2 | 10 | 20 | 50 | 5 |

SELECTION PRICE: All the selections to 10.00.

SELECTION NPRICE: All selections are set price line 1.

DECIMAL POINT POSITION: 2.

TEMPERATURE BOILER EXP: ---- °C.

TEMPERATURE BOILER INSTANT: 86.0 °C.

WATER/POWDER DELAYS: Powder delays to water 0.6 sec.

TYPE OF KEYBOARD: Numerical keyboard.

LANGUAGE: Italiano, Francaise, Espanol, English, Deutsch, Danska, Norge, Svenska, Ungaria, Nederlands e Czech

MODIFICHE.

SW CPU / SCHEDA ATTUAZIONI REV. 0.5:

Implementazione gestioni sportelli e taniche per autoalimentazione.

Aggiornamento dosi.

SW CPU / SCHEDA ATTUAZIONI REV. 0.7:

Pulsante lavaggi in programmazione come nella rev. 0.5.

Gestione funzionamento ventola sulla mensola del FB montata in posizione centrale.

SW CPU / SCHEDA ATTUAZIONI REV. 0.8 UK:

Aggiunta dei menù d’impostazione dell’ID per l’unità fredda (Tecn. 5.3) e del tempo di movimentazione ugelli per le bevande fredde e selezione acqua calda (Tecn. 5.4).

Modifiche per l’Operator friendly (pulsanti aggiunti, gestione lavaggi ed acqua calda).

Inserimento delle lingue danese, svedese e norvegese

SW CPU / SCHEDA ATTUAZIONI REV. 0.9:

Eliminati i layout L03-L04-L05.

Sistemazione alcune stringhe in programmazione.

Modifica ciclo di erogazione delle bevande carbonate con una coda di acqua fredda.

Aggiunta menù di abilitazione delle aggiunte di acqua calda (Tecn. 5.5).

Implementazione MDB, EVADTS, GSM e SETUP AUTOMATICO (Fill>6.x / Fill>7.x /

Tecn>5.6 / Tecn>6.x).

SW CPU / SCHEDA ATTUAZIONI REV. 1.0:

Tempi di sgocciolamento programmabili.

Menu mdb per impostazione banconote accettate sotto il livello.

SW CPU / SCHEDA ATTUAZIONI REV. 1.1:

Modifica alla struttura del database (l’upgrade necessita l’inizializzazione del d.a.).