LAYOUT L05.

TABELLA PRODOTTI.

| POSIZ. | PRODOTTO | N° DOSAT. | TARAT. DOS. (grammi/sec.) | N° ELETTR. | TAR. EV (cc/sec.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Whitener / Milk | 1 | 1,5 | 1 | 9 |
| 2 | Topping | 2 | 1,5 | 1 | 9 |
| 3 | Fresh Brew 1 | 3 | 1,7 | 2 | 16 |
| 4 | Fresh Brew 1 |  |  |  |  |
| 5 | Coffee Ins. / Soup * | 4 | 0,6 / 2,5 * | 4 | 10 / 15 * |
| 6 | Fresh Brew 2 | 6 | 1,7 | 5 | 14 |
| 7 | Fresh Brew 2 |  |  |  |  |
| 8 | Chocolate | 7 | 3,0 | 6 | 9 |
| 9 | Sugar | 9 | 2,3 | 1 | 9 |
| / | / | / | / | 3 | 20 |

DISPOSIZIONE TASTIERA L05.

| Strength - | ¤ | Strength + | ¤ |
| --- | --- | --- | --- |

| SELEZIONE | SUGAR N. | EX. SUGAR N. | NO SUGAR N. |
| --- | --- | --- | --- |
| ## Coffee Fresh Brew Black | 17 | 18 | 19 |
| Coffee Fresh Brew White | 21 | 22 | 23 |
| Coffee Fresh Brew Ex. White | 24 | 25 | 26 |
| Chocolate | -- | -- | 27 |
| Soup * | -- | -- | 28 * |
| Hot Water | -- | -- | 29 |
| Coffee Instant Black | 37 | 38 | 39 |
| Coffee Instant White | 41 | 42 | 43 |
| Coffee Instant Extra White | 44 | 45 | 46 |
| FB Tea White | 51 | 52 | 53 |
| FB Tea Extra White | 54 | 55 | 56 |
| Fresh Brew Tea Black | 57 | 58 | 59 |
| ### Coffee Fresh Brew Cappuccino | 61 | -- | 62 |
| Coffee Fresh Brew Espresso | 63 | -- | 64 |
| Coffee Fresh Brew EspressoChoc | 65 | -- | 66 |
| ChocoMilk | -- | -- | 67 |
| Instant Cappuccino | 71 | -- | 72 |
| Instant EspressoChoc | 73 | -- | 74 |
|  | STILL |  | CARBONATED |
| Chilled Water | 81 |  | 84 |
| Orange ( Syrop 1 ) | 82 |  | 85 |
| Lemon (Syrop 2 ) | 83 |  | 86 |

* Quando si sostituisce il Coffee Instant con il  Soup, e viceversa, e’ necessario  disabilitare le selezioni della polvere da sostituire ed abilitare quelle della nuova, quindi  modificare del valori di taratura del dosatore ed elettrov. corrispondente come indicato nella tabella prodotti.

PRESELEZIONI L05.

| PRESELEZIONE (N.) | VARIAZ. DOSE | DESCRIZIONE |
| --- | --- | --- |
| ## Strength - | -10 % | Riduce la polvere di Coffee/Tea (3 step). |
| Strength + | +10 % | Aumenta la polvere di Coffee/Tea (3 step). |
| Extra Sugar | + 66% | Aumenta la dose di zucchero del 66% |
| No Sugar | - 100% | Azzera la dose di zucchero della selezione |

SELEZIONI L05.

| SELEZIONE (N.) | DOSI ACQUA | ZUCCH. (a porta) | ### DOSI POLVERI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fresh Brew Black ( 17 – 18 – 19 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| Fresh Brew White ( 21 – 22 – 23 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 24 – 25 – 26 ) | Coffee FB 110 cc |  | Coff. FB 9,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Chocolate ( 27 ) | Chocolate 80 cc |  | Chocolate 19,0 gr. |  |  |  |
|  | Hot Water 60 cc (tot. 140 cc) |  |  |  |  |  |
| Soup * ( 28 ) | Soup 150 cc |  | Soup 8,0 gr. |  |  |  |
| Hot Water ( 29 ) | Hot Water 150 cc |  |  |  |  |  |
| Coffee Black ( 37 – 38 – 39 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |
| Coffee White ( 41 – 42 – 43 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |

| SELEZIONE (N.) | DOSI ACQUA | ZUCCH. (a porta) | ### DOSI POLVERI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coffee Ex. White ( 44 – 45 – 46 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 3,0 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Fresh Brew White ( 51 – 52 – 53 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Fr. Br. Ex.White ( 54 – 55 – 56 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 2,1 gr. | Sugar 5,3 gr. |  |  |
| Fresh Brew Black ( 57 – 58 – 59 ) | Tea FB 110 cc |  | Tea FB 3,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| FB Cappuccino ( 61 – 62 ) | Coffee FB 80 cc |  | Coff. FB 6,5 gr. |  |  |  |
|  | Topping 55 cc |  | Topping 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 30 cc (tot. 165 cc) |  | Chocolate 3,5 gr. |  |  |  |
| FB Espresso ( 63 – 64 ) | Coffee FB 50 cc |  | Coff. FB 3,3 gr. |  |  |  |
|  | Sugar 60 cc (tot. 110 cc) |  | Sugar 5,3 gr. |  |  |  |
| EspressoChoc ( 65 – 66 ) | Coffee FB 55 cc |  | Coff. FB 3,8 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 140 cc) |  | Chocolate 13,0 gr. |  |  |  |

| SELEZIONE (N.) | DOSI ACQUA | ZUCCH. (a porta) | ### DOSI POLVERI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Chocomilk ( 67 ) | Chocolate 70 cc |  | Chocolate 16,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 130 cc) |  | Milk 1,4 gr. |  |  |  |
| Inst. Cappuccino ( 71 – 72 ) | Coffee Instant 40 cc |  | Coffee 1,6 gr. |  |  |  |
|  | Topping 60 cc |  | Topping 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 40 cc (tot. 140 cc) |  | Chocolate 9,6 gr. |  |  |  |
| Ins.EspressoChoc ( 73 – 74 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Chilled Water ( 81 ) | Still 100 cc |  |  |  |  |  |
| Chilled Water ( 84 ) | Carbonated 100 cc |  |  |  |  |  |
| Orange ( 82 ) | Still 84 cc |  | Syrop 1 32 ds |  |  |  |
| Orange ( 85 ) | Carbonated 84 cc |  | Syrop 1 32 ds |  |  |  |
| Lemon ( 83 ) | Still 84 cc |  | Syrop 2 32 ds |  |  |  |
| Lemon ( 86 ) | Carbonated 84 cc |  | Syrop 2 32 ds |  |  |  |

DISPOSIZIONE INFORMAZIONI GENERALI.

TABELLA PESI VALIDATORE:

| Linea | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Peso | 1 | 2 | 10 | 20 | 50 | 5 |

PREZZO SELEZIONI: Tutte le selezioni a 10.00.

NPREZZO SELEZIONI: Tutte le selezioni puntano al prezzo 1.

POSIZIONE PUNTO DECIMALE: 2.

TEMPERATURA CALDAIA EXP: ---- °C.

TEMPERATURA CALDAIA SOLUBILI: 86.0 °C.

RITARDI ACQUE / POLVERI: Ritardo polvere rispetto all’acqua 0.6 sec.

MODIFICHE.

SW CPU / SCHEDA ATTUAZIONI REV. 0.5:

Implementazione gestioni sportelli e taniche per autoalimentazione.

Aggiornamento dosi.

SW CPU / SCHEDA ATTUAZIONI REV. 0.7:

Pulsante lavaggi in programmazione come nella rev. 0.5.

Gestione funzionamento ventola sulla mensola del FB montata in posizione centrale.