LAYOUT L04.

PRODUCT TABLE.

| POSIT. | PRODUCT | N° DOSING | SETT. DOS. (gram./sec.) | N° ELECTRO. | SETT. EV (cc/sec.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Whitener / Milk | 1 | 1,5 | 1 | 9 |
| 2 | Sugar | 2 | 2,3 | 1 | 9 |
| 3 | Fresh Brew 1 | 3 | 1,7 | 2 | 16 |
| 4 | Fresh Brew 1 |  |  |  |  |
| 5 | Coffee Instant | 4 | 0,6 | 4 | 10 |
| 6 | Coffee Instant Dec. | 6 | 0,6 | 4 | 10 |
| 7 | Chocolate | 7 | 3,0 | 5 | 9 |
| 8 | Soup | 8 | 2,5 | 6 | 15 |
| / | Hot Water | / | / | 3 | 20 |
| / | Carbonated Water | / | / | 7 | 17 |
| / | Still Water | / | / | 8 | 16 |

KEYBOARD SELECTION L04.

| Strength - |  | Strength + |  |
| --- | --- | --- | --- |
| ### Sugar |  | ### Extra Sugar |  |
| #### Coffee Black |  | ### Fresh Brew Black |  |
| Coffee White |  | Fresh Brew White |  |
| Coffee Black Dec. |  | Soup |  |
| Coffee White Dec. |  | Espresso |  |
| Cappuccino |  | ### Dec. Cappuccino |  |
| EspressoChoc |  | Dec. Espr. Choc |  |
| Chocomilk |  | Blank |  |
| Chocolate |  | Hot Water |  |

ALTERNATIVE SELECTIONS L01.

| Chilled Water |  | Carbonated Chilled Water |  |
| --- | --- | --- | --- |
| Orange |  | Carbonated Orange |  |
| Lemon |  | Carbonated Lemon |  |

PRESELECTION L04.

| PRESELECTION (N.) | DOSE VARIAT. | DESCRIPTION |
| --- | --- | --- |
| ## Strength - | -10 % | Reduces the powder of Coffee/Tea (3 step). |
| Strength + | +10 % | Increase the powder of Coffee/Tea (3 step). |
| Sugar (1) | + 100% | Insert the dose of sugar of the selection |
| Extra Sugar (10) | + 166% | Increase the dose of sugar to 166% |

All selections are without sugar by default.

Default Sugar: 0 gr.

Sugar: 5.3 gr.

Extra Sugar: 8.8 gr.

Tutte le selezioni di default sono senza zucchero.

SELECTION L04.

| SELECTION (N.) | WATER DOSES | SUGAR (to door) | ### POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coffee Black ( 2 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |
| Coffee White ( 3 ) | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Coffee Black Dec. ( 4 ) | Coffee Instant Dec. 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 45 cc |  | Sugar 5,3 gr. |  |  |  |
|  | Hot Water 55 cc (tot. 140 cc) |  |  |  |  |  |
| Coffee White Dec ( 5 ) | Coffee Instant Dec. 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Hot Water 50 cc (tot. 150 cc) |  |  |  |  |  |
| Cappuccino ( 6 ) | Coffee Instant 40 cc |  | Coffee 1,6 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 40 cc (tot. 140 cc) |  | Chocolate 9,6 gr. |  |  |  |
| EspressoChoc ( 7 ) | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee Instant 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Chocomilk ( 8 ) | Chocolate 70 cc |  | Chocolate 16,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 130 cc) |  | Milk 1,4 gr. |  |  |  |
| Chocolate ( 9 ) | Chocolate 80 cc |  | Chocolate 19,0 gr. |  |  |  |
|  | Hot Water 60 cc (tot. 140 cc) |  |  |  |  |  |
| Fresh Brew Black ( 11 ) | Tea 110 cc |  | Tea 3,0 gr. |  |  |  |
|  | Sugar 60 cc (tot. 170 cc) |  | Sugar 5,3 gr. |  |  |  |
| Fresh Brew White ( 12 ) | Tea 110 cc |  | Tea 3,0 gr. |  |  |  |
|  | Milk 60 cc (tot. 170 cc) |  | Milk 1,5 gr. | Sugar 5,3 gr. |  |  |
| Soup ( 13 ) | Soup 150 cc |  | Soup 8,0 gr. |  |  |  |
| Espresso ( 14 ) | Coffee Instant 35 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Sugar 30 cc (tot. 65 cc) |  | Sugar 5,3 gr. |  |  |  |
| Dec. Cappuccino ( 15 ) | Coffee Instant Dec. 40 cc |  | Coffee 1,6 gr. |  |  |  |
|  | Milk 60 cc |  | Milk 8,0 gr. | Sugar 5,3 gr. |  |  |
|  | Chocolate 40 cc (tot. 140 cc) |  | Chocolate 9,6 gr. |  |  |  |
| Dec. Espr. Choc ( 16 ) | Milk 35 cc |  | Milk 1,8 gr. | Sugar 5,3 gr. |  |  |
|  | Coffee Instant Dec. 40 cc |  | Coffee 1,4 gr. |  |  |  |
|  | Chocolate 50 cc (tot. 125 cc) |  | Chocolate 13 gr. |  |  |  |
| Hot Water ( 18 ) | Hot Water 150 cc |  |  |  |  |  |
| Chilled Water ( 21 ) | Still 160 cc |  |  |  |  |  |
| Carbonated Chill. Water | Carbonated 140 cc |  |  |  |  |  |
| ( 22 ) | Still 20 cc (tot. 160) |  |  |  |  |  |
| Orange ( 23 ) | Still 150 cc |  | Syrop 1 32 ds |  |  |  |
| Carbonated Orange | Still 35 cc |  | Syrop 1 0 ds |  |  |  |
| ( 24 ) | Carbonated 95 cc |  | Syrop 1 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |
| Lemon ( 25 ) | Still 150 cc |  | Syrop 2 32 ds |  |  |  |
| Carbonated Lemon | Still 35 cc |  | Syrop 2 0 ds |  |  |  |
| ( 26 ) | Carbonated 95 cc |  | Syrop 2 32 ds |  |  |  |
|  | Still 20 cc (tot. 150) |  |  |  |  |  |

INFORMAZIONI GENERALI.

TABELLA PESI VALIDATORE:

| Linea | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Peso | 1 | 2 | 10 | 20 | 50 | 5 |

PREZZO SELEZIONI: Tutte le selezioni a 10.00.

NPREZZO SELEZIONI: Tutte le selezioni puntano al prezzo 1.

POSIZIONE PUNTO DECIMALE: 2.

TEMPERATURA CALDAIA EXP: ---- °C.

TEMPERATURA CALDAIA SOLUBILI: 86.0 °C.

RITARDI ACQUE / POLVERI: Ritardo polvere rispetto all’acqua 0.6 sec.

TIPO DI TASTIERA: Tastiera Astro.

LINGUE: Italiano, Francaise, Espanol, English, Deutsch, Danska, Norge, Ungaria, Nederlands, Svenska, Czech, Polski and Finnish.

Per l’impostazione del sistema di pagamento si utilizza il menu 2.1.2.1. Nel caso in cui il distributore non abbia sistema di pagamento bisogna impostare OFF.

Gestione Astro-Sfera in batteria master-slave. (La Rev. Astro 0.5 SUK può lavorare solo con la Rev. Sfera 1.5 o superiori).

GENERAL INFORMATION.

PAYMENT SYSTEMS DETAILS:

| Line | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Weight | 1 | 2 | 10 | 20 | 50 | 5 |

SELECTION PRICE: All the selections to 10.00.

SELECTION NPRICE: All selections are set price line 1.

DECIMAL POINT POSITION: 2.

TEMPERATURE BOILER EXP: ---- °C.

TEMPERATURE BOILER INSTANT: 86.0 °C.

WATER/POWDER DELAYS: Powder delays to water 0.6 sec.

TYPE OF KEYBOARD: Astro keyboard.

LANGUAGE: Italiano, Francaise, Espanol, English, Deutsch, Danska, Norge, Ungaria, Nederlands, Svenska, Czech, Polski and Finnish.

For the payment system setting it is used the menu 2.1.2.1. In case of situation without payment system it is needed to set this parameter to OFF.

The Sw. Astro Rev. 0.5 SUK can only work with Sfera/Rondò Rev. 1.5 or upper).

MODIFICHE.

SW CPU / SCHEDA ATTUAZIONI REV. 0.2:

Aggiunto layout L03.

SW CPU / SCHEDA ATTUAZIONI REV. 0.3:

Modifica del parametro CUP SETTLING TIME.

SW CPU / SCHEDA ATTUAZIONI REV. 0.4:

Modificato valore contrasto LCD per gestione display BLU.

Inserito layout L04.

SW CPU / SCHEDA ATTUAZIONI REV. 0.5:

Inserite selezioni fredde come alternative.

CHANGES.

SW CPU - I/O BOARD REV. 0.2:

Inserted layout L03

SW CPU - I/O BOARD REV. 0.3:

Modified CUP SETTLING TIME parameter.

SW CPU - I/O BOARD REV. 0.4 UK:

Changed LCD contrast value for blue display management.

Inserted layout L04.

SW CPU - I/O BOARD REV. 0.5 UK:

Inserted cold selections as alternative ones.