NAZIONE: UK.

COUNTRY: UK.

## TABLE DOSES VERSION 2FB SETUP 1

## DISPOSIZIONE SELEZIONI VERSIONE 2FB SETUP 1

| Light |  | Strong |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SUGAR |  | EXTRA SUGAR |  | Hot water |  |
| Freshly brewed white coffee |  | Freshly brewed white tea |  | Lemon tea |  |
| # Freshly brewed black coffee |  | Freshly brewed black tea |  | Still orange |  |
| # Espresso 3 |  | Cappuccino with chocolate |  | Carbonated orange |  |
| Instant white coffee 2 |  | Cappuccino without choc. |  | Still lemon |  |
| # Instant black coffee 2 |  | Espressochoc 3 |  | Carbonated lemon |  |
| Instant white coffee 3 |  | Chocolate |  | Still cold water |  |
| # Instant black coffee 3 |  | Chocomilk |  | Carbonated cold water |  |

PRESELEZIONI / PRESELECTIONS

| PRESELEZIONE | VAR. DOSE | DESCRIZIONE |
| --- | --- | --- |
| Zucchero | / | Eroga lo zucchero nelle selezioni dove è presente. |
|  |  | Dispense sugar powder if present. |
| Extra zucchero | +66% | Aumenta la dose di zucchero (1 step). |
|  |  | Increases the sugar powder dose (1 step). |
| Strong | +10% | Aumenta la polvere del caffè solub, the e fb (3 step). |
|  |  | Increases the powder dose of coffee sol, the and fb (3 step). |
| Light | -20% | Riduce la polvere del caffè solub, the e fb (3 step). |
|  |  | Reduces the powder dose of coffee sol, the and fb (3 step). |

Temperatura caldaia solubili: 86.0 °C.

Instant boiler temperature: 86.0 °C.

## MINI-DIP CONFIGURATION VERSION 2FB SETUP 1

CONFIGURAZIONE MINI-DIP VERSIONE 2FB SETUP 1

| Minidip 5 | Minidip 4 | Minidip 3 | Minidip 2 | Minidip 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OFF | ON | ### ON | #### OFF | OFF |  |  |  |

## TABELLA DOSI / TABLE DOSES

| SELEZIONE SELECTION | DOSI ACQUA WATER DOSES | DOSI POLVERI POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Freshly brewed white coffee | milk 40 cc | milk 3,2 gr. | sugar 7,5 gr. |  |  |
|  | coffee 110 cc (tot. 150 cc) | coffee 9,5 gr. |  |  |  |
| Freshly brewed black coffee | coffee 110 cc | coffee 9,5 gr. |  |  |  |
|  | sugar 40 cc (tot. 150 cc) | sugar 7,5 gr. |  |  |  |
| Espresso 3 | coffee 55 cc | coffee 1,2 gr. |  |  |  |
|  | sugar 60 cc (tot. 115 cc) | sugar 7,5 gr. |  |  |  |
| Instant white coffee 2 | milk 60 cc | milk 1,8 gr. | sugar 7,5 gr. |  |  |
|  | coffee 40 cc | coffee 1,2 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Instant coffee black 2 | coffee 40 cc | coffee 1,2 gr. |  |  |  |
|  | sugar 60 cc | Sugar 7,5 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Instant white coffee 3 | Milk 60 cc | Milk 1,8 gr. | sugar 7,5 gr. |  |  |
|  | coffee 40 cc | Coffee 1,2 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |

| SELEZIONE SELECTION | DOSI ACQUA WATER DOSES | DOSI POLVERI POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Instant coffee black 3 | coffee 40 cc | coffee 1,2 gr. |  |  |  |
|  | sugar 60 cc | sugar 7,5 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Freshly brewed white tea | Milk 60 cc | milk 1,5 gr. | sugar 7,5 gr. |  |  |
|  | Tea 110 cc (tot. 170 cc) | tea 3,0 gr. |  |  |  |
| Freshly brewed black tea | Tea 110 cc | tea 3,0 gr. |  |  |  |
|  | sugar 60 cc (tot. 170 cc) | sugar 7,5 gr. |  |  |  |
| Cappuccino with chocolate | milk 50 cc | milk 2,1 gr. | sugar 7,5 gr. |  |  |
|  | coffee 65 cc | coffee 1,2 gr. |  |  |  |
|  | chocolate 30 cc (tot. 145 cc) | chocolate 3,5 gr. |  |  |  |
| Cappuccino without chocolate | milk 65 cc | milk 2,1 gr. | sugar 7,5 gr. |  |  |
|  | coffee 80 cc (tot. 145 cc) | coffee 1,2 gr. |  |  |  |
| Espressochoc 3 | milk 35 cc | milk 1,8 gr. | sugar 7,5 gr. |  |  |
|  | coffee 40 cc | coffee 1,2 gr. |  |  |  |
|  | chocolate 50 cc (tot. 125 cc) | chocolate 14,5 gr. |  |  |  |
| Chocolate | chocolate 100 cc | chocolate 19,0 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |

| SELEZIONE SELECTION | DOSI ACQUA WATER DOSES | DOSI POLVERI POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Chocomilk | chocolate 80 cc | chocolate 16,0 gr. |  |  |  |
|  | milk 60 cc (tot. 140 cc) | milk 1,4 gr. | sugar 7,5 gr. |  |  |
| Hot water | water 150 cc (tot. 150 cc) |  |  |  |  |
| Lemon tea | tea 100 cc (tot. 100 cc) | tea 9,0 gr. |  |  |  |
| Still orange | water 84 d.s. |  |  |  |  |
|  | orange 32 d.s. (tot. 150 cc) |  |  |  |  |
| Carbonat. orange | water 84 d.s. |  |  |  |  |
|  | orange 32 d.s. (tot. 150 cc) |  |  |  |  |
| Still lemon | water 84 d.s. |  |  |  |  |
|  | lemon 32 d.s. (tot. 150 cc) |  |  |  |  |
| Carbonat. lemon | water 84 d.s. |  |  |  |  |
|  | lemon 32 d.s. (tot. 150 cc) |  |  |  |  |
| Still cold water | water 100 d.s. (tot. 180 cc) |  |  |  |  |
| Carb. cold water | water 100 d.s. (tot. 180 cc) |  |  |  |  |

LEGENDA/LEGEND:                                             TARATURA DOSATORI/DOSEUR SETTING:

| Posizione 1 | Chocolate |  | Dosatore 1 | 3,0 gr/s |
| --- | --- | --- | --- | --- |
| Posizione 2 | Coffee 1 |  | Dosatore 2 | 0,6 gr/s |
| Posizione 3-4 | Grup. coffee Fresh Brew |  | Dosatore 3 | 1,7 gr/s |
| Posizione 5 | Coffee 2 |  | Dosatore 4 | 0,6 gr/s |
| Posizione 6-7 | Grup. tea Fresh Brew |  | Dosatore 6 | 1,7 gr/s |
| Posizione 8 | Lemon tea |  | Dosatore 7 | 2,5 gr/s |
| Posizione 9 | Whitener for tea |  | Dosatore 9 | 1,5 gr/ s |
| Posizione 10 | Creamer for coffee |  | Dosatore 10 | 1,5 gr/ |
| Posizione 11 | Sugar |  | Dosatore 11 | 2,3 gr/s |
| Posizione 12 | Non utilizzata |  | Dosatore 12 |  |
| Posizione 13 | Non utilizzata |  | Dosatore 13 |  |

TARATURA ELETTROV / SOLENOID SETTING:

| Elettrov. 1 | 9 cc/s |
| --- | --- |
| Elettrov. 2 | 9 cc/s |
| Elettrov. 3 | 14 cc/s |
| Elettrov. 4 | 9 cc/s |
| Elettrov. 5 | 14 cc/s |
| Elettrov. 6 | 12 cc/s |
| Elettrov. 7 | 15 cc/s |
| Elettrov. 8 | 9 cc/s |

## LAYOUT VERSION 2FB SETUP 1

LAYOUT MACCHINA VERSIONE 2FB SETUP 1

1               2             3        4                 5            6        7                 8               9              10         11

12

13

TABLE DOSES VERSION 2FB-02 SETUP 1

DISPOSIZIONE SELEZIONI VERSIONE 2FB-02 SETUP 1

| Light |  | Strong |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SUGAR |  | EXTRA SUGAR |  | Hot water |  |
| Freshly brewed white coffee |  | Freshly brewed white tea |  | Lemon tea |  |
| # Freshly brewed black coffee |  | Freshly brewed black tea |  | Cappuccino 2 |  |
| # Espresso 3 |  | Cappuccino with chocolate |  |  |  |
| Instant white coffee 2 |  | Cappuccino without choc. |  | Cappuccino 2 with chocol. |  |
| # Instant black coffee 2 |  | Espressochoc 3 |  |  |  |
| Instant white coffee 3 |  | Chocolate |  | Espressochoc 2 |  |
| # Instant black coffee 3 |  | Chocomilk |  |  |  |

MINI-DIP CONFIGURATION VERSION 2FB-02 SETUP 1

CONFIGURAZIONE MINI-DIP VERSIONE 2FB-02 SETUP 1

| Minidip 5 | Minidip 4 | Minidip 3 | Minidip 2 | Minidip 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OFF | ON | ### ON | #### OFF | ### ON |  |  |  |

## TABLE DOSES VERSION 2FB SETUP 2

## DISPOSIZIONE SELEZIONI VERSIONE 2FB SETUP 2

| Light |  | Strong |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SUGAR |  | EXTRA SUGAR |  | Hot water |  |
| Freshly brewed white coffee |  | Freshly brewed white tea |  | Lemon tea |  |
| # Freshly brewed black coffee |  | Freshly brewed black tea |  | Still orange |  |
| # Espresso 3 |  | Cappuccino with chocolate |  | Carbonated orange |  |
| Instant white coffee 2 |  | Cappuccino without choc. |  | Still lemon |  |
| # Instant black coffee 2 |  | Espressochoc 3 |  | Carbonated lemon |  |
| Instant white coffee 3 |  | Chocolate |  | # Still cold water |  |
| # Instant black coffee 3 |  | Chocomilk |  | Carbonated cold water |  |

## MINI-DIP CONFIGURATION VERSION 2FB SETUP 2

CONFIGURAZIONE MINI-DIP VERSIONE 2FB SETUP 2

| Minidip 5 | Minidip 4 | Minidip 3 | Minidip 2 | Minidip 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ### ON | ON | ### ON | #### OFF | OFF |  |  |  |

TABELLA DOSI / TABLE DOSES

| SELEZIONE SELECTION | DOSI ACQUA WATER DOSES | DOSI POLVERI POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Freshly brewed white coffee | milk 40 cc | milk 3,0 gr. | sugar 6,0 gr. |  |  |
|  | coffee 110 cc (tot. 150 cc) | coffee 5,0 gr. |  |  |  |
| Freshly brewed black coffee | coffee 110 cc | coffee 5,0 gr. |  |  |  |
|  | sugar 40 cc (tot. 150 cc) | sugar 6,0 gr. |  |  |  |
| Espresso 3 | coffee 55 cc | coffee 5,0 gr. |  |  |  |
|  | sugar 60 cc (tot. 115 cc) | sugar 6,0 gr. |  |  |  |
| Instant white coffee 2 | milk 60 cc | milk 3,0 gr. | sugar 6,0 gr. |  |  |
|  | coffee 40 cc | coffee 1,3 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Instant coffee black 2 | coffee 40 cc | coffee 1,3 gr. |  |  |  |
|  | sugar 60 cc | sugar 6,0 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Instant white coffee 3 | milk 60 cc | milk 3,0 gr. | sugar 6,0 gr. |  |  |
|  | coffee 40 cc | coffee 1,3 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Instant coffee black 3 | coffee 40 cc | coffee 1,3 gr. |  |  |  |
|  | sugar 60 cc | sugar 6,0 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| SELEZIONE SELECTION | DOSI ACQUA WATER DOSES | DOSI POLVERI POWDER DOSES |  |  |  |
| Freshly brewed white tea | milk 60 cc | milk 2,0 gr. | sugar 6,0 gr. |  |  |
|  | tea 110 cc (tot. 170 cc) | tea 1,2 gr. |  |  |  |
| Freshly brewed black tea | tea 110 cc | tea 1,2 gr. |  |  |  |
|  | sugar 60 cc (tot. 170 cc) | sugar 6,0 gr. |  |  |  |
| Cappuccino with chocolate | milk 50 cc | topping 6,0 gr. | sugar 6,0 gr. |  |  |
|  | coffee 65 cc | coffee 1,3 gr. |  |  |  |
|  | chocolate 30 cc (tot. 145 cc) | chocolate 6,0 gr. |  |  |  |
| Cappuccino without chocolate | milk 65 cc | topping 6,0 gr. | sugar 6,0 gr. |  |  |
|  | coffee 80 cc (tot. 145 cc) | coffee 1,3 gr. |  |  |  |
| Espressochoc 3 | milk 35 cc | milk 1,8 gr. | sugar 6,0 gr. |  |  |
|  | coffee 40 cc | coffee 1,3 gr. |  |  |  |
|  | chocolate 50 cc (tot. 125 cc) | chocolate 14,5 gr. |  |  |  |
| Chocolate | chocolate 100 cc | chocolate 20,0 gr. |  |  |  |
|  | hot water 50 cc (tot. 150 cc) |  |  |  |  |
| Chocomilk | chocolate 80 cc | chocolate 16,0 gr. |  |  |  |
|  | milk 60 cc (tot. 140 cc) | milk 1,4 gr. | sugar 6,0 gr. |  |  |
| Hot water | water 150 cc (tot. 150 cc) |  |  |  |  |
| Lemon tea | tea 100 cc (tot. 100 cc) | tea 8,0 gr. |  |  |  |

| SELEZIONE SELECTION | DOSI ACQUA WATER DOSES | DOSI POLVERI POWDER DOSES |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Still orange | water 84 d.s. |  |  |  |  |
|  | orange 32 d.s. (tot. 150 cc) |  |  |  |  |
| Carbonat. orange | water 84 d.s. |  |  |  |  |
|  | orange 32 d.s. (tot. 150 cc) |  |  |  |  |
| Still lemon | water 84 d.s. |  |  |  |  |
|  | lemon 32 d.s. (tot. 150 cc) |  |  |  |  |
| Carbonat. lemon | water 84 d.s. |  |  |  |  |
|  | lemon 32 d.s. (tot. 150 cc) |  |  |  |  |
| Still cold water | water 100 d.s. (tot. 180 cc) |  |  |  |  |
| Carb. cold water | water 100 d.s. (tot. 180 cc) |  |  |  |  |

LEGENDA/LEGEND:                                             TARATURA DOSATORI/DOSEUR SETTING:

| Posizione 1 | Chocolate |  | Dosatore 1 | 3,5 gr/s |
| --- | --- | --- | --- | --- |
| Posizione 2 | Coffee 1 |  | Dosatore 2 | 0,6 gr/s |
| Posizione 3-4 | Grup. coffee Fresh Brew |  | Dosatore 3 | 1,4 gr/s |
| Posizione 5 | Coffee 2 |  | Dosatore 4 | 0,6 gr/s |
| Posizione 6-7 | Grup. tea Fresh Brew |  | Dosatore 6 | 1,7 gr/s |
| Posizione 8 | Lemon tea |  | Dosatore 7 | 3,0 gr/s |
| Posizione 9 | Whitener for tea |  | Dosatore 9 | 5,0 gr/ s |
| Posizione 10 | Creamer for coffee |  | Dosatore 10 | 3,7 gr/ |
| Posizione 11 | Sugar |  | Dosatore 11 | 2,3 gr/s |
| Posizione 12 | Non utilizzata |  | Dosatore 12 |  |
| Posizione 13 | Non utilizzata |  | Dosatore 13 |  |

TARATURA ELETTROV / SOLENOID SETTING:

| Elettrov. 1 | 9 cc/s |
| --- | --- |
| Elettrov. 2 | 9 cc/s |
| Elettrov. 3 | 14 cc/s |
| Elettrov. 4 | 9 cc/s |
| Elettrov. 5 | 14 cc/s |
| Elettrov. 6 | 12 cc/s |
| Elettrov. 7 | 15 cc/s |
| Elettrov. 8 | 9 cc/s |

## LAYOUT VERSION 2FB BUNZL SETUP 2

LAYOUT MACCHINA VERSIONE 2FB BUNZL SETUP 2

1               2             3        4                 5            6        7                 8               9              10         11

12

13

TABLE DOSES VERSION 2FB-02 SETUP 2

DISPOSIZIONE SELEZIONI VERSIONE 2FB-02 SETUP 2

| Light |  | Strong |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SUGAR |  | EXTRA SUGAR |  | Hot water |  |
| Freshly brewed white coffee |  | Freshly brewed white tea |  | Lemon tea |  |
| # Freshly brewed black coffee |  | Freshly brewed black tea |  | Cappuccino 2 |  |
| # Espresso 3 |  | Cappuccino with chocolate |  |  |  |
| Instant white coffee 2 |  | Cappuccino without choc. |  | Cappuccino 2 with chocol. |  |
| # Instant black coffee 2 |  | Espressochoc 3 |  |  |  |
| Instant white coffee 3 |  | Chocolate |  | Espressochoc 2 |  |
| # Instant black coffee 3 |  | Chocomilk |  |  |  |

MINI-DIP CONFIGURATION VERSION 2FB-02 SETUP 2 CONFIGURAZIONE MINI-DIP VERSIONE 2FB-02 SETUP 2

| Minidip 5 | Minidip 4 | Minidip 3 | Minidip 2 | Minidip 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ### ON | ON | ### ON | #### OFF | ### ON |  |  |  |